| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Hierarchical Network Study Report with change mark |  |
| Date Submitted | 2012-01-18 |  |
| Source(s) | Project Planning Committee Editor: Rakesh Taori | E-mail: rakesh.taori@samsung.com |
| Re: | This document is the Study report on Hierarchical Network |  |
| Abstract | Study report on Hierarchical Network |  |
| Purpose | To be adopted by the PPC |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Study Report on Hierarchical Networks

1	Introduction	4

2	Usage Models	6

2.1	Single Radio Access Technology	6

2.1.1	Multi-tier Networks	6

2.1.1.1	Multi-Tier Deployment Scenarios	7

2.1.1.2	Spectrum Usage across Tiers	9

2.1.2	Client Tier	11

2.1.2.1	In-band Client cooperation (In-band CC)	11

2.1.3	BS Energy Saving	12

2.2	Multiple Radio Access Technology	12

2.2.1	Unlicensed Spectrum Access	14

2.2.2	Efficient Multi-Radio Operation for Client Devices	14

2.2.3	Interworking Schemes	14

2.2.3.1	BS-AP Interworking	14

2.2.3.2	Interworking via Higher-layer	15

3	Hierarchical Network System Architecture	15

3.1	Protocol and Functional Structure	15

3.1.1	Single-RAT Protocol and Functional Structure	16

Multi-RAT Protocol Structure	17

3.2	Multi-RAT Network Architecture	19

3.2.1.1	Network Discovery and Selection Server	19

4	Key Features and Requirements	19

4.1	Single Radio Access Technology	19

4.1.1	Advanced Interference Management in Multi-tier Networks	20

4.1.1.1	Advanced Interference Management for Single Carrier Deployment	20

4.1.1.2	Advanced Coordinated Multi-cell Technologies for Co-channel Interference Management	20

4.1.1.3	Advanced Interference Management for Multiple Carrier Deployment	21

4.1.1.4	Power Setting or Power Control of Femtocell	21

4.1.2	Advanced Mobility Management	21

4.1.3	Special Energy Saving Mode of Base Station	21

4.1.4	Advanced Self-Organized Networking (SON)	21

4.1.5	Client Cooperation (CC) Support	22

4.2	Multi Radio Access Technology	22

5	Standards Implications	23

5.1	Single Radio Access Technology	23

5.1.1	Advanced Interference Management in Multi-tier Networks	23

5.1.1.1	Advanced Interference Management for Single Carrier Deployment	23

5.1.1.2	Advanced Coordinated Multi-cell Technologies for Co-channel Interference Management	23

5.1.1.3	Advanced Interference Management for Multiple Carrier Deployment	23

5.1.1.4	Power Setting or Power Control of Femtocell	23

5.1.2	Advanced Mobility Management	23

5.1.3	Special Energy Saving Mode of Base Station	24

5.1.4	Advanced Self-Organized Networking (SON)	24

5.1.5	Client Cooperation (CC) Support	24

5.2	Multi Radio Access Technology	24

5.2.1	General	24

5.2.2	Virtual Carrier	24

5.2.2.1	Multi-RAT Network Discovery/Access Management	24

5.2.2.2	Flow Mobility management	25

5.2.3	Multi-RAT Client Cooperation	25

5.2.4	Support of BS-AP Interworking	25

6	Conclusions & Recommendations	25

7	List of Acronyms and Definitions	25

8	References	26

9	Appendix A: An example of Smart Resource Allocation in Multi-tier Networks	27

10	Appendix B: Multitier Evaluation Methodology	27

10.1	Hierarchical Network Simulation Methodology	27

10.2	Multi-tier Femtocell Overlay Networks	27

10.2.1	General Simulation Settings	28

10.2.2	FAP and Subscribers Deployment Model	29

10.2.3	Channel Model	31

10.2.4	Interference Modeling	32

10.2.5	Performance Metrics	32

11	Appendix C: Multi Radio Access Deployments employing cognitive radio techniques	33

11.1	Disaster Relief	33

11.2	Key Features and Requirements	34

[Editor: This study report is based on the output of the PPC adhoc on hierarchical network, documented in C802.16ppc-11/0007r5]

# Introduction

Several recent studies have pointed to the explosive growth in mobile data demand driven by compelling devices such as the iPhone and netbooks. For example, studies by Cisco suggest 66x growth in mobile internet traffic, from 2008-2013, corresponding to a CAGR of 131% [1]. Therefore, a critical challenge for future broadband networks is to provide significantly enhanced capacity to meet this exponential growth in demand.

While capacity demands on future networks are increasing, network operators are facing flattening revenues as their revenue mix moves from being voice-centric with “minutes of use” billing to “flat-rate” data centric plans. Therefore, it is imperative that the network operators find cost-effective ways to add capacity, while continuing to add network services that can enhance their revenues.  This situation is well-described in [1], which points out that future networks must drastically reduce cost/bit, while adding new services.  Hierarchical networks, which encompass multi-tier, multi-radio network architecture, represent a disruptive approach towards low cost/bit capacity enhancements, which efficiently utilize all spectral resources in the system. In addition other metrics such as client quality of service, coverage etc. are also enhanced.

Figure 1 below illustrates the hierarchical network architecture for future 802.16 networks as described on [1].

Figure 1: Overview of Hierarchical and Multi-Radio Architecture

The network architecture shown in the figure represents an evolution and integration of existing network elements in a multi-tier or hierarchical deployment. In the multi-tier hierarchy shown, large cells provide ubiquitous coverage to clients as well as support mobility. The smaller network elements such as relays, pico, and femto access points (APs) take connectivity closer to the clients thereby increasing the available capacity in the system. Key elements of the “multi-tier” network are shown in “green”. The lower cost structure associated with the smaller APs makes this an attractive method of adding lower cost/bit capacity. Small base stations (BSs) including micro, pico, and femto APs can cooperate to reduce interference and increase the capacity, where macro BS and small BSs may be connected in a wireless manner and the cooperative small BSs form a cluster to support the clients. The channel information and user context can be shared among BSs through the wireless link between macro BS and small BS. Further, clients (mobile stations) can also be utilized as another tier in the network hierarchy without incurring additional infrastructure deployment cost. Here intelligent client cooperation can improve capacity as well as connectivity for the client.

The figure also shows multiple radio access technologies (RAT) being integrated and managed as part of a single hierarchical network (multi-radio network elements are indicated in “blue”). The additional spectrum and connectivity available across these different networks may be exploited synergistically to further improve system capacity and client quality of service.

The examples include:

- Offloading “best-effort” traffic to IEEE 802.11 hotspots.

- Offloading traffic from other licensed band such as 3GPP LTE/EUTRA, and vice versa.

The cost associated with this additional capacity may be significantly lower when offloading to unlicensed spectrum. Also new network devices, such as the integrated IEEE 802.11/16 femto AP shown, can implement tighter coupling across these two radio technologies and efficiently utilize the spectrum available across both licensed and unlicensed bands.

The network is also expected to be “self organizing” (SON) providing for low-cost deployment,, configuration and management of network infrastructure.

Also note that client devices form an important part of the multi-tier network hierarchy and can take on new roles as network elements. In this study report we explore several uses cases that allow clients to serve as access points and relays and have the ability to cooperate with each other to improve network capacity and link quality of service.

As the operators (cost perspective) and the regulatory authorities (environmental perspective) increasingly demand energy saving mechanisms, future wireless networks will incorporate energy saving mechanisms. Mechanisms for energy saving in the radio access network, for both single-RAT and multi-RAT scenarios include, but are not limited to,

- BS energy saving mechanisms

  - low duty mode (Section 4.1.3)

  - switching on/off of BS (e.g. during off peak hours BS can be switched off) (Section 4.3)

- Network configurations

  - multi-hop operation

  - coordinated multi-cell operation (Section 4.1.1.2)

  - mechanisms for joint radio resource management for multi-radio devices or network (Section 5.2.2.1)

To adapt to the changing network conditions, adequate procedures need to be defined for supporting the foregoing mechanisms. To ensure interoperability and integrate energy saving mechanisms in the radio access network, standardized procedures and functionalities to support the abovementioned mechanisms should be defined.

To summarize, this study report focuses on “low cost” network architectures and enabling techniques to maximize system capacity and user quality of service.

Strict requirements will be set for this objective and should be followed by features investigated to satisfy them. As mentioned above, the objectives can best be approached by “hierarchical network” architectures, comprising low-cost infrastructure and clients acting as network elements, where spectrum (especially unlicensed spectrum) across multiple radio access technologies may also be utilized for low-cost capacity enhancement. Hence the study report will evaluate both Single-RAT multi-tier and Multi-RAT based hierarchical networks. Specifically, the following techniques, enabling such architectures, will be investigated.

- Enhanced coordination techniques between devices, between cells in the same tier and multi tiers by allow multiple signaling to be communicated per channel (i.e., multi-cell coordination techniques along with multi-user precoding/decoding techniques among tiers)

- Enhanced interference mitigation techniques by allowing a single transmission per channel among tiers (i.e., resource allocation among tiers in views of time/frequency/code-domain) for Single-RAT and/or Multi-RAT

- Enhanced spectrum utilization between Multi-RAT systems

- Seamless mobility of connection flows among Multi-RATs (e.g. selective data offloading, handover)

- Enhanced interworking with other RAT (e.g. IEEE802.11) involving access points (i.e. IEEE802.11) and IEEE802.16 BS.

- Collaboration between Multi-RAT devices (i.e. restricted to terminal, or client, only)

- Enhanced Energy Saving mechanisms and optimization.

To address the aforementioned features, the following sections below consist of key usage scenarios, network architecture, requirements and IEEE 802.16 standards implications for hierarchical network topologies, based on single or multiple radio access technologies.

# Usage Models

## Single Radio Access Technology

### Multi-tier Networks

Multi-tier networks refer to a hierarchical or overlay deployments of cells which may have increasingly base stations of smaller sizes: macro base station, micro base station, pico base station, femto base station, and relay base station (Figure 2). The hierarchy shown in Figure 2 is not strict and is mainly illustrative of the increasingly smaller cell sizes that may be included as part of a multi-tier deployment.  For example, in a given deployment, a 2 tier hierarchical relationship may exist between a Macro and both pico and/or a femto cells, with the pico and femto cells comprising the same tier in the hierarchy. Typical deployment would consist of the tiers operating on the same radio access technology (RAT). Femto and relay are included as a part of the IEEE 802.16m specification [3]. Coordination techniques between devices and between cells across the multiple tiers and among cooperating small BSs are important aspects of multi-tier network design to achieve cell capacity enhancements and interference mitigation techniques across tiers are also critical to achieving the user throughput enhancements. However, it is not fully enabled in IEEE 802.16m because multiple types of base stations (including Macro, Pico, Femto, Relay) will be deployed in multi-tier networks and this will cause more complicated interference among tiers. Therefore enhanced interference mitigation and coordination techniques are needed with the enhanced channel measurement, downlink signaling, and uplink feedback schemes, etc. In particular, multi-tier deployments affect the areal capacity (bps/Hz/square meters) of the system due to the deployment of significantly more cells in a given area. The highest tier macro cells are still needed to provide broader coverage and seamless mobility.

Figure 2: An example of Single RAT Hierarchical (Multi-tier) Architecture Framework

#### Multi-Tier Deployment Scenarios

As indicated, small base stations may be used in a hierarchical network deployment. The type and location of these base stations will play a significant role in determining the cost and performance of multi-tier deployments. For example, indoor femto cell deployments can utilize the existing back-haul thereby significantly lowering the cost of such deployments. With outdoor pico-cellular deployments, the operator will need to provide back-haul capability and manage more critical spectrum reuse challenges. Especially, the wireless back-haul can be used to reduce CAPEX/OPEX between macro BS and small BSs, and to ease the deployment of small BSs. As mentioned in section 1 and shown in Figure 2, a group of small BSs cooperates to increase the system coverage and capacity. To efficiently form and manage a cluster of cooperative small BSs, the interference management among cooperative small BSs is of high importance. Other deployment models cover indoor enterprise or outdoor campus deployments that may impose different manageability and reliability requirements. Multi-tier deployments across this range of scenarios are not fully addressed by the IEEE 802.16m standard.

##### Self & Operator Managed Deployments

Smaller base-stations like femtos are typically user-deployed and managed. With increasing density of cells, self organization of network will be critical to reduce operational expenses as well as improve the response time to fix network problems. However, the operators are looking for a network solution that finds the optimum middle ground between low-cost, consumer-managed and deployed private femto-cells versus operator owned and managed public pico base stations. Hence, low-cost but improved manageability of hierarchical deployments will be a key consideration.

The cooperative cluster of small BSs can help to improve manageability and reduce operational expenses of hierarchical deployments. For example, the cluster can be formed without any control of operator according to the condition of the network (e.g., nearby small BSs and user devices). Also, it can detect a broken small BS, which cannot serve user devices anymore, by communicating with each other and reconfigure the cluster by itself to provide seamless service to users served by the broken small BS, resulting in improved manageability with low cost.

Figure 3 Cluster reconfiguration with the broken small base-station

##### Access Rules in Multi-Tier Deployments

As mentioned, hierarchical networks may be deployed by using a variety of lower tier network elements in different locations. The deployment scenario will determine whether access to the lower tier network is available to all users in the network. For instance user-deployed, in-home femto base stations may only allow access to users who are part of the household. Such access rules are captured for femto-cellular deployments, as part of 802.16m. The terms Closed Subscriber Groups (CSG) and Open Subscriber Groups (OSG) are used to refer to private and public femto base stations respectively. However, the performance aspects of the different access rules are not well-evaluated.

##### Deployments of cooperative small BSs

We can achieve higher system capacity by increasing the number of small BSs. However, it may cause serious interference problem if small BSs do not cooperate with other small BSs. One solution of the interference problem can be a cooperative cluster of small BSs. To form a cluster of small BSs, the system may keep candidate group lists based on physical information of each small BS (e.g. location), which is called a physical cluster. Each user may be served in a subset of a physical cluster, which is called a logical cluster. In order to determine logical clusters, several factors such as user’s SINR(signal-to-interference and noise ratio), user mobility, their requested service rates, or nearby other BSs’ interference, should be considered. Also, a cluster of cooperative small BSs may be dynamically reconfigured by adding or subtracting small BSs.

Figure 4 Deployments of cooperative physical and logical clusters of small base-stations

#### Spectrum Usage across Tiers

As mentioned, spectrum allocation across multiple tiers is an important aspect of deployment and use of hierarchical architectures.  Currently, multi-tier deployments are possible for both

According to the spectrum used, multi-tier cell deployments are possible for the following cases

- Single carrier case:

The multi-tier cells are deployed on a single carrier. This can also be called co-channel deployment.

- Multiple carriers case:

The multi-tier cells are deployed on multiple carriers. When multiple carriers are available, choices can be made to enable flexible cell deployment. For example, the macrocell and small cell can be deployed on distinct carriers, or on the same set of carriers while having joint carrier and power assignment/selection for macrocell and small cell in order to better manage inter-cell interference.

IEEE 802.16m supports both types of deployments, but more efficient spectrum usage across tiers using time/frequency/spatial domain is required.

##### Single Carrier Case

Due to the overlaying architecture, when macrocell and the overlaid small cells, such as pico and femto, are deployed on the same channel, interference management across tiers becomes an important design aspect that must be addressed,

There are many scenarios that lead to interferences. One particular scenario is of small cells (such as femto supporting CSG) which are overlaid by a macrocell. In this case the device connected to a macrocell may fall under a coverage hole caused by a nearby strong interfering small cell if the device is not a subscriber of the small cell. The device connected to the macrocell may not even detect the control channel of the macrocell due to a strong interference from the small cell, while the device cannot handover to the small cell since it is not its subscriber (i.e., not part of the CSG). Another scenario can be of small cells (such as picos) interfered by the overlaying macrocell, while the device connected to the pico cell is impacted by such interference, hence the pico offloading traffic capability is weakened. Current IEEE 802.16m has limited solutions for the co-channel small cells overlaid by a macrocell, especially for the control channel design (for example, the synchronization channel and superframe header collide in the time domain for small cells and overlaying macrocell, which may cause an outage to devices connected to a macrocell impacted by interference from the small cell).

Advanced interference management solutions for both control channel and data channel are needed to enable such deployment. Some of the control channels of IEEE 802.16m may need to be re-designed to support this deployment. Enhanced schemes to improve the data channel coverage and throughput need to be investigated.

The deployment can use smart resource (power, time, frequency, space, etc.) allocation. For example, the combination of the efficient resource allocation across tiers and fractional frequency reuse (FFR) can enhance the cell capacity. Appendix A compares the performance of a femto cellular network deployment with that of a macro-only network and illustrates the issues and additional performance possible through single frequency deployments.  As can be seen, single frequency deployments when coupled with smart interference management schemes are effective in providing improved data rates for indoor users, while retaining macro-cellular throughput performance.  Further investigation related to efficient resource allocation is needed to support a co-channel deployment.

The deployment can use coordinated multi-cell transmission technologies, such as multi-cell joint scheduling, coordinated beamforming, single-cell precoding with multi-cell coordination. With coordinated multi-cell transmission technologies applied on multiple cells across tiers, resource (power, time, frequency, space, etc.) allocation can be made more effectively. IEEE 802.16m has provided support for multi-cell related technologies, so as to have better system performance such as increasing the cell-edge throughput. However, the main focus there is addressing macrocells, while multi-tier networks are not in focus. With multi-tier deployment, multi-cell technologies need to be revisited, so that the design can be taking into account the multi-tier networks with large cells overlaying small cells, especially addressing resource allocation for interference and mobility management.

##### Multiple Carriers Case

Large cells, such as macrocells, and small cells, such as femto or pico cells, can be deployed over the distinct set of carriers to avoid inter-tier interference. Large cells and small cells can also be deployed over the same set of carriers or having carriers overlapped, where resource (carrier, power, time, space, etc.) allocation can be made, to avoid or mitigate the inter-tier interference. With good design of resource allocation, the coverage hole problem caused by the small cell to the non-member mobile station can be solved. For cells of the same tier, resource (carrier, power, time, space, etc.) allocation can also be made in order to better manage intra-tier interference.

One particular scenario is a deployment of multi-carrier cells. For example, a Carrier Aggregation (CA) method can be used, where the carriers can be aggregated to cover a larger band, and by doing so the system can have more resources and more flexibility to properly allocate the resources. IEEE 802.16m has provided support for multi-carrier cells, however, the main focus is in addressing macrocells while multi-tier networks are not in focus. With multi-tier deployment, multi-carrier technologies need to be revisited, so that the design can be taking into account the multi-tier networks with large cells overlaying small cells, especially addressing resource allocation for interference and mobility management. For example, to support co-existing picocells and femtocells, there is a need to further study the advanced design of multi-carrier systems in combination with SON algorithms. In order to avoid inter-tier interference, separate frequencies/carriers can be assigned for the link between the macro BS and the small BS (backhaul link), and the link between the small BS and the user (access link).

In distinct frequency/carrier multi-tier deployments, the carrier management schemes among tiers may be applied. For example, base stations of multi-tiers may use respectively different carriers among multi-carriers based on the measurement of interference from other base stations. Or the different region of time/frequency resource in multi-carriers is allocated to DL/UL control channels of each base station for robust transmission of control channels, respectively.

As mentioned in both single carrier and multi-carrier cases, an advanced interference mitigation scheme needs to achieve cell throughput enhancement. For example, the interference by the signal of CSG femto is critical to devices in macro cell. The efficient spectrum reuse is considered to avoid the interference among tiers. And the interference cancellation/ mitigation techniques of transmitter/receiver such as enhanced MIMO scheme (PMI coordination, etc) or the signaling/ feedback methods between base station of infrastructure tiers and/or devices can be considered. The techniques and methods are not fully addressed by the IEEE 802.16m standard.

### Client Tier

As mentioned Clients form an important part of the network hierarchy and can cooperate to assist in improving capacity coverage and quality of service in the network. In this section we address the client cooperation techniques for clients that support a single RAT.

#### In-band Client cooperation (In-band CC)

As shown in Fig., client cooperation is a function which enables relay through a mobile station or cooperative data transmission by cooperating clients to achieve throughput enhancement, reduced device power consumption, and interference reduction to neighbor cell. In-band CC means that the communication between cooperative clients is performed in-band. In other words, the same radio access technology as macro-cell is used for communication link between cooperative clients. If WWAN technology is used for in-band CC, then, client cooperation can be exploited between distant devices (e.g., widely spread M2M devices belonging to the same service provider). Moreover, it is possible to efficiently control communication link between cooperating clients since all the communication links are based on the same radio access technology which is controllable in the macro BS side.

Figure 5: In-band Client Cooperation using WirelessMAN Advanced Air Interface

### BS Energy Saving

One of the most known techniques for energy saving in the RAN is the reconfiguration of the network according to conditions of the network, e.g. traffic load and user density. Examples of such dynamic configuration include switching the BS on or off, or MIMO configuration change (multi stream encoding for smaller cells to cooperative MIMO for larger cells).

Proper handling is required in the standard to accommodate the inclusion and integration of ES mechanisms in the access network with minimal impact on the network performance and stability.

## Multiple Radio Access Technology

Future hierarchical network deployments are expected to be heterogeneous and integrate multiple radio access technologies. Synergistic utilization of the several available multiple radio interfaces, offers a rich dimension for enhancing the capabilities of hierarchical network deployments. This section describes some usages exploiting multi-radio access technologies (Multi-RATs). Especially, we emphasize that an integrated IEEE 802.16/802.11  BS can be used to efficiently manage diverse IEEE 802.11 networks to improve network/user performance, quality of service, battery power and so on.

Multi-RAT deployment combined with cognitive radio techniques can be advantageous in several scenarios. Of particular importance can be its applicability to service continuity in disaster situations in that there is a possibility that a certain radio access network may be disrupted or degraded for a sustained period of time. In such situations, Multi-RAT deployments with cognitive techniques can be useful in providing continuity of communications. Appendix C provides an example of this application.

2.2.1 Multi-RAT Deployment Scenarios

As indicated, an IEEE 802.16 BS may be used to efficiently manage communication between the Multi-RAT networks. This usage model calls for synergistic use of licensed IEEE 802.16 and unlicensed spectrum, IEEE 802.11, to improve the overall network capacity and user quality of service in order to address additional applications and business models.

As shown in the figure 4, two different scenarios are possible depending on whether a Multi-RAT device connects to an IEEE 802.11 AP with wireless or wired backhaul (in this case, the scenario is “Virtual Carrier”) or cooperative dual RAT device (in that case, the scenario is “Enhanced Tethering”).

  - Virtual Carrier

- A Multi-RAT device is able to communicate with IEEE 802.16 BS while also communicating with IEEE 802.11 AP using IEEE 802.11 air interface.

- The Multi-RAT device uses both IEEE 802.16 and IEEE 802.11 connectivity in order to provide enhanced spectrum utilization.

- IEEE 802.11 link can be used to offload certain or all traffic off a Multi-RAT device with IEEE 802.16 connectivity.

  - Enhanced Tethering (Multi-RAT Client Cooperation)

- A Multi-RAT device is able to communicate with IEEE 802.16 BS while also communicating with other Multi-RAT device(s) using IEEE 802.11 air interface.

- In order to provide enhanced spectrum utilization, both the direct IEEE 802.16 link and indirect IEEE 802.11 link can be used to communicate with the IEEE 802.16 BS.

- Depending on link quality, a source Multi-RAT device can choose one of the direct IEEE 802.16 link and indirect IEEE 802.11 link to communicate with the IEEE 802.16 BS.

In these usage scenarios, IEEE 802.11 link of each Multi-RAT device can be coordinated by the IEEE 802.16 BS which provides IEEE 802.11 neighbor discovery / selection and access information in order to increase the interworking benefits.

Figure 4. An example of multi-RAT deployment scenarios

Several techniques are applicable for utilizing the IEEE 802.11 link to improve multiple metrics. Error: Reference source not found describes the multiple different ways the IEEE 802.11 link may be used and their associated advantages.

| Techniques | Description | Target Gains |
| --- | --- | --- |
| Handoff to IEEE 802.11 controlled by same IEEE 802.16 operator (Baseline) | Offload to IEEE 802.11 networks when possible | Interference avoidance gains ~3x for indoor users; |
| Interference Avoidance | Dynamically switch between 802.11 & 802.16 to avoid interference | Increase system throughput > 3x. |
| Diversity/Redundancy Transmission | Use added spectrum to improve diversity, code rates with incremental redundancy | Increases SINR ~3-5 dB, HARQ gains, decreases cell-edge outage, latency advantage |
| Aggregation of Multi-RAT carriers | Use added spectrum to transmit independent data streams (potentially different connections) | Increases peak throughput ~2-3x |
| Multi-Service Flow QoS | QoS-aware mapping of service flows to different Multi-RAT carriers | Improves QoS |
| Load Balancing | Dynamically switch flows between IEEE 802.11 and IEEE 802.16 networks. | Improves system capacity, potential improvement to QoS |
| Reduced Overhead w/ Unified Control across IEEE 802.16 and IEEE 802.11 | Unified control of paging, security, Multi-RAT connection, etc. | Saves power consumption Reduces overhead |

Table 1: Use cases and advantages for Multi-RAT Deployment Scenarios

### Unlicensed Spectrum Access

IEEE 802.16 allows the use of unlicensed spectrum for its operation. While this allows for access to additional spectrum, it may create co-existence issues with other radio access technologies. Here IEEE 802.16 may be used to facilitate communication between the multi-radio protocols.

### Efficient Multi-Radio Operation for Client Devices

As client devices evolve to support multiple radio protocols, cost considerations may drive towards increasing hardware reuse across protocols. Here additional protocol support from the network may be required to enable efficient hardware reuse. An example of this protocol support is that the network may support periodic silence periods, to allow devices to listen to other networks while using the same hardware.

### Interworking Schemes

In order to provide higher level of interworking function, cellular IEEE 802.16 BS and an IEEE 802.11 AP may interact with each other through the wireless or wired connection. Interworking features depicted in the study report are based on and mostly affected by coordination between the BS and AP.

As mentioned above, there are several benefits those are provided by enabling interworking schemes for a Hierarchical Network, among which are the improvement of link quality and maximized capacity on both IEEE802.16 and IEEE802.11 networks, achieved by means of device offloading or seamless handover from one network to another as well as interactive interference management of IEEE802.11 networks through the IEEE802.16 link. Such benefits may be achieved by BS coordination (i.e. 802.16 BS may provide configuration/coordination information to its subordinate IEEE 802.11 APs).

#### BS-AP Interworking

In order to minimize the power consumption and access delay for devices discovering IEEE802.11 APs and establishing connection with them, the cellular network can provide the devices with information for efficient discovery and selection of IEEE802.11 APs via the IEEE 802.16 link. Management of IEEE802.11 AP’s can be established via the IEEE802.16 BS. Tightly coupled BS-AP interworking is a scheme where an IEEE802.16 BS communicates through the IEEE802.16 link with subordinate access points that support both IEEE802.11 and IEEE802.16 protocols. This implies that the IEEE802.16BS may be able to manage (or facilitate network management) and coordinate the APs under its coverage. Such coordination through the wireless connection can provide low-access latency and enhanced interworking for, e.g., BS-AP frame alignment when interference is a concern (e.g. same operating frequency), common scheduling for both technologies, coordinated handover and SON operation etc.

Figure xx illustrates coordinated IEEE 802.11 AP for BS-AP interworking.

Figure N. Example of interworking IEEE 802.16 BS and IEEE 802.11 AP

#### Interworking via Higher-layer

As aforementioned, a MS can be provided interworking services by using upper layer protocols (i.e. above L2). Signaling procedures of L2.5 protocols are already defined in other standards. Example implementations of Layer 2.5 can be based on ORAT, MIH/802.21 or possibly Internet Protocols such as IETF Mobile IP.

# Hierarchical Network System Architecture

## Protocol and Functional Structure

Figure x shows the IEEE 802.16 HN functional entities within the Protocol Structure.

Figure x: IEEE 802.16 HN Functional decomposition within the Protocol Structure

### Single-RAT Protocol and Functional Structure

In order to support single-RAT hierarchical network operation, following functions may be enhanced.

- Radio resource management

- Multi-carrier support

- Mobility management

- Interference Management

- Scheduling and Resource multiplexing

- Control Signaling

‘Radio resource management’ and ‘Scheduling and resource multiplexing’ are the functions that help to utilize the resources in views of time/frequency/space/power allocation and help to mitigate the interference and/or coordinate transmission among multi-tiers.

‘Multi-carrier support’ allows data and control channels to be allocated over the distinct set or the same set of available carriers across tiers.

‘Mobility management’ function assists MS to search for the cells not in the advertized or configured neighboring list without significant searching overhead.

‘Interference management’ function controls severe inter-cell interference in the present of multiple cells in a multi-tier network and enhances the user throughput.

‘Control signal’ may be enhanced to support downlink control and uplink control and measurement feedback handling multiple channels used by multi-tiers.

### Multi-RAT Protocol Structure

In order to support Multi-RAT hierarchical network operation, following functional blocks are included over existing IEEE 802.16m protocol.

- Multi-RAT Discovery & Selection Management

- Multi-RAT Service Flow Management

- Multi-RAT Connection Management

- Multi-RAT Handover Management

- Multi-RAT Coordination

The Multi-RAT Discovery & Selection Management entity helps the network discovery process managing the scanning procedures of neighboring multi-RAT networks (APs) or multi-RAT devices. It may include query and response to/from an information server containing data related to pertinent accessible networks and/or a function providing pertinent information on neighboring Multi-RAT devices/networks. The Multi-RAT Discovery & Selection Management entity also supports a function that selects a specific AP or cooperative device.

The Multi-RAT Service Flow Management entity manages service flow related operations. The entity is QoS-aware and enables mapping of service flows to the various Multi-RAT connections and IP flow management to different Multi-RAT carriers, etc.

The Multi-RAT Connection Management entity is enabling the establishment and release of the Multi-RAT connection(s) between AP and device or between peer devices. The Multi-RAT Connection Management entity also handles connection loss.

The Multi-RAT Handover Management entity supports functions related to performing Multi-RAT handovers and Inter-AP handovers.

The Multi-RAT Coordination entity can enable the configuring of transmission parameters of APs, enables AP registration/updating and Inter-AP coordination.

In order to support multi-RAT hierarchical network operation, the functions in the following entities may be enhanced.

- Control Signaling

- Network Entry Management

- Multi-Radio Coexistence

- Security Management

- Data forwarding

Figure 7 illustrates the position of Layer 2.5 in a protocol stack and the interaction of L 2_5 with the 802.16 and 802.11 protocol stacks.

All exchanges between the L2.5 and other functional entities occur through service primitives, grouped in service access points (SAPs).

Example implementations of Layer 2.5 can be modeled based on ORAT, MIH/802.21 or possibly Internet Protocols such as IETF Mobile IP.

Figure 6: Protocol architecture of Multi-RAT devices

The reference model includes the following SAPs:

a) L_2.5_SAP: Media independent interface with the upper layers of the protocol stack

b) LSAP: Media dependent interface of L_2.5 with the lower layers of the 802.11 protocol stacks

c) CS_SAP: Media dependent interface of L_2.5 with the convergence sublayer of the 802.16 protocol stacks

d) M-SAP: Provides the interface between L_2.5 and the IEEE 802.16 management plane functions

e) C-SAP: Provides the interface between the L_2.5 and the IEEE 802.16 control plane

f) MLME-SAP: Defines the interface between L_2.5 and the management plane of the IEEE 802.11 MAC layer

g) PLME-SAP: Defines the interface between L_2.5 and the management plane of the IEEE 802.11

## Multi-RAT Network Architecture

Figure 7: Network architecture of Multi-RAT system

#### Network Discovery and Selection Server

The ND&S Server contains data management necessary to provide network discovery and selection assistance data as per operators' policy.

The ND&S Server may be able to initiate data transfer to the device, based on network triggers, and/or respond to information query from the device.

Example of ND&S Server can be found in section 4.8.2 of TS 23.402 and section 7.2 of WMF-T37-010-R016v01, WiMAX-WiFi Interworking.

# Key Features and Requirements

In this document Single Radio, Dual Radio and Multi Radio devices follow the definition provided in 802.16m section 3.

## Single Radio Access Technology

The key advantages of a single RAT multi-tier deployment are summarized as follows:

- Gains in areal capacity: Preliminary performance evaluation of multi-tier cell deployments with full spectrum reuse, indicate that the areal capacity increases linearly w/ increasing number of cells

- Favorable cost structure: Significant cost benefits are available for smaller lower-cost infrastructure tiers, as well as reduced CAPEX and OPEX from base-station site development and maintenance. For example, femto cells are user deployed in home and use existing backhauls [6].

- Improved coverage: Coverage is substantially improved as the base-station move closer to the users.  This improvement is especially needed for indoor deployments, where the signal from the macro base station may be weaker and an in-home device like the femto can significantly improve coverage.

To fully achieve the advantages, we have to consider the below points:

- For limited client access to Multi-tier networks, the level of network access shall be selectively provided by the network in terms of client membership to the IEEE 802.16 network.

- Network access information may be provided by Network Discovery and Selection Server.

To achieve the advantages, the following features and requirements are considered. The following sub-clauses include features that are common to one or more multi-tier deployment scenarios. It is possible to have requirements or features independently according to the deployment scenarios.

### Advanced Interference Management in Multi-tier Networks

Interference between devices and/or BSs in different tiers shall be mitigated.

#### Advanced Interference Management for Single Carrier Deployment

Control/data channel interference shall be mitigated to support small cell in a single carrier multi-tier deployment, where the small cell is deployed on the same carrier (co-channel) as the overlaying macrocell.

Co-channel interference can happen between overlaying macrocell and overlaid small cell, in both downlink and uplink. The most difficult problem is the CSG femtocell, which may cause a coverage hole to a nearby non-member mobile station due to strong interference and not being able to handover to the CSG femtocell. Such coverage hole may happen between CSG femtocell and other cells (i.e., macrocell, picocell, other femtocells). To solve the problem, advanced designs addressing interference mitigation/avoidance shall be made on the control/data channels.

Pico cell can be used to offload the macrocell traffic and a device may prefer to stay connected to the pico cell even though a macrocell with stronger signal strength is found. A device connected to a picocell may suffer interference from the overlaying macrocell, and likewise the device connected to a picocell may also cause uplink interference to the other devices connected to the macrocell. Advanced interference management is required to support pico cell in a co-channel deployment environment.

Smart resource allocation (power, time, frequency, space, etc.) shall be incorporated, such as the coordinated resource reservation across tiers, fractional frequency reuse (FFR), etc.

#### Advanced Coordinated Multi-cell Technologies for Co-channel Interference Management

Advanced coordinated multi-cell technologies across multiple tiers can help mitigate the interference across tiers. With advanced coordinated multi-cell technologies, such as multi-cell joint scheduling coordinated beamforming, single-cell precoding with multi-cell coordination, with multi-tier cells, resource (power, time, space, etc.) allocation can be made more effectively. The design shall taking into account the multi-tier networks with large cells overlaying small cells, especially along the resource allocation for interference management.

#### Advanced Interference Management for Multiple Carrier Deployment

Control/data channel interference shall be mitigated to enable multiple carrier multi-tier deployment, where the cells are deployed on multiple available carriers. Large cells and small cells can be deployed over a distinct set of carriers to avoid interference across the tier of large cells and the tier of small cells. Large and small cells can also be deployed over the same set of carriers or having carriers overlapped, where resource (carrier, power, time, space, etc.) allocation can be made, to avoid or mitigate the interference across tiers. Enhanced interference management schemes can be used for managing interference among cooperating small BSs. Examples of such schemes are joint scheduling and coordinated beamforming.

Carrier Aggregation (CA) shall be supported with the considerations interference mitigation or avoidance across tiers. This will account for the overlaying macrocell, overlaid pico and femto cells (including CSG cells). Multi-carrier system may work together with SON algorithms.

#### Power Setting or Power Control of Femtocell

Femtocell’s downlink transmission power shall be controlled as to avoid introducing source of interferences to other cells. Standards support to enable power setting or power control of femtocells may be defined.

### Advanced Mobility Management

Energy efficient and optimized devices searching for neighboring cells shall be supported. It shall support the ability of a device to search for cells not in its neighbor list in efficient ways without much overhead. Cell search shall differentiate access triggering conditions for each tier. Handover to a preferred type of cells shall be supported as well as support for cell selection and reselection based on factors such as cell type preference, cell load, etc.

For limited client access to multi-tier networks, the level of network access may be selectively provided by the network in terms of client membership to the IEEE 802.16 network.

### Special Energy Saving Mode of Base Station

Base stations, especially small base stations such as femtocell, may support special energy saving mode, such as low duty cycle mode, when there is no MS attached or trying to attach it. The mode can save energy consumption, also mitigate the interference to others. With such mode, efficient way for the MS to identify and search for the femtocells shall be supported.

This section may also serve as a place holder for green technologies, which are to be described in later stages.

### Advanced Self-Organized Networking (SON)

Advanced self-organized networking shall be supported in order to enable multi-tier deployment. Advanced SON shall be able to work together with advanced interference management, mobility management, base station mode change, etc.

### Client Cooperation (CC) Support

The following requirement shall be supported:

- An IEEE 802.16 device communicating with another IEEE 802.16 device for Single-RAT CC shall be controlled by an IEEE 802.16 BS.

- An IEEE 802.16 system supporting single-RAT CC should minimize the hardware change of the IEEE 802.16 BS.

- An IEEE 802.16 BS supporting single-RAT CC shall be able to configure the candidate set for a certain source device. The candidate set consists of single-RAT CC-capable devices.

## Multi Radio Access Technology

- IEEE 802.16 device supporting multi-RAT functionality shall support one or more radio access technologies (e.g., IEEE 802.11, IEEE 802.16)

- IEEE 802.16 device supporting multi-RAT should be able through any serving RAT to discover and select an access network through the use of Network Discovery and Selection Server

- An IEEE 802.16 device supporting multi-RAT function should be able to communicate with other IEEE 802.16 devices supporting multi-RAT function through IEEE 802.11 radio access technology.

- IEEE 802.16 devices supporting multi-RAT function shall be able to switch its radio access technologies based on a certain condition.

- Connection of an IEEE 802.16 device supporting multi-RAT function with an IEEE 802.16 BS shall be continuously supported when the IEEE 802.16 device communicates with the IEEE 802.16 BS using other RAT link.

- An IEEE 802.16 device supporting multi-RAT function shall be able to be connected to one or more radio access technologies.

- IEEE 802.16 device supporting multi-RAT function should scan and report its channel status on each radio access technology.

- IEEE 802.16 device supporting multi-RAT function shall be able to discover its neighbor IEEE 802.16 devices including ones that support multi-RAT function.

- IEEE 802.16 devices supporting multi-RAT function should be able to transmit the data packets, which are received from neighbor IEEE 802.16 devices supporting multi-RAT function to IEEE 802.16 BS.

- IEEE 802.16 devices supporting multi-RAT function should be able to transmit the data packets, which are received from IEEE802.16 BS, to its neighbor IEEE 802.16 devices supporting multi-RAT function.

- Data PDU shall be formatted to be automatically transferred from one radio interface to other radio interface within IEEE 802.16 devices supporting multi-RAT function.

- IEEE 802.16 devices supporting multi-RAT function shall be able to perform handover between IEEE802.16 BS and IEEE802.11 AP.

- IEEE 802.16 device supporting multi-RAT function shall be able to enable its IEEE802.16 radio interface by input provided through its IEEE802.11 protocol and vice versa.

- IEEE 802.16 BS supporting multi-RAT capability shall be able to communicate with an IEEE 802.11 AP under its coverage via wired and/or wireless control connection for interworking operation. IEEE802.16 BS should be able to provide or facilitate configuration and control of the IEEE802.11 APs under its coverage.

# Standards Implications

## Single Radio Access Technology

### Advanced Interference Management in Multi-tier Networks

Enhanced interference mitigation is required during data transmission between base stations, between cooperative devices, and between base station and cooperative devices.

Smart resource allocation (power, time, frequency, space, etc.) shall be supported, such as the coordinated resource reservation across tiers, fractional frequency reuse (FFR), etc

#### Advanced Interference Management for Single Carrier Deployment

Control/data channel interference shall be mitigated to support small cell deployment in a single carrier multi-tier deployment, where the small cell is deployed on the same carrier (co-channel) as the overlaying macrocell.

Control channel redesign at the physical layer may be needed to support co-channel deployment of overlay macrocell and overlaid femtocell in particular one supporting CSG.

The standards shall enable a mobile station to detect a picocell and attach to it if prefers to be served by pico cell even whe subject to strong interference by a nearby macrocell.

#### Advanced Coordinated Multi-cell Technologies for Co-channel Interference Management

Advanced coordinated multi-cell technologies across multiple tiers are supported in order to mitigate the interference across tiers. Technologies such as multi-cell MIMO, multi-cell joint scheduling, coordinated beamforming, single-cell precoding with multi-cell coordination in multi-tier system may be enhanced when compared to a single tier system.

#### Advanced Interference Management for Multiple Carrier Deployment

In multiple carrier multi-tier deployment, joint resource allocation using time/frequency/spatial domain is required to mitigate the interference. Technologies such as Carrier Aggregation in multi-tier system may be enhanced when compared to a single tier system.

#### Power Setting or Power Control of Femtocell

Femtocell’s transmission power shall be controlled in order to minimize interferences to other cells and devices. Standards support to enable power setting or power control of femtocells may be defined.

### Advanced Mobility Management

Operations and corresponding standards support shall be enhanced for energy efficient optimized devices in order to efficiently search adequate neighboring cells. This includes efficient search for the cells not included in the neighboring list with minimum overhead.

Configuration of pertinent Single-RAT information at the Network Discovery and Selection server may be required

### Special Energy Saving Mode of Base Station

Energy efficient small cells with special mode of transmission reduced may be defined.

Corresponding MAC support is needed to search for and entering the cells in such mode.

This section may also serve as a place holder for green technologies, which are to be described in later stages.

### Advanced Self-Organized Networking (SON)

Advanced Self-Organized Networking functionality shall be supported in order to enable multi-tier deployment. Advanced SON shall be able to work together with advanced interference management, mobility management, base station mode change, etc.

### Client Cooperation (CC) Support

The framework for direct communication link between a source device and a cooperative device shall be defined. Examples may include frame structure in the device, scheduling information, HARQ feedback, and channel measurement and feedback for the link.

Enabling Client Cooperation capability within a single-RAT IEEE 802.16 device may be negotiated through either a forced network (re-)entry procedure or possibly through dynamically configuration updated during or after Initial Network Entry (INE).

## Multi Radio Access Technology

### General

- The security procedure may be enhanced to support data communication between the Multi-RAT devices and the PoAs. A BS supporting Multi-RAT may manage the security associations of both the IEEE 802.16 and IEEE 802.11.

- Changes to the network entry/re-entry procedure for Multi-RAT operation (e.g., Multi-RAT capability negotiation, security related procedure) may be required.

- Configuration of pertinent Multi-RAT information at the Network Discovery and Selection server should be required.

### Virtual Carrier

#### Multi-RAT Network Discovery/Access Management

- A scanning procedure can be modified to support the IEEE 802.11 AP scanning. The BS may transmit the network information of the IEEE 802.11 APs within its coverage. The network shall provide neighboring IEEE 802.11 AP information via Information Server. IEEE802.16 devices supporting Multi-RAT may perform the scanning procedure for the IEEE 802.11 APs based on the provided information, predefined criteria (e.g., carried data characteristics or channel quality) or explicit signaling (e.g., SCN-RSP MAC control message) transmitted by the BS.

- Access to the IEEE802.11 AP may be controlled by the IEEE802.16 BS for power efficiency.

- Devices supporting virtual carrier may maintain data connections with the IEEE 802.11 based on the signaling transmitted by the BS for power efficiency.

- Data communication on the virtual carrier may require a new association with the specific AP. The BS may manage connection information (e.g., association, re-association and disassociation) for data packets transmitted on the virtual carrier and help the device to easily join/release to the AP. A device supporting Multi-RAT may transmit its preferred AP(s) to a BS and/or a BS may inform a device of the selected AP, based on the device’s preferred AP(s), channel quality or network loads.

- A device supporting Multi-RAT may transmit its preferred AP(s) to a BS and/or a BS, through the use of IS (Information Server), may inform a device of the selected AP. The information may be based on the device’s preferred AP(s) and/or network loads.

#### Flow Mobility management

- Data communication on the virtual carrier may require an IP flow mobility procedure between Multi-RATs to support seamless Multi-RAT switching. The flow connections between multi-RATs may require a QoS mapping to support the seamless IP flow mobility.

- Devices supporting virtual carrier may require a flow mobility management between IEEE 802.11 neighbors to support continuous data communication of high data rate. A device may transmit information (e.g., channel quality) for its IEEE 802.11 neighbors and a BS may help the device to easily join to the neighbor AP.

### Multi-RAT Client Cooperation

- To enable Multi-RAT client cooperation, neighbor discovery and cooperative device selection protocols may be provided. Also, Multi-RAT supporting systems may help an IEEE 802.16 device supporting Multi-RAT to discover other IEEE 802.16 devices supporting Multi-RAT and to select the cooperative device.

- Change to cooperation addressing and security operation may be required.

- Control signaling for Multi-RAT supporting systems may be supported for data transmission of the communication link between source device and cooperative device.

- To sustain good connectivity, link management between source device and cooperative device may be coordinated by IEEE802.16 BS

### Support of BS-AP Interworking

In order to support a tightly coupled interworking AP, the following procedures between IEEE802.16BS and IEEE802.11AP are required to be supported or defined:

- The IEEE802.11 AP shall support a wired link or IEEE802.16 air interface link with the IEEE802.16 BS.

- Enable registration of the IEEE802.11AP with the IEEE802.16BS (e.g., capability negotiation, AP white list, etc)

- Enable AP configuration procedures by or via the IEEE802.16BS. These include but not limited to AP transmission power, timing, operational mode, etc.

- Enable reporting from the AP to or via the IEEE802.16BS. This includes but not limited to neighbor AP scan reports, interference reports, data load reports, etc.

# Conclusions & Recommendations

# List of Acronyms and Definitions

| HN | Hierarchical Networks |
| --- | --- |
| CC | Client Cooperation |
| Capillary Networks | Multiple networks fanning out with smaller granularity than a network hub |
| Client | Implies a subscriber station |
| FFR | Fractional Frequency Reuse |
| FFZ | Femto free zoning (Reserved resources where femtos do not transmit) |
| GLL | General Link Layer |
| In-Band | In the same frequency band |
| RF | Radio Frequency |
| WAN | Wide Area Networks |
| WiMAX | Worldwide Interoperability for Microwave Access |
| WLAN | Wireless Local Area Network |
| WPAN | Wireless Personal Area Networks |
| WWAN | Wireless Wide Area Networks |

# References

- IEEE C802.16ppc-10_0008r3, “Study Report on Hierarchical Networks,” Inuk Jung, Nageen Himayat, January, 2011

- IEEE C80216-10_0016r1, “Future 802.16 network: Challenges and Possibilities,” IEEE 802 Call for Interest Tutorial, March, 2010.

- IEEE P802.16m/D11

- IEEE C80216-10_0004, “Multi-tier topologies in future broadband networks,” January, 2010.

- S. Yeh, S. Talwar, S. Lee and H. Kim, “WiMAX Femtocells: A perspective on network architecture, capacity and coverage,” IEEE Communication Magazine, Oct. 2008.

- Johansson at al, “A Methodology for Estimating Cost and Performance of Heterogeneous Wireless Access Networks,” PIMRC’07.

- IEEE 802.16m Evaluation Methodology Document (EMD), January 2009

- “Guidelines for evaluation of radio transmission technologies for IMT-2000”, Rec. ITU-R M.1225

- IST-WINNER II Deliverable D1.1.1 V1.1, WINNER II interim channel models, September 2007.

# Appendix A: An example of Smart Resource Allocation in Multi-tier Networks

Currently, multi-tier deployments are possible for both

a) Single frequency/carrier across tiers

b) Multiple frequencies/carriers across tiers

IEEE 802.16m supports both types of deployments, but more efficient spectrum usage across tiers using time/frequency/spatial domain is required. For example, in single frequency/carrier multi-tier deployments, the combination of the efficient resource allocation across tiers and fractional frequency reuse (FFR) makes the cell capacity enhanced. Error: Reference source not foundComparing performance of a femto cellular network deployment with that of a macro-only network illustrates the issues and shows how additional performance is possible through single frequency deployments. As can be seen single frequency deployments when coupled with smart interference management schemes are effective in providing improved data rates for indoor users, while retaining macro-cellular throughput performance. The provided results use intelligent resource management and combine fractional frequency reuse (FFR) in macro cells with femto free zoning (FFZ) to reduce network interference. Further investigation of these ideas is warranted for both control and data channels.

| Transmission Scheme | Outdoor Outage (%) | Indoor Outage (%) | 50% Outdoor rate (Mbps) | 50% Indoor rate (Mbps) |
| --- | --- | --- | --- | --- |
| FFR + NO Femto AP on 10 MHz | 3.0 | 17.0 | 0.07 | 0.03 |
| FFR Macro on 5 MHz, Femto on different 5 MHz | 3.0 | 0.2 | 0.06 | 10.7 |
| FFR + FFZ + 0dBm Femto AP power on 10 MHz | 3.0 | 0.5 | 0.06 | 11.3 |

Table 2: Performance improvements for indoor and out-door users with overlay femto-cell networks, utilizing intelligent interference management schemes.  FAP refers to femto access points [4]. See Appendix B for the methodology used for the simulation.

# Appendix B: Multitier Evaluation Methodology

## Hierarchical Network Simulation Methodology

This annex will address the simulation methodology for hierarchical network simulations. This simulation methodology addresses the performance gain and interference issues critical to hierarchical network performance evaluation. Multi-tier femto-cell networks are used as a representative case to illustrate the methodology. Note that two possible usage scenarios may be considered for overlay devices serving as access points in multi-tier networks: open access and closed subscriber group (CSG). The open access devices are available to all subscribers. They are usually public infrastructure based, like picocell base stations (BS) and relay stations, and the deployment is planned by the operators. On the contrary, only a restricted group of users have permission to access a CSG device. This is the typical scenario for femtocells since Femto access points (FAP) are usually privately owned and deployed by users.

## Multi-tier Femtocell Overlay Networks

This subsection describes the simulation methodology for femtocell overlay networks, considering both CSG and OSG (open access) types of FAPs. The methodology covers general settings for spectrum usage, traffic model, scheduling, and FAP device parameters. We also describe a deployment model to locate FAP and subscriber stations within the macro-cellular network. In addition, channel models, interference modeling and performance metrics are all addressed in this document.

### General Simulation Settings

The simulation methodology for hierarchical networks is built on top of existing 802.16m evaluation methodology document Error: Reference source not found. Parameters specified in Error: Reference source not found should be adopted in simulations for femtocell overlay networks. If not stated explicitly, the simulation setting from Error: Reference source not found will be directly inherited. Additional parameter settings are described in the following.

FAP spectrum usage:

Table 3 – FAP spectrum usage

| Scenario | Description |
| --- | --- |
| Co-channel operation | FAPs share the same carrier as MBSs (macro base stations). |
| Separate channel operation | FAPs transmit at different carrier as MBSs. |

The operating band for FAPs can be either the same as MBSs or using a separate spectrum. The co-channel operating case where FAPs share the same spectrum with the MBSs, is the case of most interest as it targets the desired goal of full spectrum reuse between tiers. Additionally, it also fully captures interference issue present in multi-tier deployments.

Traffic Model:

For simplicity, only full buffer traffic is considered at this stage. More advanced traffic patterns may be considered in the future.

Scheduling

Simple round-robin scheduling may be used for initial performance evaluation. More realistic scheduling schemes, like proportional-fair scheduling, may also be considered.

MBS, Subscriber Station and FAP parameters:

For MBSs and subscriber stations, the parameters from Error: Reference source not found are adopted. For FAPs, the parameters are summarized as follows.

Table 4 – FAP settings

| Parameters | Value |
| --- | --- |
| Antenna Gain | 0dB |
| Antenna Height | 2 meters + floor height |
| Antenna Pattern | A(θ) = 0 (omni-directional) |
| Maximum Transmit Power Level | -10, 0, 10, 20 dBm |

Macrocell Deployment:

The macrocell deployment model is the same as in Error: Reference source not found. A hexagonal grid with 19 cell sites, each with 3 sectors (Total 57 sectors), is considered. Additional 6 clusters wrap around can be added. The deployment is show in Figure 8.

Figure 8 Macrocells deployment

Two more scenarios, in addition to the baseline settings in [7], should be evaluated.

The small cell scenario:

    - Cell radius = 500 meters (Site to site distance = 866 meters)

    - BS TX Power = 36dBm

The large cell scenario:

    - Cell radius = 1500 meters (Site to site distance = 2598 meters)

    - BS TX Power = 46dBm

### FAP and Subscribers Deployment Model

The FAP deployment is illustrated in Figure 9. Single floor circular houses with 10 meters radius are assumed. There is one FAP in each house located at the center of the house. The house locations are determined as follows. First a square grid with 20m minimum separation is formed and then house locations are randomly select from this grid. A fully populated grid has around 538 houses per sector (2500 FAPs/km2).

The FAP density is a programmable parameter. Two representative deployment densities may be considered: dense deployment with about 100 FAPs per sector (~465 FAPs/km2) and sparse deployment with about 10 FAPs per sector (~46 FAPs/km2).

Figure 9. Illustration of FAP deployment

For the subscribers, indoor and outdoor users are separately deployed. Indoor user locations are uniformly distributed within the houses they are in. The probabilities that there are 1, 2, 3 and 4 users per house are 80%, 12%, 6% and 2%, respectively. The outdoor users are uniformly distributed over the area outside of houses, i.e., 10 meters away from all FAPs. The ratio of the number of indoor users to outdoor users is programmable. Typically, there can be equal number of indoor and outdoor users or 70% subscribers being indoors.

For simplicity, all FAPs are assumed to be CSG (or OSG). Only users within the same house as the FAP are assumed to have access permission to the CSG FAP. CSG user will choose between all MBSs and its FAP and pick the serving base station based on association rules such as picking the one with maximum received power at SS. Users not in CSG can only associate with MBSs or OSG FAPs and will choose the serving base station based on the association rule used in the network. The cell association scenario is demonstrated in Figure 10.

Statistics are only collected from SSs associated with MBSs and FAPs locating inside the center cell. However, to take into account the shadowing effect and to better capture the interference behavior, both FAPs and SSs are deployed inside a hexagon with radius equals five times of the macrocell radius.

Figure 10 CSG cell association

### Channel Model

- Path loss Models and Shadowing Models

The path loss, shadow fading (SF) and penetration loss under different cases are summarized in Table 5. A combination of ITU channel models Error: Reference source not found and Winner models Error: Reference source not found9] is used for static channel modeling.

Table 5 Channel Models

|  | Path Loss | SF | Penetration |
| --- | --- | --- | --- |
| Macro-BS to outdoor SS | (>500m) ITU vehicular: 40(1-4×10-3hb)log10(R[km]) + 21log10(f[MHz]) + 80 – 18log10(hb) (≤500m) ITU pedestrian: 40log10(R[km]) + 30log10(f[MHz]) + 49 | 10 dB | 0 |
| Macro-BS to indoor SS | (>500m) ITU vehicular (≤500m) ITU pedestrian | 12 dB | Mean 12dB, Std 8dB |
| Femto-AP to indoor SS | Winner A1 NLOS (through wall): PLfree_space = 46.4 + 20log10(R[m]) + 20log10(f[GHz]/5) | 6 dB | One light wall (3dB) every 3 meters |
| Femto-AP to outdoor SS | Winner A2 NLOS: max( PLfree_space, PLB1) If d<dBP, PLB1 = 41 + 22.7log10(d[m]*) + 20log10(f[GHz]/5) If d≥dBP, PLB1 = 41 + 22.7log10(d[m]*) + 40log10(d[m]/dBP) + 20log10(f[GHz]/5) | 7 dB | PLtw=(14+15(1-cosθ)2 ); PLin = 0.5din; |
| Femto-AP to neighbor SS | Same as above | 7 dB | Above + 12dB wall loss |

*dBP = 4h’BSh’MSfc/c, where fc is the center frequency in Hz, c = 3.0×108 m/s is the propagation velocity in free space, and h’BS and h’MS are computed as h’BS = hBS – 1[m] and h’MS = hMS – 1[m], where hBS and hMS are the actual antenna heights and the effective environment height in urban environments is assumed to be equal to 1 meter.

Fast fading models in Error: Reference source not found and Error: Reference source not found can also be used to model the fast fading effects in femtocell overlay networks.

### Interference Modeling

The interference modeling for dynamic simulation generally follows Error: Reference source not found. The following describes the procedure, and the differences are highlighted in bold.

- Determine the path loss, BS/FAP antenna gain, and shadowing from all interfering sectors and FAPs to MS.

- Rank the interfering sectors and FAPs in order of received power (based on pathloss, BS/FAP antenna gain, and shadowing).

- Model the channels of the strongest (strong I ) interferers as the signal path. (account for the pathloss, BS antenna gain, shadowing, and fast fading variations.) The value of strong I is set to 8 for MBSs and 18 for FAPs.

- Model the remaining sectors as spatially white Gaussian noise processes whose variances are based on a spectrally flat Rayleigh fading process. The power of the Rayleigh fading process includes the effects of pathloss, BS antenna gain, and shadowing. The fading processes for all links between MS and BS are assumed to be independent, and the Doppler rate is determined by the speed of the mobile. At any instant in time, the total received interference power is the summation of the receive power from of all weak interferers. Hence, the interference power is varying in time during a simulation drop.

### Performance Metrics

The following metrics should be considered for multi-tier network performance evaluation.

  - 5% outdoor throughput

  - 5% indoor throughput

  - 50% outdoor throughput

  - 50% indoor throughput

  - Overall areal throughput

  - Percentage of subscribers associated with FAPs.

An example of simulation results is shown in the following table:

Table 6 Example Simulation Results (~ 10 FAPs per sector, 48% SS with FAPs)

| FFR, w/ FFZ | FFZ ratio | 5% Outdoor Rate (Mbps) | 5% Indoor Rate (Mbps) | 50% Outdoor Rate (Mbps) | 50% Indoor Rate (Mbps) | Center Cell Throughput |
| --- | --- | --- | --- | --- | --- | --- |
| Macro Only |  | 0.0753 | 0.0537 | 0.4164 | 0.4556 | 44.4224 |
| FAP Tx Pwr = 10dBm | 4/8 | 0.1319 | 1.5578 | 0.4182 | 13.7848 | 809.7991 |
| Macro Only |  | 0.0762 | 0.0544 | 0.4176 | 0.4539 | 44.3857 |
| FAP Tx Pwr = 10dBm | 5/8 | 0.1716 | 1.2557 | 0.4558 | 10.3334 | 613.4495 |
| Macro Only |  | 0.0762 | 0.0534 | 0.4199 | 0.4545 | 44.3714 |
| FAP Tx Pwr = 10dBm | 6/8 | 0.1842 | 1.0322 | 0.4464 | 7.1107 | 417.1955 |
| Macro Only |  | 0.0756 | 0.0539 | 0.4190 | 0.4510 | 44.4351 |
| FAP Tx Pwr = 10dBm | 7/8 | 0.1352 | 0.6857 | 0.4501 | 3.5303 | 220.1630 |

# Appendix C: Multi Radio Access Deployments employing cognitive radio techniques

## Disaster Relief

Figure 11 shows a Multi-RAT deployment wherein the following cognitive radio functionalities are utilized.

  - Surveillance of radio access network

- A Multi-RAT device is able to recognize the state (signal quality, load, etc) of each radio access network.

- Alternatively, the network can inform the terminal about the status of the different RANs.

- The intelligent access node having Multi-RAT radio interfaces for WAN and LAN may cooperate with the network and the user terminals.

  - Cooperative controlling of Multi-RAT

- A Multi-RAT device is able to communicate through appropriate radio access network based on the application type and its destination, without producing undue overhead.

- In order to provide continuous operation, user’s traffic is routed to appropriate radio access network.

IEEE802.11 network

BS

BS

IEEE 802.16x

Internet

Ad hoc network

IEEE802.11 mesh network

Mesh network

Cellular

Satellite

Intelligent access node

LAN Interface

WAN Interface

user terminals

Figure 11 System architectures with intelligent access node

## Key Features and Requirements

- IEEE 802.16 device supporting cognitive radio function shall support key features and requirements for multi-RAT functionality.

- IEEE 802.16 device supporting cognitive radio function should recognize performance of a radio interface.

- IEEE 802.16 device supporting cognitive radio function should be able to transmit the data packets to an appropriate radio interface based on its application and destination.

- IEEE 802.16 device supporting cognitive radio function should be able to provide the required communication quality at least by combining multiple RATs.