| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Abnormal power down signaling header protection |  |
| Date Submitted | 2012-03-08 |  |
| Source(s) | Anil Agiwal, Hyunjeong Kang, Youngkyo Baek, Anshuman Nigam, Rakesh Taori Samsung Electronics | anilag@samsung.com, hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | Sponsor Ballot |  |
| Abstract | This contribution proposes method to protect Abnormal power down signaling header |  |
| Purpose | To be discussed and adopted in IEEE P802.16.1b/D3 |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Abnormal Power down Signaling Header Protection

Anil Agiwal, Hyunjeong Kang, Youngkyo Baek, Anshuman Nigam, Rakesh Taori

Samsung Electronics

- Introduction

The section 6.2.29 in draft [1] defines the mechanism to signal the abnormal power down indication to the BS. The draft [1] proposes a method to indicate abnormal power down indication using abnormal power down signaling header. The protection mechanism for abnormal power down signaling header is not defined.

In this contribution we propose a method of protecting abnormal power down signaling header. One bit EC field indicates whether abnormal power down signaling header is protected or not. A 16 bit short CMAC is used to provide integrity protection to abnormal power down signaling header. The frame number and ROC is used to provide the replay attack. The 8 bit ROC provides ensures that key refresh does not happen frequently. A separate key derived from CMAC-TEK prekey is used to generate short CMAC as the existing CMAC derivation and replay protection method used for protecting control messages is different than the methods used for protecting abnormal power down signaling header.

- Proposed Text Change

Adopt the following remedies in IEEE P802.16.1b/D3.

[Remedy 1: Modify Table 13b on page 7 as follows]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

Table 13b – M2M abnormal power down header format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| M2M abnormal power down header () { |  |  |
| FID | 4 | Flow Identifier. Set to 0b0010 |
| Type | 5 | MAC Signaling header type = 0b01001 |
| Length | 3 | Indicates the length of the signaling header in bytes |
| STID | 12 | STID of the M2M device that transmits this signaling header |
| STID_Valid_Offset | 3 | STID_Valid_Offset of the M2M device that sends emergency report. If the assigned STID is not shared with other M2M devices, M2M device shall set this field to zero. |
| Emergency Type | 1 | 0b0: Power Outage 0b1: Reserved |
| Authentication Code | 20 |  |
| EC | 1 | 0: Unprotected 1: Protected using CMAC. |
| If (EC == 1) { |  |  |
| ROC | 3 | 3 LSBs of Roll over counter (see 6.2.5.7 for details) |
| CMAC | 16 | CMAC (see 6.2.5.7 for details) |
| } else { |  |  |
| Reserved | 3 |  |
| } |  |  |
| } |  |  |

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 2: Insert the new section]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

6.2.5.7 Abnormal power down signaling header protection using Cipher-based message Authentication Code (CMAC)

An M2M device may support abnormal power down signaling header integrity protection based on CMAC-together with the AES block cipher. The CMAC construction as specified in NIST Special Publication 800-38B shall be used. The calculation of the keyed hash value contained in the CMAC Digest attribute and the CMAC Tuple shall use the CMAC algorithm with AES. The UL authentication key CMAC_SIG_KEY_U shall be used for authenticating abnormal power down signaling header transmitted by M2M device in the UL direction. The CMAC_SIG_KEY_U is derived as follows:

CMAC_SIG_KEY_U = Dot16KDF (CMAC-TEK prekey,”CMACSIG”, 128).

The 8 bit ROC is the rollover counter for the frame number, i.e., when the frame number reaches 0x0000000 (from 0xFFFFFF) it is incremented by 1 mod 2^8. ROC shall be reset to zero upon obtaining a new CMAC-TEK prekey. Using this method, one abnormal power down signaling header per frame using the same CMAC_SIG_KEY_U can be supported. A new encryption key (CMAC_SIG_KEY_U) is required every 2^24 * 2^8 = 2^32 frames. The reauthorization process should be initiated to establish a new PMK/AK before the ROC reaches the end of its number space.

The CMAC value shall be calculated over a field consisting of the AK ID followed by the super frame number and frame index expressed as an unsigned 24-bit number, followed by the 12-bit STID and 4-bit FID (FID of signaling header), followed by 8 bit ROC, followed by 16-bit of zero padding (for the header to be aligned with AES block size) and followed by the contents of abnormal power down signaling header other than CMAC. The CMAC value for abnormal power down signaling header shall be calculated as follows:

CMAC value = Truncate(CMAC (CMAC_SIG_KEY_U, AK ID |22 bit super frame number|2 bit frame index |STID|FID|ROC|16-bit zero padding |Abnormal power down signaling header contents other than CMAC & ROC), 16). The LSB 16-bit of the outcome of AES-CMAC calculation shall be used for CMAC value.

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

- Reference

[1] IEEE P802.16.1b/D2