| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Some parameters for multicast feedback |  |
| Date Submitted | 2012-03-01 |  |
| Source(s) | Eldad Zeira InterDigital | E-mail: eldad.zeira@interdigital.com |
| Re: | LB 38 |  |
| Abstract | The text proposal in this contribution provides some missing parameters for flexible feedback channel for multicast |  |
| Purpose | To be discussed and accepted by GRIDMAN |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Some parameters for multicast feedback

Eldad Zeira

# Summary

Some parameters are missing from related messages, proposed here.

# Text Proposal

- Modify table 57 as follows:

| N_SIZE_LOG | TBD | Indicates the number of distinct sizes of logical channels, measured in terms of number of physical channels associated with them | Included if multicast feedback is supported |
| --- | --- | --- | --- |
| For( i=0; i< N_SIZE_LOG; i++) { |  | N_SIZE_LOG indicates the number of distinct sizes of logical channels, measured in terms of number of physical channels associated with them. N_SIZE_LOG is [1..8] |  |
| p, Log base 2 of the number of physical channel per logical channel | 3 | The number is 2p |  |
| N_of_Size | TBD 12 | Indicates number of logical channels that have size 2p |  |
| } |  |  |  |
| N_frame | TBD 2 | Delay in frames between starting frame for the reception of multicast and the first frame of the feedback channel associated with it 0b01: 1 frame 0b10: 2 frames 0b11: 3 frames 0b00: 4 frames (i.e. same frame in next superframe) |  |
| K_subframe | 3 | Subframe indicator. 0b000: first subframe 0b001: second subframe … 0b111: eighth subframe |  |
| Feedback_ranging_format | 2 | 00b: S-RCH, 01b: NS-RCH format 0, 10b: NS-RCH format 1, 11b: reserved |  |
| Subcarrier start | TBD 11 | The starting sub-carrier for the ranging preamble |  |
| Starting code index | TBD 8 | The first code used |  |
| Code spacing | 4 | The spacing between codes that are used for feedback |  |
| Total number of codes | TBD 8 | The total number of channels used (note that the number of codes may span multiple subbands and sub-frames) which implicitly defines the number of subbands |  |

- Modify table 83 as follows:

| For (i=0; i<Num of Multicast Group ID and FID (M); i++) { |  | Num of Multicast Group ID and FID (M) is the number of Multicast Group IDs to add [1..16] | Present when ABS initiates AAI-DSA-REQ |
| --- | --- | --- | --- |
| Multicast Group ID | 12 | ID of a group to which the flow is added | Present only if Num of Multicast Group ID and FID (M)> 0 |
| FID | 4 | Multicast specific FID that is associated with Multicast Group ID | Present only if Num of Multicast Group ID and FID (M)> 0 |
| Feedback request indicator FRI | 2 | 00-no feedback, 01-ACK only, 10-NAK only, 11-reserved | May be omitted if feedback isn't supported; |
| If FRI ≠ 00b { |  |  |  |
| Logical channel indicator FBACK_LCI | TBD 12 | Indicates the index of the logical channel assigned to this multicast | May be omitted if sent in REG-RSP |
| If FRI = 10b { |  |  |  |
| Probability indicator of sending ranging preamble, pi | 10 | Indicates the probability of sending the NAK if NAK is indicated, probability = 2-pi | pi ≥ 0 |
| } |  |  |  |
| RNG-ACK_Used | 1 | 0b indicates no RNG-ACK is to be expected, 1b it is |  |
| If RNG-ACK_Used = 1 { |  |  |  |
| N_frame_rampup | 4 | Delay in frames to retransmission, =0 indicates no retransmissions |  |
| Number of retransmissions | 4 | Indicates number of retransmissions if no RNG-ACK |  |
| Power step | 4 | Indicates power difference in dB for retransmissions |  |
| } |  |  |  |
| } (of FRI ≠ 00b) |  |  |  |
| } |  |  |  |
| } // End if (Multicast is supported) |  |  |  |

- Modify table 84 as follows:

| For (i=0; i<Num of Multicast Group ID and FID (M); i++) { |  | Num of Multicast Group ID and FID (M) is the number of Multicast Group IDs to add [1..16] | Present when ABS initiates AAI-DSA-REQ |
| --- | --- | --- | --- |
| Multicast Group ID | 12 | ID of a group to which the flow is added | Present only if Num of Multicast Group ID and FID (M)> 0 |
| FID | 4 | Multicast specific FID that is associated with Multicast Group ID | Present only if Num of Multicast Group ID and FID (M)> 0 |
| Feedback request indicator FRI | 2 | 00-no feedback, 01-ACK only, 10-NAK only, 11-reserved | May be omitted if feedback isn't supported; |
| If FRI ≠ 00b { |  |  |  |
| Logical channel indicator FBACK_LCI | TBD 12 | Indicates the index of the logical channel assigned to this multicast | May be omitted if sent in REG-RSP |
| If FRI = 10b { |  |  |  |
| Probability indicator of sending ranging preamble, pi | 10 | Indicates the probability of sending the NAK if NAK is indicated, probability = 2-pi | pi ≥ 0 |
| } |  |  |  |
| RNG-ACK_Used | 1 | 0b indicates no RNG-ACK is to be expected, 1b it is |  |
| If RNG-ACK_Used = 1 { |  |  |  |
| N_frame_rampup | 4 | Delay in frames to retransmission, =0 indicates no retransmissions |  |
| Number of retransmissions | 4 | Indicates number of retransmissions if no RNG-ACK |  |
| Power step | 4 | Indicates power difference in dB for retransmissions |  |
| } |  |  |  |
| } (of FRI ≠ 00b) |  |  |  |
| } |  |  |  |
| } // End if (Multicast is supported) |  |  |  |

- Modify table 86 as follows:

| For (i=0; i<Num of Multicast Group ID and FID (M); i++) { |  | Num of Multicast Group ID and FID (M) is the number of Multicast Group IDs to add [1..16] | Present when ABS initiates AAI-DSA-REQ |
| --- | --- | --- | --- |
| Multicast Group ID | 12 | ID of a group to which the flow is added | Present only if Num of Multicast Group ID and FID (M)> 0 |
| FID | 4 | Multicast specific FID that is associated with Multicast Group ID | Present only if Num of Multicast Group ID and FID (M)> 0 |
| Feedback request indicator FRI | 2 | 00-no feedback, 01-ACK only, 10-NAK only, 11-reserved | May be omitted if feedback isn't supported; |
| If FRI ≠ 00b { |  |  |  |
| Logical channel indicator FBACK_LCI | TBD 12 | Indicates the index of the logical channel assigned to this multicast | May be omitted if sent in REG-RSP |
| If FRI = 10b { |  |  |  |
| Probability indicator of sending ranging preamble, pi | 10 | Indicates the probability of sending the NAK if NAK is indicated, probability = 2-pi | pi ≥ 0 |
| } |  |  |  |
| RNG-ACK_Used | 1 | 0b indicates no RNG-ACK is to be expected, 1b it is |  |
| If RNG-ACK_Used = 1 { |  |  |  |
| N_frame_rampup | 4 | Delay in frames to retransmission, =0 indicates no retransmissions |  |
| Number of retransmissions | 4 | Indicates number of retransmissions if no RNG-ACK |  |
| Power step | 4 | Indicates power difference in dB for retransmissions |  |
| } |  |  |  |
| } (of FRI ≠ 00b) |  |  |  |
| } |  |  |  |
| } // End if (Multicast is supported) |  |  |  |