| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Missing parameters for HR-MS paging |  |
| Date Submitted | 2012-03-01 |  |
| Source(s) | Eldad Zeira InterDigital | E-mail: eldad.zeira@interdigital.com |
| Re: | LB 38 |  |
| Abstract | The text proposal in this contribution provides some missing parameters for HR-MS paging |  |
| Purpose | To be discussed and accepted by GRIDMAN |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Missing parameters for HR-MS paging

Eldad Zeira

# Summary

Some parameters are missing from related messages, proposed here.

# Text Proposal

Modify as follows:

- 6.2.3.65.46   AAI-MSPG-GRP

To add or remove an HR-MS to or from a pagers group the HR-BS shall unicast an AAI-MSPG-GRP described in Table 106a:

Table 106a – AAI-MSPG-GRP message field description

| Field | Size (bits) | Value / Description | Condition |
| --- | --- | --- | --- |
| Purpose | 1 | 0b – remove from group, 1b – add to group |  |
| Pager group ID | TBD 8 |  |  |

- 6.2.3.65.47  AAI-MSPG-PG

Message AAI-MSPG-PG multicast to a group of paging HR-MS lists of paged HR-MS ID and the access resources to be used by them. The format is defined in Table 106b.

Table 106b – AAI-MSPG-PG message field description

| Field | Size (bits) | Value / Description | Condition |
| --- | --- | --- | --- |
| Pager group ID | TBD 8 |  | May be omitted if can be uniquely mapped multicast stream identifier, TBD |
| N_PI | TBD 6 | Indicates the number of distinct paging indicators |  |
| For (i=0; i<N_PI; i++) { |  |  | Support of WirelessMANOFDMA with FDM-based UL PUSC Zone |
| PI PID | 12 | Paging ID |  |
| Sub-frame offset of the RCH | 2 |  |  |
| Start RP code information of the RCH | 4 |  |  |
| Number of RP codes allocated | 6 |  |  |
| } |  |  |  |
| For (i=0; i<N_PI; i++) { |  |  | For AAI mode |
| PI PID | TBD 12 | Paging ID |  |
| Start RP code information of the S-RCH | 4 | Indicates the ks that is the parameter controlling the start root index of the RP codes (rs0). rs0 = 6 × ks + 1 The range of values is 0 ≤ ks ≤ 15 |  |
| Number of RP codes allocated | TBD 6 | Indicates number of codes allocated for the (group of) paged HR-MS |  |
| } |  |  |  |

Modify 6.12.3.2.1.1 as follows:

- After the ranging status has become Success, HR-BS shall allocate bandwidth in the uplink to allow the new HR-MS to transmit AAI-RNG-REQ message. The forwarding HR-MS shall recognize this allocation IE in the A-MAP and inform the new HR-MS accordingly (using a CDMA Allocation IE).

- The HR-BS may request the forwarding HR-MS to report any ranging attempt, whether they have crossed the response threshold or not, for management purposes. The HR-BS may request this information by sending an AAI-FN-RNG-FLU to an HR-MS that did not report any ranging. Ranging attempts that were received but didnt cross the reporting threshold are then reported. A null message is sent if no ranging attempts were detected.

The above FTN operation is illustrated in Error: Reference source not found. Note that after the new HR-MS has been associated as a forwarded HR-MS, the forwarding HR-MS periodically transmit Periodic NCI (P-NCI). The transmission of P-NCI is described in Error: Reference source not found.

Modify table 200 as follows:

- —The PI IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| N_PI | TBD 6 | Indicates the number of distinct paging indicators |
| If (Support of WirelessMANOFDMA with FDM-based UL PUSC Zone){ |  |  |
| For (i=0; i<N_PI; i++) { |  |  |
| PI PID | TBD 12 | Paging ID |
| Sub-frame offset of the RCH | 2 | Indicates the sub-frame offset OSF of the RCH allocation. The range of values is 0 ≤ OSF ≤ 3 |
| Start RP code information of the RCH | 4 | Indicates the kns, which is the parameter for start of the RP code group (rns0). rns0(kns) = 16 × kns + 1, kns = 0,1, ...,15 |
| Number of RP codes allocated | TBD 6 | Indicates number of codes allocated for the (group of) paged HR-MS |
| } |  |  |
| else { |  |  |
| For (i=0; i<N_PI; i++) { |  |  |
| PI PID | TBD 12 | Paging ID |
| Start RP code information of the S-RCH | 4 | Indicates the ks that is the parameter controlling the start root index of the RP codes (rs0). rs0 = 6 × ks + 1 The range of values is 0 ≤ ks ≤ 15 |
| Number of RP codes allocated | TBD 6 | Indicates number of codes allocated for the (group of) paged HR-MS |
| } |  |  |
| } |  |  |