| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Missing parameters for HR-MS paging |  |
| Date Submitted | 2012-03-13 |  |
| Source(s) | Eldad Zeira, Anh Tuan Hoang InterDigital & I2R | E-mail: eldad.zeira@interdigital.com |
| Re: | LB 37 |  |
| Abstract | The text proposal in this contribution provides some missing parameters for HR-MS paging |  |
| Purpose | To be discussed and accepted by GRIDMAN |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Missing parameters for HR-MS paging

# Summary

Some parameters are missing from related messages, proposed here.

# Text Proposal

Modify as follows:

- 6.3.2.3.98.27 MSPG-GRP

To add or remove an HR-MS to or from a pagers group the HR-BS shall unicast an MSPG-GRP described in Table 229a:

Table 229a —  MSPG-GRP message format

| Syntax | Size [bits] | Notes |
| --- | --- | --- |
| MSPG-GRP message format () { |  |  |
| Purpose | 1 | 0b – remove from group, 1b – add to group |
| Pager group ID | TBD 8 |  |
| Padding bits | variable | As required for octet boundary |
| } |  |  |

- 6.3.2.3.98.28 MSPG-PG

MSPG-PG multicast to a group of paging HR-MS lists of paged HR-MS ID and the access resources to be used by them. The format is defined in Table 229b.

Table 229b — MSPG-PG message format

| Syntax | Size [bits] | Notes |
| --- | --- | --- |
| MSPG-PG message format () { |  |  |
| Pager group ID | TBD 8 | May be omitted if can be uniquely mapped multicast stream identifier, TBD |
| N_PI | TBD 6 | Indicates the number of distinct paging indicators |
| Sub-channel offset for ranging | 5 | Counted from the beginning of FCZ assigned for transmissions by forwarded HR-MS |
| Ranging method | 1 | 0b0: Initial ranging over two symbols 0b1: BR/periodic ranging over one symbol |
| For (i=0; i<N_PI; i++) { |  |  |
| PI PID | TBD 12 |  |
| Initial ranging code | 5 | CDMA codes used for initial ranging as in table {PIM} |
| } |  |  |
| Padding bits | variable | As required for octet boundary |
| } |  |  |

Modify 16.3.2.1.1 as follows:

- After the ranging status has become Success, HR-BS shall allocate bandwidth in the uplink to allow the new HR-MS to transmit RNG-REQ message. The forwarding HR-MS shall recognize this allocation IE in the UL-MAP and inform the new HR-MS accordingly (using a CDMA Allocation IE).

- The HR-BS may request the forwarding HR-MS to report any ranging attempt, whether they have crossed the response threshold or not, for management purposes. The HR-BS may request this information by sending an FN-RNG-FLU to an HR-MS that did not report any ranging. Ranging attempts that were received but didnt cross the reporting threshold are then reported. A null message is sent if no ranging attempts were detected.

###### Modify PI message as follows

6.3.2.3.98.26 PI Message

The PI Message contains the ID of paged HR-MS or groups of HR-MS. How HR-MS or groups of HR-MS receive their paging ID is out of scope for this amendment. The contents and format of the PI IE is given in Table 229c. PI Message is transmitted by forwarding HR-MS using a broadcast CID.

Table 229c — PI Message Format

| Syntax | Size [bits] | Notes |
| --- | --- | --- |
| PI message format () { |  |  |
| N_PI | TBD 6 | Indicates the number of distinct paging indicators |
| Sub-channel offset for ranging | 5 | Counted from the beginning of FCZ assigned for transmissions by forwarded HR-MS |
| Ranging method | 1 | 0b0: Initial ranging over two symbols 0b1: BR/periodic ranging over one symbol |
| For (i=0; i<N_PI; i++) { |  |  |
| PI PID | TBD 12 | Paging ID |
| Initial ranging code | 5 | Indicates the CDMA codes used for initial ranging toward the forwarding HR-MS. 2 MSBs indicate index of starting code: 0b00: starting code index = 0 0b01: starting code index = 4 0b10: starting code index = 8 0b11: starting code index = 16 3 LSBs indicate number of codes used. The codes are contiguous from the starting code. |
| } |  |  |
| Padding bits | variable | As required for octet boundary |
| } |  |  |