| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Missing parameters for multicast feedback |  |
| Date Submitted | 2012-03-13 |  |
| Source(s) | Eldad Zeira, Anh Tuan Hoang InterDigital & I2R | E-mail: eldad.zeira@interdigital.com |
| Re: | LB 37 |  |
| Abstract | The text proposal in this contribution provides some missing parameters for flexible feedback channel for multicast |  |
| Purpose | To be discussed and accepted by GRIDMAN |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Missing parameters for multicast feedback

# Summary

Some parameters are missing from related messages, proposed here.

# Text Proposal

Remedy 1

Modify figure 520 as below:

Discovery

&

Registration

Connection Establishment

/

Configuration

REG

-

REQ

/

RSP

HR

-

MS

Receiving multicast control channel

Receiving multicast data

DSA

-

REQ

/

RSP

HR

-

BS

Multicast assignment MAP

Multicast burst

Transmitting feedback (if used)

Modify 16.9.2.2 as follows:

- 16.9.2.2 Feedback for multicast information

To ensure robust multicast and provide the network operator with specific or statistical information of its reception a feedback operation is defined between an HR-MS that is an addressee of a multicast transmission and its serving HR-BS or HR-RS.

The conditions for providing feedback are defined by the network per each multicast channel and include positive feedback only (logical ACK), negative feedback only (logical NAK) or both (logical ACK/NAK). It is expected that all intended recipients of a multicast channel obey the same rules but those can be changed by the network. UL resources for the feedback are also provided by the HR-BS. Feedback parameters may be unicast or multicast.

Feedback operation is supported by multicast addressees in connected as well as in idle states.

To set up feedback operation, the HR-BS shall include the following parameters in multicast setup DSA-REQ, DSA-RSP and DSC-REQ.

- Feedback request indicator FRI: indicates type of feedback, if any;

- Logical channel indicator: index of the logical channel for feedback associated with the service.

- Probability for sending feedback (NAK feedback only);

- RNG-ACK indicator: indicator if ACK for feedback is to be expected (ACK feedback only)

- Retransmissions and ramp-up of feedback (if RNG-ACK is used only)

If the above message is multicast then the HR-BS sends the HR-MS its logical channel assignment in a unicast message when setting up the multicast service.

The HR-BS may include the following in UCD:

N_SIZE_LOG: Indicates the The number of distinct sizes of logical channels, measured in terms of number of physical channels associated with them.

For each distinct size of logical channel:

- p, Log base 2 of the number of physical channel per logical channel

- N_of_Size number of logical channels of the size

N_frame:  The delay in frames between starting frame for the reception of multicast and the first frame of the feedback channel associated with it.

Sub-channel offset for ranging: Counted from the beginning of FCZ assigned for UL. TBD

Ranging method – initial or periodic; 0b0: Initial ranging over two symbols, 0b1: BR/periodic ranging over one symbol.

Initial ranging code, number of codes and spacing between codes: Indicates the CDMA codes used for initial ranging toward the forwarding HR-MS.

2 MSbs indicate index of starting code:

- 0b00: starting code index = 0

- 0b01: starting code index = 4

- 0b10: starting code index = 8

- 0b11: starting code index = 16

6LSbs indicate number of codes used.

Code spacing: The spacing between codes that are used for feedback

Remedy 2

Include TLV descriptors as follows:

### 11.13.a Feedback request indicator FRI

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].x | 1 | 2 LSBs represent: 00-no feedback, 01-ACK only, 10-NAK only, 11-reserved | DSA-REQ/DSA-RSP/DSC-REQ |

### 11.13.a+1 Logical channel indicator FBACK_LCI

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].(x+1) | 2 | 12LSB Indicate the index of the logical channel assigned to this multicast | DSA-REQ/DSA-RSP/DSC-REQ |

### 11.13.a+2 Probability indicator of sending ranging preamble, pi

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].(x+2) | 2 | 10 LSBs indicate the probability of sending the NAK if NAK is indicated, probability = 2-pi | DSA-REQ/DSA-RSP/DSC-REQ |

### 11.13.a+3 RNG-ACK parameters

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].(x+3) | 2 | First 4 LSBs: Delay in frames to retransmission, =0b0000 indicates no retransmissions Next 4 bits: Indicates maximum number of retransmissions if no RNG-ACK Next 4 bits: Indicates power difference in dB for retransmissions Last 4 bits: reserved. | DSA-REQ/DSA-RSP/DSC-REQ |

Add to table 675 the following

| Name | Type (1 byte) | Length | Value |
| --- | --- | --- | --- |
| N_SIZE_LOG | 225 | 1 | 3 LSB Indicate the number of distinct sizes of logical channels, measured in terms of number of physical channels associated with them |
| Physical_channel_count | 226 | 1 | 3 LSB represent the base-2 logarithm of the number of physical channels in the logical channel |
| N_OF_SIZE | 227 | 2 | 12 LSB represent the number of logical channels with a specific count of physical channels in Physical_channel_count |
| Frame_delay | 228 | 1 | 2 LSB represent Delay in frames between starting frame for the reception of multicast and the first frame of the feedback channel associated with it. 0b01-1 frame, 0b10-2 frames, 0b11- 3 frames and 0b00 - 4 frames (i.e. same frame in next superframe) |
| Ranging_method | 229 | 1 | 1 LSB indicates the ranging method; 0b0: Initial ranging over two symbols, 0b1: BR/periodic ranging over one symbol. |
| ranging_codes | 230 | 1 | 2 MSB indicate index of starting code: 0b00: starting code index = 0 0b01: starting code index = 4 0b10: starting code index = 8 0b11: starting code index = 16 Next 6b indicate number of code used Next 4b indicate the spacing between codes used |
| Subchannel_offset | 231 | 1 | 7 LSB indicate subchannel offset for ranging, counted from the beginning of FCZ assigned for UL |