| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Clean-up for RNG-REQ Message |  |
| Date Submitted | 2012-03-05 |  |
| Source(s) | Ping-Heng Kuo ITRI | E-mail: pinghengkuo@itri.org.tw |
| Re: | Call for contributions on the IEEE P802.16p/D3 |  |
| Abstract | The text in Section 6.3.2.3.5 requires some clean-up for consistency |  |
| Purpose | To be discussed and adopted by 802.16p WG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Clean-up for RNG-REQ Message

Ping-Heng Kuo

ITRI

# Introduction

The text in Section 6.3.2.3.5 requires some clean-up for consistency. In particular, the descriptions for the 8 bits of ranging purpose indication should be written in a consistent fashion.

- Text proposal for inclusion in IEEE 802.16p/D3

--------------------------------- Start of Proposed Text Part 1 ---------------------------------

6.3.2.3.5 RNG-REQ (ranging request) message

Change the paragraph as indicated

The following TLV parameter shall be included in the RNG-REQ message when the MS is attempting to perform reentry, HO, or location update or abnormal power down reporting:

Ranging Purpose Indication

The presence of this item in the message indicates the following MS action:

If Bit 0: Reentry indication. When this bit is set to 1, in combination with a serving BSID, it indicates that the MS is currently attempting to HO or reentry; or, in combination with a Paging Controller ID, indicates that the MS is attempting network reentry from idle mode to the BS.

If Bit 1: Location update indication. When this bit is set to 1, it indicates that the MS is initiating the idle mode location update process.

Bit 2: Seamless HO indication. When this bit is set to 1 in combination with other included information elements, it indicates the MS is initiating ranging as part of seamless HO procedure.

Bit 3: Ranging Request for Emergency Call Setup. When this bit is set to 1, it indicates MS action of Emergency Call Process.

Bit 4: MBS update. When this bit is set to 1, the MS is currently attempting to perform location update due to a need to update service flow management encodings for MBS flows.

Bits 5-7: Power down reporting indication. When these bits are set to 001, by an M2M device it indicates that an abnormal or involuntary power down has occurred.

Change the paragraph as indicated

The following TLV parameter shall be included in the RNG-REQ message when the MS is attempting to perform reentry or location update:

Paging Controller ID (see 11.1.8.2)

The Paging Controller ID is a logical network identifier for the serving BS or other network entity retaining MS service and operational information and/or administering paging activity for the MS while in idle mode. This parameter shall not be included when a RNG-REQ is transmitted by an M2M device in Localized Idle Mode.

Add the following texts at the end of the subclause 6.3.2.3.5 as indicated

The MIMO Feedback information TLV parameter may be included in a RNG-REQ message when the Matrix A or Matrix B is supported by a fixed M2M device:

MIMO Feedback information

This TLV includes the 1 bit Matrix indicator indicating the preferred STC/MIMO matrix and 4-bit DL Effective CINR as defined in Table 520.

The following parameter may be included in a RNG-REQ message when an M2M device performs ranging:

Ranging Retries

The number of ranging retries in the current ranging attempt.

The following TLV parameter may be included in a RNG-REQ message when M2MCID is updated in an M2M device.

M2MCID Update Acknowledgement Indicator

This TLV is used to acknowledge the receipt of an M2MCID update.

The following TLV parameter may be included in a RNG-REQ message when an M2M device does not have the information of new M2M Group Zone.

M2MCID update request

This TLV is used to request M2MCIDs update.

--------------------------------- End of Proposed Text Part 1 ---------------------------------