| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI-MTE-IND clean-up |  |
| Date Submitted | 2012-03-06 |  |
| Source(s) | Jeongki Kim, Giwon Park, Jinsam Kwak LG Electronics | Email: jeongki.kim@lge.com |
| Re: | Call for contributions for 802.16 WG Sponsor Ballot (IEEE 802.16.1b) |  |
| Abstract | This contribution proposes the text to clean up the AAI-MTE-IND message |  |
| Purpose | To be discussed and adopted for 802.16.1b amendment working document. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

AAI-MTE-IND clean-up

Jeongki Kim, Giwon Park and Jinsam Kwak

LG Electronics

# Introduction

At the last meeting, it was decided that AAI-MTE-IND and AAI-MGMC can be multicast or broadcast. The message including only one MGID will be multicast by using MGID and the messages including multiple MGIDs will be broadcast.

Therefore, AAI-MTE-IND message should be cleaned up similar to AAI-MGMC.

# Text proposal

---- Start of proposed text ----

[Remedy #1: Modify the Table 106a and subclause 6.2.3.64 as follows, Page 40, line 15:]

6.2.3.64 AAI-MTE-IND (Multicast transmission end indication) message

The ABS shall multicast (using MGID) send AAI-MTE-IND message to M2M devices to indicate the end of multicast transmission by using either broadcast manner or multicast manner. In case of broadcast manner, the PHY burst carrying the AAI-MTE-IND message shall be sent by the Broadcast Assignment A-MAP IE with function index = 0b11 and 16 bit CRC masked with masking prefix = 0b0, message type indicator = 0b010, and masking code = 4094. If the Multicast SA is established for the group of these M2M devices then the AAI-MTE-IND shall be encrypted using the established Multicast SA (see 6.2.5.5 for details on encryption method). The PHY burst with the MAC PDU carrying the AAI-MTE-IND message is indicated by the M2M group assignment A-MAP IE and the FID in the MAC PDU carrying the unencrypted or encrypted AAI-MTE-IND message is set to zero or one respectively. In case AAI-MTE-IND message is encrypted the multicast SA corresponding to the M2M device group identified by the MGID in M2M group assignment A-MAP IE is used to decrypt the AAI-MTE-IND message. If an M2M device in idle mode receives the AAI-MTE-IND message, the M2M device may enter the paging unavailable interval as specified in 6.2.18.2.

Table 106a—AAI-MTE-IND Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| For (i=0; i<Num_MGID;i++) { |  | Number of MGID to be terminated [1..4]. |  |
| MGID | 12 | MGID related to the terminated multicast traffic |  |
| M2M Group Zone Index | 2 | M2M_Group_Zone_Index of the corresponding M2M_GROUP_ZONE_ID that the MGID belongs to. |  |
| } |  |  |  |

---- End of proposed text ----