| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Update for MGMC Format |  |
| Date Submitted | 2012-3-6 |  |
| Source(s) | Chia-Lung Tsai, Wei-Chieh Huang, Ping-Heng Kuo, Jen-Yuan Hsu ITRI | E-mail: chickout2@itri.org.tw |
| Re: | Call for contributions on the IEEE P802.16p/D3 |  |
| Abstract | The table for MGMC Format requires updates on the number of M2MCID |  |
| Purpose | To be discussed and adopted during Session 78 |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Update for MGMC Format

Chia-Lung Tsai, Wei-Chieh Huang, Ping-Heng Kuo, Jen-Yuan Hsu

ITRI

# Introduction

Due to the adoption of contribution 0013 in the previous meeting, the number of M2MCID is defined by 4bits. However, Table 229b has not been updated accordingly. Therefore, this contribution suggests such update in Table 229b.

Proposed text

Black text: text in the current 802.16p AWD

Red Text: delete text

Blue text: new text

---------------------------------------------------------Start of the Text #1--------------------------------------------------------

[Note to Editor – Modify the text of section 6.3.2.3.99, page 14, line 34 in the current 802.16p AWD as follows]

Table 229b-MGMC message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MGMC_Message_Format(){ |  |  |
| Management Message Type = 111 | 8 |  |
| Num_M2MCID | TBD4 | Number of M2MCID |
| For (i=1; i<= Num_M2MCID;i++){ |  |  |
| M2MCID | 16 |  |
| Action Code | 2 | Use to identify the purpose if this message 0b00: re-assignment of M2MCID value 0b01-0b11: Reserved |
| If(Action Code == 0x00){ |  |  |
| New M2MCID | 16 | New M2MCID value to be assigned |
| { |  |  |
| { |  |  |
| { |  |  |

# References

[1] IEEE 802.16p/D3 WirelessMAN-Advanced Enhancements to Support Machine-to-Machine Applications

[2] IEEE 802.16-12-0013-05-000p