| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Changes for MS Forwarding to Network in 802.16n |  |
| Date Submitted | 2012-03-07 |  |
| Source(s) | Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow, Shoukang Zheng, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | In response to Letter Ballot #37 on P802.16n/D1 |  |
| Abstract |  |  |
| Purpose |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Changes for MS Forwarding to Network in 802.16n

Anh Tuan Hoang, Wai Leong Yeow, Haiguang Wang, Jaya Shankar,

Shoukang Zheng, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

Introduction

This contribution proposes modifications for the existing specifications for HR-MS Forwarding to Network.

# References

- [1] IEEE P802.16n/D1, Feb. 2012.

Proposed Text Modifications to IEEE P802.16n/D1

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in YELLOW-HIGHLIGHTED RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in YELLOW-HIGHLIGHTED BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

{Remedy 1: Modify section 16.3.2.1.2 as indicated}

        - Connection management MPDU transmissions involving forwarding HR-MS

Management and transport connections between a forwarded HR-MS and serving HR-BS/RS are defined in the same way as specified in 6.3.1.1. Each forwarding HR-MS keeps track of all basic/primary/secondary and transport CIDs allocated to all of its associated forwarded HR-MSs.

Serving HR-BS/RS schedules resource for the FTN transmissions to/from forwarded HR-MS using FTN Assignment IE, which is an extended UL-MAP IE (UIUC = 15) with Extended UIUC = 0xC (Table 1). The detailed operation depends on whether the forwarded HR-MS is inside or outside of HR-BS/RS coverage and is described in  and 16.3.2.1.2.2.

- Table 1 — FTN Assignment IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| FTN Allocation IE () { | — | — |
| Extended UIUC | 4 | 0xC |
| Length | 4 | Set to 0x3 |
| UIUC | 4 | Value is from 1 to 10, for different burst profiles |
| Duration | 10 | In OFDMA slots |
| Repetition coding indication | 2 | 0b00: No repetition coding 0b01: Repetition coding of 2 used 0b10: Repetition coding of 4 used 0b11: Repetition coding of 6 used. |
| Transmission Indicator | 2 | 0b00: For HR-BS to transmit to forwarding HR-MS 0b01: For forwarding HR-MS to transmit to HR-BS 0b10: For forwarding HR-MS to transmit to forwarded HR-MS 0b11: For forwarded HR-MS to transmit to forwarding HR-MS |
| Frame Offset | 2 | Number of frames, counting from the next frame, that this assignment is applied to. |
| Reserved | 4 |  |
| } |  |  |

{Remedy 2: Modify Section 16.3.2.1.2.1as indicated}

- 16.3.2.1.2.1Forwarding of UL traffic for an inside-of-coverage forwarded HR-MS

The procedure for forwarding of UL traffic for an inside-of-coverage forwarded HR-MS consists of the following steps:

- The serving HR-BS/RS assigns an UL transmission resource for the forwarded HR-MS through an UL-MAP IE. The forwarded HR-MS, upon receiving such an UL-MAP IE, transmits its MPDU as per normal, i.e., just like transmitting directly to the serving HR-BS/RS.

- As the forwarding HR-MS keeps track of the basic CIDs of all of its forwarded HR-MS, it is aware of the UL resource assignment from the serving HR-BS/RS and shall attempt to receive, if it is not scheduled to transmit uplink traffic on the same frame.

- If the serving HR-BS/RS wants the forwarding HR-MS to forward the received MPDU from forwarded HR-MS, it shall assign UL resource to the forwarding HR-MS in one of the subsequent frames, using an FTN Assignment IE. The FTN Assignment IE is transmitted with the basic CID of the forwarded HR-MS and with the field “Transmission Indicator” set to 0b01.

- The forwarding HR-MS shall buffer the MPDU received from the forwarded HR-MS for at most [TBD] FTN_Buffering_Duration frames, where FTN_Buffering_Duration is a parameter exchanged in the SBC-REQ/RSP messages. If during those frames, the corresponding FTN Assignment IE is not received from serving HR-BS/RS, the forwarding HR-MS shall discard the received MPDU.

{ Remedy 3: Modify Section 16.3.2.1.2.2as indicated }

          - Forwarding of traffic for an outside-of-coverage forwarded HR-MS

The procedure for forwarding of DL traffic for an outside-of-coverage forwarded HR-MS consists of the following steps:

- The serving HR-BS/RS sends a DL-MAP IE with the basic CID of the forwarded HR-MS. As the forwarding HR-MS keeps track of the basic CIDs of all of its forwarded HR-MS, it is aware of the DL resource assignment and shall attempt to receive.

- In one of the subsequent frames, the serving HR-BS/RS assigns an UL resource to forwarding HR-MS to forward the received MPDU toward the forwarded HR-MS, using an FTN Assignment IE. The FTN Assignment IE is transmitted with the basic CID of the forwarded HR-MS and with the field “Transmission Indicator” set to 0b10. The forwarding HR-MS shall buffer the MPDU received from the forwarded HR-BS for at most FTN_Buffering_Duration frames, where FTN_Buffering_Duration is a parameter exchanged in the SBC-REQ/RSP messages. If during those frames, the corresponding FTN Assignment IE is not received from serving HR-BS/RS, the forwarding HR-MS shall discard the received MPDU.

- Upon receiving the FTN Assignment IE, the forwarding HR-MS conveys the assigned resource to the forwarded HR-MS through an F-MAP IE in the coming F-MAP.

- The forwarding HR-MS then proceeds to transmit the MPDU received from the serving HR-BS/RS toward the forwarded HR-MS.

The procedure for forwarding of UL traffic for an outside-of-coverage forwarded HR-MS consists of the following steps:

- The serving HR-BS/RS assigns UL resource for the forwarded HR-MS to transmit its MPDU toward the forwarding HR-MS using an FTN Assignment IE, using the basic CID of the forwarded HR-MS and with the field “Transmission Indicator” set to 0b11.

- Upon receiving the FTN Assignment IE, the forwarding HR-MS conveys the assigned UL resource to the forwarded HR-MS through an F-MAP IE in the coming F-MAP. The forwarding HR-MS subsequently attempts to receive the MPDU transmitted by the forwarded HR-MS in the assigned UL resource.

- In one of the subsequent frames, the serving HR-BS/RS sends a UL-MAP IE with the basic CID of the forwarded HR-MS. The forwarding HR-MS is aware of the UL resource assignment and uses that to forward the MPDU received from the forwarded HR-MS toward the serving HR-BS/RS.

- The forwarding HR-MS shall buffer the MPDU received from the forwarded HR-MS for at most [TBD] FTN_Buffering_Duration frames, where FTN_Buffering_Duration is a parameter exchanged in the SBC-REQ/RSP messages. If during those frames, the corresponding FTN Assignment IE UL-MAP IE with the basic CID of the forwarded HR-MS is not received from the serving HR-BS/RS, the forwarding HR-MS shall discard the received MPDU.

{ Remedy 4: Modify Section 16.3.2.2.1.1 as indicated }

          - Forwarding zone prefix

M-FCH carries Forwarding zone prefix information. The structure of Forwarding zone prefix is similar to the DL frame prefix transmitted by serving HR-BS, with a the 1-bit field “Forwarding_HR-MS_Indicator” set to 0b1 to let outside-of-coverage HR-MS know that the transmission is by a forwarding HR-MS. The content of Forwarding zone prefix is described in Error: Reference source not found for FFT sizes of 2048, 1024, and 512, and in Error: Reference source not found for FFT size of 128.

{Remedy 5: Insert the following row at the end of Table 695 -  SBC-REQ/RSP management message encodings (OFDMA PHY-specific)}

| Type | Parameter |
| --- | --- |
| 206 | FTN_Buffering_Duration |

{Remedy 6: Insert the following new subclause at the end of Section 11.8}

11.8.xx OFDMA FTN Buffering Capability

The TLV FTN_Buffering_Duration is defined to allow a HR-MS acting as forwarding HR-MS and its serving HR-BS to agree on the maximum duration that the HR-MS shall buffer the MPDU to/from a forwarded HR-MS.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 227 | 1 | 3 LSBs indicate the value of FTN_Buffering_Duration, in units of frames | SBC-REQ (see 6.3.2.3.23) SBC-RSP (see 6.3.2.3.24) |

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.