| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Changes for BS Dual-mode and Related Operations in 802.16.1a |  |
| Date Submitted | 2012-03-07 |  |
| Source(s) | Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow, Shoukang Zheng, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | In response to Letter Ballot #38 on P802.161a/D1 |  |
| Abstract |  |  |
| Purpose |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Changes for BS Dual-mode and Related Operations in 802.16.1a

Anh Tuan Hoang, Wai Leong Yeow, Haiguang Wang, Jaya Shankar,

Shoukang Zheng, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

Introduction

This contribution proposes modifications for the existing specifications for HR-BS dual-mode operation and related HR-RS operation.

# References

- [1] IEEE P802.16.1a/D1, WirelessMAN-Advanced Air Interface for Broadband Access Systems – Draft Amendment: Higher Reliability Networks, Feb. 2012.

Proposed text changes for the IEEE P802.16.1a/D1

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in YELLOW-HIGHLIGHTED RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in YELLOW-HIGHLIGHTED BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

{Remedy 1: Modify section 6.12.1.1.2.2.1 as indicated}

            - When R-TTI = 0

When R-TTI = 0, the affected HR-BS shall keep its original PHY-layer configuration, including IDCell, frame configuration, and FFR pattern. In addition, the affected HR-BS shall set the AAI_Relay_zone_AMS_allocation_indicator field in AAI-SCD and AAI-ARS-CONFIG-CMD messages to 0b0. The operation of the affected/dual-role HR-BS can be described as follows.

In BS Operation Mode:

- The affected/dual-role HR-BS shall only communicate with its subordinate AMS/HR-MS/HR-RS and shall not be available to receive from or transmit to its serving HR-BS.

- The manner in which the affected/dual-role HR-BS control and communicate with its subordinate AMS/HR-MSs/HR-RSs shall be the same as that of a normal HR-BS. The serving HR-BS is not expected to know the specific configuration of the dual-role HR-BS during BS Operation. When the affected/dual-role HR-BS transmits to or receives from its subordinate AMS/HR-MS/HR-RS during BS Operation, it does so independently to the serving HR-BS.

- The affected/dual-role HR-BS transmits AAI-ARS-CONFIG-CMD control message to tell the subordinate HR-RSs when the HR-BS will switch to RS Operation.

In RS Operation Mode:

- The affected/dual-role HR-BS shall communicate with the serving HR-BS and with the subordinate AMS/HR-MS. It may or may not communicate with its subordinate HR-RS during this mode of operation. The frame structure of the affected HR-BS is divided into DL Access zone, DL Relay zone, UL Access zone, and UL Relay zone. Note that as R-TTI = 0, no time gap need to be inserted into the last OFDM symbol of the last subframe in the DL Access zone.

- As the affected HR-BS still transmits the same SA-Preamble, the subordinate AMS/HR-MS are oblivious to the mode change of the affected HR-BS. The affected HR-BS continues to transmit to its subordinate AMS/HR-MS in the DL Access zone, and receive from its subordinate AMS/HR-MS in the UL Access zone.

- The affected/dual-role HR-BS receives from and transmits to its serving HR-BS during the DL Relay zone and UL Relay zone, respectively. The PHY-layer configuration for DL/UL Relay zones shall be sent by the serving HR-BS toward the affected HR-BS in the AAI-ARS-CONFIG-CMD message.

- The affected/dual-role HR-BS can communicate with its subordinate HR-RSs in the following ways:

- The affected/dual-role HR-BS can instruct its subordinate HR-RSs to transmit UL data during the DL Relay zone, i.e., when the affected/dual-role HR-BS also receives from the serving HR-BS. While doing so, the affected/dual-role HR-BS shall, through AAI-ARS-CONFIG-CMD message,  instruct the transmitting HR-RSs to use the same PHY-layer configuration as used in the DL Relay zone of the serving HR-BS. More detailed operation is described in 6.12.5.2.

- The affected/dual-role HR-BS can instruct its subordinate HR-RSs to receive DL messages during the UL Relay zone, i.e., when the affected/dual-role HR-BS also transmits to the serving HR-BS. While doing so, the HR-BS shall, through AAI-ARS-CONFIG-CMD message,  instruct the transmitting HR-RSs to use the same PHY-layer configuration as used in the UL Relay zone of the serving HR-BS. Furthermore, if an R-RTI = Ts is inserted in the first OFDMA symbol of the first subframe of the UL Relay zone, the dual-role HR-BS shall let its subordinate HR-RSs be aware of this insertion. More detailed operation is described in 6.12.5.2.

{Remedy 2: Modify Section 6.12.5.2.1 as indicated}

        - Resource assignment and HARQ operation

When the dual-role HR-BS assumes BS Operation mode, resource assignment through A-MAP and HARQ timing/operation follow what specified in 6.6. All resource assignments made in BS Operation mode become inactive when the dual-role HR-BS switches to RS Operation.

If the dual-role HR-BS does not transmit to/receive from its subordinate HR-RSs during its RS Operation periods, as indicated when the field “Supporting_Subordinate_HR-RS in_RS_Operation_Indicator” in the AAI-ARS-CONFIG-CMD message is set to 0b0, no resource assignment shall be made to HR-RS during such RS Operation periods. The dual-role HR-BS and subordinate HR-RS may or may not preserve the contexts of the HARQ channels that were on-going during the preceding BS Operation period. This is indicated by the field “Preserving_HARQ_context” in the AAI-ARS-CONFIG-CMD message.

If the dual-role HR-BS continues to receive from/transmit to its subordinate HR-RSs during its RS Operation periods, as indicated when the field “Supporting_Subordinate_HR-RS in_RS_Operation_Indicator” in the AAI-ARS-CONFIG-CMD message is set to 0b1, resource assignments shall be made using DL RS-RS Assignment A-MAP IE/UL RS-RS Assignment A-MAP IE for the DL Relay zone/UL Relay zone of the dual-role HR-BS. These assignment IEs are specified in Error: Reference source not found and Error: Reference source not found.

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.