| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Changes for MS Forwarding to Network in 802.16.1a |  |
| Date Submitted | 2012-03-07 |  |
| Source(s) | Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow, Shoukang Zheng, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | In response to Letter Ballot #38 on P802.161a/D1 |  |
| Abstract |  |  |
| Purpose |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Changes for MS Forwarding to Network in 802.16.1a

Anh Tuan Hoang, Wai Leong Yeow, Haiguang Wang, Jaya Shankar,

Shoukang Zheng, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

Introduction

This contribution proposes modifications for the existing specifications for HR-MS Forwarding to Network.

- References

- [1] IEEE P802.16.1a/D1, WirelessMAN-Advanced Air Interface for Broadband Access Systems – Draft Amendment: Higher Reliability Networks, Feb. 2012.

Proposed text changes for the IEEE P802.16.1a/D1

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in YELLOW-HIGHLIGHTED RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in YELLOW-HIGHLIGHTED BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

{Remedy 1: Modify Table 106qq, AAI-FN-CONFIG-CMD message, as indicated}

Table 106a – AAI-FN-CONFIG-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Superframe Number Action (nstart) | 4 | LSBs of the superframe number in which the coverage-extending cycle should be started. | Mandatory |
| IDCell | 10 | IDCell (SA-Preamble index Idx and subcarrier set index n) to be used by the scheduled HR-MS (If Two-Phase Discovery is used, the HR-BS should assign pre-access SA-Preamble to groups of HR-MS based on their service characteristics.) | Mandatory |
| Number of Preamble-only Superframes (mprep) | 4 | Indicates the number of superframes (starting from superframe with number nstart) in which the scheduled HR-MS should broadcast PA/SA-Preambles. No NCI shall be transmitted during these superframes. | Mandatory |
| Number of Superframes with I/S-NCI (mnci ) | 2 | Indicates the number of superframes (starting from superframe with number nstart + mprep) that the scheduled HR-MS transmits I/S-NCI. | Mandatory |
| Subframe index for S-NCI | 3 | The 2 LSB bits indicate the index of the UL subframe in which S-NCI is transmitted. The MSB bit indicates the frame in which S-NCI is transmitted. MSB bit == 0 indicate the 1st frame while MSB bit = 1 indicate the 3rd frame. | Mandatory |
| LRU starting index for S-NCI | 6 | Indicates the starting index of the LRUs allocated for S-NCI | Mandatory |
| Frame containing the ranging opportunity | 1 | 0b0: The ranging opportunity is located at 2nd frame within a superframe 0b1: The ranging opportunity is located at 4th frame within a superframe | Mandatory |
| Number of Ranging Opportunities (mrng) | 2 | The number of ranging opportunities that are allocated after each S-NCI | Mandatory |
| If (Support of WirelessMANOFDMA with FDM-based UL PUSC Zone){ |  |  |  |
| Subframe offset of the RCH | 2 | Indicates the subframe offset (OSF) of the RCH allocation. The range of values is 0 ≤ OSF ≤ 3 |  |
| Start RP code information of the RCH | 4 | Indicates the kns, which is the parameter for start of the RP code group (rns0). rns0(kns) = 16 × kns + 1, kns = 0,1, ...,15 |  |
| Number of RP codes allocated for coverage extension ranging | 2 | Indicates the number of RP codes and the particular codes that can be used for ranging with the forwarding HR-MS. |  |
| }else{ |  |  |  |
| Subframe offset of the S-RCH | 2 | Indicates the subframe offset (OSF) of the S-RCH allocation |  |
| Start RP code information of the S-RCH |  | Indicates the ks that is the parameter controlling the start root index of the RP codes (rs0). rs0 = 6 × ks + 1 The range of values is 0 ≤ ks ≤ 15 |  |
| Transmission timing offset of SRCH | 3 | Indicates NRTO, which is the parameter used for the calculation of the sample number, TRTO, which is applied to advance the ranging signal transmission timing relative to the defined uplink transmission timing point based on the frame structure from AMS perspective when AMS conducts initial or handover ranging in a femtocell. TRTO = floor(NRTO × (Tg – 2) × FS)(samples) where NRTO = min( RTD ⁄ (Tg–2) , 7), and RTD is the round trip delay from the femto-ABS to the overlay macro-ABS. The range of values is 0 ≤ NRTO ≤ 7. |  |
| } |  |  |  |
| HR-MS Preamble Timing Advance (tadv) | [TBD] 11 | Timing advanced that should be used by scheduled HR-MS when transmitting PA/SA-Preamble, relative to the beginning of each frame as seen by the scheduled HR-MS. Value in units of 0.1 μs. | Optional |
| HR-MS EIRP | 5 | Unsigned integer from 1 to 64 in units of 1 dBm, where 0b00000 = 0dBm and 0b11111 = 31dBm | Mandatory |
| HR-MS to HR-MS Feedback Resource Index | 11 | 512 FFT size: 0 in first2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size. | Optional |
| HR-MS to HR-BS Report Resource Index | 11 | 512 FFT size: 0 in first2 MSB bits + 9 bits for resource index 1024 FFT size: 11 bits for resource index 2048 FFT size: 11 bits for resource index Resource index includes location and allocation size. | Optional |
| If(Two-Phase Discovery){ |  |  |  |
| Threshold | [TBD] 4 | Indicates the SINR threshold for the ranging signal above which the forwarding HR-MS should report to serving HR-BS/RS for instruction whether to proceed with the 2nd phase. The 4 bit value from 0b0000 to 0b1111 represent values among {–9, –8.5, –8, –7.5, –7, –6.5,–6, –5.5, –5, –4.5, –4, –3.5, –3, –2.5,–2, –1.5} dB |  |
| If(Post-access parameters to be pre-assigned){ |  |  |  |
| Post IDCell | 10 | IDCell (SA-Preamble index Idx and subcarrier set index n) to be used by the scheduled HR-MS after detecting ranging signal above a specified threshold. (The HR-BS should assign post-access SA-Preamble such that they will be unique within physical proximity.) |  |
| Number of Superframes with NCI (mnci ) | 2 | Indicates the number of superframes (starting from superframe with number nstart + mprep) that the scheduled HR-MS transmits NCI. | Mandatory |
| Start RP code information of the S-RCH | 4 |  |  |
| Number of Ranging Opportunities (mrng) | 2 | Indicates the number of ranging opportunities given to outside-of-coverage HR-MS | Mandatory |
| Subframe offset of Ranging Channel | 2 | Indicates the subframe offset (OSF) of the RCH allocation. The range of values is 0 ≤ OSF ≤ 3 | Mandatory |
| } |  |  |  |
| } |  |  |  |

{Remedy 2: Modify Section 6.12.3.2.2.1 as indicated}

          - Frame structure for HR-MS FTN

A forwarding HR-MS and an inside-of-coverage forwarded HR-MS fully follow the superframe and frame configurations set by the serving HR-BS/RS, as described in 6.3.3, 6.6.6, and 6.6.4. A forwarded HR-MS that is out-of-coverage of the serving HR-BS/RS obtains essential superframe and frame configurations through A-Preambles and control channels from its corresponding forwarding HR-MS. These forwarding control channels include I-NCI, S-NCI, Forwarding MAP (F-MAP).

From the point of view of forwarding and forwarded HR-MS, the frames within each superframe are classified into forwarding and reversed frames. The forwarding HR-MS transmits to its forwarded HR-MS in uplink subframes of forwarding frames. The forwarding HR-MS receives from its forwarded HR-MS in uplink subframes of reversed frames. The 1st and 3rd frames of each superframe are classified as forwarding frames while the 2nd and 4th frames of each superframe are classified as reversed frames. The restriction in transmitting/receiving modes of the forwarding and forwarded HR-MS does not apply to the transmission and reception of PA/SA-Preambles, which is described in .

Resources for HR-MS Direct Communications and HR-MS Forwarding to Network shall be scheduled by the serving HR-BS/RS. Serving HR-BS/RS can schedule direct communication in an on-demand and dynamic manner, and can multiplex this with transmissions between HR-MS and HR-BS / HR-RS.

To optimize the signaling and switching cost and improve QoS provisioning to HR-MS direct communication, serving HR-BS / HR-RS can schedule resource for DC/FTN zone for multiplexing DC/FTN transmissions. An HR-MS DC / FTN Zone is an area of continuous OFDMA resources in time and logical subchannels or resource units. The size and location of DC/FTN zone is dynamically or semi-stationary determined by the serving HR-BS.

{Remedy 2: Modify Section 6.12.3.2.2.2.1 as indicated}

- 6.12.3.2.2.2.1 Transmission of A-Preambles

As described in Error: Reference source not found, under the instruction of the serving HR-BS/RS, an HR-MS transmits PA/SA-Preambles, together with I/S-NCI, to help an out-of-coverage HR-MS to carry out network entry and get associated with the serving HR-BS/RS. After the out-of-coverage HR-MS has successfully completed network entry, it becomes a forwarded HR-MS that is attached to the helping/forwarding HR-MS.

A forwarding HR-MS shall periodically transmit PA/SA-Preambles as long as there is at least one out-of-coverage forwarded HR-MS attached to it. PA-Preamble shall be transmitted in the first OFDMA symbol of the 2nd frame of a superframe, while SA-Preamble shall be transmitted in the first OFDMA symbol of the 3rd frame of a superframe. The periodicity of PA/SA-Preamble transmission can be different from that applied during the network entry of an out-of-coverage HR-MS. The periodicity of PA/SA-Preamble transmission is informed to the forwarded HR-MS at the end of the network entry process and is updated through the field “Preamble Periodicity” in the S-NCI P-NCI.

{Remedy 3: Modify Table 201 – I-NCI IE format, as indicated}

- Table 201—I-NCI IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| BS IDcell | 10 |  |
| Frame Configuration Index | 6 | The mapping between value of this index and frame configuration is listed in Table 149, Table 150, and Table 151 |
| If (WirelessMAN-OFDMA with FDM-based UL PUSC Zone Support){ |  | True if Frame configuration index is equal to – 5, 7, 9, 11, 13, 15, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29 or 30 for 5/10 MHz channel bandwidth according to Table 149; – 4, 6, 8 or 10 for 8.75 MHz channel bandwidth according to Table 150; – 3 or 5 (with CP=1/8) for 7 MHz channel bandwidth according to Table 151. False if Frame configuration index is something else |
| UL_Permbase | 7 | Indicates UL_Permbase used in WirelessMAN-OFDMA system with FDM-based UL PUSC Zone. |
| Reserved | [TBD] 15/10/6 | For 2048 FFT size, 15 bits For 1024 FFT size, 10 bits For 512 FFT size, 6 bits |
| }else{ |  |  |
| USAC | 5/4/3 | Indicates the number of subbands KSB as defined in Table 246 to Table 248 in 6.3.7.2.1 For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |
| UFPC | 4/3/3 | Indicate the frequency partition configuration as defined in Table 249 to Table 251 in 6.3.7.2.3 For 2048 FFT size, 4 bits For 1024 FFT size, 3 bits For 512 FFT size, 3 bits |
| UFPSC | 3/2/1 | Indicate the number of subbands allocated to FPi (i > 0) in 6.3.7.2.3 For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bits |
| UCASSB0 | 5/4/3 | Indicates the number of subband-based CRUs in FP0 in 6.3.7.3.1 For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |
| UCASMB0 | 5/4/3 | Indicates the number of miniband-based CRUs in FP0 in 6.3.7.3.1 For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |
| } |  |  |
| Subframe index for S-NCI | 3 | The 2 LSB bits indicate the index of the UL subframe in which S-NCI is transmitted. The MSB bit indicates the frame in which S-NCI is transmitted. MSB bit == 0 indicate the 1st frame while MSB bit = 1 indicate the 3rd frame. |
| LRU starting index for S-NCI | 6 | Indicates the starting index of the LRUs allocated for S-NCI |
| P-NCI Location Indicator | 1 | 0b0: P-NCI is transmitted in the 1st frame of a superframe 0b1: P-NCI is transmitted in the 3rd frame of a superframe |
| P-NCI Periodicity Indicator | 2 | 0b00: P-NCI is transmitted every superframe 0b01: P-NCI is transmitted every 2 superframes 0b10: P-NCI is transmitted every 4 superframes 0b11: P-NCI is transmitted every 8 superframes |

{Remedy 4: Modify Section 6.12.3.2.2.2.4, P-NCI, as indicated}

- 6.12.3.2.2.2.4 P-NCI

The P-NCI shall be transmitted in NP-NCI contiguous DLRUs of an uplink subframe. A forwarding HR-MS shall periodically transmit P-NCI as long as there is at least one out-of-coverage forwarded HR-MS attached to it. During this operation, S-NCI P-NCI can be transmitted in either 1st or 3rd frame of a superframe. The frame-location and periodicity of P-NCI transmission shall be made known to the forwarded HR-MS through a control message [TBD] I-NCI.

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.