| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Missing Parameter in Multicast Security Update in 802.16.1a |  |
| Date Submitted | 2012-03-06 |  |
| Source(s) | Wai-Leong Yeow, Joseph Teo Chee Ming, Jaya Shankar, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 | Voice: +65 6408-2254 E-mail: wlyeow@i2r.a-star.edu.sg |
| Re: | In response to Letter Ballot #38 on P802.161a/D1 |  |
| Abstract |  |  |
| Purpose |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Missing Parameter in Multicast Security Update in 802.16.1a

Wai-Leong Yeow, Joseph Chee Ming Teo,

Jaya Shankar, Hoang Anh Tuan, Wang Haiguang, Zheng Shoukang

Institute for Infocomm Research (I2R)

Introduction

This contribution proposes modifications to address a missing parameter in Multicast Security Update during location update or network reentry for HO. MC_Nonce is needed in order to derive subsequent keys including MTEK. It should be present in the AAI-RNG-RSP message.

- References

- [1] IEEE P802.16.1a/D1, WirelessMAN-Advanced Air Interface for Broadband Access Systems – Draft Amendment: Higher Reliability Networks, Feb. 2012.

Proposed text changes for the IEEE P802.16.1a/D1

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in YELLOW-HIGHLIGHTED RED color: the removal of existing 802.16.1a Amendment Draft Standard Text

The text in YELLOW-HIGHLIGHTED BLUE color: the new text added to the 802.16.1a Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

{Remedy 1: Change Table 28 in section 6.2.3.2 as indicated: }

Table 28 —AAI-RNG-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| If (Location Update Response == 0x0) { |  |  |  |
| … | … | … | … |
| Multicast Group ID | 12 | Multicast Group ID to update METKMTEK or MC_Nonce | Shall be present if needed to update MTEK or MC_Nonce in HR-network |
| FID | 4 | FID to update MTEK | Shall be present if needed to update MTEK or MC_Nonce in HR-network |
| MC_Nonce | 128 | The key used to derive the MCMAC-MTEK Prekey | Shall be present if needed to update MTEK or MC_Nonce in HR-network |
| COUNTER_MTEK | 16 | COUNTER_MTEK used for deriving current MTEK | Shall be present if needed to update MTEK or MC_Nonce in HR-network |
| MEKS | 2 | Encryption key sequence number for current MTEK | Shall be present if needed to update MTEK or MC_Nonce in HR-network |
| }//end of If (Location Update Response == 0x0 ) |  |  |  |
| … | … | … | … |
| If (it is under network reentry for HO){ |  |  |  |
| … | … | … | … |
| New Multicast Group ID | 12 |  |  |
| New FID | 4 |  |  |
| } |  |  |  |
| Multicast Group ID | 12 | Multicast Group ID to update METKMTEK or MC_Nonce | Shall be present if needed to update MTEK or MC_Nonce in HR-network |
| FID | 4 | FID to update MTEK | Shall be present if needed to update MTEK or MC_Nonce in HR-network |
| MC_Nonce | 128 | The key used to derive the MCMAC-MTEK Prekey | Shall be present if needed to update MTEK or MC_Nonce in HR-network |
| COUNTER_MTEK | 16 | COUNTER_MTEK used for deriving current MTEK | Shall be present if needed to update MTEK or MC_Nonce in HR-network |
| MEKS | 2 | Encryption key sequence number for current MTEK | Shall be present if needed to update MTEK or MC_Nonce in HR-network |
| }//end of If (it is under network reentry for HO) |  |  |  |
| … | … | … | … |

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.