| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up to AAI-PAG-ADV message |  |
| Date Submitted | 2012-03-09 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao ITRI | Voice: E-mail: mhtao@itri.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | sb010b |  |
| Abstract | This contribution proposes changes to the Section 6.2.3.23 of IEEE P802.16.1b/D2 |  |
| Purpose | To be discussed and adopted by TGp |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Clean-up to AAI-PAG-ADV message

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

# Introduction

This contribution tries to clarify some ambiguous sentences in the section 6.2.3.23.

# Text Proposal

Adopt the following text modifications to IEEE P802.16.1b/D2:

--------------------------  Begin Text Proposal   -------------------------------

[Remedy #1: Insert the blue text and delete the red text from page 24, line 14]

Table 49—AAI-PAG-ADV message field description

| Field | Size (bits) | Value | Condition |
| --- | --- | --- | --- |
| Initial ranging backoff start | 4 | Indicate the initial backoff window size for M2M devices. This parameter is applied for all M2M devices that are instructed to perform network reentry, uplink report, or location update by this message. | Shall be included when the ABS pages the fixed M2M devices in localized idle mode. |

[Remedy #2: Insert the blue text and delete the red text from page 28, line 6]

Table 49—AAI-PAG-ADV message field description

| Field | Size (bits) | Value | Condition |
| --- | --- | --- | --- |
| Multicast transmission start time (MTST) | 8 | Least significant 8 bits of the frame number in which the ABS starts sending DL multicast data. | Shall be present when the ABS is aware of when to start sending DL multicast data the MTST needs to be included in this message. |

[Remedy #3: Insert the blue text and delete the red text from page 31, line 31]

Table 49—AAI-PAG-ADV message field description

| Field | Size (bits) | Value | Condition |
| --- | --- | --- | --- |
| Extension Flag | 1 | … If this flag is set to 0b1, AMS and M2M device shall remain awake and monitors the subsequent AAI subframe unless its their identifiers is are found in the received segments. |  |

-----------------------------  End Text Proposal   ----------------------------------

# Reference

[1] IEEE 802.16p-10/0004r3: IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD).

[2] IEEE P802.16.1b/D2: WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems

Enhancements to Support Machine-to-Machine Applications

[3] IEEE 802.16p-10/0005r1: Machine to Machine (M2M) Communications Technical Report.