To:

Roger Marks

Chair, IEEE 802.16 Working Group

r.b.marks@ieee.org

Reference: TWG Inter-Operability Problem Reports

March 10, 2011

Subject: Liaison Statement to IEEE 802.16 WG on modifications to the IEEE Std 802.16 needed to support WiMAX certification.

Dear Dr. Marks,

In the course of development and validation of product certification test cases based on IEEE Std 802.16, the WiMAX Forum TWG has identified critical issues with the 802.16 specification that impede product interoperability. The WiMAX Forum TWG believes these issues require clarification and/or correction.  TWG respectfully requests that the IEEE 802.16 working group take the following actions.

- Review the attached problem statements and/or WiMAX contemplated remedies for each one of the problem statements as provided in Annexes A through F

- Develop a remedy for each one of the issues

- Inform the WiMAX Forum TWG of the results of IEEE 802.16 working group’s actions on this matter.

Should the IEEE 802.16 working group develop any specific remedy in response to the problems identified in the Annexes, and should these remedies be incorporated into IEEE Std 802.16, the WiMAX Forum Technical Working Group (TWG) would appreciate further communication giving specific details of the remedies including affected IEEE Std 802.16 sections.

- Thank you very much for your attention to this matter of mutual importance.

Sincerely,

- Wonil Roh

Chair, WiMAX Forum Technical Working Group (TWG)

- Non-Adaptive Precoding in Mixed Mode (IOPR TIO_62351 r1)

  - Interoperability Problem Statement

Distributed PUSC belongs to DLRU so that non-adaptive precoding shall be applied to DLRU in mixed mode. Non-adaptive precoding is applied based on matrix W that changes every N1PSC (72 subcarriers) contiguous physical subcarriers. When supporting FDM based UL PUSC zone, the number of used tone is not the multiple of N1PSC subcarriers, so we need to define how to apply the cycling pattern of matrix W in mixed mode. For simplicity, the non-adaptive precoding matrix W can be started from physical subcarrier index=0 in FDM based UL PUSC zone.

  - Outline or Idea for the Remedy

Hence, following change is required.

  - Possible Changes in the IEEE 802.16 Standards

[Adopt the following change in line 57 on page 811 in section 6.3.9.1.2.1 in IEEE P802.16.1/D3:]

In a RU allocated in a AAI subframe with MEF = 0b00 (SFBC) or 0b01 (VE) and non-adaptive precoding, the matrix W changes every N1PSC contiguous physical subcarriers according to Equation (316), and it does not depend on the AAI subframe number. When supporting FDM based UL PUSC zone, the matrix W for non-adaptive precoding changes every N1PSC contiguous physical subcarriers that start from physical subcarrier index=0. The Nt × Mt precoding matrix W(k) applied on subcarrier k in physical subband s is selected as the codeword of index i in the open-loop codebook subset of rank Mt, where i is given by Equation (316).

- Number of ACID in Multicarrier (IOPR TIO- 62357 r2)

  - Interoperability Problem Statement

Due to the feedback delay in TDD mode, RTT(round trip time) is increased in some subframes.

When it comes to DL:UL=6:2, 10 ACIDs are required in DL as follows.

W can

In general, 16m systems operate with two LRU types such as DLRU and CLRU for control and traffic channels. In order to maximize throughout, it needs to allocate bursts in both DLRU and CLRU. If two burst are allocated in every DL subframe, at least 14 ACIDs are required in case of DL:UL=5:3.

For an AMS operating over carriers with sum of FFT size that is 2048 subcarriers or less, in DL or UL, the maximum number of total HARQ channel per AMS is 16. In multicarrier mode, at least two bursts per carrier need to be allocated in every DL subframe to maximize throughput. Hence, the required number of ACIDs is increased in twice in multicarrier mode than in single carrier mode. If number of ACID in system is less than the required number of ACID, DL throughput will be decreased. Following Table shows DL throughput between in 10MHz single carrier and in 10MHz carrier of multicarrier.

|  | 10MHz BW (40 SLRU, 8 DLSRU) | 10MHz (in 10+10 MHz MC) (40 SLRU, 8 DLSRU per 10MHz) | 10MHz BW (36 SLRU, 12 DLSRU) | 10MHz (in 10+10 MHz MC) (36 SLRU, 12 DLSRU per 10MHz) | 10MHz BW (32 SLRU, 16 DLSRU) | 10MHz (in 10+10 MHz MC) (32 SLRU, 16 DLSRU per 10MHz) |
| --- | --- | --- | --- | --- | --- | --- |
| DL:UL=5:3 | 76.2 Mbps | 73.1 Mbps | 66.3 Mbps | 59.6 Mbps | 65.5 Mbps | 55.1 Mbps |
| DL:UL=6:2 | 93.0 Mbps | 76.8 Mbps | 78.5 Mbps | 61.5 Mbps | 75.7 Mbps | 57.0 Mbps |

In order to maximize throughput in multicarrier, the maximum number of HARQ channel per AMS needs to be 16 in a carrier. For an AMS operating over carrier, the maximum number of total HARQ channel per AMS needs to be increased with number of used carriers as (16 x number of used carrier).

  - Outline or Idea for the Remedy

Hence, following change is required.

  - Possible Changes in the IEEE 802.16 Standards

[Adopt the following change in line 35 on page 431 in section 6.2.14.2.1.1 in IEEE P802.16.1/D3:]

The maximum number of HARQ channels per AMS in a carrier, in DL, is 16. For an AMS operating over carriers in DL, the maximum number of total HARQ channels per AMS, NDL_HARQ_CH, is (16 × number of used carriers). If the i-th physical carrier is assigned to an AMS, the ACID in the physical carrier index i is calculated as (16 × i + n) mod NDL_HARQ_CH, where n is the 4-bit ACID value signaled in DL assignment A-MAP IE, and i is indexed from 0 to i.

For an AMS operating over carriers with a sum of FFT sizes that is 2048 subcarriers or less, in DL, the maximum number of total HARQ channels per AMS is 16. For an AMS operating over carriers with the sum of FFT sizes that is larger than 2048 subcarriers, in DL, the maximum number of total HARQ channels per AMS, NDL_HARQ_CH, is floor(16 × sum of FFT sizes of all used carriers / 2048). If the i-th physical carrier is assigned to an AMS, the ACID in the physical carrier index i is calculated as (16 × ri + n) mod NDL_HARQ_CH, where n is the 4-bit ACID value signaled in DL assignment A-MAP IE, and ri = ceil(Si / 2048) – 1. Si is the sum of FFT sizes of active carriers that are assigned to an AMS among the carriers indexed from 0 to i.

[Adopt the following change in line 17 on page 432 in section 6.2.14.2.1.2 in IEEE P802.16.1/D3:]

The maximum number of HARQ channels per AMS in a carrier, in UL, is 16. For an AMS operating over carriers in UL, the maximum number of total HARQ channels per AMS, NUL_HARQ_CH, is (16 × number of used carriers). If the i-th physical carrier is assigned to an AMS, the ACID in the physical carrier index i is calculated as (16 × i + n) mod NUL_HARQ_CH, where n is the 4-bit ACID value signaled in UL assignment A-MAP IE, and i is indexed from 0 to i.

For an AMS operating over carriers with a sum of FFT sizes that is 2048 subcarriers or less, in UL, the maximum number of total HARQ channels per AMS is 16. For an AMS operating over carriers with a sum of FFT sizes that is larger than 2048 subcarriers, in UL, the maximum number of total HARQ channels per AMS, NUL_HARQ_CH, is floor(16 × sum of FFT sizes of all used carrier / 2048). If the i-th physical carrier is assigned to an AMS, the ACID in the physical carrier index i is calculated as (16 × ri + n) mod NUL_HARQ_CH, where n is the 4-bit ACID value signaled in DL assignment A-MAP IE, and ri = ceil(Si / 2048) – 1. Si is the sum of FFT sizes of active carriers that are assigned to an AMS among the carriers indexed from 0 to i.

- Modulation Order Table for Subframe Type-3 (IOPR TIO- 62361 r2)

  - Interoperability Problem Statement

The code rate of allocated burst is determined by burst size, allocation size, STC rate, and modulation order. Burst size, allocation size and modulation order are jointly determined so that these parameters are considered together in scheduler.

In case of type-3 subframe that has five OFDMA symbols, some combinations of Burst size, allocation size, and modulation order make code rate be greater than 1. Scheduling with Table of Burst size table, Table of Minimal size index as a function of the allocation size, and Table of Rules for modulation order, 336 cases shows the code rate to be greater than 1.

These 336 cases show various combinations of TNS, STC rate, I_sizeoffset, LRU size, and modulation order. Since burst size, allocation size, and modulation order are jointly determined by scheduler, it is difficult to escape these 336 cases in processing of scheduling. If the code rate scheduled is greater than 1, the scheduler needs to reschedule resource allocation. Due to this problem, scheduling time will be increased, which is too much burden in implementation.

This problem happens only in Type-3 subframe so that it needs to adopt new modulation order table for Type-3 subframe.

  - Outline or Idea for the Remedy

Hence, following changes are required.

  - Possible Changes in the IEEE 802.16 Standards

[Adopt the following change on page 825 in section 6.3.10.1.2 in IEEE P802.16.1/D3:]

The modulation order Nmod (2 for QPSK, 4 for 16-QAM and 6 for 64-QAM) depends on the parameter ISizeOffset and AAI subframe type according to Table 305 or Table xxx. Allocation size of 1 or 2 are special cases (separate columns in the table). For allocation size of at least 3, the modulation order depends only on ISizeOffset. The allocation size and the value of ISizeOffset are set by the ABS scheduler, which takes into account the resulting modulation order and effective code rate, and according to the link adaptation.

[Adopt the following change on page826 in section 6.3.10.1.2 in IEEE P802.16.1/D3:]

Table 305—Rules for modulation order (for default TTI burst transmitted other than Type-3 subframe or long-TTI burst transmitted over subframes not including Type-3 subframe)

| ISizeOffset | Nmod (allocation size>2) | Nmod (allocation size=2) | Nmod (allocation size=1) |
| --- | --- | --- | --- |
| 0 ~ 9 | 2 | 2 | 2 |
| 10 ~ 15 | 2 | 2 | 4 |
| 16 ~ 18 | 2 | 4 | 6 |
| 19 ~ 21 | 4 | 4 | 6 |
| 22 ~ 23 | 4 | 6 | 6 |
| 24 ~ 31 | 6 | 6 | 6 |

Table xxx—Rules for modulation order (for default TTI burst transmitted in Type-3 subframe or long-TTI burst transmitted over subframes including Type-3 subframe)

| ISizeOffset | Nmod (allocation size>2) | Nmod (allocation size=2) | Nmod (allocation size=1) |
| --- | --- | --- | --- |
| 0 ~ 7 | 2 | 2 | 2 |
| 8 ~ 13 | 2 | 2 | 4 |
| 14 ~ 16 | 2 | 4 | 6 |
| 17 ~ 19 | 4 | 4 | 6 |
| 20 ~ 21 | 4 | 6 | 6 |
| 22 ~ 31 | 6 | 6 | 6 |

- Paging Cycle Request in AAI-DREG-REQ (IOPR TIO- 62369 r2)

  - Interoperability Problem Statement

ABS can decide a paging cycle with paging cycle field in AAI-DREG-RSP message. AMS can also request paging cycle to AMS with paging cycle request field in AAI-DREG-REQ message. However, the value of paging cycle field is not specified.

According to AMS’s battery conservation policy, each AMS may want different paging cycle. If AMS requests paging cycle to ABS, the ABS will determine the paging cycle in AAI-DREG-RSP message based on AMS’s request.

In order to receive AMS’s request on paging cycle, it needs to define the value of paging cycle request field in AAI-DREG-REQ message.

  - Outline or Idea for the Remedy

The value of paging cycle is defined in AAI-DREG-RSP message. Hence, the value of paging cycle request field can be defined as the same value of paging cycle field in AAI-DREG-RSP message.

  - Possible Changes in the IEEE 802.16 Standards

[Adopt the following change in Table 47on page 135 in section 6.2.3.21 in IEEE P802.16.1/D3:]

6.2.3.21 AAI-DREG-REQ message

Table 47—AAI-DREG-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Deregistration_Request_ Code | 3 | Used to indicate the purpose of this  message 0x00: AMS deregistration request from ABS and network 0x01: request for AMS deregistration from S-ABS and initiation of AMS idle mode. 0x02: response for the unsolicited AAI-DREG-RSP message with action code 0x05 by the ABS. 0x03: reject for the unsolicited AAI-DREG-RSP message with action code 0x05 by the ABS. This code is applicable only when an AMS has a pending UL data to transmit. 0x04: request for AMS deregistration from S-ABS to enter DCR mode 0x05: response for the unsolicited AAI-DREG-RSP message with action code 0x00, 0x01, 0x02, or 0x03 0x06–0x07: Reserved |  |
| If (Deregistration_Request_Cod e == 0x01) { |  |  |  |
| Paging cycle request | 4 | Used to indicate Paging cycle recom-mended by AMS 0b0000–0b1111: Reserved 0b0000: 4 superframes 0b0001: 8 superframes 0b0010: 16 superframes 0b0011: 32 superframes 0b0100: 64 superframes 0b0101: 128 superframes 0b0110: 256 superframes 0b0111: 512 superframes 0b1000–0b1111: Reserved |  |
| Idle Mode Retain Information element | 5 | Provided as part of this message indicative only. Network reentry from idle mode process requirements may change at time of actual reentry. For each bit location, a value of 0 indicates the information for the associated reentry control messages shall not be retained and managed; a value of 1 indicates the information for the associated reentry control message shall be retained and managed. Bit 0: Retain MS service and operational information associated with AAI-SBCREQ/ RSP messages. Bit 1: Retain MS service and operational information associated with AAI-PKMREQ/ RSP messages. Bit 2: Retain MS service and operational information associated with AAI-REGREQ/ RSP messages. Bit 3: Retain MS service and operational information associated with network address. Bit 4: Retain MS state information. The information retained by setting bit 4 includes configuration of all Service Flows in the AMS as set by successful AAI-DSA and AAI-DSC transactions. In particular it includes FIDs and related description (QoS descriptors and CS classifier information), |  |

- Direct Handover between 16e BS and 16m ABS in Idle Mode (IOPR TIO- 62379 r4)

  - Interoperability Problem Statement

When direct handover in idle mode is completed, the context in previous paging controller needs to be deleted. Hence, it needs to inform paging controller ID and paging information in serving base station to corresponding target base station.

In order to get rid of ongoing jobs in previous paging controller, it needs to define new ranging purpose indication code for network reentry from idle mode in case of direct handover from legacy BS or AAI-only BS.

For network reentry from idle mode in case of direct handover from legacy BS, paging controller ID used in legacy BS needs to be informed to the target 16m ABS. For network reentry from idle mode in case of direct handover from AAI-only BS, paging controller ID, DID, and paging cycle used in 16m ABS need to be informed to the target 16e BS.

  - Outline or Idea for the Remedy

Following change is required in both IEEE P802.16.1/D3 and IEEE P802.16Rev3/D3.

  - Possible Changes in the IEEE 802.16 Standards

[Adopt the following change in Table 27on page 56 in section 6.2.3.1 in IEEE P802.16.1/D3:]

Table 27—AAI-RNG-REQ message field description (continued)

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Ranging Purpose Indication | 4 | 0b0000 = Initial network entry 0b0001 = HO reentry 0b0010 = Network reentry from idle mode 0b0011 = Idle mode location update 0b0100 = DCR mode extension 0b0101 = Emergency call setup (e.g., E911) 0b0110 = Location update for  updating service flow management encodings of E-MBS flows 0b0111 = Location update for transi-tion to DCR mode from idle mode 0b1000 = Reentry from DCR mode, coverage loss or detection of different ABS restart count 0b1001 = Network reentry from a R1 BS 0b1010 = Zone switch to MZONE from LZONE 0b1011 = Location update due to power down 0b1100 = Interference mitigation request to a CSG Femto ABS when experiencing interference from the CSG Femto ABS 0b1101 = NS/EP call setup 0b1110 = Network Reentry from idle mode of AMS which has entered Idle Mode in R1 BS 0–0b1111 = Reserved |  |
| CMAC indicator | 1 | Indicate whether this message is  protected by CMAC tuple 0b0: not protected 0b1: protected |  |

[Adopt the following change in Table 27on page 63 in section 6.2.3.1 in IEEE P802.16.1/D3:]

Table 27—AAI-RNG-REQ message field description (continued)

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Initial Offset for uplink power control (OffsetInitial) | 5 | The bit size represents power level ranging from –15 dB (0x00) to 16dB (0x1F) with 1 dB step The value is determined by AMS after successful initial ranging process. |  |
| }//end of Ranging Purpose Indication }else if (Ranging Purpose Indication == 0b1110) { |  | // Network Reentry from idle mode of AMS which has entered idle mode in R1 BS |  |
| Paging Controller ID | 48 | The Paging Controller ID to which the AMS previously belonged in serving legacy BS |  |
| }//end of Ranging Purpose Indication |  |  |  |
| for (i = 0; i < N_CSG_ID_Infos; i++) { |  | N_CSG_ID_Infos is the number of CSG ID Information blocks. 1≤ N_CSG_ID_Infos≤15 | Optional for loop. May be included for quick CSG membership detection or ABS reselection assistance. |
| Operator ID of the CSG Femtocell | 24 | The Operator ID of the CSG  Femtocell | Present if the Operator ID is different from the one of the ABS |
| for (j = 0; j < N_CSG_IDs; j++) { |  | N_CSG_IDs is the number of CSG IDs belongs to this Operator ID. |  |
| CSGID | variable | The CSGID within the Operator ID. It may be part of the BS ID, with certain bits inside indicating its length. If the CSG has single BS, it may be of maximum length, which is the LSB-24- bits of the full BS ID. |  |
| } |  |  |  |
| } |  |  |  |

[Adopt the following change in line58 on page 122 in section 6.3.2.3.5 in IEEE P802.16Rev3/D3:]

The following TLV parameter shall be included in the RNG-REQ message when the MS is attempting to perform reentry, HO, or location update:

Ranging Purpose Indication

The presence of this item in the message indicates the following MS action:

If Bit 0 is set to 1, in combination with a serving BSID, it indicates that the MS is currently

attempting to HO or reentry; or, in combination with a Paging Controller ID, indicates that

the MS is attempting network reentry from idle mode to the BS.

If Bit 1 is set to 1, it indicates that the MS is initiating the idle mode location update

process.

Bit 2: Seamless HO indication. When this bit is set to 1 in combination with other included

information elements, it indicates the MS is initiating ranging as part of seamless HO

procedure.

Bit 3: Ranging Request for Emergency Call Setup. When this bit is set to 1, it indicates

MS action of Emergency Call Process.

Bit 4: MBS update. When this bit is set to 1, the MS is currently attempting to perform

location update due to a need to update service flow management encodings for MBS

flows.

Bit 5: Network Reentry from idle mode of MS which has entered idle mode in AAI-only ABS

Bits 6–7: Reserved

Bits 5.–7: Reserved

The following TLV parameter shall be included in the RNG-REQ message when the MS is attempting to

perform reentry or location update:

[Adopt the following change in line62 on page 123 in section 6.3.2.3.5 in IEEE P802.16Rev3/D3:]

The following TLV shall be included whenever the CMAC tuple is included in the RNG-REQ message during re-entry, secure Location Update or handover.

CMAC_KEY_COUNT

This field contains the MSs current value of the CMAC_KEY_COUNT, which is used to generate the CMAC_KEY_U used to generate the CMAC Tuple included in this message. See 7.2.2.2.9.

The following parameter shall be included in the RNG-REQ message when Bit 5 in Ranging Purpose Indication is set to 1.

Paging Controller ID from AAI-only ABS (see 11.5)

Deregistration ID (see 11.5)

Paging cycle from AAI-only ABS (see 11.5)

The following parameter shall be included in the RNG-REQ message when the MS is attempting to perform Network Re-Entry from idle mode, Keep-Alive check in sleep mode, Secure Location Update, or HO and the MS has a hashed message authentication code (HMAC)/cipher-based message authentication code (CMAC) tuple necessary to expedite security authentication.

HMAC/CMAC Tuple (see 11.1.2)

The HMAC/CMAC Tuple shall be the last attribute in the message.

[Adopt the following change in Table 686 on page 1481 in IEEE P802.16Rev3/D3:]

Table 686.—RNG-REQ message encodings

| Name | Type (1 byte) | Length | Value (variable length) | PHY scope |
| --- | --- | --- | --- | --- |
| Requested Downlink Burst Profile | 1 | 1 | Bits 0.–3: DIUC of the DL burst profile requested by the SS for DL traffic. Bits 4.–7: 4 LSB of Configuration Change Count value of DCD defining the burst profile associated with DIUC. | All |
| SS MAC Address | 2 | 6 | The MAC address of the SS. | All |
| Ranging Anomalies | 3 | 1 | A parameter indicating a potential error condition detected by the SS during the ranging process. Setting the bit associated with a specific condition indicates that the condition exists at the SS. Bit 0 .— SS already at maximum power. Bit 1 .— SS already at minimum power. Bit 2 .— Sum of commanded timing adjustments is too large. | All |
| AAS broadcast capability | 4 | 1 | 0 = SS can receive broadcast messages. 1 = SS cannot receive broadcast messages. | OFDM, OFDMA |
| Serving BSID | 5 | 6 | The unique identifier of the former serving BS. | — |
| Ranging Purpose Indication | 6 | 1 | Bit 0: HO indication (when this bit is set to 1 in combination with other included information elements indicates the MS is currently attempting to HO or network reentry from idle mode to the BS) Bit 1: Location update request (when this bit is set to 1, it indicates MS action of idle mode location update process) Bit 2: Seamless HO indication (when this bit is set to 1 in combination with other included information elements indicates the MS is currently initiating ranging as part of the seamless HO procedure) Bit 3: Ranging Request for Emergency Call Setup (when this bit is set to 1, it indicates MS action of Emergency Call Process) Bit 4: MBS update. When this bit is set to 1, the MS is currently attempting to perform location update due to a need to update service flow management encodings for MBS flows. Bit 5: Network Reentry from idle mode of MS which has entered idle mode in AAI-only ABS Bits 6–7: Reserved Bits 5.–7: Reserved | — |
| HO ID | 7 | 1 | ID assigned by the target BS for use in initial ranging during MS HO to it (see 6.3.20). | — |
| Power down Indicator | 8 | 1 | Presence of item in message indicates the MS is currently attempting to switch power off, regardless of value. | — |
| Requested downlink repetition coding level | 12 | 1 | This parameter indicates repetition coding level Indication requested by the MS for DL traffic. If this TLV is not present in the RNG-REQ, it shall be assumed that repetition coding is not requested. Bit 0.–1: Repetition coding level: 0b00 .– No repetition 0b01 .– Repetition coding of 2 0b10 .– Repetition coding of 4 0b11 .– Repetition coding of 6 The BS shall ignore this field if the DIUC requested in the Requested Downlink Burst Profile TLV refers to modulations higher than QPSK. Bits 2.–7: Reserved | — |
| CMAC_KEY_COU NT | 13 | 2 | CMAC_KEY_COUNT A 16-bit counter used in the generation of the CMAC keys (refer to 7.2.2.2.9.1). | — |
| Power_Saving_Class_Parameters | 21 | variable | Compound TLV to specify Power Saving Class definition and/or operation. | — |
| Unified TLV encoding for Power Saving Class Parameters | 22 | variable | MS may use this TLV encoding instead of the Power_Saving_Class_Parameters TLV encoding to specify a Power Saving Class (see Table 684). | — |
| Paging Cycle Change | 23 | 2 | Requested Paging Cycle length expressed in number of frames. An MS in Idle Mode may use this TLV encoding to request a change of the MS’s Paging Cycle. | — |
| SS attribute | 24 | 1 | Bit 0: normal SS. Bit 1: SS with forwarding capabilities. | — |
| Paging Controller ID from AAI-only ABS | 25 | 6 | Paging controller to which the MS previously belonged in serving AAI-only ABS | — |
| Deregistration ID | 26 | 3 | DID to which the MS previously belonged in serving AAI-only ABS | — |
| Paging cycle from AAI-only ABS | 27 | 1 | Paging cycle to which the MS previously belonged in serving AAI-only ABS | — |

- ARQ Parameters Update for Direct Handover from 16e BS to 16m ABS (IOPR TIO- 62416 r1)

  - Interoperability Problem Statement

In case of direct handover from 16e BS to 16m ABS, ARQ parameters need to be updated by DSC process. DSC transaction only for updating ARQ parameters in direct handover from 16e BS to 16m ABS increases latency in handover. In order to minimize the latency in handover procedures, ARQ parameters can be updated in AAI-RNG-RSP message.

  - Outline or Idea for the Remedy

Hence, following change is required.

  - Possible Changes in the IEEE 802.16 Standards

[Adopt the following change in Table 47on page 135 in section 6.2.3.21 in IEEE P802.16.1/D3:]

Table 28—AAI-RNG-RSP message field description (continued)

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Resource Retain Time | 16 | The default duration in units of  100 ms to which the ABS set the ABS_Resource_Retain_Timer. | May be included if Reentry Process Optimization Bit 3 is set to 1. |
| For (i = 0; i<N_FIDs; i++) { |  | N_FIDs is the number of FIDs to be included here. The maximum number of N_FIDs is 24. | Shall be included if the ABS needs to update the AMS’s existing flows. FIDs that are not present here shall be regarded as guaranteed by the ABS. |
| SFID | 32 | SFID of the existing flow to be updated. |  |
| FID | 4 |  |  |
| Update or delete | 1 | 0: delete the flow 1: updated QoS Info exists. |  |
| Updated QoS Info | variable | The following parameters shall be included if the parameter needs to be updated. The parameter that does not have to be updated is not included in this field. — Traffic Priority parameter — Maximum Sustained Traffic Rate parameter — Maximum Traffic Burst parameter — Minimum Reserved Traffic Rate parameter — Maximum Latency parameter — Paging Preference parameter — Tolerated Jitter parameter — UL scheduling type — Unsolicited Grant Interval parameter — Unsolicited Polling Interval parameter — Primary Grant Polling Interval parameter — Primary Grant Size parameter — Secondary Grant Polling Inter-val parameter — Secondary Grant Size parameter — Adaptation Method — Access Class — Differentiated BR Timer — Predefined BR index — BR action — BR size — Initial Backoff Window Size — Maximum Backoff Window Size — Backoff Scaling Factor — MAC in-order delivery indicator — Vendor ID |  |
| ROHC support | 1 | 0: not support 1: support | Present if needed. |
| PHS support | 1 | Indicates the level of PHS support. 0: Reserved 1: Packet PHS | Present if needed. |
| If (ARQ parameters are needed) { |  |  |  |
| ARQ Enable | 1 | 0 = ARQ Not Requested/Accepted 1 = ARQ Requested/Accepted | Present if needed |
| ARQ_WINDOW_SIZE | 16 | >0 and ≤(ARQ_BSN_MODULUS/2) | Present if needed |
| ARQ_BLOCK_LIFETIME | 16 | 0 = Infinite 1–6553500 μs (100 μs granularity) | Present if needed |
| ARQ_SYNC_LOSS_TIMEOUT | 16 | 0 = Infinite 1–6553500 μs (100 μs granularity) | Present if needed |
| ARQ_RX_PURGE_TIMEOUT | 16 | 0 = Infinite 1–6553500 μs (100 μs granularity) | Present if needed |
| ARQ_SUB_BLOCK_SIZE | 3 | Bit 0–2: encoding for selected block size (P), where the selected block size is equal to 2(P+3), 0≤P≤7. ARQ subblock size is byte unit | Present if needed |
| ARQ_ERROR_DETECTION_T IMEOUT | 16 | 0 = Infinite 1–6553500 s (100 s granularity) | Present if needed |
| ARQ_FEEDBACK_POLL_RET RY_TIMEOUT | 16 | 0 = Infinite 1–6553500 s (100 s granularity) | Present if needed |
| } |  |  |  |
| } |  |  |  |