| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modulation Order Table for Subframe Type-3 |  |
| Date Submitted | 2012-3-13 |  |
| Source(s) | Zheng Yan-Xiu, Maintenance Task Group | zhengyanxiu@itri.org.tw |
| Re: | IEEE 802.16-12-0218-00-WGLS, Liaison Statement to IEEE 802.16 WG on modifications to the IEEE Std 802.16 needed to support WiMAX certification. |  |
| Abstract | Changes are provided to accommodate Modulation Order Table for Subframe Type-3 |  |
| Purpose | As basis of maintenance data base for IEEE 802.16.1 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modulation Order Table for Subframe Type-3

Zheng Yan-Xiu

Maintenance Task Group

# Introduction

This contribution is intended to incorporate changes agreed based on the liaison IEEE 802.16-12-0218-00-WGLS.

The code rate of allocated burst is determined by burst size, allocation size, STC rate, and modulation order. Burst size, allocation size and modulation order are jointly determined so that these parameters are considered together in scheduler.

In case of type-3 subframe that has five OFDMA symbols, some combinations of Burst size, allocation size, and modulation order make code rate be greater than 1. Scheduling with Table of Burst size table, Table of Minimal size index as a function of the allocation size, and Table of Rules for modulation order, 336 cases shows the code rate to be greater than 1.

These 336 cases show various combinations of TNS, STC rate, I_sizeoffset, LRU size, and modulation order. Since burst size, allocation size, and modulation order are jointly determined by scheduler, it is difficult to escape these 336 cases in processing of scheduling. If the code rate scheduled is greater than 1, the scheduler needs to reschedule resource allocation. Due to this problem, scheduling time will be increased, which is too much burden in implementation.

This problem happens only in Type-3 subframe so that it needs to adopt new modulation order table for Type-3 subframe.

# Proposed Text

[Adopt the following change on page 831 in section 6.3.10.1.2 in IEEE P802.16.1/D4:]

The modulation order Nmod (2 for QPSK, 4 for 16-QAM and 6 for 64-QAM) depends on the parameter ISizeOffset </insert>and AAI subframe type<insert> according to Table 305 </insert>or Table xxx<insert>. Allocation size of 1 or 2 are special cases (separate columns in the table). For allocation size of at least 3, the modulation order depends only on ISizeOffset. The allocation size and the value of ISizeOffset are set by the ABS scheduler, which takes into account the resulting modulation order and effective code rate, and according to the link adaptation.

[Adopt the following change on page832 in section 6.3.10.1.2 in IEEE P802.16.1/D4:]

Table 305—Rules for modulation order </insert>(for default TTI burst transmitted other than Type-3 subframe or long-TTI burst transmitted over subframes not including Type-3 subframe)<insert>

| ISizeOffset | Nmod (allocation size>2) | Nmod (allocation size=2) | Nmod (allocation size=1) |
| --- | --- | --- | --- |
| 0 ~ 9 | 2 | 2 | 2 |
| 10 ~ 15 | 2 | 2 | 4 |
| 16 ~ 18 | 2 | 4 | 6 |
| 19 ~ 21 | 4 | 4 | 6 |
| 22 ~ 23 | 4 | 6 | 6 |
| 24 ~ 31 | 6 | 6 | 6 |

</insert>Table xxx—Rules for modulation order (for default TTI burst transmitted in Type-3 subframe or long-TTI burst transmitted over subframes including Type-3 subframe)

| ISizeOffset | Nmod (allocation size>2) | Nmod (allocation size=2) | Nmod (allocation size=1) |
| --- | --- | --- | --- |
| 0 ~ 7 | 2 | 2 | 2 |
| 8 ~ 13 | 2 | 2 | 4 |
| 14 ~ 16 | 2 | 4 | 6 |
| 17 ~ 19 | 4 | 4 | 6 |
| 20 ~ 21 | 4 | 6 | 6 |
| 22 ~ 31 | 6 | 6 | 6 |

<insert>