| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ARQ Parameters Update for Direct Handover from 16e BS to 16m ABS |  |
| Date Submitted | 2012-03-13 |  |
| Source(s) | Zheng Yan-Xiu, Maintenance Task Group | zhengyanxiu@itri.org.tw |
| Re: | IEEE 802.16-12-0218-00-WGLS, Liaison Statement to IEEE 802.16 WG on modifications to the IEEE Std 802.16 needed to support WiMAX certification. |  |
| Abstract | Changes are provided to accommodate ARQ Parameters Update for Direct Handover from 16e BS to 16m ABS |  |
| Purpose | As basis of maintenance data base for IEEE 802.16.1 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ARQ Parameters Update for Direct Handover from 16e BS to 16m ABS

Zheng Yan-Xiu

Maintenance Task Group

# Introduction

This contribution is intended to incorporate changes agreed based on the liaison IEEE 802.16-12-0218-00-WGLS.

# Proposed Text

[Adopt the following change in Table 29 on page 70 in section 6.2.3.21 in IEEE P802.16.1/D4:]

Table 29—AAI-RNG-RSP message field description (continued)

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Resource Retain Time | 16 | The default duration in units of  100 ms to which the ABS set the ABS_Resource_Retain_Timer. | May be included if Reentry Process Optimization Bit 3 is set to 1. |
| For (i = 0; i<N_FIDs; i++) { |  | N_FIDs is the number of FIDs to be included here. The maximum number of N_FIDs is 24. | Shall be included if the ABS needs to update the AMS’s existing flows. FIDs that are not present here shall be regarded as guaranteed by the ABS. |
| SFID | 32 | SFID of the existing flow to be updated. |  |
| FID | 4 |  |  |
| Update or delete | 1 | 0: delete the flow 1: updated QoS Info exists. |  |
| Updated QoS Info | variable | The following parameters shall be included if the parameter needs to be updated. The parameter that does not have to be updated is not included in this field. — Traffic Priority parameter — Maximum Sustained Traffic Rate parameter — Maximum Traffic Burst parameter — Minimum Reserved Traffic Rate parameter — Maximum Latency parameter — Paging Preference parameter — Tolerated Jitter parameter — UL scheduling type — Unsolicited Grant Interval parameter — Unsolicited Polling Interval parameter — Primary Grant Polling Interval parameter — Primary Grant Size parameter — Secondary Grant Polling Inter-val parameter — Secondary Grant Size parameter — Adaptation Method — Access Class — Differentiated BR Timer — Predefined BR index — BR action — BR size — Initial Backoff Window Size — Maximum Backoff Window Size — Backoff Scaling Factor — MAC in-order delivery indicator — Vendor ID |  |
| ROHC support | 1 | 0: not support 1: support | Present if needed. |
| PHS support | 1 | Indicates the level of PHS support. 0: Reserved 1: Packet PHS | Present if needed. |
| <insert> If (ARQ parameters are needed) { |  |  | </insert> |
| <insert> ARQ Enable | 1 | 0 = ARQ Not Requested/Accepted 1 = ARQ Requested/Accepted | Present if needed</insert> |
| <insert> ARQ_WINDOW_SIZE | 16 | >0 and ≤(ARQ_BSN_MODULUS/2) | Present if needed</insert> |
| <insert> ARQ_BLOCK_LIFETIME | 16 | 0 = Infinite 1–6553500 μs (100 μs granularity) | Present if needed</insert> |
| <insert> ARQ_SYNC_LOSS_TIMEOUT | 16 | 0 = Infinite 1–6553500 μs (100 μs granularity) | Present if needed</insert> |
| <insert> ARQ_RX_PURGE_TIMEOUT | 16 | 0 = Infinite 1–6553500 μs (100 μs granularity) | Present if needed</insert> |
| <insert> ARQ_SUB_BLOCK_SIZE | 3 | Bit 0–2: encoding for selected block size (P), where the selected block size is equal to 2(P+3), 0≤P≤7. ARQ subblock size is byte unit | Present if needed</insert> |
| <insert> ARQ_ERROR_DETECTION_T IMEOUT | 16 | 0 = Infinite 1–6553500 s (100 s granularity) | Present if needed</insert> |
| <insert> ARQ_FEEDBACK_POLL_RET RY_TIMEOUT | 16 | 0 = Infinite 1–6553500 s (100 s granularity) | Present if needed</insert> |
| /insert> } |  |  | </insert> |
| } |  |  |  |