| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text clarification for comment # 1013 |  |
| Date Submitted | 2012-03-14 |  |
| Source(s) | Anil Agiwal, Hyunjeong Kang, Youngkyo Baek, Anshuman Nigam, Rakesh Taori Samsung Electronics | anilag@samsung.com, hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | Sponsor Ballot |  |
| Abstract | This contribution proposes the modified text for comment # 1013 to address the reply comment of Erik. |  |
| Purpose | To be discussed and adopted in IEEE P802.16.1b/D3 |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Text clarification for comment # 1013

Anil Agiwal, Hyunjeong Kang, Youngkyo Baek, Anshuman Nigam, Rakesh Taori

Samsung Electronics

- Introduction

The mapping of SA for the control connection carrying encrypted multicast control message is defined by comment # 1013. However the proposed text is not clear as mentioned by Erik in his reply comment. This contribution proposes the modified text to address the reply comment of Erick.

- Proposed Text Change

Adopt the following remedies in IEEE P802.16.1b/D3.

Modify the section 6.2.5.2.2.1 as follows

6.2.5.2.2.1 Mapping of flows to SAs

The following rules for mapping flows to SAs apply:

a) The unicast transport flows shall be mapped to an SA.

b) The multicast or broadcast transport flows shall be mapped to Null SA.

c) The encrypted unicast control flows shall be mapped to the Primary SA.

d) The non-encrypted unicast control flows shall not be mapped to any SA.

e) The broadcast control flows shall not be mapped to any SA.

f) The multicast transport flow for an M2M group shall be mapped to Null SA or Multicast SA.

g) The encrypted multicast control flow shall be dynamically mapped to different multicast SA. The encrypted multicast control flow carrying the control message for an M2M device group shall be mapped to the multicast SA established for that M2M device group.

- Reference

[1] IEEE P802.16.1b/D2