| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up to the Indication of Dedicated Ranging Channel Allocation |  |
| Date Submitted | 2012-03-14 |  |
| Source(s) | Giwon Park LG Electronics | Voice: E-mail: giwon.park@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | sb010b |  |
| Abstract | This contribution proposes changes to the Section 6.2.3.23 of IEEE P802.16.1b/D2 |  |
| Purpose | To be discussed and adopted by TGp |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Clean-up to the Indication of Dedicated Ranging Channel Allocation

Giwon Park

LG Electronics

# Introduction

This contribution tries to clarify the indication of dedicated ranging channel allocation in AAI-PAG-ADV message.

# Text Proposal

Adopt the following text modifications to IEEE P802.16.1b/D2:

--------------------------  Begin Text Proposal   -------------------------------

[Insert the blue text and delete the red text from page 51 line 15]

6.2.18.7.2 Network re-entry from idle mode for M2M devices

The ABS may indicate the dedicated ranging channel (i.e., M2M ranging opportunity subframe index, Periodicity of the M2M ranging) allocated by the AAI-SCD in AAI-PAG-ADV message.

If the dedicated ranging channel (i.e., M2M ranging opportunity subframe index, Periodicity of the M2M ranging) which is allocated through AAI-PAG-ADV message is different with the dedicated ranging channel which is allocated via AAI-SCD message, Dedicated Channel Allocation Timer is included in AAI-PAG-ADV message and the allocated dedicated ranging channel is valid within the Dedicated Channel Allocation Timer.

-----------------------------  End Text Proposal   ----------------------------------

# Reference

[1] IEEE 802.16p-10/0004r3: IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD).

[2] IEEE P802.16.1b/D2: WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems

Enhancements to Support Machine-to-Machine Applications

[3] IEEE 802.16p-10/0005r1: Machine to Machine (M2M) Communications Technical Report.