| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on UGS |  |
| Date Submitted | 2012-03-15 |  |
| Source(s) | Seho Kim, Samsung Electronics Co. LTD. | E-mail: seho42.kim@samsung.com |
| Re: | Call for comments |  |
| Abstract | It proposes revisions to IEEE P802.16.1/D4 for clarification on UGS |  |
| Purpose | To be adopted by Maint TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Introduction

Since there is no specification on unsolicited grant service (UGS) in 16m standard, it means that UGS in 16m shall follow 16e standardization. In 16e standard, piggyback bandwidth request on UGS is provided by the grant management subheader. To support this operation, grant management extended header is also required in 16m.

# Proposed text

[Remedy 1: Adopt the following change in Table 6 on page 37 in section 6.2.2.1.3 in IEEE P802.16.1/D4:]

Table 6— Type field encodings for MAC signaling header type

| Type field (5-bits) | MAC signaling header type |
| --- | --- |
| 00000 | BR with STID |
| 00001 | BR without STID |
| 00010 | Service specific scheduling control header |
| 00011 | Sleep control |
| 00100 | AMS battery level report |
| 00101 | Uplink power status report |
| 00110 | Correlation matrix feedback |
| 00111 | MIMO feedback |
| <insert>01000 | Grant Management </insert> |
| <delete>01000</delete><insert>01001</insert>-11111 | Reserved |

[Remedy 2: Adopt the following change in line 38 on page 46 in IEEE P802.16.1/D4:]

<insert>

6.2.2.1.3.9 Grant management header (GMH)

The GMH is used by the AMS to convey bandwidth management needs to the ABS. This header is encoded differently based upon the type of UL scheduling service for the connection (as given by the FID). The GMH is shown in Table xx. Its fields are defined in Table yy.

Table xx—Grant Management header format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| GMEGH { | — | — |
| FID | 4 | Flow Identifier. Set to 0010. |
| Type | 5 | MAC signaling header type = 0b01000 |
| Length | 3 | Indicates the length of the signaling header in bytes. |
| BR FID | 4 | The FID for which UL bandwidth is requested. |
| if (Uplink grant scheduling type== UGS) { | — | — |
| SLI | 1 |  |
| PM | 1 |  |
| FLI | 1 | — |
| FL | 4 | — |
| Reserved | 9 | Shall be set to zero |
| } else if (Uplink grant scheduling type== Extended rtPS) { | — | — |
| Extended piggyback request | 11 | — |
| FLI | 1 | — |
| FL | 4 | — |
| } else { | — | — |
| PiggyBack Request | 16 | — |
| } | — | — |
| } | — | — |

Table yy—GMH fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| FLI | 1 | Frame latency indication 0 = FL field disabled for this grant 1 = FL field enabled for this grant |
| FL | 4 | Frame latency. The number of frames previous to the current one in which the transmitted data was available. When the latency is greater than 15 then the FL field shall be set to 15. |
| Extended Piggyback Request | 11 | The number of bytes of UL bandwidth requested by the AMS. The request shall not include any PHY overhead. In case of Extended rtPS, the ABS changes its grant size to the size specified in this field. |
| PBR | 16 | Piggyback request. The number of bytes of UL bandwidth requested by the AMS. The request shall not include any PHY overhead. The request shall be incremental. |
| PM | 1 | Poll me 0 = No action 1 = Used by the AMS to request a bandwidth poll. |
| Reserved | 9 | - |
| SLI | 1 | Slip indicator 0 = No action 1 = Used by the AMS to indicate a slip of UL grants relative to the UL queue depth. |

</insert>