| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on coverage loss |  |
| Date Submitted | 2012-03-15 |  |
| Source(s) | Seho Kim, Samsung Electronics Co. LTD. | E-mail: seho42.kim@samsung.com |
| Re: | Call for comments |  |
| Abstract | It proposes revisions to IEEE P802.16.1/D4 for clarification on coverage loss |  |
| Purpose | To be adopted by Maint TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Introduction

Initial Offset for uplink power control (OffsetInitial) is required to UL power control and calculated by AMS during initial ranging process. Initial Offset for uplink power control (OffsetInitial) is also required in handover case. Hence, it needs to be added in AAI-RNG-REQ message.

In coverage loss case, if AMS sends AAI-REG-REQ message with CRID without Previous STID, ABS will request context retrieve from paging controller. Since paging controller does not have context of AMS in active mode, paging controller sends failure indication to ABS. Then, AMS shall perform full reentry. If AMS perform full reentry from coverage loss, it will take long time to finish network reentry process. Hence, to reduce network reentry latency from coverage loss, AMSID or AMSID*, MAC version and Initial Offset for uplink power control (OffsetInitial) are required in AAI-RNG-REQ message.

# Proposed text

[Remedy 1: Adopt the following change in Table 28 on page 64 in section 6.2.3. 1 in IEEE P802.16.1/D4:]

Table 28—AAI-RNG-REQ Message Field description (continued)

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| … |  |  |  |
| For (i=0; i<N_FIDs; i++) { |  | N_FIDs is the number of transport FIDs that the AMS has. 1 N_FIDs 24  |  |
| FID | 4 | Flow identifier |  |
| FID change count | 4 | FID change count |  |
| DL/UL indicator | 1 | 0b0: Downlink 0b1: Uplink |  |
| } |  |  |  |
| <insert> Initial Offset for uplink power control (OffsetInitial) | 5 | The bit size represents power level ranging from –15 dB (0x00) to 16dB (0x1F) with 1dB step The value is determined by AMS after successful handover ranging process. The value is calculated as the same way with initial ranging channel power control. | Optional. Present if code based ranging is done. </insert> |
| } else if (Ranging Purpose Indication == 0b0010) { |  | // Network reentry from idle mode |  |
| … |  |  |  |

[Remedy 2: Adopt the following change in Table 28 on page 67 in section 6.2.3. 1 in IEEE P802.16.1/D4:]

Table 28—AAI-RNG-REQ Message Field description (continued)

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| … |  |  |  |
| For (i=0; i<N_FIDs; i++) { |  | N_FIDs is the number of transport FIDs that the AMS has. 1 N_FIDs 24  |  |
| FID | 4 | Flow identifier |  |
| FID change count | 4 | FID change count |  |
| DL/UL indicator | 1 | 0b0: Downlink 0b1: Uplink |  |
| } |  |  |  |
| <insert>If (S-SFH Network Configuration bit == 0b0 and AMSID privacy is enabled) { |  |  | </insert> |
| <insert>AMSID* | 48 | The AMSID hash value. Refer to 6.2.5.3.1 | </insert> |
| <insert>} else if (S-SFH Network Configuration bit == 0b1 or AMSID privacy is disabled){ |  |  | </insert> |
| <insert>AMS MAC address | 48 | AMS’s real MAC address | </insert> |
| <insert>} |  |  | </insert> |
| <insert>MAC version | 8 | See 11.1.3 in IEEE 802.16REV3/D4 Editor's note: Need to update cross reference before sponsor ballot finalization] | </insert> |
| <insert>Initial Offset for uplink power control (OffsetInitial) | 5 | The bit size represents power level ranging from –15 dB (0x00) to 16dB (0x1F) with 1dB step The value is determined by AMS after successful initial ranging process | </insert> |
| } else if (Ranging Purpose Indication == 0b1001) { |  | // Network reentry from a R1 BS |  |
| … |  |  |  |