| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on ROHC |  |
| Date Submitted | 2012-03-15 |  |
| Source(s) | Seho Kim, Samsung Electronics Co. LTD. | E-mail: seho42.kim@samsung.com |
| Re: | Call for comments |  |
| Abstract | It proposes revisions to IEEE P802.16.1/D4 for clarification on ROHC |  |
| Purpose | To be adopted by Maint TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Introduction

To support ROHC, ROHC Max Context ID, Large Context IDs, ROHC MRRU, ROHC Profiles, ROHC Feedback Channel fields are required. However, ROHC Profiles, ROHC Feedback Channel fields are missed in AAI-DSA-REQ and AAI-DSC-REQ message.

# Proposed text

[Remedy 1: Adopt the following change in Table 84 on page 212 in section 6.2.3.47.1 in IEEE P802.16.1/D4:]

Table 84—AAI-DSA-REQ Message Field description (continued)

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| … |  |  |  |
| inner IPv6 Flow Label | 8 | IPv6 Flow Label of inner IP header | Present if needed |
| } //End If (Packet Classification Rule) |  |  |  |
| If (ROHC Parameter is needed) { |  |  |  |
| ROHC Max Context ID | 16 | Non-negative integer |  |
| Large Context IDs | 8 | 0: FALSE (Small Context ID) 1: TRUE (Large Context ID) 2–255: Reserved |  |
| ROHC MRRU | 16 | 0: no segmentation 1..65535: MRRU Maximum reconstructed reception unit |  |
| <insert>ROHC Profiles | variable | A set of nonnegative integers, where each integer indicates a 16 bit profile identifier of a ROHC profile supported by the decompressor. | </insert> |
| <insert>ROHC Feedback Channel | 32 | 0: no associated ROHC feedback Otherwise: SFID for ROHC feedback. | </insert> |
| } |  |  |  |
| Packet Error Rate (PER) | 8 | MSB (bit 7): 0–PER measured by the application, post-ARQ and post-HARQ process 1–PER measured on the airlink, before the application of ARQ and HARQ Bit 6: 0–Interpret bits 0–5 as an integer %, i.e., if bits 0–5 are the binary represen-tation of the integer N, then the PER = N/100 (= N%) 1—Interpret bits 0–5 as 10 times a negative exponent of 10, i.e., if bits 0– 5 are the binary representation of the integer N, then the PER = 10-N/10 LSB 6 bits (bits 0–5): PER value If bit 6 =0, [0 to 63%] PER If bit 6=1, [~510–7 to 110–0] PER | Present if needed |
| … |  |  |  |

[Remedy 2: Adopt the following change in Table 87 on page 231 in section 6.2.3.47.4 in IEEE P802.16.1/D4:]

Table 87—AAI-DSC-REQ Message Field description (continued)

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| … |  |  |  |
| ARQ_FEEDBACK_POLL_RE TRY_TIMEOUT | 16 | 0=Infinite 1-6553500s (100s granularity) | Present if needed |
| } |  |  |  |
| If (ROHC Parameter is needed) { |  |  | Present if needed |
| ROHC Max Context ID | 16 | Non-negative integer |  |
| Large Context IDs | 8 | 0: FALSE (Small Context ID) 1: TRUE (Large Context ID) 2–255: Reserved |  |
| ROHC MRRU | 16 | 0: no segmentation 1..65535: MRRU Maximum reconstructed reception unit |  |
| <insert>ROHC Profiles | variable | A set of nonnegative integers, where each integer indicates a 16 bit profile identifier of a ROHC profile supported by the decompressor. | </insert> |
| <insert>ROHC Feedback Channel | 32 | 0: no associated ROHC feedback Otherwise: SFID for ROHC feedback. | </insert> |
| } |  |  |  |
| … |  |  |  |