| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on AAI-RNG-REQ considering Direct handover in idle mode |  |
| Date Submitted | 2012-04-13 |  |
| Source(s) | Seho Kim, Hyunjeong Kang Samsung Electronics Co. LTD. Kenji Saito UQ Communications Inc. | seho42.kim@samsung.com kenji@uqc.jp |
| Re: | IEEE P802.16.1 Sponsor Ballot Recirculation #2 |  |
| Abstract | It proposes revisions to IEEE P802.16.1/D5 for clarification on AAI-RNG-REQ |  |
| Purpose | To be discussed and adopted for 802.16.1 D6 draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on AAI-RNG-REQ considering Direct handover in idle mode

# Introduction

According to the P802.16.1/D5, Paging Controller ID is only included in AAI-RNG-REQ in case of Network Re-Entry from Idle Mode. However, Full Network Re-Entry should be performed when an AMS in state of Idle Mode goes across between WirelessMAN-OFDMA R1 Reference System and WirelessMAN-Advanced System due to the lack of necessary information for Direct Handover. So re-assignment of IP Address should be needed in Direct Idle Handover, user’s session will be disconnected after Direct Idle Handover as a result.

Currently, AAI-RNG-REQ (Direct_Idle_HO) does not include AMS’s MAC address and AK Count so that ASN cannot continue AMS’s IP session

This contribution proposes to add AMS’s MAC address and AK count in AAI- RNG-REQ for Network Reentry from idle mode of AMS which has entered Idle Mode in R1 BS.

# Proposed text

[Remedy 1: Adopt the following change in line 48 on page 70 in IEEE P802.16.1/D5:]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Initial Offset for uplink power control (OffsetInitial) | 5 | The bit size represents power level ranging from –15 dB (0x00) to 16dB (0x1F) with 1 dB step The value is determined by AMS after successful initial ranging process. |  |
| }else if (Ranging Purpose Indication == 0b1110) { |  | // Network Reentry from idle mode of AMS which has entered idle mode in R1 BS |  |
| Paging Controller ID | 48 | The Paging Controller ID to which the AMS previously belonged in serving legacy BS |  |
| <insert>if (S-SFH Network Configuration bit == 0b1 or AMSID privacy is disabled){ |  |  |  |
| AMS MAC address | 48 | AMS’s real MAC address |  |
| } |  |  |  |
| If (CMAC indicator == 0b1){ |  |  |  |
| AK_COUNT | 16 | The AMS’s current value of the AK_COUNT, which is used to update the security keys in the T-ABS. | Shall be present if the AMS has a CMAC Tuple necessary to expedite security authentication. |
| } </insert> |  |  |  |
| }//end of Ranging Purpose Indication |  |  |  |
| for (i = 0; i < N_CSG_ID_Infos; i++) { |  | N_CSG_ID_Infos is the number of CSG ID Information blocks. 1≤ N_CSG_ID_Infos≤15 | Optional for loop. May be included for quick CSG membership detection or ABS reselection assistance. |
| Operator ID of the CSG Femtocell | 24 | The Operator ID of the CSG  Femtocell | Present if the Operator ID is different from the one of the ABS |
| for (j = 0; j < N_CSG_IDs; j++) { |  | N_CSG_IDs is the number of CSG IDs belongs to this Operator ID. |  |
| CSGID | variable | The CSGID within the Operator ID. It may be part of the BS ID, with certain bits inside indicating its length. If the CSG has single BS, it may be of maximum length, which is the LSB-24- bits of the full BS ID. |  |
| } |  |  |  |
| } |  |  |  |

[Remedy 2: Adopt the following change in line 36 on page 955 in IEEE P802.16.1/D5:]

NsEpCallSetup ::= SEQUENCE {

macAddress		 MACAddress,

macVersion 		MACVersion,

initialOffsetUlpc INTEGER (0..31),

...

}

<insert>NetworkReentryFromIdleModeR1 ::= SEQUENCE {

pagingControllerId		 PCID,

macAddress	MACAddress	OPTIONAL,

akCount	AKCount	OPTIONAL,

...

}</insert>

[Remedy 3: Adopt the following change in line 37 on page 959 in IEEE P802.16.1/D5:]

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Ranging Request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-RNG-REQ ::= SEQUENCE {

-- Indicate whether this message is protected by CM

cmacIndicator CMACI,

rangingPurposeDiffMessage CHOICE {

initialNetworkEntry InitialNetworkEntry,

handoverReentry HandoverReentry,

networkReentryFromIdleMode NetworkReentryFromIdleMode,

idleModeLocationUpdate LocationUpdate,

dcrModeExtension DCRModeExtension,

emergencyCallSetup EmergencyCallSetup, -- e.g., E911

-- Location update for updating service flow management encoding

-- of E-MBS flows

locationUpdateEmbsFlows LocationUpdate,

-- Location update for transition to DCR mode from idle mode

locationUpdateToDcrMode LocationUpdate,

-- Reentry from DCR mode, coverage loss or detection of

-- different ABS restart count

reentryFromDcr ReentryFromDCR,

-- Network reentry from a R1 BS

networkReentryFromR1 NetworkReentryFromR1,

-- Zone switch to MZONE from LZONE

zoneSwitch ZoneSwitch,

locationUpdatePowerDown LocationUpdate,

-- experiencing "femto interference"

femtoInterference FemtoInterference,

-- NS/EP Call Setup

nsEpCallSetup 	NsEpCallSetup,

<insert>networkReentryFromIdleModeR1  NetworkReentryFromIdleModeR1,</insert>

...

},

-- CSG information

csgInformation SEQUENCE (SIZE (1..15)) OF CsgInfoItem OPTIONAL,

...

}