| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on M2M abnormal power down report header |  |
| Date Submitted | 2012-05-02 |  |
| Source(s) | Soojung Jung, Jaesun Cha, Wooram Shin, Anseok Lee, Kwangjae Lim, Sungcheol Chang ETRI | Voice: +82-42-860-5928 E-mail: sjjung@etri.re.kr *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Sponsor Ballot(IEEE 802.16.1b/D3) |  |
| Abstract | This contribution proposes to clarify M2M abnormal power down report header. |  |
| Purpose | To be discussed and adopted by the 802.16 M2M TG. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Clarification on M2M abnormal power down report header

Soojung Jung, Jaesun Cha, Wooram Shin, Anseok Lee, Kwangjae Lim, Sungcheol Chang

ETRI

# Introduction

Maximum length of contents in MAC signaling header format is 36 bits. In current draft, the length of contents in M2M abnormal power down report header format is 56 or 40 bits. It is due to parameter “authentication code” which was replaced by other parameters, i.e., “EC”, “ROC”, and “CMAC”. Therefore, the parameter “authentication code” should be removed. Therefore, we propose to clarify the related texts.

# References

[1]IEEE P802.16.1b/D3, “DRAFT Amendment to IEEE Standard for WirelessMAN-Advanced Air interface for Broadband Wireless Access Systems: Enhancements to Support Machine-to-Machine Applications,” March. 2012

# Proposed text

We differentiated any texts modified by this contribution by using a different color as follows.

- Text added by this contribution: blue underline texts

- Text deleted by this contribution: red strikeout texts

-------------------------------Start of the text proposal-----------------------------------------------------------------------

6.2.2.1.3.10 M2M abnormal power down report header

Modify Table 13b as indicated

Table 13b – M2M abnormal power down report header format

| Syntax | Size(bits) | Notes |
| --- | --- | --- |
| M2M Abnormal Power Report Down Report (){ |  |  |
| FID | 4 | Flow Identifier. Set to 0b0010 |
| Type | 5 | MAC signaling header type = 0b01001 |
| Length | 3 | Indicates the length of the signaling header in bytes |
| STID | 12 | Indicates STID of the M2M device that transmits this M2M abnormal power down report signaling header. |
| STID-Valid-offset | 3 | Indicates STID-Valid-Offset of the M2M device that sends this M2M abnormal power down report signaling header. If the assigned STID is not shared with other M2M devices, M2M device shall set this field to zero. |
| Emergency Type | 1 | 0b0:power outage 0b1: Reserved |
| Authentication code | 20 |  |
| EC | 1 | 0: Unprotected 1: Protected using CMAC |
| If(EC==1) |  |  |
| ROC | 3 | 3 LSBs of Roll over counter( see 6.2.5.7 for details) |
| CMAC | 16 | CMAC( see 6.2.5.7 for details) |
| }else{ |  |  |
| Reserved | 3 2 |  |
| } |  |  |
| } |  |  |

------------------------------End of the text proposal--------------------------------------------------------------------------