| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on M2M abnormal power down reporting in idle state |  |
| Date Submitted | 2012-05-02 |  |
| Source(s) | Soojung Jung, Jaesun Cha, Wooram Shin, Anseok Lee, Kwangjae Lim, Sungcheol Chang ETRI Anshuman Nigam Samsung Electronics | Voice: +82-42-860-5928 E-mail: sjjung@etri.re.kr *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Sponsor Ballot(IEEE 802.16.1b/D3) |  |
| Abstract | This contribution proposes to clarify M2M abnormal power down reporting in idle state. |  |
| Purpose | To be discussed and adopted by the 802.16 M2M TG. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

# Clarification on M2M abnormal power down reporting in idle state

Soojung Jung, Jaesun Cha, Wooram Shin, Anseok Lee, Kwangjae Lim, Sungcheol Chang

ETRI

Anshuman Nigam

Samsung

# Introduction

According to baseline document [2], a Ranging Purpose Indication with value 0b1110 is used for other purpose (Network Reentry from idle mode of AMS which has entered Idle Mode in R1 BS). However, in current draft [1], a Ranging Purpose Indication with value 0b1110 is used for abnormal power down indication. So we need to change the value for abnormal power down indication.

# References

[1]IEEE P802.16.1b/D3, “DRAFT Amendment to IEEE Standard for WirelessMAN-Advanced Air interface for Broadband Wireless Access Systems: Enhancements to Support Machine-to-Machine Applications,” March. 2012

[2]IEEE P802.16.1/D6,” IEEE draft for WirelessMAN-Advanced Air interface for Broadband Wireless Access Systems:

# Proposed text

We differentiated any texts modified by this contribution by using a different color as follows.

- Text added by this contribution: blue underline texts

- Text deleted by this contribution: red strikeout texts

-------------------------------Start of the text proposal-----------------------------------------------------------------------

6.2.3.1  AAI-RNG-REQ

Modify Table 27 as indicated

Table 27 – AAI-RNG-RSP message field description

| Field | Size(bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Ranging Purpose Indication | 4 | 0b0000 = Initial network entry 0b0001 = HO reentry 0b0010 =Network reentry from idle mode … 0b1101= NS/EP call setup <del>0b1110 = Abnormal Power down indication 0b1110-0b1111 =Reserved </del> <ins> 0b1110 = Network Reentry from idle mode of AMS which has entered Idle mode in R1 BS 0b1111 = Ranging Purpose Indicator Extension</ins> |  |
| <ins> If (Ranging Purpose Indication == 0b1111) {</ins> |  |  |  |
| <ins> Ranging Purpose Indicator Extension</ins> | <ins> 3</ins> | <ins> 0b000 = Abnormal power down 0b001-0b111 Reserved</ins> |  |
| } |  |  |  |
| …… |  |  |  |
|  |  | //Abnormal or involuntary power down |  |
| } |  |  |  |
| …. |  |  |  |
| Retrials | 2 | The number of failed trials in this ranging process Bits 0-1: Indicates the number of retrials in the channel ranging access as follows 00 – Success in the first attempt 01 – Success in the second attempt 10 – Success in the third attempt 11 – Success in the 4th or later attempt | May be included by M2M devices after initial ranging during network entry or reentry, periodic ranging, or HO ranging |

6.2.29.2 Abnormal power down reporting in idle state

Modify the text on page 59,line 29 as indicated

The ABS, upon receiving the ranging code, may include a CDMA Allocation  A-MAP IE in the next frame identifying the M2M device and provide an allocation sufficiently large to allow the M2M device to transmit an AAI-RNG-REQ message including a Ranging Purpose Indication and the CMAC Tuple. Upon receiving this allocation, the M2M device shall transmit an AAI-RNG-REQ message including a Ranging Purpose Indication with value <del>0b1110</del> <ins>0b1111 and Ranging Purpose Indicator Extension with value 0b000</ins>(power outage) and a valid CMAC Tuple. The M2M device shall not repeat sending of a ranging code if it does not receive an allocation from the ABS.

------------------------------End of the text proposal--------------------------------------------------------------------------