| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Alternative Path in 16.1a Network |  |
| Date Submitted | 2012-05-04 |  |
| Source(s) | Anseok Lee, Eunkyung Kim, Jaesun Cha, Wooram Shin, Kwangjae Lim, Dongseung Kwon ETRI | E-mail: alee@etri.re.kr |
| Re: | In response to LB #38a on P802.16.1a/D2 |  |
| Abstract | This provides AWD text proposals of clarification on Alternative Path in IEEE 802.16.1a Network. |  |
| Purpose | To be discussed and adopted by 802.16 GRIDMAN TG |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Clarification of Alternative Path in 16.1a Network

Anseok Lee, Eunkyung Kim, Jaesun Cha, Wooram Shin, Kwangjae Lim, Dongseung Kwon

ETRI

# Introductions

This contribution provides clarification on Alternative Path in 16.1a Network.

# Proposed Text for the 802.16.1a Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16.1a Amendment Draft Standard

The text in RED color: the removal of existing 802.16.1a Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16.1a Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy 1: Accept the following modifying to Section 6.12.7.1.3]

- 6.12.7.1.3 Alternative path information and fast switching to HR-infrastructure station

The alternative path information is transmitted to HR-MSs by the serving HR-infrastructure station using the AAI-HO-CMD message with mode = 0b11, Extended HO Mode = 0b00 (Alternative Path), HO Reentry Mode = 0b0 and one target HR-infrastructure station. In addition, the message facilitates a fast network reentry procedure defined by a trigger condition, as described below.

When the trigger condition specified in the TLV of the last recent AAI-HO-CMD described in 6.2.3.12 is met, the alternative path is activated by an HR-MS. If the action time is non-zero, the HR-MS shall perform network reentry as described in 6.2.6.3.5 after the action time expires. The action time allows the target HR-station of the alternative path to perform mode change into an HR-infrastructure station when necessary.

To support switching to alternative path with fast network reentry, the AAI-HO-CMD message with mode = 0b11, Extended HO Mode = 0b00 (Alternative Path), HO Reentry Mode = 0b0 and CDMA_RNG_FLAG = 1 and a recommended ranging code and slot from the ranging region of the target HR-infrastructure station of the alternative path. This reduces contention in ranging when multiple HR-MSs perform fast network recovery. However, how to select the ranging code and slot is out of this specification.

The MAC context information of HR-MS may be shared between the serving and target HR-infrastructure stations to facilitate optimized network reentry through the alternative path. An indication of whether MAC context information of the subordinate HR-MS is shared by infrastructure stations shall be indicated in the AAI-HO-CMD message.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

[1] IEEE P802.16.1a/D2, “Draft Amendment for IEEE Standard for WirelessMAN-Advanced Air Interface for Broadband Access Systems - Amendment: Higher Reliability Networks”