| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of HARQ parameters in talk-around direct communication |  |
| Date Submitted | 2012-05-04 |  |
| Source(s) | Sungcheol Chang, Seokki Kim, Miyoung Yun, Won-Ik Kim, Hyun Lee, Kim Eunkyung Kim, Sungkyung Kim, Chulsik Yoon ETRI | E-mail: scchang@etri.re.kr |
| Re: | Letter Ballot #38a on P802.16.1a/D2 |  |
| Abstract | This provides text proposals of clarifying HARQ parameters of talk-around direct communications |  |
| Purpose | To be discussed and adopted by 802.16 GRIDMAN TG |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

# Clarification of HARQ parameters in talk-around direct communication

Sungcheol Chang, Seokki Kim, Miyoung Yun, Won-Ik Kim, Hyun Lee, Kim Eunkyung Kim,

Sungkyung Kim, Chulsik Yoon

ETRI

# Introductions

This contribution provides clarification of HARQ parameters of talk-around direct communications between HR-MSs as the followings:

- Maximum number of HARQ retransmission

A PHY burst is delivered to a receiver and its delay is limited by the number of HARQ retransmission. The maximum transmission delay of the burst is 100msec because the maximum delay of HARQ feedback is four TDC frames (80ms). To get the operation characteristic the maximum number of HARQ retransmission field is set to a value that the delay of packet for service flow is allowed. For voice traffic the maximum number of HARQ retransmission is set to 1.

The maximum delay of a PHY burst depends on an allowed maximum delay of service flows established between HR-MSs. When the allowed maximum delay is modified by establishment, modification, and deletion of service flows, the maximum delay also is changed. This contribution provides that the maximum number of HARQ retransmission field is updated for all the procedures of service flow.

# Proposed Text for P802.16.1a/D2

Note:

The text in BLACK color: the existing text in the 802.16.1a Draft Standard Text

The text in RED color: the removal of existing 802.16.1a Draft Standard Text

The text in BLUE color: the new text added to the 802.16.1a Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

## [Remedy1: Modify the following text in Section 6.2.3.65.31 in P802.16.1a/D2]

- 6.2.3.65.31 AAI-DC-DSA-CMD

An HR-MS transmits an AAI-DC-DSA-CMD message to create a new service flow on one-way point-to-point and point-to-multipoint TDC links.

Table 106a – AAI-DC-DSA-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender. | Shall always be present |
| For (i=0; i<N_Flow_Est; i++) { |  | N_Flow_Est is the number of flows on which the sender of this message sends MAC PDUs. |  |
| FID | 4 | Flow identifier assigned by the source of packets on the flow |  |
| Traffic Priority | 3 | 0 to 7: Higher numbers indicate higher priority Default: 0 |  |
| CS Specification Parameters | 8 | 0–15: Reserved 16: Voice Codec G.729A 17: Voice Codec AMR 18–255: Reserved |  |
| MAC Header Type | 1 | Indicates whether AGMH or SPMH is presented at the start of MAC PDUs of the service flow. 0 : AGMH (Advanced Generic MAC Header) 1 : SPMH (Short-Packet MAC header) default value is 0. |  |
| } |  |  |  |
| For (i=0; i<N_ResourceID_HARQ_Tx; i++) { |  | N_ResourceID_HARQ_Tx is the number of ResourceID that a PHY burst is retransmitted on the resources and its Maximum Number of HARQ Retransmission parameter is changed. |  |
| Resource ID | 4 | Resource ID |  |
| Maximum Number of HARQ Retransmission | 2 | Indicates maximum number of PHY burst retransmission for HARQ operation |  |
| Reserved | 2 |  |  |
| } |  |  |  |
| Reserved |  |  |  |

        - AAI-DC-DSC-CMD

An HR-MS transmits an AAI-DC-DSC-CMD message to change the parameters of an existing service flow on one-way point-to-point and point-to-multipoint TDC links.

Table 106b – AAI-DC-DSC-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender. | Shall always be present |
| For (i=0; i<N_Flow_Chg; i++) { |  | N_Flow_Chg is the number of flows on which the sender of this message sends MAC PDUs. | Present if 0 < N_Flow_Chg |
| FID | 4 | Flow identifier assigned by the source of packets on the flow |  |
| Traffic Priority | 3 | 0 to 7: Higher numbers indicate higher priority Default: 0 |  |
| CS Specification Parameters | 8 | 0–15: Reserved 16: Voice Codec G.729A 17: Voice Codec AMR 18–255: Reserved |  |
| MAC Header Type | 1 | Indicates whether AGMH or SPMH is presented at the start of MAC PDUs of the service flow. 0 : AGMH (Advanced Generic MAC Header) 1 : SPMH (Short-Packet MAC header) default value is 0. |  |
| } |  |  |  |
| For (i=0; i<N_ResourceID_HARQ_Tx; i++) { |  | N_ResourceID_HARQ_Tx is the number of ResourceID that a PHY burst is retransmitted on the resources and its Maximum Number of HARQ Retransmission parameter is changed. |  |
| Resource ID | 4 | Resource ID |  |
| Maximum Number of HARQ Retransmission | 2 | Indicates maximum number of PHY burst retransmission for HARQ operation |  |
| Reserved | 2 |  |  |
| } |  |  |  |

        - AAI-DC-DSD-CMD

An HR-MS transmits an AAI-DC-DSD-CMD message to delete an existing service flow on one-way point-to-point and point-to-multipoint TDC links.

Table 106c – AAI-DC-DSD-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender. | Shall always be present |
| For (i=0; i<N_Flow_Rel; i++) { |  | N_Flow_Rel is the number of flows which the sender of this message is to release. Range [0..12] |  |
| FID | 4 | Flow identifier assigned by the source of packets on the flow |  |
| } |  |  |  |
| For (i=0; i<N_ResourceID_HARQ_Tx; i++) { |  | N_ResourceID_HARQ_Tx is the number of ResourceID that a PHY burst is retransmitted on the resources and its Maximum Number of HARQ Retransmission parameter is changed. |  |
| Resource ID | 4 | Resource ID |  |
| Maximum Number of HARQ Retransmission | 2 | Indicates maximum number of PHY burst retransmission for HARQ operation |  |
| Reserved | 2 |  |  |
| } |  |  |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

[1] IEEE P802.16.1a/D2, “WirelessMAN-Advanced Air Interface for Broadband Access Systems -  Draft Amendment: Higher Reliability Networks,” April 2012.

# Title of Contribution

[Authors of Contribution]

[Affiliation of authors, per <http://standards.ieee.org/faqs/affiliationFAQ.html>]

# Template Instructions

This page provides instructions on using the template and submitting a contribution. Before submitting, delete these instructions or replace them with your text to be submitted.

# Document Format

Edit the cover page. Replace the square-bracketed sections with the correct text for your document. Eliminate the brackets. Leave the non-bracketed words on the cover page. Limit the cover page information to one page, shrinking the font if necessary.

Begin the main document with a page break after the cover page. Restate the document title and author name. You should use heading styles that support an outline view of the document.

# Header

The document number <http://ieee802.org/16/submit.html> goes in the upper right of the header. It takes the form “IEEE 802.16-YY-#####-RR-FFFF”, where:

• Every IEEE 802.16 document number begins with “IEEE 802.16-”.

• Following “IEEE 802.”, the remainder of the document number (“16-YY-#####-RR-FFFF”) is the Document Control Number (DCN), used by IEEE Mentor as the basis of the file name.

• “YY” represents the final two digits of the calendar year of original submission, as assigned by the IEEE Mentor documentation system.

• “####” represents the document serial number, as assigned by the IEEE Mentor documentation system. The serial number is uniquely assigned for each calendar year.

• “RR” represents the revision number, as assigned by the IEEE Mentor documentation system. The initial version of the document uses the revision number “00”.

• “FFFF” represents the Mentor “file code” <http://ieee802.org/16/submit.html>. It is selected by the submitter as the most appropriate identifier among the list of  IEEE 802.16 file codes in the IEEE Mentor documentation system.

# Fonts

Use only the fonts “Times” (or “Times New Roman”), “Arial” (or “Helvetica”),  and “Symbol,” since these fonts need not be embedded in PDF files. You may use other fonts only if necessary.

# Document Submission

To contribute a document to the IEEE 802.16 Working Group or a subgroup, create it using this template or another template at <http://ieee802.org/16/submit.html>. (If it cannot be put into one of the available formats, then create at least a cover page using an available template.) Submit the document following the instructions in the Call for Contributions or the IEEE 802.16 Document Submission Instructions <http://ieee802.org/16/submit.html>.