| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Message Confirm Indication in response to MAC control message in talk-around direct communication |  |
| Date Submitted | 2012-05-04 |  |
| Source(s) | Sungcheol Chang, Seokki Kim, Miyoung Yun, Won-Ik Kim, Hyun Lee, Kim Eunkyung Kim, Sungkyung Kim, Chulsik Yoon ETRI | E-mail: scchang@etri.re.kr |
| Re: | Letter Ballot #38a on P802.16.1a/D2 |  |
| Abstract | This provides text proposals for Message Change Indication in response to MAC control message in talk-around direct communication |  |
| Purpose | To be discussed and adopted by 802.16 GRIDMAN TG |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

# Message Confirm Indication in response to MAC control message in talk-around direct communication

Sungcheol Chang, Seokki Kim, Miyoung Yun, Won-Ik Kim, Hyun Lee, Kim Eunkyung Kim, Sungkyung Kim, Chulsik Yoon

ETRI

# Introductions

This contribution provides a Message Confirm Indication on feedback channel in response to MAC control message as the followings:

- Definition of Message Confirm Indication on feedback channel

- Receiving Message Confirm Indication in response

- Priority to other feedback sequences on feedback channel

# Proposed Text for P802.16.1a/D2

Note:

The text in BLACK color: the existing text in the 802.16.1a Draft Standard Text

The text in RED color: the removal of existing 802.16.1a Draft Standard Text

The text in BLUE color: the new text added to the 802.16.1a Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

## [Remedy1: Modify the following text in Section 6.12.2.3.1.5.1]

6.12.2.3.1.5.1 Control connections

One pair of bi-directional unicast control connectionsUnicast control connections are automatically established to send MAC control messages when two HR-MSs perform twoone-way unicast link establishment with two-way handshake of control messages. MAC control messages sent on one-way unicast connection are confirmed by Message Confirm Indication feedback sequences if the target HR-MS receives them successfully.

One multicast control connection is Multicast control connections are established when HR-MSs perform multicast link establishment multicasting one-way MAC control message.

## [Remedy2: Modify the following text in Section 6.12.2.3.2.8]

6.12.2.3.2.8 HARQ

HARQ operation is enabled on one-way unicast link only.

The sending HR-MS sends a data packet on dedicated channel in a DC frame. If the receiving HR-MS fails to decode the data packet successfully, it sends NAK feedback within four DC frames. There are four types of NAK feedbacks and each type of NAK feedbacks indicates a frame number that data packet corrupts. The sending HR-MS retransmits the data packet with NAK indicated frame number at next frame.

If the receiving HR-MS has no feedback, ACK feedback are sent on the supplementary channel. It indicates that the receiving HR-MS receives all the unacknowledged packets on the dedicated channel successfully.

When the receiving HR-MS receives a MAC control message successfully, it sends Message Confirm Indication feedback sequence on feedback channel in response to the MAC control message on dedicated channel. Message Confirm Indication feedback sequence has priority to the other feedback sequences on feedback channel. On decoding the feedback sequence the sending HR-MS knows that the receiving HR-MS receives the MAC control message successfully.

## [Remedy3: Modify the following text in Section 6.12.2.3.2.4.3]

6.12.2.3.2.4.3 Feedback channel

The Sup-CH can be used to transmit the feedback channel, that includes ACK channel, NAK channel, MCS Change Confirm , RCHG Indication, etc carries feedback sequences including ACK, NAKs, Message Confirm Indication, RCHG Request Indication, Send Indication, Token Request Indication, etc. The feedback channel is transmitted using the slots which are not used by the ranging channel and the CQI channel. The feedback channel uses the same codeword set and symbol sequence to subcarrier mapping as the CQI channel. The codeword sequences and mapping of feedback channel are defined in .

Table 199—Sequences and mapping of feedback channel

| Index | Sequence | Usage |
| --- | --- | --- |
| 0 | 1111111111 | ACK |
| 1 | 0010110001 | NAK for frame 0 |
| 2 | 0100100110 | NAK for frame 1 |
| 3 | 1001101000 | NAK for frame 2 |
| 4 | 1011000100 | NAK for frame 3 |
| 5 | 0110001010 | MCS Change Confirm Message Confirm Indication |
| 6 | 0000011101 | RCHG Request Indication |
| 7 | 1101010011 | Send Indication |
| 8 | 1100011000 | Token Request Indication |
| 9 | 0001010110 | Reserved |
| 10 | 0111000001 | Reserved |
| 11 | 1010001111 | Reserved |
| 12 | 1000100011 | Reserved |
| 13 | 0101101101 | Reserved |
| 14 | 0011111010 | Reserved |
| 15 | 1110110100 | Reserved |

## [Remedy4: Modify the following text in Section 6.12.2.3.2.4]

6.12.2.3.2.4 Supplementary Channel

There are two subframes to transmit supplementary channel in a super frame. One subframe for supplementary channel is located in the first uplink subframe of the second frame, and the other subframe for supplementary channel is located in the first uplink subframe of the fourth frame. A supplementary sub-channel is one-to-one mapped with each dedicated sub-channel. By using the supplementary sub-channel, the following PHY signalings and short feedbacks messages related with the corresponding dedicated sub-channel are transmitted.

- PHY signalings: periodic ranging sequence, CQIsounding signal et. al.

- Short feedbacks messages: ACK, NACKs, MCS Change ConfirmMessage Confirm Indication, RCHG Request iIndication, Send iIndication, Token rRequest iIndication and CQI.

## [Remedy5: Update the texts with the following two proposals]

- Replace “dedicated sub-channel” with “dedicated channel” in IEEE P802.16.1a/D2.

- Replace “supplementary sub-channel” with “supplementary channel” in IEEE P802.16.1a/D2.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

[1] IEEE P802.16.1a/D2, “WirelessMAN-Advanced Air Interface for Broadband Access Systems -  Draft Amendment: Higher Reliability Networks,” April 2012.

# Title of Contribution

[Authors of Contribution]

[Affiliation of authors, per <http://standards.ieee.org/faqs/affiliationFAQ.html>]

# Template Instructions

This page provides instructions on using the template and submitting a contribution. Before submitting, delete these instructions or replace them with your text to be submitted.

# Document Format

Edit the cover page. Replace the square-bracketed sections with the correct text for your document. Eliminate the brackets. Leave the non-bracketed words on the cover page. Limit the cover page information to one page, shrinking the font if necessary.

Begin the main document with a page break after the cover page. Restate the document title and author name. You should use heading styles that support an outline view of the document.

# Header

The document number <http://ieee802.org/16/submit.html> goes in the upper right of the header. It takes the form “IEEE 802.16-YY-#####-RR-FFFF”, where:

• Every IEEE 802.16 document number begins with “IEEE 802.16-”.

• Following “IEEE 802.”, the remainder of the document number (“16-YY-#####-RR-FFFF”) is the Document Control Number (DCN), used by IEEE Mentor as the basis of the file name.

• “YY” represents the final two digits of the calendar year of original submission, as assigned by the IEEE Mentor documentation system.

• “####” represents the document serial number, as assigned by the IEEE Mentor documentation system. The serial number is uniquely assigned for each calendar year.

• “RR” represents the revision number, as assigned by the IEEE Mentor documentation system. The initial version of the document uses the revision number “00”.

• “FFFF” represents the Mentor “file code” <http://ieee802.org/16/submit.html>. It is selected by the submitter as the most appropriate identifier among the list of  IEEE 802.16 file codes in the IEEE Mentor documentation system.

# Fonts

Use only the fonts “Times” (or “Times New Roman”), “Arial” (or “Helvetica”),  and “Symbol,” since these fonts need not be embedded in PDF files. You may use other fonts only if necessary.

# Document Submission

To contribute a document to the IEEE 802.16 Working Group or a subgroup, create it using this template or another template at <http://ieee802.org/16/submit.html>. (If it cannot be put into one of the available formats, then create at least a cover page using an available template.) Submit the document following the instructions in the Call for Contributions or the IEEE 802.16 Document Submission Instructions <http://ieee802.org/16/submit.html>.