| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of MS forwarding to network in talk-around direct communications |  |
| Date Submitted | 2012-05-04 |  |
| Source(s) | Sungcheol Chang, Seokki Kim, Miyoung Yun, Won-Ik Kim, Hyun Lee, Kim Eunkyung Kim, Sungkyung Kim, Chulsik Yoon ETRI | E-mail: scchang@etri.re.kr |
| Re: | Letter Ballot #38a on P802.16.1a/D2 |  |
| Abstract | This provides text proposals for clarification of MS forwarding to network in talk-around direct communications |  |
| Purpose | To be discussed and adopted by 802.16 GRIDMAN TG |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

# Clarification of MS forwarding to network in talk-around direct communications

Sungcheol Chang, Seokki Kim, Miyoung Yun, Won-Ik Kim, Hyun Lee, Kim Eunkyung Kim, Sungkyung Kim, Chulsik Yoon

ETRI

# Introductions

This contribution provides clarifications of resource change procedure as the followings:

- Message definitions : AAI-DC-ML-ADV, AAI-DC-LU-REQ, AAI-DC-LU-RSP, AAI-DC-MS-ADV

# Proposed Text for P802.16.1a/D2

Note:

The text in BLACK color: the existing text in the 802.16.1a Draft Standard Text

The text in RED color: the removal of existing 802.16.1a Draft Standard Text

The text in BLUE color: the new text added to the 802.16.1a Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

## [Remedy1: Modify the following text in Section 6.2.3]

6.2.3 MAC control messages

Change Table 26 as indicated:

Table 26—MAC control messages

| No. | Functional Areas | Message names | Message description | Security | Connection |
| --- | --- | --- | --- | --- | --- |
| …… | …… | …… | …… | …… | …… |
| TBD | Forwarding MS List | AAI-DC-MMML-ADV | MS list Advertisement |  | Broadcast or multicast or unicast |
| TBD | Forwarding MS list Update | AAI-DC-LU-REQ | MS List Update Request |  | Unicast |
| TBD | Forwarding MS list Update | AAI-DC-LU-RSP | MS List Update Response |  | Unicast |
| …… | …… | …… | …… | …… | …… |

## [Remedy2: Modify the following text in Section 6.2.3.65.48]

6.2.3.65.48 AAI-DC-MMML-ADV

An HR-BS transmits an AAI-DC-MMML-ADV message to advertise an MS list for HR-MS forwarding.

Table 106uu1 – AAI-DC-ML-ADV message field description

| Field | Size (bits) | Value / Description | Condition |
| --- | --- | --- | --- |

## [Remedy3: Modify the following text in Section 6.2.3.65.49]

6.2.3.65.49 AAI-DC-LU-REQ

A forwarding HR-MS transmits an AAI-DC-LU-REQ message to update a MS list for HR-MS forwarding.

Table 106uu2 – AAI-DC-LU-REQ message field description

| Field | Size (bits) | Value / Description | Condition |
| --- | --- | --- | --- |

## [Remedy4: Modify the following text in Section 6.2.3.65.50]

6.2.3.65.50 AAI-DC-LU-RSP

An HR-BS transmits a AAI-DC-LU-RSP message in response to a received AAI-DC-LU-REQ.

Table 106uu3 – AAI-DC-LU-RSP message field description

| Field | Size (bits) | Value / Description | Condition |
| --- | --- | --- | --- |

## [Remedy5: Modify the following text in Section 6.12.3.3]

6.12.3.3 Talk-around HR-MS forwarding to network

6.12.3.3.1 Medium access control

HR-MS discoveries

A forwarding HR-MS shall maintain a list of HR-MSs that are in communication range using talk-around direct communication.

An HR-BS shall maintain a list of HR-MSs that are collected from forwarding HR-MSs for HR-MS forwarding. An HR-BS broadcast the HR-MS list for HR-MS forwarding to forwarding HR-MSs using AAI-DC-MMML-ADV message. When new HR-MS is added or HR-MSs are deleted, the forwarding HR-MS shall update the HR-MS list by an exchange of MAC Management messages with HR-BS such as AAI-DC-LU-REQ/RSP.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

[1] IEEE P802.16.1a/D2, “WirelessMAN-Advanced Air Interface for Broadband Access Systems -  Draft Amendment: Higher Reliability Networks,” April 2012.

# Title of Contribution

[Authors of Contribution]

[Affiliation of authors, per <http://standards.ieee.org/faqs/affiliationFAQ.html>]

# Template Instructions

This page provides instructions on using the template and submitting a contribution. Before submitting, delete these instructions or replace them with your text to be submitted.

# Document Format

Edit the cover page. Replace the square-bracketed sections with the correct text for your document. Eliminate the brackets. Leave the non-bracketed words on the cover page. Limit the cover page information to one page, shrinking the font if necessary.

Begin the main document with a page break after the cover page. Restate the document title and author name. You should use heading styles that support an outline view of the document.

# Header

The document number <http://ieee802.org/16/submit.html> goes in the upper right of the header. It takes the form “IEEE 802.16-YY-#####-RR-FFFF”, where:

• Every IEEE 802.16 document number begins with “IEEE 802.16-”.

• Following “IEEE 802.”, the remainder of the document number (“16-YY-#####-RR-FFFF”) is the Document Control Number (DCN), used by IEEE Mentor as the basis of the file name.

• “YY” represents the final two digits of the calendar year of original submission, as assigned by the IEEE Mentor documentation system.

• “####” represents the document serial number, as assigned by the IEEE Mentor documentation system. The serial number is uniquely assigned for each calendar year.

• “RR” represents the revision number, as assigned by the IEEE Mentor documentation system. The initial version of the document uses the revision number “00”.

• “FFFF” represents the Mentor “file code” <http://ieee802.org/16/submit.html>. It is selected by the submitter as the most appropriate identifier among the list of  IEEE 802.16 file codes in the IEEE Mentor documentation system.

# Fonts

Use only the fonts “Times” (or “Times New Roman”), “Arial” (or “Helvetica”),  and “Symbol,” since these fonts need not be embedded in PDF files. You may use other fonts only if necessary.

# Document Submission

To contribute a document to the IEEE 802.16 Working Group or a subgroup, create it using this template or another template at <http://ieee802.org/16/submit.html>. (If it cannot be put into one of the available formats, then create at least a cover page using an available template.) Submit the document following the instructions in the Call for Contributions or the IEEE 802.16 Document Submission Instructions <http://ieee802.org/16/submit.html>.