| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Path management |  |
| Date Submitted | 2012-05-11 |  |
| Source(s) | Hoang Vinh Dien, Ming-Tuo Zhou, Xin Zhang, Liru Lu (Alina), Masayuki Oodo, Hiroshi Harada National Institute of Information and Communications Technology (NICT) | E-mail: hvdien@nict.com.sg; mingtuo@nict.com.sg; zhangxin@nict.com.sg; liru@nict.com.sg; moodo@nict.go.jp; harada@nict.go.jp |
| Re: | In response to Letter Ballot Recirc #38a on P802.16.1a/D2 |  |
| Abstract | This provides a clarification on message format AP-NBR-REP alternative path in 802.16.1a networks |  |
| Purpose | To be discussed and adopted by TGn for draft 802.16.1a AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Clarification on Message formats for determining alternative path in path management of 802.16.1a GRIDMAN networks

Hoang Vinh Dien, Ming-Tuo Zhou, Xin Zhang, Liru Lu (Alina), Masayuki Oodo, Hiroshi Harada

NICT

# Introduction

# There's a difference between PM-NBR-REQ message in IEEE.802.16Rev3 and AP-NBR-REQ message in IEEE802.16.1a, it should be corrected.

# Text Proposal in AWD

The text in BLACK color: the existing text in the Amendment Draft Standard

The text in RED color: the removal of existing Amendment Draft Standard Text

The text in BLUE color: the new text added to the Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Insert the following text into Table 106yy ---- AP-NBR-REP message field and description  of 802.16.1a AWD (page 88, line 6)]

Table 1 ---- AP-NBR-REP message field and description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| For (i = 0; i < Number of neighbor HR-MSs; i ++){ |  |  |  |
| Index | 4 | Index of the neighbor HR-MSs |  |
| SINR | 4 | Indicates the received SINR of the ranging code from neighbor HR-MSs. The 4 bit value from 0b0000 to 0b1111 represent values among {–9, –8.5, –8, –7.5, –7, –6.5,–6, –5.5, –5, –4.5, –4, –3.5, –3, –2.5,–2, –1.5} dB |  |
| Hop | 2 | Number of hop to HR-BS 0b00: 1 hop 0b01: 2 hop 0b10: 3 hop or more 0b11: reserved |  |
| } |  |  |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, March 2012.

- IEEE 802.16n-11/0009r1, “802.16n Amendment Working Draft,” May 2011.