| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modified downlink multicast control channel in IEEE 802.16.1a |  |
| Date Submitted | 2012-07-09 |  |
| Source(s) | Sungkyung Kim, Sungcheol Chang, Chulsik Yoon, Won-Ik Kim, Hyun Lee, Miyoung Yun, Seokki Kim ETRI | E-mail: scchang@etri.re.kr |
| Re: | Letter Ballot #38b on P802.16.1a/D3 |  |
| Abstract | This provides text proposals for downlink control of multicast operation on GRIDMAP Draft Standard |  |
| Purpose | To be discussed and adopted by 802.16 GRIDMAN TG |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

# Modified Downlink Multicast Control Channel in IEEE 802.16.1a

Sungkyung Kim, Sungcheol Chang, Chulsik Yoon, 
Won-Ik Kim, Hyun Lee, Miyoung Yun, Seokki Kim

ETRI

# Introductions

This contribution provides a change on the downlink control for multicast communication to enhance the multicast performance. From contribution [2], we recognized the need for fragmented transmission under the MCS levels with low coding rate. To increase the resource utilization, “TI” field was inserted into HR-Multicast DL Assignment A-MAP IE at the session #78. Nevertheless, using “TI” is not the best solution because the restriction on resource allocation during the Lifetime interval is still remained. Hence, to improve power-saving operation and resource utilization for multicast communication, we propose something to modify Multicast Control Channel (i.e., HR-Multicast DL Assignment A-MAP IE).

Unlike Persistent Allocation, HR-Multicast DL Assignment A-MAP IE is transmitted every allocation for HR-MS(s) in idle mode or sleep mode. HR-Multicast DL Assignment A-MAP IE must not change during the Lifetime. So, HR-MS does not need to receive the HR-Multicast DL Assignment A-MAP IE before the expiration of Lifetime. However, in practice, HR-MS parses all MAP IEs to find its own MAP IE(s) in awake/listening interval. That is, the benefit of using Lifetime is not much as we think. Moreover, due to randomness of traffic generation, it is not easy for HR-BS to select the proper value of Lifetime. Figure 1 shows the outage rate of PTT service for multicast communication using HR-Multicast DL Assignment A-MAP IE. As shown the figure, when Lifetime is set to more than 2 superframes (8frames), it’s hard to satisfy QoS of PTT service.

Figure 1. outage rate of multicast based PTT service with varying Lifetime
(Number of PTT Groups: 60/sector, 10MHz, MCS of multicast burst: QPSK, 1/6)

# Modified HR-Multicast DL Assignment A-MAP IE

To provide fragmented transmission of multicast traffic and to help power-saving operation of HR-MS, Next Allocation Frame Interval and Next Allocation Subframe Index of HR-Multicast DL Assignment A-MAP IE is proposed. And, Lifetime and persistent allocation rule during the Lifetime are deleted from the HR-Multicast DL Assignment A-MAP IE. For the freedom of resource allocation, Resource allocation flag and dynamic allocation information for multicast burst are considered. Figure 2 shows an example of multicast transmission with modified HR-Multicast DL Assignment A-MAP IE.

Figure 2- an example of multicast transmission with modified HR-Multicast DL Assignment A-MAP IE

# Proposed Text for P802.16.1a/D3

Note:

The text in BLACK color: the existing text in the 802.16.1a Draft Standard Text

The text in RED color: the removal of existing 802.16.1a Draft Standard Text

The text in BLUE color: the new text added to the 802.16.1a Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

## [Remedy: Modify the following text in Section 6.12.9.2.1]

6.12.9.2.1 downlink control channel for multicast communication

HR-multicast control channel (i.e., HR-Multicast DL Assignment A-MAP IE) carries configuration information (including allocation/change/release) the information of resource allocation and the transmission time of next HR-Multicast DL Assignment A-MAP IE for multicast communication for on multicast zone in an HR-BS. In HR-Multicast DL Assignment A-MAP, Next Allocation Frame Interval and Next Allocation Subframe Index indicate that a position of next HR-Multicast DL Assignment A-MAP IE. Resource allocation flag indicates whether the HR-Multicast DL Assignment A-MAP IE contains the information of Multicast burst allocation or not. When Both Next Allocation Frame Interval and Next Allocation Subframe Index are set to 0b000, it indicates that the Multicast DL Assignment A-MAP IE does not include the information of next transmission time. It means the transmission of the multicast connection will be stopped for a certain time. To resume the transmission for multicast connection, HR-BS should transmit HR-Multicast DL Assignment A-MAP IE. allocation period indicates a period of persistent allocation of multicast resource and Lifetime is timer indicating the next instance of HR-Multicast DL-Assignment A-MAP IE. During the allocation period, transmission indication indicates whether multicast resource is fragmented. If the transmission indication (TI) is set to 00, it indicates no fragmented traffic is transmitted until the next allocation instance. If TI is set to 01, it indicates the first fragmented traffic and more fragmented traffic is expected to transmit until the TI is set to 11. More fragmented traffic is transmitted with the value of TI setting to 10 or 11. If TI is set to 10, it indicates more fragmented traffic is transmitting until it is set to 11. If TI is set to 11, no more fragmented traffic (i.e., last fragmented traffic) is transmitted. Unless the Lifetime expires, this HR-Multicast DL Assignment A-MAP excluding the value of TI, dose not change during the allocation duration. At the time the Lifetime expires, the HR-Multicast DL Assignment A-MAP shall change or release the allocation.

Table 208 – HR-Multicast DL Assignment A-MAP IE*

| Syntax | Size (bit) | Description/Notes |
| --- | --- | --- |
| HR-Multicast_DL_Assignment_A-MAP_IE() { |  |  |
| A-MAP IE Type | 4 | HR-Multicast DL Assignment A-MAP IE |
| Allocation periodNext Allocation Frame Interval | 23 | Period of persistent allocation of multicast resource. If (Allocation Period==0b00), it indicates the deallocation of persistent resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 6 frames Next transmission allocation interval of multicast DL assignment A-MAP IE. If (Next Allocation Frame Interval == 0b000) and (Next Allocation subframe Index == 0b000), it indicates that the Multicast DL assignment A-MAP IE dose not express the transmission time of next Multicast DL assignment A-MAP IE. 0b000: current frame 0b001: 1 frame 0b010: 2 frames 0b011: 3 frames 0b100: 4 frames 0b101: 8 frames 0b110 ~ 0b111: reserved |
| Next Allocation Subframe Index | 3 | Subframe index in the next allocation frame. When the Next Allocation Frame Interval ==0b000 and Resource allocation flag == 0b 1(the current frame is allocated), the Next Allocation Subframe Index shall be one of the subsequent subframes in this frame. |
| Resource allocation flag | 1 | It indicates whether the current Multicast DL Assignment A-MAP IE allocates a Multicast Data Burst or not. 0b0: not included 0b1: included |
| If (Allocation Period == 0b00) { |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long TTI Indicator | 1 | Indicates number for AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default TTI) 0b1: 4 DL AAI subframe for FDD or all DL AAI subframes for TDD (long TTI) |
| Reserved | 22 |  |
| } else if(Allocation Period != 0b00) { |  |  |
| If (Resource allocation flag == 0b1) { |  |  |
| Isizeoffset | 5 | Offset used to compute burst size index |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Multi-layer encoding 0b11: CDR |
| If (MEF ==0b01) { |  | Parameter for vertical encoding |
| Mt | 3 | Number of streams in transmission Mt<=Nt Nt: Number of transmit antennas at the HR-BS 0b000: 1 stream 0b001: 2streams 0b010: 3streams 0b011: 4streams 0b100: 5streams 0b101: 6streams 0b110: 7streams 0b111: 8streams |
| Reserved | 1 |  |
| } else if (MEF == 0b10) { |  | Parameters for multi-layer encoding |
| Si | 4 | Index to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO, and the modulation constellation of paired user in the case of 2 stream transmission 0b0000: 2 streams with PSI=stream1 and other modulation = QPSK 0b0001: 2 streams with PSI=stream1 and other modulation = 16QAM 0b0010: 2 streams with PSI=stream1 and other modulation = 64QAM 0b0011: 2 streams with PSI=stream1 and other modulation information not available 0b0100: 2 streams with PSI=stream2 and other modulation =QPSK 0b0101: 2 streams with PSI=stream2 and other modulation =16QAM 0b0110: 2 streams with PSI=stream2 and other modulation =64QAM 0b0111: 2 streams with PSI=stream2 and other modulation information not available 0b1000: 3 streams with PSI=stream1 0b1001: 3 streams with PSI=stream2 0b1010: 3 streams with PSI=stream3 0b1011: 4 streams with PSI=stream1 0b1100: 4 streams with PSI=stream2 0b1101: 4 streams with PSI=stream3 0b1110: 4 streams with PSI=stream4 0b1111: n/a |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long TTI Indicator | 1 | Indicates number for AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default TTI) 0b1: 4 DL AAI subframe for FDD or all DL AAI subframes for TDD (long TTI) |
| Lifetime(L) | 4 | Indicates the time to transmit next HR-Multicast DL Assignment A-MAP and the information excluding Transmission Indication (TI) of this HR-Multicast DL Assignment A-MAP does not change during the allocation duration. The next HR-Multicast DL Assignment A-MAP is at the superframe whose superframe number, Nsuperframe, satisfies the following condition. Nsuperframe modulo L+1 = 0 |
| Transmission Indication (TI) | 2 | Indicates whether additional traffic is transmitting before allocation period 0b00: no (no additional transmission) 0b01: first (first transmission) 0b10: continue (more transmission) 0b11: last (no more transmission) |
| Reserved | 56 |  |
| } |  |  |
| } |  |  |

*A 16bit CRC is generated based on the randomized contents of the HR-Multicast DL Assignment A-MAP 
IE. The CRC is masked by the 16-bit CRC mask (with masking prefix=0b0 and message type indicator=
0b010) generated according to Table 192 as describe in 6.3.5.5.2.4.

[-----------------------------------------------------End of Text Proposal---------------------------------------------------]

# References

[1] IEEE P802.16.1a/D3, “WirelessMAN-Advanced Air Interface for Broadband Access Systems -  Draft Amendment: Higher Reliability Networks,” June 2012.

[2] IEEE 802.16-12-0187-00-010a, “Proposed change on downlink control for multicast communication over IEEE 802.16.1a,” March 2012.