| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Flow establishment for token handover in talk-around direct communications |  |
| Date Submitted | 2012-07-09 |  |
| Source(s) | Sungcheol Chang, Miyoung Yun, Seokki Kim, Won-Ik Kim, Hyun Lee, Kim Eunkyung Kim, Sungkyung Kim, Chulsik Yoon ETRI | E-mail: scchang@etri.re.kr |
| Re: | Letter Ballot #38b on P802.16.1a/D3 |  |
| Abstract | This provides text proposals for flow establishment after a token is passed over in talk-around direct communications |  |
| Purpose | To be discussed and adopted by 802.16 GRIDMAN TG |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

# Flow establishment for token handover in talk-around direct communications

Sungcheol Chang, Miyoung Yun, Seokki Kim, Won-Ik Kim, Hyun Lee, Kim Eunkyung Kim,

Sungkyung Kim, Chulsik Yoon

ETRI

# Introductions

When a source HR-MS has established a one-way link to a target HR-MS using the AAI-DC-LEST-REQ/RSP messages, flow identifiers are used to identify unidirectional flows from the source HR-MS to the target HR-MS. After a token is passed in to the target HR-MS, the target HR-MS has no any flows from the target HR-MS to the source HR-MS. Though the target HR-MS has a right to send bursts on the dedicated channel, the target HR-MS has no any flow on which MAC PDU are sent

This contribution provides that the target HR-MS establishes flows backward in the following cases:

- A unicast link: Additional flows from the target HR-MS to the source HR-MS are established using the AAI-DC-LEST-RSP message.

- A multicast link: The token-taken HR-MS sends the AAI-DC-LEST-CMD messages with new flow identifier after receiving the AAI-DC-TKN-HO messages and before sending data MAC PDUs.

- A unicast link and unicast relay link: Additional flows are established using the AAI-DC-RELAY-RSP message and the AAI-DC-LEST-RSP message.

- A unicast link and multicast relay link: flows on the unicast relay link have established using the AAI-DC-TKN-REQ/RSP messages.

# Proposed Text for P802.16.1a/D3

Note:

The text in BLACK color: the existing text in the 802.16.1a Draft Standard Text

The text in RED color: the removal of existing 802.16.1a Draft Standard Text

The text in BLUE color: the new text added to the 802.16.1a Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

## [Remedy1: Update the following messages]

6.2.3.5.21 AAI-DC-LEST-REQ

An HR-MS transmits a AAI-DC-LEST-REQ message to establish a one way peer-to-peer TDC link.

Table 106a – AAI-DC-LEST-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Link Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, Link Change Count is incremented by one (modulo 16) by the sender. | Shall always be present |
| Flow Establishment Request for Token Handover | 4 | Indicates that the sending HR-MS requests the receiving HR-MS to establish flows and send MAC PDUs on the flows 0x0 : Request to establish flow for token handover 0x1 : Not allowed 0x2 to 0xF: Reserved. | Shall always be present |
| For (i=0; i<N_Flow_Est; i++) { |  | N_Flow_Est is the number of flows on which the sender of this message sends MAC PDUs. Range [0..1] |  |
| FID | 4 | Flow identifier assigned by the sinksource of packets on the flow |  |
| Traffic Priority | 3 | 0 to 7: Higher numbers indicate higher priority Default: 0 |  |
| CS Specification Parameters | 8 | 0–15: Reserved 16: Voice Codec G.729A 17: Voice Codec AMR 18–255: Reserved |  |
| MAC Header Type | 1 | Indicates whether AGMH or SPMH is presented at the start of MAC PDUs of the service flow. 0 : AGMH (Advanced Generic MAC Header) 1 : SPMH (Short-Packet MAC header) default value is 0. |  |
| } |  |  |  |
| Reserved |  |  |  |

6.2.3.5.22 AAI-DC-LEST-RSP

An HR-MS transmits an AAI-DC-LEST-RSP message in response to a received AAI-DC-LEST-REQ.

Table 106b – AAI-DC-LEST-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Link Change Count | 4 | Link Change Count from corresponding the AAI-DC-LEST-REQ | Shall always be present |
| Confirmation Code | 43 | Zero indicates the request was successful. Nonzero indicates failure. 0x0: accept 0x1: reject with a recommended DCH 0x2: reject 0x3 to 0xF7: reserved | Shall always be present |
| Flow Establishment Confirm for Token Handover | 1 | Zero indicates that the sending HR-MS of this message confirms flow establishment. 0 : Confirm flow establishment for token handover 1 : Not allowed | Shall always be present |
| Direct Mode Zone Type | 2 | Direct mode zone type of a recommended DCH 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3 to 0xF: Reserved. | Present if Confirmation Code == 0x1 |
| DCH Number | 6 | Recommended DCH number on the zone of Direct Mode Zone Type | Present if Confirmation Code == 0x1 |
| For (i=0; i<N_Flow_Est; i++) { |  | N_Flow_Est is the number of flows on which the sender of this message sends MAC PDUs. Range [0..1] | Present if Confirmation Code == 0x0 and Flow Establishment Confirm for Token Handover == 0x0 |
| FID | 4 | Flow identifier assigned by the source of packets on the flow | Present if Confirmation Code == 0x0 and Flow Establishment Confirm for Token Handover == 0x0 |
| Traffic Priority | 3 | 0 to 7: Higher numbers indicate higher priority Default: 0 | Present if Confirmation Code == 0x0 and Flow Establishment Confirm for Token Handover == 0x0 |
| CS Specification Parameters | 8 | 0–15: Reserved 16: Voice Codec G.729A 17: Voice Codec AMR 18–255: Reserved | Present if Confirmation Code == 0x0 and Flow Establishment Confirm for Token Handover == 0x0 |
| MAC Header Type | 1 | Indicates whether AGMH or SPMH is presented at the start of MAC PDUs of the service flow. 0 : AGMH (Advanced Generic MAC Header) 1 : SPMH (Short-Packet MAC header) default value is 0. | Present if Confirmation Code == 0x0 and Flow Establishment Confirm for Token Handover == 0x0 |
| } |  |  |  |

6.2.3.65.41 AAI-DC- RELAY -REQ

An HR-MS transmits a AAI-DC-RELAY-REQ message to request a relaying HR-MS to establish a unicast TDC link

Table 106c – AAI-DC-RELAY-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Link Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, Link Change Count is incremented by one (modulo 16) by the sender. | Shall always be present |
| For (i=0; i<N_Flow_Est; i++) { |  | N_Flow_Est is the number of flows on which the sender of this message sends MAC PDUs. Range [0..1] |  |
| FID | 4 | Flow identifier assigned by the sink of packets on the flow |  |
| Traffic Priority | 3 | 0 to 7: Higher numbers indicate higher priority Default: 0 |  |
| CS Specification Parameters | 8 | 0–15: Reserved 16: Voice Codec G.729A 17: Voice Codec AMR 18–255: Reserved |  |
| MAC Header Type | 1 | Indicates whether AGMH or SPMH is presented at the start of MAC PDUs of the service flow. 0 : AGMH (Advanced Generic MAC Header) 1 : SPMH (Short-Packet MAC header) default value is 0. |  |
| } |  |  |  |
| Target DCTID or DCGID | 24 | Indicates a receiving HR-MS (Group) address. |  |
| Flow Establishment Request for Token Handover | 4 | Indicates that the sending HR-MS requests the receiving HR-MS to establish flows and send MAC PDUs on the flows. 0x0 : Request to establish flow for token handover 0x1 : Not allowed 0x2 to 0xF: Reserved. | Present if Target DCTID exists |

6.2.3.65.42 AAI-DC-RELAY-RSP

An HR-MS transmits a AAI- DC-RELAY-RSP message in response to a received AAI- DC-RELAY-REQ message.

Table 106d – AAI-DC-RELAY-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Link Change Count | 4 | Link Change Count from the corresponding AAI-DCRELAY-REQ | Shall always be present |
| Confirmation Code | 4 | Zero indicates the request was successful. Nonzero indicates failure. 0x0: accept 0x1: reject with a recommended DCH 0x2: wait next response message 0x3 to 0xF: reserved | Shall always be present |
| Direct Mode Zone Type | 2 | Direct mode zone type of a recommended DCH 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3 to 0xF: Reserved. | Present if Confirmation Code == 0x1 |
| DCH Number | 6 | Recommended DCH number on zone of Direct Mode Zone Type | Present if Confirmation Code == 0x1 |
| Flow Establishment Confirm for Token Handover | 1 | Zero indicates that the sending HR-MS of this message confirms flow establishment. 0 : Confirm flow establishment for token handover 1 : Not allowed | Present if Confirmation Code == 0x0 and the sending HR-MS of this message receives the “Flow Establishment Request for Token Handover” field |
| For (i=0; i<N_Flow_Est; i++) { |  | N_Flow_Est is the number of flows on which the sender of this message sends MAC PDUs. Range [0..1] | Present if Confirmation Code == 0x0 and Flow Establishment Confirm for Token Handover == 0 |
| FID | 4 | Flow identifier assigned by the source of packets on the flow | Present if Confirmation Code == 0x0 and Flow Establishment Confirm for Token Handover == 0 |
| Traffic Priority | 3 | 0 to 7: Higher numbers indicate higher priority Default: 0 | Present if Confirmation Code == 0x0 and Flow Establishment Confirm for Token Handover == 0 |
| CS Specification Parameters | 8 | 0–15: Reserved 16: Voice Codec G.729A 17: Voice Codec AMR 18–255: Reserved | Present if Confirmation Code == 0x0 and Flow Establishment Confirm for Token Handover == 0 |
| MAC Header Type | 1 | Indicates whether AGMH or SPMH is presented at the start of MAC PDUs of the service flow. 0 : AGMH (Advanced Generic MAC Header) 1 : SPMH (Short-Packet MAC header) default value is 0. | Present if Confirmation Code == 0x0 and Flow Establishment Confirm for Token Handover == 0 |
| } |  |  |  |

6.2.3.65.26 AAI-DC-TKN-REQ

An HR-MS transmits an AAI-DC-TKN-REQ message to change a token for half duplex communication.

Table 106e – AAI-DC-TKN-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Direct Mode Zone Type | 2 | Direct mode zone type of a recommended DCH 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3 to 0xF: Reserved. | Shall always be present |
| DCH Number | 4 | Indicates a number of dedicated channel with DCH Number. | Shall always be present |
| For (i=0; i<N_Flow_Est; i++) { |  | N_Flow_Est is the number of flows on which the sender of this message sends MAC PDUs. Range [0..1] |  |
| FID | 4 | Flow identifier assigned by the source of packets on the flow |  |
| Traffic Priority | 3 | 0 to 7: Higher numbers indicate higher priority Default: 0 |  |
| CS Specification Parameters | 8 | 0–15: Reserved 16: Voice Codec G.729A 17: Voice Codec AMR 18–255: Reserved |  |
| MAC Header Type | 1 | Indicates whether AGMH or SPMH is presented at the start of MAC PDUs of the service flow. 0 : AGMH (Advanced Generic MAC Header) 1 : SPMH (Short-Packet MAC header) default value is 0. |  |
| } |  |  |  |

## [Remedy2: Insert the following text in section 6.12.3.3.1.8]

6.12.2.3.1.8 Token management

When an HR-MS has a token, the HR-MS sends packets on a shared link for half-duplex transmission. The AAI-DC-TKN-HO message passes the token over to other HR-MS.

- —Message procedures of token management for unicast one-way link

If the receiving HR-MS requests the token, TKN-REQIND code on supplementary channel is sent to the sending HR-MS. If accepting it, the sending HR-MS send AAI-DC-TKN-HO message to pass the token over to the receiver HR-MS in response to TNK-REQIND code. When the receiving HR-MS gets the token, it becomes the sending HR-MS and sends packets on the dedicated channel. The sending HR-MS sends MAC PDUs on flows that has been established at unicast link establishment using the AAI-DC-LEST-REQ/RSP messages.

For multicast one-way link, an HR-MS requests a token by sending AAI-DC-TKN-REQ message. If the sending HR-MS accepts the token request, it sends AAI-DC-TKN-RSP message in response to the AAI-DC-TKN-REQ message. AAI-DC-TKN-REQ/RSP messages are sent in a frame that the sending HR-MS does not send data packets. After the AAI-DC-TKN-HO message sent, the token-receiving HR-MS sends the Ded-CH preamble and the AAI-DC-LEST-CMD message at consecutive two frames on the dedicated channel.

- —Procedures of token management with token request for unicast (left) and multicast (right) one-way links

## [Remedy3: Insert the following text after section 6.12.2.3.1.6.3]

6.12.3.3.1.6.4 Link maintenance

For a multicast link the sending HR-MS on the link sends the AAI-DC-LEST-CMD message periodically or at aperiodic intervals. The sending HR-MS sends consecutive three frames at a trial: the Ranging Notification header, the Ded-CH preamble, and the AAI-DC-LEST-CMD message piggybacked by the AAI-DC-RTS message.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

[1] IEEE P802.16.1a/D3, “WirelessMAN-Advanced Air Interface for Broadband Access Systems -  Draft Amendment: Higher Reliability Networks,” June 2012.

# Title of Contribution

[Authors of Contribution]

[Affiliation of authors, per <http://standards.ieee.org/faqs/affiliationFAQ.html>]

# Template Instructions

This page provides instructions on using the template and submitting a contribution. Before submitting, delete these instructions or replace them with your text to be submitted.

# Document Format

Edit the cover page. Replace the square-bracketed sections with the correct text for your document. Eliminate the brackets. Leave the non-bracketed words on the cover page. Limit the cover page information to one page, shrinking the font if necessary.

Begin the main document with a page break after the cover page. Restate the document title and author name. You should use heading styles that support an outline view of the document.

# Header

The document number <http://ieee802.org/16/submit.html> goes in the upper right of the header. It takes the form “IEEE 802.16-YY-#####-RR-FFFF”, where:

• Every IEEE 802.16 document number begins with “IEEE 802.16-”.

• Following “IEEE 802.”, the remainder of the document number (“16-YY-#####-RR-FFFF”) is the Document Control Number (DCN), used by IEEE Mentor as the basis of the file name.

• “YY” represents the final two digits of the calendar year of original submission, as assigned by the IEEE Mentor documentation system.

• “####” represents the document serial number, as assigned by the IEEE Mentor documentation system. The serial number is uniquely assigned for each calendar year.

• “RR” represents the revision number, as assigned by the IEEE Mentor documentation system. The initial version of the document uses the revision number “00”.

• “FFFF” represents the Mentor “file code” <http://ieee802.org/16/submit.html>. It is selected by the submitter as the most appropriate identifier among the list of  IEEE 802.16 file codes in the IEEE Mentor documentation system.

# Fonts

Use only the fonts “Times” (or “Times New Roman”), “Arial” (or “Helvetica”),  and “Symbol,” since these fonts need not be embedded in PDF files. You may use other fonts only if necessary.

# Document Submission

To contribute a document to the IEEE 802.16 Working Group or a subgroup, create it using this template or another template at <http://ieee802.org/16/submit.html>. (If it cannot be put into one of the available formats, then create at least a cover page using an available template.) Submit the document following the instructions in the Call for Contributions or the IEEE 802.16 Document Submission Instructions <http://ieee802.org/16/submit.html>.