| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Release of dedicated channels from a TDC link in talk-around direct communications |  |
| Date Submitted | 2012-07-09 |  |
| Source(s) | Sungcheol Chang, Miyoung Yun, Seokki Kim, Won-Ik Kim, Hyun Lee, Eunkyung Kim, Sungkyung Kim, Chulsik Yoon ETRI | E-mail: scchang@etri.re.kr |
| Re: | Letter Ballot #38b on P802.16.1a/D3 |  |
| Abstract | This provides text proposals for usages of multiple dedicated channels for a TDC link in talk-around direct communications |  |
| Purpose | To be discussed and adopted by 802.16 GRIDMAN TG |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

# Release of dedicated channels from a TDC link in talk-around direct communications

Sungcheol Chang, Miyoung Yun, Seokki Kim, Won-Ik Kim, Hyun Lee, Eunkyung Kim,

Sungkyung Kim, Chulsik Yoon

ETRI

# Introductions

A TDC link can operate with multiple dedicated channels in the purpose of communication coverage extension and/or transmission rate enhancement. The way to add a dedicated channel to a link is to exchange the AAI-DC-RTS/CTS messages on the dedicated channel and the AAI-DC-RTS/CTS messages contain the “Resource ID” field. This Resource ID field allows the sending HR-MS to use all the dedicated channels for transmission of a burst or bursts. However, there is no way to remove a dedicated channel out of a group of dedicated channels for a TDC link.

This contribution provides that the concept of resource change procedure is extended to removal of dedicated channel

- The AAI-DC-RCHG-CMD message has dedicated channel identifiers for release

- The AAI-DC-RCHG-REQ/RSP messages have dedicated channel identifiers for release

# Proposed Text for P802.16.1a/D3

Note:

The text in BLACK color: the existing text in the 802.16.1a Draft Standard Text

The text in RED color: the removal of existing 802.16.1a Draft Standard Text

The text in BLUE color: the new text added to the 802.16.1a Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

## [Remedy1: Update the following messages]

6.2.3.65.35 AAI-DC-RCHG-CMD

An HR-MS transmits an AAI-DC-RCHG-CMD message to change communication resource on one-way point-to-point and point-to-multipoint TDC links.

Table 106ii – AAI-DC-RCHG-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| For (i=0; i<N_DCH; i++) { |  | N_ DCH is the number of DCHs which are changed to new DCHs |  |
| Old Direct Mode Zone Type | 2 | Direct mode zone type for old DCH 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3: Reserved. |  |
| Old DCH Number | 4 | Indicates old DCH number. |  |
| New Direct Mode Zone Type | 2 | Direct mode zone type for new DCH 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3: Reserved. |  |
| New DCH Number | 4 | Indicates new DCH number. |  |
| } |  |  |  |
| For (i=0; i<N_DCH_Rel; i++) { |  | N_ DCH_Rel is the number of DCHs which are released from TDC communication |  |
| Direct Mode Zone Type | 2 | Direct mode zone type for release 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3: Reserved. |  |
| DCH Number | 4 | Indicates DCH number for release |  |
| } |  |  |  |

6.2.3.65.24 AAI-DC-RCHG-REQ

An HR-MS transmits an AAI-DC-RCHG-REQ message to change radio resource for dedicated channel.

Table 106x – AAI-DC-RCHG-REQ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| For (i=0; i<N_DCH; i++) { |  | N_DCH is the number of dedicated channels |  |
| Old Direct Mode Zone Type | 2 | Direct mode zone type for old DCH 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3 to 0xF: Reserved. |  |
| Old DCH Number | 4 | Indicates old DCH number. |  |
| New Direct Mode Zone Type | 2 | Direct mode zone type for new DCH 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3: Reserved. |  |
| New DCH Number | 4 | Indicates new DCH number. |  |
| } |  |  |  |
| For (i=0; i<N_DCH_NI; i++) { |  | N_DCH_NI is the number of recommended dedicated channels for candidates of new DCHs | Present if 0 < N_DCH_NI |
| Direct Mode Zone Type | 2 | Direct mode zone type of a recommended DCH 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3: Reserved. |  |
| DCH Number | 6 | DCH number of a candidate |  |
| Noise and Interference Level Mean | 8 | Noise and Interference mean. This is noise plus interference power level that is divided by the number of subcarriers in the frequency domain and averaged over the dedicated channel. -134dBm to -30dBm in units of 1dB. -134 dBm is encoded as 0x00, -30 dB is encoded as 0x69, 0x69 to 0xFF is reserved. |  |
| Noise and Interference Level Variance | 4 | Noise and Interference variance. 0 dB to 15 dB in units of 1dB. |  |
| } |  |  |  |
| For (i=0; i<N_DCH_Rel; i++) { |  | N_ DCH_Rel is the number of DCHs which are released from TDC communication |  |
| Direct Mode Zone Type | 2 | Direct mode zone type for release 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3: Reserved. |  |
| DCH Number | 4 | Indicates DCH number for release |  |
| } |  |  |  |

6.2.3.65.25 AAI-DC-RCHG-RSP

An HR-MS transmits a AAI-DC-RCHG-RSP message in response to a received AAI-DC-RCHG-REQ.

Table 106y – DC-RCHG-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Confirmation Code | 4 | Zero indicates the request was successful. Nonzero indicates failure 0x0: accept with new mapping of DCHs 0x1: reject (continue to use current DCHs) 0x2 to 0xF: reserved | Shall always be present |
| For (i=0; i<N_DCH; i++) { |  | N_ DCH is the number of DCHs which are changed to new DCHs | Present if Confirmation Code == 0x0 |
| Old Direct Mode Zone Type | 2 | Direct mode zone type for old DCH 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3: Reserved. |  |
| Old DCH Number | 4 | Indicates old DCH number. |  |
| New Direct Mode Zone Type | 2 | Direct mode zone type for new DCH 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3: Reserved. |  |
| New DCH Number | 4 | Indicates new DCH number. |  |
| } |  |  |  |
| For (i=0; i<N_DCH_Rel; i++) { |  | N_ DCH_Rel is the number of DCHs which are released from TDC communication |  |
| Direct Mode Zone Type | 2 | Direct mode zone type for release 0x0: Common direct mode zone (CDMZ) 0x1: Common direct mode zone extended (CDMZ-E) 0x2: Cell specific direct mode zone (CSDMZ) 0x3: Reserved. |  |
| DCH Number | 4 | Indicates DCH number for release |  |
| } |  |  |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

[1] IEEE P802.16.1a/D3, “WirelessMAN-Advanced Air Interface for Broadband Access Systems -  Draft Amendment: Higher Reliability Networks,” June 2012.

# Title of Contribution

[Authors of Contribution]

[Affiliation of authors, per <http://standards.ieee.org/faqs/affiliationFAQ.html>]

# Template Instructions

This page provides instructions on using the template and submitting a contribution. Before submitting, delete these instructions or replace them with your text to be submitted.

# Document Format

Edit the cover page. Replace the square-bracketed sections with the correct text for your document. Eliminate the brackets. Leave the non-bracketed words on the cover page. Limit the cover page information to one page, shrinking the font if necessary.

Begin the main document with a page break after the cover page. Restate the document title and author name. You should use heading styles that support an outline view of the document.

# Header

The document number <http://ieee802.org/16/submit.html> goes in the upper right of the header. It takes the form “IEEE 802.16-YY-#####-RR-FFFF”, where:

• Every IEEE 802.16 document number begins with “IEEE 802.16-”.

• Following “IEEE 802.”, the remainder of the document number (“16-YY-#####-RR-FFFF”) is the Document Control Number (DCN), used by IEEE Mentor as the basis of the file name.

• “YY” represents the final two digits of the calendar year of original submission, as assigned by the IEEE Mentor documentation system.

• “####” represents the document serial number, as assigned by the IEEE Mentor documentation system. The serial number is uniquely assigned for each calendar year.

• “RR” represents the revision number, as assigned by the IEEE Mentor documentation system. The initial version of the document uses the revision number “00”.

• “FFFF” represents the Mentor “file code” <http://ieee802.org/16/submit.html>. It is selected by the submitter as the most appropriate identifier among the list of  IEEE 802.16 file codes in the IEEE Mentor documentation system.

# Fonts

Use only the fonts “Times” (or “Times New Roman”), “Arial” (or “Helvetica”),  and “Symbol,” since these fonts need not be embedded in PDF files. You may use other fonts only if necessary.

# Document Submission

To contribute a document to the IEEE 802.16 Working Group or a subgroup, create it using this template or another template at <http://ieee802.org/16/submit.html>. (If it cannot be put into one of the available formats, then create at least a cover page using an available template.) Submit the document following the instructions in the Call for Contributions or the IEEE 802.16 Document Submission Instructions <http://ieee802.org/16/submit.html>.