| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on HR-MS neighbor discovery |  |
| Date Submitted | 2012-07-15 |  |
| Source(s) | Anh Tuan Hoang I2R | E-mail: athoang@i2r.a-star.edu.sg |
| Re: | “IEEE 802.16-12-400-00-Gdoc,” in response to Letter Ballot Recirc #38b on P802.16n/D3 |  |
| Abstract |  |  |
| Purpose | To discuss and adopt the proposed text in the draft amendment document on GRIDMAN |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on HR-MS neighbor discovery

Anh Tuan Hoang

Institute for Infocomm Research

# Introduction

We propose some modifications to the specification of HR-MS neighbor discovery to make the description clearer.

# References

[1] IEEE P802.16n/D3, Air Interface for Broadband Wireless Access Systems - Draft Amendment: Higher Reliability Networks, June 2012.

[2] IEEE P802.16.1a/D3, WirelessMAN-Advanced Air Interface for Broadband Access Systems - Draft Amendment: Higher Reliability Networks, June 2012.

# Proposed Text for the 802.16.1a AWD

Note:

The text in BLACK color: the existing text in the IEEE P802.16n/D3

The text in RED color: the removal of existing IEEE P802.16n/D3

The text in BLUE color: the new text added to the IEEE P802.16n/D3

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Modify Section 6.3.2.3.99.22 in IEEE P802.16n/D3 as indicated]

          - HR-RNG-CMD message

HR-BS/RS sends HR-RNG-CMD message to instruct one or group of its associated HR-MSs to carry out neighbor discovery. The field “Action” in HR-RNG-CMD message tells the receiving HR-MS whether it (them) should to transmit or receive the specified ranging signals. Using the field “Reversed action offset”, Tthe serving HR-BS/HR-RS can schedule transmissions of ranging signal in both directions between a pair of HR-MSs. allocate ranging resources to both involved HR-MSs in a single assignment. This allows the receiving HR-MS to transmit back a ranging sequence right after successfully processing the ranging sequence transmitted by the other HR-MS.

Table 229a — HR-RNG-CMD message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| HR-RNG-CMD_Message_Format() { | — | — |
| Management Message Type = [TBD] | 8 | — |
| Rendezvous time | 4 | Indicates the time offset, in number of frames, when the HR-BS shall provide dedicated ranging opportunity for the transmission of this discovery ranging signal |
| Dedicated CDMA code | 8 | CDMA code assigned for discovery/ranging purpose, shall be taken from the code set for initial ranging |
| Transmission opportunity offset | 4 | The offset is in number of symbol duration |
| Action | 2 | 0b00: ranging for neighbor discovery and reception node to carry out transmission 0b01: ranging for neighbor discovery and receiving node to carry out reception 0b10: DC/FTN periodic ranging transmission 0b11: Reserved |
| Reserved | 6 | Shall always be set to 0 |
| if (Action == 0b00){ |  |  |
| Transmit power level | 5 | Unsigned integer from 0 to 31 in units of 1 dBm, where 0b00000 = 0dBm and 0b11111 = 31dBm |
| Reversed action offset | 2 | 0b01-0b11: Indicates number of frames that this HR-MS should switch to receive the same ranging code in the same ranging slot 0b00: no such reversed action. |
| Reserved | 1 | Shall always be set to 0 |
| } |  |  |
| elseif(Action == 0b01){ |  |  |
| Reporting mode | 1 | Indicates if the report mode is exclusive or triggered by threshold. 0b0: exclusive reporting 0b1: triggered-based reporting |
| if(Reporting mode == 0b0){ |  |  |
| Reversed action offset | 2 | 0b01-0b11: Indicates number of frames that this HR-MS should switch to transmit the same ranging code in the same ranging slot 0b00: no such reversed action. |
| If(Reversed action offset > 0b00){ |  |  |
| Transmit power level | 5 | Unsigned integer from 0 to 31 in units of 1 dBm, where 0b00000 = 0dBm and 0b11111 = 31dBm |
| }else{ |  |  |
| Reserved | 5 | Shall always be set to 0 |
| } |  |  |
| else{ |  | Reporting mode == 0b1 |
| SINR threshold | 2 | Indicates the SINR threshold for the ranging signal above which report should be made by receiving station. The 2 bit value from 0b00 to 0b11 represent values among {–9, –8, –6, –4 } dB |
| Reversed action offset | 2 | 0b01-0b11: Indicates number of frames that this HR-MS should switch to transmit the same ranging code in the same ranging slot 0b00: no such reversed action. |
| If(Reversed action offset > 0b00){ |  |  |
| Transmit power level | 3 | Unsigned integer from 0 to 31 in units of 4 dBm, where 0b000 = 0dBm and 0b111 = 32dBm |
| }else{ |  |  |
| Reserved | 3 | Shall always be set to 0 |
| } |  |  |
| elseif(Action == 0b10){ |  | For DC/FTN periodic ranging |
| Initial transmit power | 3 | Unsigned integer from 0 to 31 in units of 4 dBm, where 0b000 = 0dBm and 0b111 = 32dBm |
| Periodicity | 2 | Indicates the periodicity of periodic ranging: 0b00: transmit ranging signal every 2 frames 0b01: transmit ranging signal every 4 frames 0b10: transmit ranging signal every 16 frames 0b11: transmit ranging signal every 32 frames |
| Tx/Rx Offset | 2 | Indicate the offset between transmitting and receiving ranging from the other HR-MS: 0b00: No offset, unidirectional ranging 0b01: Offset = 1 frame 0b10: Offset = 3 frame 0b11: Reserved |
| Reserved | 1 | Shall always be set to 0 |
| } |  |  |
| } |  |  |

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| HR-RNG-CMD_Message_Format() { | — | — |
| Management Message Type = [TBD] | 8 | — |
| Rendezvous time | 4 | Indicates the time offset, in number of frames, when the HR-BS shall provide dedicated ranging opportunity for the transmission of this discovery ranging signal |
| Dedicated CDMA code | 8 | CDMA code assigned for discovery/ranging purpose, shall be taken from the code set for initial ranging |
| Transmission opportunity offset | 4 | The offset is in number of symbol duration |
| Action | 2 | 0b00: ranging for neighbor discovery and reception node to carry out transmission 0b01: ranging for neighbor discovery and receiving node to carry out reception 0b10: DC/FTN periodic ranging transmission 0b11: Reserved |
| Reserved | 6 | Shall always be set to 0 |
| if (Action == 0b00){ |  |  |
| Transmit power level | 5 | Unsigned integer from 0 to 31 in units of 1 dBm, where 0b00000 = 0dBm and 0b11111 = 31dBm |
| Reversed action offset | 2 | 0b01-0b11: Indicates the number of frames, counting from the frame that this HR-MS transmits the dedicated ranging code, until the frame in which this HR-MS should switch to receive the same ranging code in the same ranging slot 0b00: no such reversed action. |
| Reserved | 1 | Shall always be set to 0 |
| if (Reserved action offset > 0b00){ |  |  |
| Reporting mode | 1 | Indicates if the report mode is exclusive or triggered by threshold. 0b0: exclusive reporting 0b1: triggered-based reporting |
| if(Reporting mode == 0b1){ |  |  |
| SINR threshold | 4 | Indicates the SINR threshold for the ranging signal above which report should be made by receiving station. The 4 bit value from 0b0000 to 0b1111 represent values among {–9, –8.5, –8, –7.5, –7, –6.5,–6, –5.5, –5, –4.5, –4, –3.5, –3, –2.5,–2, –1.5} dB |
| Reserved | 3 | Shall always be set to 0 |
| }else{ |  |  |
| Reserved | 7 | Shall always be set to 0 |
| } |  |  |
| } //end of if (Reserved action offset > 0b00) |  |  |
| } //end of if (Action == 0b0) |  |  |
| elseif(Action == 0b01){ |  |  |
| Reporting mode | 1 | Indicates if the report mode is exclusive or triggered by threshold. 0b0: exclusive reporting 0b1: triggered-based reporting |
| Reversed action offset | 2 | 0b01-0b11: Indicates the number of frames, counting from the frame that this HR-MS attempt to receive the dedicated ranging code, until the frame in which this HR-MS should switch to transmit the same ranging code in the same ranging slot 0b00: no such reversed action. |
| if(Reporting mode == 0b1){ |  |  |
| SINR threshold | 4 | Indicates the SINR threshold for the ranging signal above which report should be made by receiving station. The 4 bit value from 0b0000 to 0b1111 represent values among {–9, –8.5, –8, –7.5, –7, –6.5,–6, –5.5, –5, –4.5, –4, –3.5, –3, –2.5,–2, –1.5} dB |
| Reserved | 4 | Shall always be set to 0 |
| } |  |  |
| If(Reversed action offset > 0b00){ |  |  |
| Transmit power level | 5 | Unsigned integer from 0 to 31 in units of 1 dBm, where 0b00000 = 0dBm and 0b11111 = 31dBm |
| }else{ |  |  |
| Reserved | 5 | Shall always be set to 0 |
| } |  |  |
| } //end of if (Action == 0b1) |  |  |
| elseif(Action == 0b10){ |  | For DC/FTN periodic ranging |
| Initial transmit power | 3 | Unsigned integer from 0 to 31 in units of 4 dBm, where 0b000 = 0dBm and 0b111 = 32dBm |
| Periodicity | 2 | Indicates the periodicity of periodic ranging: 0b00: transmit ranging signal every 2 frames 0b01: transmit ranging signal every 4 frames 0b10: transmit ranging signal every 16 frames 0b11: transmit ranging signal every 32 frames |
| Tx/Rx Offset | 2 | Indicate the offset between transmitting and receiving ranging from the other HR-MS: 0b00: No offset, unidirectional ranging 0b01: Offset = 1 frame 0b10: Offset = 3 frame 0b11: Reserved |
| Reserved | 1 | Shall always be set to 0 |
| } |  |  |
| } |  |  |

[Remedy2: Modify Section 16.2.2.1.1 in IEEE P802.16n/D3 as indicated]

        - HR-MS Neighbor Discovery

For associated HR-MSs to discover each other, the serving HR-BS/HR-RS shall schedule some HR-MSs to broadcast ranging signals so that while other HR-MSs can try to attempt to receive and verify their neighbor relationship. The process can be described as follows:

- The serving HR-BS/HR-RS sends HR-RNG-CMD message described in 6.3.2.3.99.22 to schedule one or multiple registered HR-MSs to broadcast ranging sequences in assigned channels. Multiple HR-MSs may share the same ranging sequence or the same assigned channel.

- Using HR-RNG-CMD message, the serving HR-BS/HR-RS also schedules some other HR-MSs to listen on those channels scheduled for ranging signals.

- Each HR-MS that is scheduled to receive ranging sequences shall determine what sequences it can properly decode, together with related information such as estimations of time/frequency offsets and signal strength.

- The HR-MS receiving a ranging sequence that has met the reporting criteria defined by the “Reporting mode” and possibly “SINR threshold” as specified in HR-RNG-CMD message shall report their measurements to the serving HR-BS/HR-RS using HR-RNG-REP message described in 6.3.2.3.99.23.

The format of HR-RNG-CMD message is described in 6.3.2.3.99.22. The HR-BS unicasts HR-RNG-CMD message to a single HR-MS or multicasts the message to a group of HR-MSs that are supposed to broadcast the ranging signal. The HR-BS unicasts HR-RNG-CMD message to a single HR-MS or multicasts the message to a group of HR-MSs that are supposed to attempt to receive the ranging signal. The HR-BS can also broadcast the HR-RNG-CMD message to all of its subordinates HR-MS. In such a case, all HR-MS that are not involved in UL transmission during the ranging opportunity index shall attempt to receive the ranging signal.

[-------------------------------------------------End of Text Proposal---------------------------------------------------]