| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes on AAI-HR-RNG-CMD message in 16.1a Network |  |
| Date Submitted | 2012-07-18 |  |
| Source(s) | Anseok Lee, Eunkyung Kim, Jaesun Cha, Wooram Shin, Kwangjae Lim, Dongseung Kwon ETRI | E-mail: alee@etri.re.kr |
| Re: | In response to LB #38b on P802.16.1a/D3 |  |
| Abstract | This provides AWD text proposals of change on AAI-HR-RNG-CMD message in IEEE 802.16.1a Network. |  |
| Purpose | To be discussed and adopted by 802.16 GRIDMAN TG |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Proposed Changes on AAI-HR-RNG-CMD message in 16.1a Network

Anseok Lee, Eunkyung Kim, Jaesun Cha, Wooram Shin, Kwangjae Lim, Dongseung Kwon

ETRI

# Introductions

This contribution provides clarification on AAI-HR-RNG-CMD message in 16.1a Network.

# Proposed Text for the 802.16.1a Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16.1a Amendment Draft Standard

The text in RED color: the removal of existing 802.16.1a Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16.1a Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy 1: Accept the following updates in Table 106l of Section 6.2.3.65.12]

Table 106l – AAI-HR-RNG-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Frame Identifier | 4 | Frame which contains the ranging channel. The frame identifier is the 4 least significant bits of the frame number. |  |
| Subframe Index | 3 | Indicates the subframe index of the allocated ranging opportunity. |  |
| Dedicated ranging code index | 5 | Indicates the index of dedicated ranging code |  |
| Action | 2 | 0b00: ranging for neighbor discovery and receiving node to carry out transmission 0b01: ranging for neighbor discovery and receiving node to carry out receiving 0b10: DC/FTN periodic ranging transmission 0b11: Reserved |  |
| Reversed action offset | 4 | 0b0001-0b1111: Indicates number of frames that this HR-MS should switch to receive the same ranging code in the same ranging slot 0b0000: no such reversed action. |  |
| if (Action == 0b00¦¦0b10) ¦¦ (Reversed action offset > 0x0){ |  |  |  |
| Dedicated ranging code index | 5 | Indicates the index of dedicated ranging code |  |
| Transmit power level | 5 | Unsigned integer from 0 to 31 in units of 1 dBm, where 0b00000 = 0dBm and 0b11111 = 31dBm |  |
| Reversed action offset | 4 | 0b0001-0b1111: Indicates number of frames that this HR-MS should switch to receive the same ranging code in the same ranging slot 0b0000: no such reversed action. |  |
| } |  |  |  |
| else if(Action == 0b01) { |  |  |  |
| Ranging code index to receive | 5 | Indicates the index of ranging code to receive in this ranging opportunity | Optional |
| } |  |  |  |
| If (Action == 0b10){ |  |  |  |
| Periodicity | 2 | Indicates the periodicity of periodic ranging: 0b00: transmit ranging signal every 2 frames 0b01: transmit ranging signal every 4 frames 0b10: transmit ranging signal every 16 frames 0b11: transmit ranging signal every 32 frames |  |
| Tx/Rx Offset | 1 | Offset between transmitting and receiving ranging from the other HR-MS: 0b0: Offset = 1 frame 0b1: Offset = 3 frames |  |
| } |  |  |  |
| Reporting mode | 1 | Indicates if the report mode is exclusive or triggered by threshold. 0b0: exclusive reporting 0b1: triggered-based reporting |  |
| if(Reporting mode == 0b1){ |  |  |  |
| SINR threshold | 4 | Indicates the SINR threshold for the ranging signal above which report should be made by receiving station. The 4 bit value from 0b0000 to 0b1111 represent values among {–9, –8.5, –8, –7.5, –7, –6.5,–6, –5.5, –5, –4.5, –4, –3.5, –3, –2.5,–2, –1.5} dB |  |
| } |  |  |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

[1] IEEE P802.16.1a/D3, “Draft Amendment for IEEE Standard for WirelessMAN-Advanced Air Interface for Broadband Access Systems - Amendment: Higher Reliability Networks”