| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal of system requirements for supporting massive MIMO in multi-tier networks |  |
| Date Submitted | 2012-09-17 |  |
| Source(s) | Sangmin Oh, Ju Yong Lee, Dong-Ho Cho KAIST IT Convergence Hyunjeong Kang Jaeweon Cho Samsung Electronics | E-mail: ohsangmin@itc.kaist.ac.kr, jylee@itc.kaist.ac.kr, dhcho@kaist.ac.kr hyunjeong.kang@samsumg.com jaeweon.cho@samsung.com |
| Re: | IEEE 802.16-12-0507 Call for contributions: multi-tier networks |  |
| Abstract | It proposes system requirements of multi-tier networks to support massive MIMO. |  |
| Purpose | To discuss and adopt in system requirement of multi-tier networks |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

proposal OF SYSTEM REQUIREMENTS For massive mimo in MULTI-TIER networks

Sangmin Oh, Ju Yong Lee, Dong-Ho Cho

KAIST IT Convergence

Hyunjeong Kang, Jaeweon Cho

Samsung Electronics

# Introduction

In order to increase the capacity in the network consisting of macro BS and cooperative small BSs, massive antenna with lots of antenna elements can help to increase the capacity abundantly, especially in the rich scattering environment. However, it has spatial limitation in the cellular band to increase the number of antenna elements in the case that the spacing among antennas are greater than the half wavelength. If antenna is designed with polarization, radiation pattern and highly accumulated antenna layout in a small space, then we can increase the number of antenna elements in a small space, and thus, the overall capacity can be increased.

Conventionally, two electrically polarized antennas have been used in the base-station. However, in the small base-stations, various types of polarized and different radiation pattern antennas can be used in the rich scattering environment such as indoor environment, where dipole antennas, loop antennas and planner inverted-F antennas (PIFA) can be combined to integrate massive antennas in a small space, which can give polarization and pattern gain and MIMO gain according to the number of integrated antennas.

This contribution suggests the deployments of massive MIMO to achieve higher system capacity and antenna selection in 802.16q multi-tier network to support up to 10 Gbps data rate of the link between macro BS and small BS.

# Proposed text change

Adopt the following remedies in IEEE 802.16q system requirements.

---------------------------------- The start of text -----------------------------------------

[Remedy: Insert the texts]

# 1. Abbreviations and acronyms

BS 		base station

MIMO 	multiple input multiple output

MS		mobile station

# 2. Functional Requirements

This section provides requirements for supporting MIMO in the link between a macro BS and a small BS in multi-tier networks.

## 2.1 MIMO support in Multi-tier Networks

## 2.1.1 MIMO support through integrated antennas in a small BS

The 802.16q system shall support MIMO with integrated antennas in a small BS. For the macro BS and the small BS, minimum [TBD] number of transmit and [TBD] number of receive antennas shall be supported.

### Antennas in a small BS shall be integrated with low inter-antenna interference.

### Integrated antennas shall support MIMO or beamforming/pre-coding operation.

### A small BS shall support antenna selection operation with low complexity.

---------------------------------- The end of text -----------------------------------------