| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications on Activation of Alternative Path to forwarding HR-MS in 16.1a Networks |  |
| Date Submitted | 2012-09-13 |  |
| Source(s) | Anseok Lee, Eunkyung Kim, Jaesun Cha, Wooram Shin, Kwangjae Lim, Dongseung Kwon ETRI | E-mail: alee@etri.re.kr |
| Re: | In response to LB #38b on P802.16.1a/D3 |  |
| Abstract | This provides AWD text proposals of clarification on Alternative Path in IEEE 802.16.1a Network. |  |
| Purpose | To be discussed and adopted by 802.16 GRIDMAN TG |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Clarifications on Activation of Alternative Path to forwarding HR-MS in 16.1a Networks

Anseok Lee, Eunkyung Kim, Jaesun Cha, Wooram Shin, Kwangjae Lim, Dongseung Kwon

ETRI

# Introductions

This contribution provides clarification on Alternative Path Management in 16.1a Network.

# Proposed Text for the 802.16.1a Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16.1a Amendment Draft Standard

The text in RED color: the removal of existing 802.16.1a Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16.1a Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy 1: Accept the following modifying to Section 6.12.7.1.4]

6.12.7.1.4 Alternative Path information and Switching to forwarding HR-MS

When HR-MS is disconnected with HR-BS in its current path, HR-MS shall try to do the network reentry through the target forwarding HR-MS.

When HR-BS realizes that the HR-MS is disconnected, HR-BS will arrange for the target forwarding HR-MS to perform the forwarding to network action as in session 6.12.3.2.1.1.

An HR-BS may schedule periodic ranging code transmission(s) for an HR-MS by using the AAI-HR-RNG-CMD message for maintenance of the connectivity with the HR-MS. The HR-BS shall measure the ranging code from the HR-MS and detect the loss of connectivity (e.g. RSSI below some threshold or consecutive failure of ranging code detection). The periodic transmission of ranging code of the HR-MS may also be used for alternative path maintenance as in 6.12.7.1.5.

When the HR-BS detects the loss of connection with the HR-MS, the HR-BS initiates the coverage extending process of the target forwarding HR-MS to facilitate the network entry of the HR-MS by transmitting the AAI-FN-CONFIG-CMD message.

The HR-MS may start the alternative path activation procedure when it detects the loss of the connection with the HR-BS and there is no alternative path with HR-infrastructure stations. The HR-MS may scan the preambles, and I/S-NCI messages from the target forwarding HR-MS and perform the network entry through the target forwarding HR-MS by using the forwarding to network function as in 6.12.3.2.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

[1] IEEE P802.16.1a/D3, “Draft Amendment for IEEE Standard for WirelessMAN-Advanced Air Interface for Broadband Access Systems - Amendment: Higher Reliability Networks”