| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications on Maintenance of Alternative Path to forwarding HR-MS in 16.1a Networks |  |
| Date Submitted | 2012-09-19 |  |
| Source(s) | Anseok Lee, Eunkyung Kim, Jaesun Cha, Wooram Shin, Kwangjae Lim, Dongseung Kwon ETRI | E-mail: alee@etri.re.kr |
| Re: | In response to SB010a on P802.16.1a/D5 |  |
| Abstract | This provides AWD text proposals of clarification on Alternative Path in IEEE 802.16.1a Network. |  |
| Purpose | To be discussed and adopted by 802.16 GRIDMAN TG |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Clarifications on Maintenance of Alternative Path to forwarding HR-MS in 16.1a Networks

Anseok Lee, Eunkyung Kim, Jaesun Cha, Wooram Shin, Kwangjae Lim, Dongseung Kwon

ETRI

# Introductions

This contribution provides clarification on Alternative Path Management in 16.1a Network.

# Proposed Text for the 802.16.1a Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16.1a Amendment Draft Standard

The text in RED color: the removal of existing 802.16.1a Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16.1a Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy 1: Accept the following modifying to Section 6.12.7.1.5]

6.12.7.1.5 Alternative path maintenance

An alternative path to a target HR-infrastructure station may be updated or changed with a new AAI-HO-CMD message. When a mode-change or mode-release occurs as described in 6.12.1, new alternative path may be selected.

An alternative path to a target forwarding HR-MS may be updated or changed by either HR-MS or HR-BS according to network conditions such as the forwarding HR-MS is moving out of reach etc.

For maintaining the alternative path to target forwarding HR-MS, the serving HR-BS shall schedule periodic ranging code transmission and reception for the HR-MS and target forwarding HR-MS respectively by using AAI-HR-RNG-CMD message. The target forwarding HR-MS shall periodically report the measurement result of ranging code from the HR-MS to the serving HR-BS by the AAI-HR-RNG-REP message.

The serving HR-BS determines the connectivity between the HR-MS and the target forwarding HR-MS by the received measurement result of the ranging code. When the serving HR-BS detects loss of link between the HR-MS and the target forwarding HR-MS, the serving HR-BS shall recover the alternative path for the HR-MS by finding another target forwarding HR-MS as in section 6.12.7.1.2.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

[1] IEEE P802.16.1a/D5, “Draft Amendment for IEEE Standard for WirelessMAN-Advanced Air Interface for Broadband Access Systems - Amendment: Higher Reliability Networks”