| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of Path Discovery and Management messages |  |
| Date Submitted | 2012-09-13 |  |
| Source(s) | Hoang Vinh Dien, Ming-Tuo Zhou, Xin Zhang, Liru Lu (Alina), Masayuki Oodo, Hiroshi Harada National Institute of Information and Communications Technology (NICT) | E-mail: hvdien@nict.com.sg; mingtuo@nict.com.sg; zhangxin@nict.com.sg; liru@nict.com.sg; moodo@nict.go.jp; harada@nict.go.jp |
| Re: | In response to SB000n on P802.16n/D5 |  |
| Abstract | This provides a Clarification of Path Discovery and Management messages |  |
| Purpose | To be discussed and adapted by TGn for draft 802.16n AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Clarification of Path Discovery and Management messages

Hoang Vinh Dien, Ming-Tuo Zhou, Xin Zhang, Liru Lu (Alina), Masayuki Oodo, Hiroshi Harada

NICT

# Introduction

# This contribution provides a clarification of Path Discovery and Management messages

# Text Proposal in IEEE 802.16n AWD

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the Multi-Carrier DG Text

[Remedy 1: Accept the following modifying to Section 6.3.2.3.99.17]

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

6.3.2.3.99.17 PM-NBR-REQ message

HR-MS shall send back a PM-NBR-REQ message to report regarding the direct communication link in response to DC-LR-REQ message.

The PM-NBR-REQ message is transmitted by a HR-MS to HR-BS to request the HR-BS to arrange for the neighbouring discovery of the HR-MS. The PM-NBR-REQ message is also used by the HR-MS to inform the HR-BS its choice of the alternative path for fast network reentry.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

[Remedy 2: Accept the following modifying to Section 6.3.2.3.99.18]

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

# 6.3.2.3.99.18 PM-NBR-REP message

HR-BS shall send back a PM-NBR-REP message to report the neighbouring discovery results to HR-MS in response to the PM-NBR-REQ message.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

[1] IEEE P802.16n/D5, “Air Interface for Broadband Wireless Access Systems Draft Amendment: Higher Reliability Networks”