| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of Path Discovery and Management messages |  |
| Date Submitted | 2012-09-19 |  |
| Source(s) | Hoang Vinh Dien, Ming-Tuo Zhou, Xin Zhang, Liru Lu (Alina), Masayuki Oodo, Hiroshi Harada National Institute of Information and Communications Technology (NICT) | E-mail: hvdien@nict.com.sg; mingtuo@nict.com.sg; zhangxin@nict.com.sg; liru@nict.com.sg; moodo@nict.go.jp; harada@nict.go.jp |
| Re: | In response to SB000n on P802.16n/D5 |  |
| Abstract | This provides a Clarification of Path Discovery and Management messages |  |
| Purpose | To be discussed and adapted by TGn for draft 802.16n AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Clarification of Path Discovery and Management messages

Hoang Vinh Dien, Ming-Tuo Zhou, Xin Zhang, Liru Lu (Alina), Masayuki Oodo, Hiroshi Harada

NICT

# Introduction

# This contribution provides a clarification of Path Discovery and Management messages

# Text Proposal in IEEE 802.16n AWD

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added to the Multi-Carrier DG Text

[Remedy 1: Change text in line#3-line#6, page 26 as follow:]

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

6.3.2.3.99.17 PM-NBR-REQ message

HR-MS shall send back a PM-NBR-REQ message to report regarding the direct communication link in response to DC-LR-REQ message.

The PM-NBR-REQ message is transmitted by a HR-MS to HR-BS to request the HR-BS to arrange for the neighbouring discovery of the HR-MS. The PM-NBR-REQ message is also used by the HR-MS to inform HR-BS of the forwarding HR-MS chosen for alternative path.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

[Remedy 2: Replace table 229q as follow:]

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

Table 229q — PM-NBR-REQ message field and description

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| PM-NBR-REQ_Message_Format(){ |  |  |
| Management Message Type = 126 | 8 |  |
| Mode requested | 2 | 0b00: Normal neighbor discovery 0b01: Discovery to prepare for HR-BS determine an alternative path 0b10: Discovery to prepare for HR-MS determine an alternative path, HR-BS shall reply HR-MS in the message PM-NBR-REP. 0b11: Inform HR-BS of the forwarding HR-MS chosen for alternative path 0b00: To request the HR-BS to arrange for the neighbor discovery of the HR-MS. 0b01: To inform HR-BS of the forwarding HR-MS chosen for alternative path 0b10-0b11: Reserved |
| If (Mode requested == 0b110b01) { |  |  |
| Index of the forwarding HR-MS | 4 | The index of HR-MS chosen as the forwarding HRMS as in the PM-NBR-REP message when Mode is 0b110b01 |
| Reserved | 2 |  |
| } |  |  |
| else { |  |  |
| Reserved | 6 |  |
| } |  |  |
| } |  |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

[Remedy 3: Insert the following text in line#10, page 26]

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

An HR-BS transmits a PM-NBR-REP message including neighbor information of the HR-MS after performing neighbor discovery as described in 16.2.2.1.1, in response to the PM-NBR-REQ message with mode 0b00.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

[Remedy 4: Replace "mode 0b10" with "mode 0b00" in the line 23, page 100]

[Remedy 5: Replace "mode 0b11" with "mode 0b01" in the line 28-29, page 100]

# References

[1] IEEE P802.16n/D5, “Air Interface for Broadband Wireless Access Systems Draft Amendment: Higher Reliability Networks”