| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Standardization of Proximity based Direct Communication: Proposed PAR and Five Criteria |  |
| Date Submitted | 2012-11-14 |  |
| Source(s) | Chanho Yoon, Seungkwon Cho, Sungkyung Kim, Soojung Jung, Hyungjin Kim, and Sungcheol Chang ETRI 218 Gajeong-dong, Yuseong-gu, Daejeon, Republic of Korea | E-mail: cyhoon@etri.re.kr |
| Re: | none |  |
| Abstract | This document proposes to the IEEE 802.16 Working Group a draft PAR and Five Criteria for a project on Proximity based Direct Communications |  |
| Purpose | This proposal requests that the 802.16 WG reviews the proposal and forward it as a draft PAR |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Annex: Proposed DRAFT PAR

P802.16.1c

Submitter Email: r.b.marks@ieee.org
Type of Project: Amendment to IEEE Standard 802.16.1-2012
PAR Request Date: 15-Oct-2012
PAR Approval Date: 
PAR Expiration Date: 
Status: Unapproved PAR, PAR for an Amendment to an existing IEEE Standard

1.1 Project Number: P802.16.1c
1.2 Type of Document: Standard
1.3 Life Cycle: Full Use

2.1 Title: WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems – Amendment for Proximity based Direct Communications (PDC).

3.1 Working Group: Broadband Wireless Access Working Group (C/LM/WG802.16)
Contact Information for Working Group Chair
   Name: Roger Marks
   Email Address: r.b.marks@ieee.org
   Phone:
Contact Information for Working Group Vice-Chair
None

3.2 Sponsoring Society and Committee: IEEE Computer Society/Local and Metropolitan Area Networks (C/LM)
Contact Information for Sponsor Chair
   Name: Paul Nikolich
   Email Address: p.nikolich@ieee.org
   Phone: 
Contact Information for Standards Representative

Name: James Glib
   Email Address: glib@ieee.org
   Phone:

3.3 Joint Sponsor: IEEE Microwave Theory and Techniques Society/Standards Coordinating Committee (MTT/SCC)
Contact Information for Sponsor Chair
   Name: Michael Janezic
   Email Address: janezic@boulder.nist.gov
   Phone: 
Contact Information for Standards Representative
   Name: Michael Janezic
   Email Address: janezic@boulder.nist.gov
   Phone:

4.1 Type of Ballot: Individual
4.2 Expected Date of submission of draft to the IEEE-SA for Initial Sponsor Ballot: 07/2014
4.3 Projected Completion Date for Submittal to RevCom: 02/2015

5.1 Approximate number of people expected to be actively involved in the development of this project: 30

5.2.a. Scope of the complete standard: This standard specifies the air interface, including the medium access control layer (MAC) and physical layer (PHY), of combined fixed and mobile point-to-multipoint broadband wireless access (BWA) systems providing multiple services, including proximity based direct communications.  The MAC is structured to support the WirelessMAN-Advanced Air Interface specifications, each suited to a particular operational environment.

5.2.b. Scope of the project:

This project will develop an amendment specifying enhancements to the WirelessMAN-Advanced Air Interface for effective use in proximity based direct communications (PDC), providing detection of device discovery in the physically close proximity.  It will focus on optimized physical (PHY) layer and medium access control (MAC) layer enhancements to provide functionalities for proximity based applications utilizing direct communication link in licensed bands, hence increase the overall network throughput.  The enhancements will be advanced mobile station (AMS) initiated discovery of neighboring AMSs, unified approach to deploy proximity based direct communication between AMSs covering both infrastructure-dependent and infrastructure-less operation scenarios, and switching mechanism between conventional ABS-centric link and direct communication link for increase of cell spectral efficiency by cellular traffic offloading.  This amendment will provide continuing support for WirelessMAN-Advanced Air Interface equipment.

5.3 Is the completion of this standard dependent upon the completion of another standard:  Yes: IEEE Std 802.16.1a

5.4 Purpose: By standardizing the PHY and MAC for proximity based direct communication in licensed bands, the standard creates new business trials and markets of proximity based applications including social commerce, advertisement, and social networking in local area.  The standard provides cost-effective 802.16.1 network deployment as a result of increased cell spectral efficiency per area compared to the conventional 802.16.1 system.

5.5 Need for the Project: In order to meet the ever growing wireless cellular data traffic, further increase in cell spectral efficiency must be provided.  This can be realized by employing a unified solution to proximity based cellular data traffic offloading in ABS coverage case and infrastructure-less proximity based direct communication in out-of-ABS coverage case.  Many proximity based applications require PHY and MAC technique that involve requirements significantly different from those used to support typical ABS-centric network access.  The ABS-centric network architecture of PHY and MAC do not have a functionality to let AMS to actively search the presence of other AMSs in its close proximity.  Such applications include proximity based social networking services, proximity based network gaming services, proximity based advertisement (e.g. retail digital signage management), proximity based service discovery, infrastructure-less synchronization and distributed coordination, and etc. 

5.6 Stakeholders for the Standard: Vendors developing IEEE 802.16.1 products, carriers using IEEE 802.16.1 products, the WiMAX ForumTM, TTA, and participants in ITU-R Working Party 5D.

Intellectual Property
6.1.a. Is the Sponsor aware of any copyright permissions needed for this project?: No
6.1.b. Is the Sponsor aware of possible registration activity related to this project?: No

7.1 Are there other standards or projects with a similar scope?: No.

7.2 Joint Development
   Is it the intent to develop this document jointly with another organization?: No.

8.1 Additional Explanatory Notes (Item Number and Explanation):

The standard will comply with IEEE Std 802, IEEE Std 802.1D, and IEEE Std 802.1Q.

FIVE CRITERIA

#### 1 Broad Market Potential

A standards project authorized by IEEE 802 LMSC shall have a broad market potential. Specifically, it shall have the potential for:

a) Broad sets of applicability.

b) Multiple vendors and numerous users.

c) Balanced costs (LAN versus attached stations).

Proximity based direct communications is a very distinct capability that enables the implementation of the “peer awareness”.  As per several market forecasts, the potential market for this is expected to be very large over the next 10 years.

- A variety of applications that are possible include traffic offloading, peer discovery, social networking, ad hoc networking, digital signage, emergency communications, internet-of-things, and etc.

- The technologies involved in enabling proximity based direct communications can be enabled by a variety of base station and mobile device vendors, chipsets can be developed by a variety of vendors and also applications can be provided using this standard by numerous industry players.

- The technologies used in proximity based direct communication (PDC) provide an appropriate degree of balance of costs to mobile devices.

#### 2 Compatibility

IEEE 802 LMSC defines a family of standards. All standards shall be in conformance with the IEEE Std 802, IEEE Std 802.D, and IEEE Std 802.1Q.  If any variances in conformance emerge, they shall be thoroughly disclosed and reviewed with 802.1 Working Group.  In order to demonstrate compatibility with this criterion, the Five Criteria statement must answer the following questions;

- Does the PAR mandate that the standard will comply with IEEE Std 802, IEEE Std 802.1D, and IEEE Std 802.1Q?

- If not, how will the Working Group ensure that the resulting draft standard is compliant or, if not, receives appropriate review from the IEEE 802.1 Working Group?

(a) The standard will comply with IEEE Std 802, IEEE Std 802.1D, and IEEE Std 802.1Q.

##### 3 Distinct Identity

Each IEEE 802 LMSC standard shall have a distinct identity. To achieve this, each authorized project shall be:

a) One unique solution per problem (not two solutions to a problem).

b) Substantially different from other IEEE 802 standards.

(a) This amendment is unique in providing enhancements for supporting proximity based applications to 802.16.1 while it supports continuing 802.16.1 user experience.

(b) The title of this amendment and the scope is distinct enough for document readers to discern the application of this standard.

##### 4 Technical Feasibility

For a project to be authorized, it shall be able to show its technical feasibility. At a minimum, the proposed project shall show:

a) Demonstrated system feasibility.

b) Proven technology, reasonable testing.

c) Confidence in reliability.

- The technical feasibility of the system is well established through the successful field test result of existing systems based on the WirelessMAN-Advanced air interface.  The fundamental operation of the system will be unchanged.

- The enhancements to be developed within the scope of the project are well understood technically and have been successfully tested in other air interfaces. The testing of the new features will not introduce fundamental complications, although peer device discovery can be expected to introduce new verification procedural test.

- Because the enhancements to be developed within the scope of the project are well understood technically and have been successfully tested in other air interfaces, no clear reliability risk factors are apparent.

4.1 Coexistence of IEEE 802 LMSC wireless standards specifying devices for unlicensed operation

A WG proposing a wireless project is required to demonstrate coexistence through the preparation of a Coexistence Assurance (CA) document unless it is not applicable.

(a) The WG will create a CA document as part of the WG balloting process.

(b) If the WG elects not to create a CA document, it will explain to the Sponsor the reason the CA document is not applicable.

The Working Group will not create Coexistence Assurance (CA) document because only licensed band devices will be supported. Hence the co-existence requirement does not apply.

##### 5 Economic Feasibility

For a project to be authorized, it shall be able to show economic feasibility (so far as can reasonably be estimated), for its intended applications. At a minimum, the proposed project shall show:

a) Known cost factors, reliable data.

b) Reasonable cost for performance.

c) Consideration of installation costs.

- The incremental cost of implementing this standard over systems based on IEEE Std 802.16.1 are nominal and within the realm of economic feasibility.  The existence of such development and field tests effort for market deployment indicates that IEEE 802.16.1 systems are expected to have a cost that is consistent with reasonable business strategies.  They are expected to be feasible in the market friendly manner, since additional costs related to required hardware and software components are not significant.

- There is no expectation of significant costs incurred to support PDC capabilities in a device or a base station beyond a reasonable amount that is actually feasible.

- Seeing the wide economic feasibility of IEEE Std 802.16.1 based systems developed for market deployment, the additional installation costs incurred for supporting this standard are within reasonable bounds.  It is expected that a new facility employing proximity based applications, including advertisement, may incur additional installation cost, however, such creates new business trials and markets.

References: