| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Periodic Ranging between HR-MSs for Direct Link Maintenance in 16n Networks |  |
| Date Submitted | 2012-11-14 |  |
| Source(s) | Anseok Lee, Eunkyung Kim, Jaesun Cha, Wooram Shin, Kwangjae Lim, Dongseung Kwon ETRI | E-mail: alee@etri.re.kr |
| Re: | In response to SB000n on P802.16n/D6 |  |
| Abstract | This provides AWD text proposals of clarification on Alternative Path in IEEE 802.16n Network. |  |
| Purpose | To be discussed and adopted by 802.16 GRIDMAN TG |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Periodic Ranging between HR-MSs for Direct Link Maintenance

in 16n Networks

Anseok Lee, Eunkyung Kim, Jaesun Cha, Wooram Shin, Kwangjae Lim, Dongseung Kwon

ETRI

# Introductions

The direct ranging function in the current draft is divided into non-periodic and periodic operation. The periodic ranging function in the current draft is in multicast or broadcast based operation.

However, in the multicast or broadcast based periodic ranging function, the reliable deliveries of ranging command message to HR-MSs are not guaranteed because of the absence of error handling mechanism. This unreliable delivery may incur the recursive failures of ranging operation.

Moreover, the current periodic ranging function only mandates the reversed action (HR-MSs changing its role as transmitter/receiver of ranging code) which designed only for BS-controlled direct communication and FTN (Forwarding To Network). The periodic ranging function should be designed also for direct link maintenance (including the alternative path to an HR-MS).

And the current draft does not support termination of the periodic ranging operation. To stop the link maintenance, termination of periodic ranging shall be needed.

In this contribution, we provide some clarifications on the ranging between HR-MSs (direct ranging) for HR-MS Neighbor Discovery in 16n Network as follows:

- Support of non-reversed action of periodic ranging operation

- Support of periodic ranging termination

- Removal of multicast or broadcast based periodic ranging

# Proposed Text for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy 1: Accept the following modifying to Section 6.3.2.3.99.21]

6.3.2.3.99.21 HR-RNG-CMD message

HR-BS/RS sends HR-RNG-CMD message to instruct its associated HR-MSs to carry out neighbor discovery or HR-MS-to-HR-MS periodic ranging. The field “Action” in HR-RNG-CMD message tells receiving HR-MS whether to transmit or receive the specified ranging signal. Using the field “Reversed action offset”, the serving HR-BS/HR-RS can schedule transmissions of ranging signal in both directions between a pair of HR-MSs.

Table 6-227u—HR-RNG-CMD message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| HR-RNG-CMD_Message_Format() { | — | — |
| Management Message Type = 130 | 8 | — |
| Rendezvous time | 4 | Indicates the time offset, in number of frames, when the HR-BS shall provide dedicated ranging opportunity for the transmission of this discovery ranging signal |
| Dedicated CDMA code | 8 | CDMA code assigned for discovery/ranging purpose, shall be taken from the code set for initial ranging |
| Transmission opportunity offset | 4 | The offset is in number of symbol duration |
| Action | 2 | 0b00: ranging for neighbor discovery and reception node to carry out transmission 0b01: ranging for neighbor discovery and receiving node to carry out reception 0b10: DC/FTN periodic ranging transmission as part of BS-control DC/FTN and alternative path management, the HR-MS is scheduled to transmit the ranging code 0b11: Reserved periodic ranging as part of BS-control DC/FTN and alternative path management, the HR-MS is scheduled to receive the ranging code |
| Reserved | 6 | Shall always be set to 0 |
| if (Action == 0b00){ |  |  |
| Transmit power level | 5 | Unsigned integer from 0 to 31 in units of 1 dBm, where 0b00000 = 0dBm and 0b11111 = 31dBm |
| Reversed action offset | 2 | 0b01-0b11: Indicates the number of frames, counting from the frame that this HR-MS transmits the dedicated ranging code, until the frame in which this HR-MS should switch to receive the same ranging code in the same ranging slot 0b00: no such reversed action. |
| Reserved | 1 | Shall always be set to 0 |
| if (Reserved Reversed action offset > 0b00){ |  |  |
| Reporting mode | 1 | Indicates if the report mode is exclusive or triggered by threshold. 0b0: exclusive reporting 0b1: triggered-based reporting |
| if(Reporting mode == 0b1){ |  |  |
| SINR threshold | 4 | Indicates the SINR threshold for the ranging signal above which report should be made by receiving station. The 4 bit value from 0b0000 to 0b1111 represent values among {–9, –8.5, –8, –7.5, –7, –6.5,–6, –5.5, –5, –4.5, –4, –3.5, –3, –2.5,–2, –1.5} dB |
| Reserved | 3 | Shall always be set to 0 |
| }else{ |  |  |
| Reserved | 7 | Shall always be set to 0 |
| } |  |  |
| } //end of if (Reserved Reversed action offset > 0b00) |  |  |
| } //end of if (Action == 0b00) |  |  |
| elseif(Action == 0b01){ |  |  |
| Reporting mode | 1 | Indicates if the report mode is exclusive or triggered by threshold. 0b0: exclusive reporting 0b1: triggered-based reporting |
| Reversed action offset | 2 | 0b01-0b11: Indicates the number of frames, counting from the frame that this HR-MS attempt to receive the dedicated ranging code, until the frame in which this HR-MS should switch to transmit the same ranging code in the same rang- ing slot 0b00: no such reversed action. |
| if(Reporting mode == 0b1){ |  |  |
| SINR threshold | 4 | Indicates the SINR threshold for the ranging signal above which report should be made by receiving station. The 4 bit value from 0b0000 to 0b1111 represent values among {–9, –8.5, –8, –7.5, –7, –6.5,–6, –5.5, –5, –4.5, –4, – 3.5, –3, –2.5,–2, –1.5} dB |
| Reserved | 4 | Shall always be set to 0 |
| } |  |  |
| If(Reserved Reversed action offset > 0b00){ |  |  |
| Transmit power level | 5 | Unsigned integer from 0 to 31 in units of 1 dBm, where 0b00000 = 0dBm and 0b11111 = 31dBm |
| }else{ |  |  |
| Reserved | 5 | Shall always be set to 0 |
| } |  |  |
| } //end of if (Action == 0b01) |  |  |
| elseif(Action == 0b10 or 0b11){ |  | For DC/FTN/Alternative Path management periodic ranging |
| Initial transmit power | 3 | Unsigned integer from 0 to 31 in units of 4 dBm, where 0b000 = 0dBm and 0b111 = 32dBm |
| Periodicity | 23 | Indicates the periodicity of periodic ranging: 0b00: transmit ranging signal every 2 frames 0b01: transmit ranging signal every 4 frames 0b10: transmit ranging signal every 16 frames 0b11: transmit ranging signal every 32 frames 0b000: the periodic ranging assignment is terminated 0b001 to 0b111: represents the value k that indicates the interval between consecutive transmissions of periodic ranging signal, where the interval is equal 2k frames |
| Duration | 8 | Indicates the duration of periodic ranging 0 : Stop the periodic ranging code transmission when it receive the suspension command 1-255 : Stop the periodic ranging code transmission after this value of periods Note that this value should be ignored when the Period is 0 |
| Tx/Rx Offset | 2 | Indicate the offset between transmitting and receiving rang- ing from the other HR-MS: 0b00: No offset, unidirectional ranging 0b01: Offset = 1 frame 0b10: Offset = 3 frames 0b11: Reserved |
| Periodic ranging for path management | 1 | 0b0: Periodic ranging for path management 0b1: Periodic ranging for DC/FTN |
| If (Periodic ranging for path management == 0b0) |  |  |
| Reporting mode | 1 | Indicates if the report mode is exclusive or triggered by threshold. 0b0: exclusive reporting 0b1: triggered-based reporting |
| if(Reporting mode == 0b1){ |  |  |
| SINR threshold | 4 | Indicates the SINR threshold for the ranging signal above which report should be made by receiving station. The 4 bit value from 0b0000 to 0b1111 represent values among {–9, –8.5, –8, –7.5, –7, –6.5,–6, –5.5, –5, –4.5, –4, –3.5, –3, –2.5,–2, –1.5} dB |
| Reserved | 2 | Shall always be set to 0 |
| }else{ |  |  |
| Reserved | 6 | Shall always be set to 0 |
| } |  |  |
| } // end of If(Periodic ranging for path management == 0b0) |  |  |
| else { |  |  |
| Reserved | 7 |  |
| } |  |  |
| Reserved | 1 | Shall always be set to 0 |
| }//end of if (Action == 0b10 or 0b11) |  |  |

HR-MS who receives the HR-RNG-CMD message shall interpret the three parameters for rendezvous time, dedicated ranging code, and transmission opportunity offset in the same way as specified in 6.3.10.4.1.

When HR-RNG-CMD message is used to schedule ranging for neighbor discovery:

—HR-BS/RS can unicast an HR-RNG-CMD message to its associated HR-MS.

—HR-BS/RS can also multicast an HR-RNG-CMD message to all HR-MSs belonging to a multicast group. While doing so, the HR-RNG-CMD message shall be transmitted using a Multicast Management CID. In this case, the “Reversed action offset” field shall be set to “0b00”.

—The HR-BS can also broadcast the HR-RNG-CMD message with “Action” field set to 0b01 to all of its subordinates HR-MS. In such a case, all HR-MS that are not involved in UL transmission during the ranging opportunity index shall attempt to receive the ranging signal. In this case, the “Reversed action offset” field shall be set to “0b00”.

The serving HR-BS/HR-RS may schedule periodic ranging with involved HR-MSs for maintenance of link between two HR-MSs. For periodic ranging code transmit/receive operation, the Periodicity and Duration fields of HR-RNG-CMD message may be set as appropriate values. The commanded HR-MSs shall schedule transmission and reception of the ranging code as in the message.

When HR-RNG-CMD message is used to schedule periodic ranging between two HR-MSs involved in BS-controlled direct communication, it is transmitted unicasted with the primary CID allocated to one of the two sides of the direct communication link, as defined in Error: Reference source not found. The HR-MS that has its primary CID used for the HR-RNG-CMD message shall transmit the ranging signal as defined by the field Rendezvous time and Periodicity. In case the field “TX/RX Offset” is 0b01 or 0b10, this HR-MS shall receive the same ranging signal in the opposite direction of the direct communication link after the corresponding offset. The other HR-MS, i.e., at the other side of the direct communication link, shall transmit the same ranging code, at the same location within a frame, and at an offset as defined by Tx/Rx Offset.

When HR-RNG-CMD message is used to schedule periodic ranging between two HR-MSs involved in BS-controlled FTN, it is transmitted unicasted using the primary CID of the forwarded HR-MS. The forwarding HR-MS shall transmit the ranging signal as defined by the field Rendezvous time and Periodicity. The forwarded HR-MS shall transmit the same ranging code, at the same location within a frame, and at an offset as defined by Tx/Rx Offset.

When HR-RNG-CMD message is used to schedule periodic ranging under alternative path management, it is unicasted using the primary CID of the involved HR-MSs. The HR-MSs shall report the measured link conditions to the HR-BS as defined in the “Reporting mode” and “SINR threshold” fields.

The periodic ranging operation shall be terminated until the Duration times of periods when Duration is not 0. The operation shall also be terminated when the HR-MS receives the HR-RNG-CMD message with Action is 0b10 or 0b11 and Periodicity is 0b000 (Termination of periodic ranging).

[Remedy 2: Modifty the following text in Table 6-51]

| 130 | HR-RNG-CMD | Neighbor discovery command | Broadcast or Primary management |
| --- | --- | --- | --- |
| 131 | HR-RNG-REP | Neighbor discovery report | Primary management |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

[1] IEEE P802.16n/D6, “Air Interface for Broadband Wireless Access Systems Draft Amendment: Higher Reliability Networks”