| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Periodic Ranging between HR-MSs for Direct Link Maintenance in 16.1a Networks |  |
| Date Submitted | 2012-11-05 |  |
| Source(s) | Anseok Lee, Eunkyung Kim, Jaesun Cha, Wooram Shin, Kwangjae Lim, Dongseung Kwon ETRI | E-mail: alee@etri.re.kr |
| Re: | In response to SB 010a on P802.16.1a/D6 |  |
| Abstract | This provides AWD text proposals to enable periodic ranging between HR-MSs for link maintenance in IEEE 802.16.1a Network. |  |
| Purpose | To be discussed and adopted by 802.16 GRIDMAN TG |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Periodic Direct Ranging between HR-MSs for Direct Link Maintenance

in 16.1a Networks

Anseok Lee, Eunkyung Kim, Jaesun Cha, Wooram Shin, Kwangjae Lim, Dongseung Kwon

ETRI

# Introductions

The direct ranging function in the current draft is divided into non-periodic and periodic operation. The periodic ranging function in the current draft is in multicast or broadcast based operation.

However, in the multicast or broadcast based periodic ranging function, the reliable deliveries of ranging command message to HR-MSs are not guaranteed because of the absence of error handling mechanism. This unreliable delivery may incur the recursive failures of ranging operation.

Moreover, the current periodic ranging function only mandates the reversed action (HR-MSs changing its role as transmitter/receiver of ranging code) which designed only for BS-controlled direct communication and FTN (Forwarding To Network). The periodic ranging function should be designed also for direct link maintenance (including the alternative path to an HR-MS).

And the current draft does not support termination of the periodic ranging operation. To stop the link maintenance, termination of periodic ranging shall be needed.

In this contribution, we provide some clarifications on the ranging between HR-MSs (direct ranging) for HR-MS Neighbor Discovery in 16.1a Network as follows:

- Support of non-reversed action of periodic ranging operation

- Support of periodic ranging termination

- Removal of multicast or broadcast based periodic ranging

# Proposed Text for the 802.16.1a Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16.1a Amendment Draft Standard

The text in RED color: the removal of existing 802.16.1a Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16.1a Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy 1: Accept the following modifying to Section 6.2.3.65.11]

6.2.3.65.11 AAI-HR-RNG-CMD message

HR-BS/RS sends AAI-HR-RNG-CMD message to instruct its associated HR-MS to carry out neighbor discovery. The field “Action” in AAI-HR-RNG-CMD message tells the receiving HR-MS whether to transmit or receive the specified ranging signal. Using the field “Reversed action offset”, the serving HR-BS/HR-RS can schedule transmissions of ranging signal in both directions between a pair of HR-MSs.

Table 6-108k—AAI-HR-RNG-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Frame Identifier | 4 | Frame which contains the ranging channel. The frame identifier is the 4 least significant bits of the frame number. |  |
| Subframe Index | 3 | Indicates the subframe index of the allocated ranging opportunity. |  |
| Action | 2 | 0b00: ranging for neighbor discovery and receiving node to carry out transmission 0b01: ranging for neighbor discovery and receiving node to carry out receiving 0b10: DC/FTN periodic ranging transmission Indicates the purpose of this message 0b00 : Ranging code transmission 0b01 : Ranging code reception 0b10 : Terminate periodic ranging code transmission or reception 0b11: Reserved |  |
| if (Action == 0b00){ |  |  |  |
| Period | 4 | Indicates the period of periodic ranging: Bits 0-3 : If bit 0-3 represent the binary representation of the integer N, then the Period = 2^(N+1) [Superframes] | Shall be present when this message is unicast |
| Duration | 8 | Indicates the duration of periodic ranging 0 : Stop the periodic ranging code transmission when it receive the suspension command 1-255 : Stop the periodic ranging code transmission after this value of periods | Shall be present when this message is unicast |
| Dedicated ranging code index | 5 | Indicates the index of dedicated ranging code. |  |
| Transmit power level | 5 | Unsigned integer from 0 to 31 in units of 1 dBm, where 0b00000 = 0dBm and 0b11111 = 31dBm |  |
| Reversed action offset | 4 | 0b0000-0b1111: Indicates the number of frames between the frame that this HR-MS transmits the dedicated ranging code until the frame that this HR-MS should try to receive the same ranging code. Indicates the reversed action 0 : No reversed action 1-15 : Number of frames between the transmit and reception frames | Present if HR-BS assigns ranging resources in both directions. |
| If (Reversed action offset != 0) { |  |  |  |
| Reporting mode | 1 | Indicates if the report mode is exclusive or triggered by threshold. 0b0: exclusive reporting 0b1: triggered-based reporting | Present if the field "Reserved action offset" is present |
| if(Reporting mode == 0b1){ |  |  |  |
| SINR threshold | 4 | Indicates the SINR threshold for the ranging signal above which report should be made by receiving station. The 4 bit value from 0b0000 to 0b1111 represent values among {-9, -8.5, -8, -7.5, -7, -6.5,-6, -5.5, -5, -4.5, -4, -3.5, -3, -2.5,-2, -1.5} dB | Present if the field "Reversed action offset" is present |
| } |  |  |  |
| } |  | // end of Reversed action offset |  |
| }elseif (Action == 0b01){ |  |  |  |
| Period | 4 | Indicates the period of periodic ranging: Bits 0-3 : If bit 0-3 represent the binary representation of the integer N, then the Period = 2^(N+1) [Superframes] | Shall be present when this message is unicast |
| Duration | 8 | Indicates the duration of periodic ranging 0 : Stop the periodic ranging code transmission when it receive the suspension command 1-255 : Stop the periodic ranging code transmission after this value of periods | Shall be present when this message is unicast |
| Reversed action offset | 4 | Indicates the reversed action 0 : No reversed action 1-15 : Number of frames between the transmit and reception frames |  |
| Reporting mode | 1 | Indicates if the report mode is exclusive or triggered by threshold. 0b0: exclusive reporting 0b1: triggered-based reporting |  |
| if(Reporting mode == 0b1){ |  |  |  |
| SINR threshold | 4 | Indicates the SINR threshold for the ranging signal above which report should be made by receiving station. The 4 bit value from 0b0000 to 0b1111 represent values among {-9, -8.5, -8, -7.5, -7, -6.5,-6, -5.5, -5, -4.5, -4, -3.5, -3, -2.5,-2, -1.5} dB |  |
| } |  |  |  |
| Reversed action offset | 4 | 0b0000-0b1111: Indicates the number of frames between the frame that this HR-MS receives the dedicated ranging code until the frame that this HR-MS should switch to transmit the same ranging code. | Present if HR-BS assigns ranging resources in both directions. |
| If (Reversed action offset != 0) { |  |  |  |
| Dedicated ranging code index | 5 | Indicates the index of dedicated ranging code. |  |
| Transmit power level | 5 | Unsigned integer from 0 to 31 in units of 1 dBm, where 0b00000 = 0dBm and 0b11111 = 31dBm | Present if the field "Reversed action offset" is present |
| } else { |  |  |  |
| Dedicated ranging code index | 5 | Indicates the index of dedicated ranging code. | Optional |
| } |  | // end of Reversed action offset |  |
| }elseif (Action == 0b10){ |  |  |  |
| Transmit power level | 5 | Unsigned integer from 0 to 31 in units of 1 dBm, where 0b00000 = 0dBm and 0b11111 = 31dBm |  |
| Periodicity | 2 | Indicates the periodicity of periodic ranging: 0b00: transmit ranging signal every 2 frames 0b01: transmit ranging signal every 4 frames 0b10: transmit ranging signal every 16 frames 0b11: transmit ranging signal every 32 frames |  |
| Tx/Rx Offset | 1 | Offset between transmitting and receiving ranging from the other HR-MS: 0b0: Offset = 1 frame 0b1: Offset = 3 frames |  |
| } |  |  |  |
| Dedicated ranging code index | 5 | Indicates the index of dedicated ranging code. | Optional when ("Action == 0b01" and "Reversed action offset is not present") Mandatory for other cases |

When an AAI-HR-RNG-CMD message is used to scheduled ranging for neighbor discovery, it may be unicast, multicast or broadcast to the receiving HR-MSs. When the AAI-HR-RNG-CMD message is multicast or broadcast, the field “Reversed action offset” shall not be present in the message.

When AAI-HR-RNG-CMD message is used to schedule periodic ranging between two HR-MSs involved in BS-controlled direct communication, it is transmitted in the DL resource assignment identified by the TWDC allocated to one of the two sides of the direct communication link. The HR-MS that has its TWDC used shall transmit the ranging signal as defined by the field “Frame Index”, “Subframe Index” and “Periodicity”. The other HR-MS, i.e., at the other side of the direct communication link, shall transmit the same ranging code, at the same location within a frame, and at an offset as defined by Tx/Rx Offset.

When AAI-HR-RNG-CMD message is used to schedule periodic ranging between two HR-MSs involved in BS-controlled FTN, it is transmitted in the DL resource assignment of the forwarded HR-MS. The forwarding HR-MS shall transmit the ranging signal as defined by the fields “Frame Index”, “Subframe Index” and “Periodicity”. The forwarded HR-MS shall transmit the same ranging code, at the same location within a frame, and at an offset as defined by Tx/Rx Offset.

When AAI-HR-RNG-CMD message is used to schedule periodic ranging between two HR-MSs (including alternative path maintenance, BS-controlled direct communication, and BS-controlled FTN), Period field may be set as an appropriate maintenance interval. When HR-MSs once received the periodic ranging code transmit or receive command, HR-MSs shall transmit or receive ranging code periodically in the allocated resource.

The periodic ranging operation shall be terminated until the Duration times of periods when Duration is not 0. The operation shall also be terminated when the HR-MS receives the AAI-HR-RNG-CMD message with Action is 0b10 (Termination of periodic ranging transmission or reception command).

[Remedy 2: Accept the following modifying to Section 6.12.2.2.1.1]

6.12.2.2.1.1 HR-MS Neighbor Discovery

For associated HR-MSs to discover each other, the serving HR-BS/HR-RS shall request some HR-MSs to broadcast ranging signals while other HR-MSs to attempt to receive and verify their neighbor relationship. The process is described as follows:

— The serving HR-BS/HR-RS sends AAI-HR-RNG-CMD message described in 6.2.3.65.11 to schedule one or multiple registered HR-MSs to broadcast ranging sequences in assigned channels. Multiple HR-MSs may share the same ranging sequence or the same assigned channel.

— Using AAI-HR-RNG-CMD message, the serving HR-BS/HR-RS also schedules some other HR-MS(s) to listen on ranging channel for the ranging signal.

— The HR-MS receiving a ranging sequence that has met the reporting criteria defined by the “Reporting mode” and possibly “SINR threshold” as specified in AAI-HR-RNG-CMD message shall report their measurements to the serving HR-BS/HR-RS using AAI-HR-RNG-REP message described in 6.2.3.65.12.

— The serving HR-BS/HR-RS may schedule periodic ranging code transmit/receive command to involving HR-MSs for maintenance of link between two HR-MSs. For periodic ranging code transmit/receive operation, the Period and Duration fields of AAI-HR-RNG-CMD message may be set as appropriate values. The commanded HR-MSs shall schedule transmission and reception of the ranging code as in the message.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

[1] IEEE P802.16.1a/D3, “Draft Amendment for IEEE Standard for WirelessMAN-Advanced Air Interface for Broadband Access Systems - Amendment: Higher Reliability Networks”