| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Periodic Ranging between HR-MSs for Direct Link Maintenance in 16.1a Networks |  |
| Date Submitted | 2012-11-14 |  |
| Source(s) | Anseok Lee, Eunkyung Kim, Jaesun Cha, Wooram Shin, Kwangjae Lim, Dongseung Kwon ETRI | E-mail: alee@etri.re.kr |
| Re: | In response to SB 010a on P802.16.1a/D6 |  |
| Abstract | This provides AWD text proposals to enable periodic ranging between HR-MSs for link maintenance in IEEE 802.16.1a Network. |  |
| Purpose | To be discussed and adopted by 802.16 GRIDMAN TG |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Periodic Direct Ranging between HR-MSs for Direct Link Maintenance

in 16.1a Networks

Anseok Lee, Eunkyung Kim, Jaesun Cha, Wooram Shin, Kwangjae Lim, Dongseung Kwon

ETRI

# Introductions

The direct ranging function in the current draft is divided into non-periodic and periodic operation. The periodic ranging function in the current draft is in multicast or broadcast based operation.

However, in the multicast or broadcast based periodic ranging function, the reliable deliveries of ranging command message to HR-MSs are not guaranteed because of the absence of error handling mechanism. This unreliable delivery may incur the recursive failures of ranging operation.

Moreover, the current periodic ranging function only mandates the reversed action (HR-MSs changing its role as transmitter/receiver of ranging code) which designed only for BS-controlled direct communication and FTN (Forwarding To Network). The periodic ranging function should be designed also for direct link maintenance (including the alternative path to an HR-MS).

And the current draft does not support termination of the periodic ranging operation. To stop the link maintenance, termination of periodic ranging shall be needed.

In this contribution, we provide some clarifications on the ranging between HR-MSs (direct ranging) for HR-MS Neighbor Discovery in 16.1a Network as follows:

- Support of non-reversed action of periodic ranging operation

- Support of periodic ranging termination

- Removal of multicast or broadcast based periodic ranging

# Proposed Text for the 802.16.1a Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16.1a Amendment Draft Standard

The text in RED color: the removal of existing 802.16.1a Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16.1a Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy 1: Accept the following modifying to Section 6.2.3.65.11]

6.2.3.65.11 AAI-HR-RNG-CMD message

HR-BS/RS sends AAI-HR-RNG-CMD message to instruct its associated HR-MS to carry out neighbor discovery. The field “Action” in AAI-HR-RNG-CMD message tells the receiving HR-MS whether to transmit or receive the specified ranging signal. Using the field “Reversed action offset”, the serving HR-BS/HR-RS can schedule transmissions of ranging signal in both directions between a pair of HR-MSs.

Table 6-108k—AAI-HR-RNG-CMD message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Frame Identifier | 4 | Frame which contains the ranging channel. The frame identifier is the 4 least significant bits of the frame number. |  |
| Subframe Index | 3 | Indicates the subframe index of the allocated ranging opportunity. |  |
| Action | 2 | 0b00: ranging for neighbor discovery and receiving node to carry out transmission 0b01: ranging for neighbor discovery and receiving node to carry out receiving 0b10: DC/FTN periodic ranging transmission as part of BS-control DC/FTN and alternative path management, the HR-MS is scheduled to transmit the ranging code 0b11: Reserved periodic ranging as part of BS-control DC/FTN and alternative path management, the HR-MS is scheduled to receive the ranging code |  |
| if (Action == 0b00){ |  |  |  |
| Transmit power level | 5 | Unsigned integer from 0 to 31 in units of 1 dBm, where 0b00000 = 0dBm and 0b11111 = 31dBm |  |
| Reversed action offset | 4 | 0b0000-0b1111: Indicates the number of frames between the frame that this HR-MS transmits the dedicated ranging code until the frame that this HR-MS should try to receive the same ranging code. | Present if HR-BS assigns ranging resources in both directions. |
| Reporting mode | 1 | Indicates if the report mode is exclusive or triggered by threshold. 0b0: exclusive reporting 0b1: triggered-based reporting | Present if the field "Reserved Reversed action offset" is present |
| if(Reporting mode == 0b1){ |  |  |  |
| SINR threshold | 4 | Indicates the SINR threshold for the ranging signal above which report should be made by receiving station. The 4 bit value from 0b0000 to 0b1111 represent values among {-9, -8.5, -8, -7.5, -7, -6.5,-6, -5.5, -5, -4.5, -4, -3.5, -3, -2.5,-2, -1.5} dB | Present if the field "Reversed action offset" is present |
| } |  |  |  |
| }elseif (Action == 0b01){ |  |  |  |
| Reporting mode | 1 | Indicates if the report mode is exclusive or triggered by threshold. 0b0: exclusive reporting 0b1: triggered-based reporting |  |
| if(Reporting mode == 0b1){ |  |  |  |
| SINR threshold | 4 | Indicates the SINR threshold for the ranging signal above which report should be made by receiving station. The 4 bit value from 0b0000 to 0b1111 represent values among {-9, -8.5, -8, -7.5, -7, -6.5,-6, -5.5, -5, -4.5, -4, -3.5, -3, -2.5,-2, -1.5} dB |  |
| } |  |  |  |
| Reversed action offset | 4 | 0b0000-0b1111: Indicates the number of frames between the frame that this HR-MS receives the dedicated ranging code until the frame that this HR-MS should switch to transmit the same ranging code. | Present if HR-BS assigns ranging resources in both directions. |
| Transmit power level | 5 | Unsigned integer from 0 to 31 in units of 1 dBm, where 0b00000 = 0dBm and 0b11111 = 31dBm | Present if the field "Reversed action offset" is present |
| }elseif (Action == 0b10 or 0b11){ |  |  |  |
| TWDC | 12 | TWDC of the HR-MS corresponding to the DC link | Present for periodic ranging for DC |
| Transmit power level | 5 | Unsigned integer from 0 to 31 in units of 1 dBm, where 0b00000 = 0dBm and 0b11111 = 31dBm |  |
| Periodicity | 23 | Indicates the periodicity of periodic ranging: 0b00: transmit ranging signal every 2 frames 0b01: transmit ranging signal every 4 frames 0b10: transmit ranging signal every 16 frames 0b11: transmit ranging signal every 32 frames 0b000: the periodic ranging assignment is terminated 0b001 to 0b 111: represents the value k that indicates the interval between consecutive transmissions of periodic ranging signal, where the interval is equal 2k frames |  |
| Duration | 8 | Indicates the duration of periodic ranging 0 : Stop the periodic ranging code transmission when it receive the suspension command 1-255 : Stop the periodic ranging code transmission after this value of periods Note that this value should be ignored when the Periodicity is 0 |  |
| Tx/Rx Offset | 1 | Offset between transmitting and receiving ranging from the other HR-MS: 0b0: Offset = 1 frame 0b1: Offset = 3 frames | Present if periodic ranging is scheduled in both direction |
| Reporting mode | 1 | Indicates if the report mode is exclusive or triggered by threshold. 0b0: exclusive reporting 0b1: triggered-based reporting | Present if this is periodic ranging for path management |
| if(Reporting mode == 0b1){ |  |  |  |
| SINR threshold | 4 | Indicates the SINR threshold for the ranging signal above which report should be made by receiving station. The 4 bit value from 0b0000 to 0b1111 represent values among {–9, –8.5, –8, –7.5, –7, –6.5,–6, –5.5, –5, –4.5, –4, –3.5, –3, –2.5,–2, –1.5} dB |  |
| } |  |  |  |
| Dedicated ranging code index | 5 | Indicates the index of dedicated ranging code. | Optional when ("Action == 0b01" and "Reversed action offset is not present") Mandatory for other cases |

When an AAI-HR-RNG-CMD message is used to scheduled ranging for neighbor discovery, it may be unicast, multicast or broadcast to the receiving HR-MSs. When the AAI-HR-RNG-CMD message is multicast or broadcast, the field “Reserved action offset” shall not be present in the message.

When AAI-HR-RNG-CMD message is used to schedule periodic ranging between two HR-MSs involved in BS-controlled direct communication, it is unicasted to the HR-MSstransmitted in the DL resource assignment identified by the TWDC allocated to one of the two sides of the direct communication link. The HR-MS that has its TWDC used included in the message shall transmit the ranging signal as defined by the field “Frame Index”, “Subframe Index” and “Periodicity”. In case the field “TX/RX Offset” is present, this HR-MS shall receive the same ranging signal in the opposite direction of the direct communication link after the corresponding offset.  The other HR-MS, i.e., at the other side of the direct communication link, shall transmit the same ranging code, at the same location within a frame, and at an offset as defined by Tx/Rx Offset.

When AAI-HR-RNG-CMD message is used to schedule periodic ranging between two HR-MSs involved in BS-controlled FTN, it is transmitted in the DL resource assignment of the forwarded HR-MS. The forwarding HR-MS shall transmit the ranging signal as defined by the fields “Frame Index”, “Subframe Index” and “Periodicity”. The forwarded HR-MS shall transmit the same ranging code, at the same location within a frame, and at an offset as defined by Tx/Rx Offset.

When AAI-HR-RNG-CMD message is used to schedule periodic ranging under alternative path management, it is unicasted to the involved HR-MSs. The HR-MSs shall report the measured link conditions to the HR-BS as defined in the “Reporting mode” and “SINR threshold” fields.

When AAI-HR-RNG-CMD message is used to schedule periodic ranging between two HR-MSs (including DC/FTN/alternative path maintenance), Periodicity field may be set as an appropriate maintenance interval. When HR-MSs once received the periodic ranging code transmit or receive command, HR-MSs shall transmit or receive ranging code periodically in the allocated resource.

The periodic ranging operation shall be terminated until the Duration times of periods when Duration is not 0. The operation shall also be terminated when the HR-MS receives the AAI-HR-RNG-CMD message with Action is 0b10 or 0b11 and Periodicity is 0b000 (Termination of periodic ranging).

[Remedy 2: Accept the following modifying to Section 6.12.2.2.1.1]

6.12.2.2.1.1 HR-MS Neighbor Discovery

For associated HR-MSs to discover each other, the serving HR-BS/HR-RS shall request some HR-MSs to broadcast ranging signals while other HR-MSs to attempt to receive and verify their neighbor relationship. The process is described as follows:

— The serving HR-BS/HR-RS sends AAI-HR-RNG-CMD message described in 6.2.3.65.11 to schedule one or multiple registered HR-MSs to broadcast ranging sequences in assigned channels. Multiple HR-MSs may share the same ranging sequence or the same assigned channel.

— Using AAI-HR-RNG-CMD message, the serving HR-BS/HR-RS also schedules some other HR-MS(s) to listen on ranging channel for the ranging signal.

— The HR-MS receiving a ranging sequence that has met the reporting criteria defined by the “Reporting mode” and possibly “SINR threshold” as specified in AAI-HR-RNG-CMD message shall report their measurements to the serving HR-BS/HR-RS using AAI-HR-RNG-REP message described in 6.2.3.65.12.

— The serving HR-BS/HR-RS may schedule periodic ranging code transmit/receive command to involving HR-MSs for maintenance of link between two HR-MSs. For periodic ranging code transmit/receive operation, the Periodicity and Duration fields of AAI-HR-RNG-CMD message may be set as appropriate values. The commanded HR-MSs shall schedule transmission and reception of the ranging code as in the message.

[Remedy 3: Add the following text in line#4, page 238, P802.16.1a/D6:]

-- Neighbor Discovery in HR-Network

aaiHrRngCmd				AAI-HR-RNG-CMD,

aaiHrRngRep				AAI-HR-RNG-REP,

[Remedy4: Add the following text in line#47, page 282, P802.16.1a/D6:]

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- Neighbor Discovery Messages

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- single type definition for neighbor discovery messages

NbrFrameIdentifier ::=		INTEGER (0..15)

SubframeIndex ::=			INTEGER (0..7)

PeriodicRangingInfo ::=		SEQUENCE {

periodicity 				INTEGER (0..15),

duration				INTEGER (0..255)

}

DedicatedRangingCodeIndex ::=	INTEGER (0..31)

TransmitPowerLevel ::=		INTEGER (0..31)

ReversedActionOffset ::=		INTEGER (0..15)

SINRThreshold ::=			INTEGER (0..15)

ReceivedSINR ::=			INTEGER (0..15)

TimingOffset ::=			INTEGER (1..16384)

FrequencyOffset ::=			INTEGER (1..256)

RangingCodeTransmission ::=	SEQUENCE {

transmitPowerLevel			TransmitPowerLevel,

reversedActionOffset			ReversedActionOffset,

sinrThreshold				SINRThreshold			OPTIONAL,

...

}

RangingCodeReception ::=		SEQUENCE {

sinrThreshold				SINRThreshold			OPTIONAL,

reversedActionOffset			ReversedActionOffset,

transmitPowerLevel			TransmitPowerLevel			OPTIONAL,

...

}

PeriodicRangingCodeTransmission ::=	SEQUENCE {

twdc					TWDC					OPTIONAL,

transmitPowerLevel			TransmitPowerLevel			OPTIONAL,

periodicRangingInfo			PeriodicRangingInfo,

txRxOffset				txRxOffset				OPTIONAL,

sinrThreshold				SINRThreshold			OPTIONAL,

…

}

PeriodicRangingCodeReception ::=	SEQUENCE {

twdc					TWDC					OPTIONAL,

transmitPowerLevel			TransmitPowerLevel			OPTIONAL,

periodicRangingInfo			PeriodicRangingInfo,

txRxOffset				txRxOffset				OPTIONAL,

sinrThreshold				SINRThreshold			OPTIONAL,

…

}

RangingCodeReporting ::=		SEQUENCE {

frameIdentifier				NbrFrameIdentifier,

subframeIndex					SubframeIndex,

dedicatedRangingCodeIndex			DedicatedRangingCodeIndex,

receivedSINR					ReceivedSINR,

timingOffset					TimingOffset			OPTIONAL,

frequencyOffset					FrequencyOffset		OPTIONAL,

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- AAI-HR-RNG-CMD Message

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-HR-RNG-CMD ::=			SEQUENCE {

frameIdentifier				NbrFrameIdentifier,

subframeIndex 			SubframeIndex,

aaiHrRngCmdMessage 			CHOICE {

rangingCodeTransmission			RangingCodeTransmission,

rangingCodeReception			RangingCodeReception,

periodicRangingCodeTransmission		PeriodicRangingCodeTransmission,

periodicRangingCodeReception		PeriodicRangingCodeReception,		...

}

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- AAI-HR-RNG-REP Message

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-HR-RNG-REP ::=			SEQUENCE {

rangingCodeReportingList			SEQUENCE (SIZE (1..16)) OF RangingCodeReporting

}

[Remedy 5: Insert follwing text in line 21, page 134:]

- Any part of a synchronous HARQ retransmission that overlaps with a time to listen on ranging channel for the ranging signal, shall not be transmitted in the designated resource. Since both the HR-BS and the HR-MS have full knowledge of neighbor discovery activity, the HR-BS and the HR-MS shall skip the retransmission that occurs in the time to listen on ranging sigal from other HR-MS, and they shall perform the retransmission in the following subframe unless a UL retransmission is allocated using a UL basic Assignment A-MAP IE.

[Remedy 6: Modify the following text in line#59, page 237, P802.16.1a/D6:]

…

-- HR Multimode operation

aaiMmAdv 				AAI-MM-ADV,

aaiMmRsReq 			AAI-MM-RS-REQ,

aaiMmRsRsp 				AAI-MM-RS-RSP,

aaiMmRlReq 				AAI-MM-RL-REQ,

aaiMmRlRsp 				AAI-MM-RL-RSP,

…

[Remedy 7: Adapt the following modification throughout P802.16.1a/D6:]

- Replace "16.2.2.1.3.10" with "6.2.2.1.3.11" in line 61, page 172

- Replace "Multicast" with "Unicast or Multicast" in line 38, page 10

- Replace "IEE Std 802.16-2009" with "IEEE Std 802.16" in line 28, page 32.

- Replace "16.6.3.3.2" with "6.6.3.3.2" in line 33, page 57.

- Replace "16.6.3.3.2" with "6.6.3.3.2" in line 43, page 57.

- Replace "16.6.3.3.2" with "6.6.3.3.2" in line 53, page 57.

- Replace "16.6.3.3.2" with "6.6.3.3.2" in line 10, page 58.

- Replace "16.6.3.5.1" with "6.6.3.5.1" in line 21 and line 29 page 58.

- Replace "IEE Std 802.16-2009" with "IEEE Std 802.16" in line 48, page 67.

- Remove "of IEEE Std 802.16m-2011" in line 53-54, page 68.

- Replace "Table 265" with "Table 6-267" in line 11, page 209.

- Replace "This PRBS is the same as that for UL (uplink) ranging code generator of 802.16e and 802.16m, described by Figure 257 of IEEE 802.16-2009." with "This PRBS is used by a PRBS generator which is the same as that for UL (uplink) ranging code generation as described in Figure 6-194." in line 53-54, page 165.

- Remove "IEEE 802.16m" in line 24, page 166.

- Replace "Table 165" with "Table 6-170" in line 42, page 239.

- Replace "See Table 152 to Ttable 154." with "See Table 6-154 to Table 6-156." in line 44, page 240.

- Replace "Table 141" with "Table 6-144" in line 47, page 240.

- Replace "See Table 149 to Table 151" with "See Table 6-151 to Table 6-153" in line 50, page 240.

- Replace "Table 141" with "Table 6-144" in line53, page 240.

- Replace "Table 119" with "Table 6-120" in line 46, page 273.

- Replace "Table 120" with "Table 6-121" in line 48, page 274.

- Replace "Table 121 and Table 122" with "Table 6-122 and Table 6-123" in line 22, page 275.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

[1] IEEE P802.16.1a/D3, “Draft Amendment for IEEE Standard for WirelessMAN-Advanced Air Interface for Broadband Access Systems - Amendment: Higher Reliability Networks”