| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Alternative Path to HR-infrastructure Station in 16.1a Networks |  |
| Date Submitted | 2012-11-13 |  |
| Source(s) | Anseok Lee, Eunkyung Kim, Jaesun Cha, Wooram Shin, Kwangjae Lim, Dongseung Kwon ETRI | E-mail: alee@etri.re.kr |
| Re: | In response to SB 010a on P802.16.1a/D6 |  |
| Abstract | This provides AWD text proposals of clarification on Alternative Path in IEEE 802.16.1a Network. |  |
| Purpose | To be discussed and adopted by 802.16 GRIDMAN TG |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

Clarification on Alternative Path to HR-infrastructure Station

in 16.1a Networks

Anseok Lee, Eunkyung Kim, Jaesun Cha, Wooram Shin, Kwangjae Lim, Dongseung Kwon

ETRI

# Introductions

The current draft supports alternative path to HR-infrastructure station for higher reliability. The alternative path is activated when the trigger condition meets.

The trigger definition for alternative path activation is delivered to an HR-MS by AAI-HO-CMD message. However, the AAI-HO-CMD message does not have fields containing such information.

Moreover, an action for alternative path activation needs to be defined in trigger condition to assign the operation when the condition is met.

In this contribution, we provide some clarifications on the alternative path for HR-infrastructure stations in 16.1a Network as follows:

- Define action for alternative path activation

- Clarify the message format to be consistent with the alternative path activation function

# Proposed Text for the 802.16.1a Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16.1a Amendment Draft Standard

The text in RED color: the removal of existing 802.16.1a Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16.1a Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy 1: Accept the following modifying to Section 6.2.3.12]

Change Table 6-40 in section 6.2.3.12 AAI-HO-CMD as indicated:

Table 6-40—AAI-HO-CMD message field description

| … |  |  |  |
| --- | --- | --- | --- |
| If (Extended HO Mode == 0b00 ) { |  |  |  |
| Role | 1 | 0b0: Stay as HR-MS; 0b1: Change to HR-RS; |  |
| Target HR-infrastructure station-specific Trigger definitions | variable | Target HR-infrastructure station-specific triggers with encoding defined in Table 6-120 | Optional Shall be present if HR-BS present Target HR-Infrastructure station-specific triggers |
| … |  |  |  |

[Remedy 2: Accept the following modifying to Section 6.2.6.2]

6.2.6.2 Trigger condition definitions

Insert the following text at the end of Section 6.2.6.2:

The HR-BS may define the Target HR-infrastructure station-specific triggers by including Target HR-infrastructure station-specific triggers in the AAI-HO-CMD message. The HR-MS evaluates Target HR-infrastructure station-specific triggers only for the Target HR-infrastructure station. Target HR-infrastructure station-specific triggers use the format in Table 6-120, where only function types 0x1, 0x2, 0x3, and 0x4 and actions types 0x2 are allowed. When present, Target HR-infrastructure station-specific triggers override any general triggers (defined in the AAI-SCD message) of the same type, function, and action.

[Remedy3: Modify the following text in line#18, page 271, P802.16.1a/D6:]

HRHandoverCommend ::=		SEQUENCE {

extendHOMode			CHOICE {

alternativePath			AlternativePath,

fbisInitiation				FBISInitiation,

fbisTermination			FBISTermination,

hoCmdMSactionAsRs		HOCmdMSActionAsRS

},

handoverCommand			HandoverCommand

}

AlternativePath ::= 			SEQUENCE {

role					BOOLEAN,

-- 0: stay as HR-MS

-- 1: change to HR-RS

nbrSpecificTrigger			Triggers			OPTIONAL

}

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

[1] IEEE P802.16.1a/D6, “Draft Amendment for IEEE Standard for WirelessMAN-Advanced Air Interface for Broadband Access Systems - Amendment: Higher Reliability Networks”