| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text on Direct Communcation in IEEE 802.16.1a |  |
| Date Submitted | 2012-11-12 |  |
| Source(s) | Haiguang Wang, Anh Tuan Hoang I2R | E-mail: hwang@i2r.a-star.edu.sg *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | In response to sponsor ballot on P802.16.1a |  |
| Abstract | Proposed text on Direct Communcation in IEEE 802.16.1a |  |
| Purpose | To discuss and adopt the proposed text in the draft amendment document on GRIDMAN |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed text on Direct Communcation in IEEE 802.16.1a

Haiguang Wang, Anh Tuan Hoang

I2R

# Introductions

This document responses to the comment resolution 0075 and propose add text for AAI-DSA_RSP and the direct communication section in IEEE P802.16.1a.

# References

- IEEE 802.16-12-0132-00, GRIDMAN System Requirement Document including SARM annex, January 2012.

- IEEE P802.16nTM/D5, Air Interface for Broadband Wireless Access Systems - Draft Amendment: Higher Reliability Networks, June 2012.

- IEEE P802.16.1aTM/D5, WirelessMAN-Advanced Air Interface for Broadband Access Systems - Draft Amendment: Higher Reliability Networks, June 2012.

- IEEE P802.16TM-2012, IEEE Standard for Air Interface for Broadband Wireless Access Systems,” August 2012.

- IEEE P802.16.1TM-2012, IEEE Standard for WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems, September 2012.

# Proposed Text on the IEEE 802.16.1a Amendment Draft Standard

The proposed texts are written in three different types of fonts according to each change purpose as follows.

The same texts in the current draft: black

The texts to be deleted by this contribution: red strikeout

The texts to be added by this contribution: blue underline

[-------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Insert the following new rows at the beginning of Table 6-86, page 47, line 37]

Table 6-86—AAI-DSA-RSP message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | FID Change Count from corresponding the AAI-DSA-REQ message | Shall always be present |
| Confirmation Code | 1 | Zero indicates the request was successful. Nonzero indicates failure | Shall always be present |
| If (Confirmation Code == 0 && AMS-initiated AAI-DSA-REQ) { |  |  |  |
| FID | 4 | An identifier of a service flow |  |
| Flag for flow setup over DC link | 1 | The presence of this flag indicates that the flow is to be setup over a DC link. The value of the flag indicates whether the DC flow setup is immediate or delayed. 0b0: the setup is immediate, with TWDC for transmitting is included 0b1: the setup is delayed. | Present when a flow is setup over a DC link and this AAI-DSA-RSP message is sent by HR-BS |
| If (Flag for flow setup over DC == 0){ |  |  |  |
| TWDC | 12 | TWDC that this HR-MS should use for transmitting over the corresponding DC link |  |
| } |  |  |  |
| } |  |  |  |
| If(received an AAI-DSA-REQ including Group Parameter Create/ Change parameter) { |  |  |  |
| … | … | … | … |

[Remedy2: Insert the following text at line 55, page 49]

6.2.3.47.3 AAI-DSA-ACK

Change Table 6-87 in 6.2.3.47.3 as indicated:

Table 6-87—AAI-DSA-ACK  Message Field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | FID Change Count from corresponding the AAI-DSA-RSP message |  |
| Confirmation code | 1 | Zero indicates the request was successful. Nonzero indicates failure |  |
| TWDC | 12 | TWDC of the transmitter of this DC flow. | Present if the flow to be deleted is over a DC link |

[Remedy3: Replace Figure 6-229d in page 137 by the following figure:]

Figure 6-229d —The establishment of direct communication between HR-MS

[Remedy4: Modify the text in line#37-48, page 137, P802.16.1a/D6 as indicated:]

Upon receiving an AAI-DSA-REQ message from an HR-MS, if the HR-BS already setup a direct communication link between the source and destination HR-MS and it intends to setup the flow over the direct communication link, then the HR-BS shall send an AAI-DSA-RSP message to source HR-MS and set the “Flag for flow setup over DC link” field to 0b1 to indicate that the assignment of TWDC for the flow is delayed while HR-BS setting up the flow on the DC link CC equals to direct-comm-setup as defined in Table 607 and STID of the direct communication link. At the same time, the HR-BS shall send AAI-DSA-REQ to the destination HR-MS with an indication of the direct communication flag and STID of direct communication link as specified in the Table 734 the TWDC address for the destination HR-MS to receive on the corresponding DC link. The destination HR-MS shall send back an AAI-DSA-RSP with indication of accept/reject of direct communication and the HR-BS sends an AAI-DSA-ACK back to the destination HR-MS. Upon acceptance from the destination HR-MS, the The HR-BS shall send an AAI-DSA-RSP to the source HR-MS with indication of accept/reject of flow setup with indication of type  the “Flag for flow setup over DC link” field set to 0b0 and the TWDC address for the source HR-MS for transmitting on the corresponding DC link. In this case, except the fields “Flag for flow setup over DC link” and “TWDC”, all the rest of the fields are the same in the two AAI-DSA-RSP messages sent by the HR-BS to the source HR-MS. If direct communication setup is rejected, the flow shall be setup on the uplink in a normal way.

[Remedy5: Modify the text in line#33-65, page 137, P802.16.1a/D6 as indicated:]

When HR-MS initiates the service flow modification, if the modification increases the resource allocated to a flow over direct communication, then the HR-BS should hold on the transaction with source HR-MS and finish the transaction with destination and then finish the transaction with source. If the modification reduces the resource allocated to a flow, the HR-BS should finish the transaction with source and then finish the transaction with destination. An HR-MS initiates service flow modification over a DC link by sending an AAI-DSC-REQ message to HR-BS. The AAI-DSC-REQ message contains the TWDC address of the transmitter corresponding to the modified flow.

When HR-BS initiates the service flow deletion and the target flow is over a direct communication link, it should send AAI-DSD-REQ to the two HR-MS respectively. When source/destination HR-MS initiates the service flow deletion and the target flow is over a direct communication link, HR-BS should also send an AAI-DSD to the destination/source HR-MS also. HR-BS shall release the resource allocated to the flow over direct communication link when the transaction of AAI-DSD for that flow finishes. The AAI-DSD-REQ message contains the TWDC address of the transmitter corresponding to the modified flow.

[Remedy6: Insert the following new rows at the beginning of Table 6-88, page 50, line 10]

Table 6-88—AAI-DSC-REQ Message Field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender | Shall always be present |
| SFID | 32 | Service flow identifier | Shall be always present |
| TWDC | 12 | TWDC of the transmitter of this DC flow. | Present if the flow to be modified is over a DC link |

[Remedy7: Insert the following text at line 42, page 52]

6.2.3.47.5 AAI-DSC-RSP

Change Table 6-89 in 6.2.3.47.5 as indicated:

Table 6-89—AAI-DSC-RSP Message Field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | FID Change Count from corresponding the AAI-DSC-REQ message | Shall always be present |
| SFID | 32 | Service flow identifier | Present when an ABS initiates AAI-DSC-REQ |
| TWDC | 12 | TWDC of the transmitter of this DC flow. | Present if the flow to be modified is over a DC link |
| Confirmation Code | 1 | 0 = Request was successful 1= Request was failure |  |
| … | … | … | … |

6.2.3.47.6 AAI-DSC-ACK

Change Table 6-90 in 6.2.3.47.6 as indicated:

Table 6-90—AAI-DSC-ACK  Message Field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | FID Change Count from corresponding the AAI-DSC-RSP message |  |
| Confirmation Code | 1 | 0 = request was successful 1 = request was failure |  |
| TWDC | 12 | TWDC of the transmitter of this DC flow. | Present if the flow to be modified is over a DC link |

6.2.3.47.7 AAI-DSD-REQ

Change Table 6-91 in 6.2.3.47.7 as indicated:

Table 6-91—AAI-DSD-REQ  Message Field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | The change count of this transaction assigned by the sender. If a new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender | Shall always be present |
| FID | 4 | Flow identifier to be deleted | Shall be always present when an existing service flow is deleted |
| TWDC | 12 | TWDC of the transmitter of this DC flow. | Present if the flow to be deleted is over a DC link |
| If(E-MBS service flow to delete exists){ |  |  |  |
| … | … | … | … |

6.2.3.47.8 AAI-DSD-RSP

Change Table 6-92 in 6.2.3.47.8 as indicated:

Table 6-92—AAI-DSD-RSP  Message Field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID Change Count | 4 | FID Change Count from corresponding the AAI-DSD-REQ message | Shall always be present |
| FID | 4 | Flow identifier to be deleted | Shall be always present when an existing service flow is deleted |
| TWDC | 12 | TWDC of the transmitter of this DC flow. | Present if the flow to be deleted is over a DC link |
| Confirmation Code | 1 | 0 = Request was successful 1= Request was failure |  |
| … | … | … | … |

[Remedy8: Insert new text after line 60 in page 260 as indicated]

fid				FID				OPTIONAL,

dcSetupFlag			BOOLEAN			OPTIONAL,

twdcAddressforTX		INTEGER (0…4095)		OPTIONAL,

…

[Remedy9: Insert new text after line 36 in page 261as indicated]

confirmationCode		ConfirmationCode,

twdcAddressofTX		INTEGER (0…4095)		OPTIONAL,

…

[Remedy10: Insert new  text after line 36 in page 261 as indicated]

absInitDscInfo			AbsInitDscInfo		OPTIONAL,

twdcAddressofTX		INTEGER (0…4095)		OPTIONAL,

…

[Remedy11: Insert new  text after line 53 in page 262 as indicated]

sfid				SFID				OPTIONAL,

twdcAddressofTX		INTEGER (0…4095)		OPTIONAL,

…

[Remedy12: Insert new  text after line 3 in page 263 as indicated]

confirmationCode		ConfirmationCode,

twdcAddressofTX		INTEGER (0…4095)		OPTIONAL,

…

[Remedy13: Insert new  text after line 21 in page 263 as indicated]

fid				FID				OPTIONAL,

twdcAddressofTX		INTEGER (0…4095)		OPTIONAL,

…

[Remedy14: Insert new  text after line 32 in page 263 as indicated]

fid				FID				OPTIONAL,

twdcAddressofTX		INTEGER (0…4095)		OPTIONAL,

…

[-------------------------------------End of Text Proposal---------------------------------------------------]