| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AWD Texts for Multi-BS MIMO in IEEE 802.16q |  |
| Date Submitted | 2013-01-14 |  |
| Source(s) | Wooram Shin, Eunkyung Kim, Jaesun Cha, Anseok Lee, Kwangjae Lim ETRI | E-mail: w.shin@etri.re.kr *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | In response to Call for Contributions on P802.16q Multi-tier Networks toward IEEE 802.16 Session #83 |  |
| Abstract | Proposal regarding multi-BS MIMO for interference mitigation in IEEE 802.16q on Multi-tier Networks |  |
| Purpose | To discuss and adopt the proposed text in Amendment Working Document (AWD) on IEEE 802.16q Multi-tier Networks |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed AWD Texts for Multi-BS MIMO in IEEE 802.16q

Wooram Shin, Eunkyung Kim, Jaesun Cha, Anseok Lee, Kwangjae Lim

ETRI

# Overview

The 802.16q amendment shall be developed in accordance with the P802.16q project authorization request (PAR) with its accompanying Five Criteria Statement [1]. The scope of the resulting project includes “MAC/PHY protocol enhancements for cooperation among base stations in multi-tier networks to enhance interference mitigation.” All contents regarding interference management included in the 802.16q amendment shall meet requirements in [2].

Multi-BS MIMO is a technique mitigating interference and enhancing cell-edge throughput via cooperative transmission/reception by multiple BSs being adjacent to or overlaid by each other, which has been adopted in IEEE Std 802.16.1-2012 for the purpose of inter-cell interference mitigation. In multi-tier networks, cross-tier interference, which is generated from overlaying macro cell(s) and overlaid small cell(s) to overlaid small cell(s) and overlaying macro cell(s), respectively, further degrades performance. Multi-BS MIMO can be an essential way to mitigate cross-tier interference as well as co-tier interference.

In this context, this document represents the general description of multi-BS MIMO (cooperative transmission/reception) for “interference management” in the 802.16q amendment working document (AWD), which includes:

- DL multi-BS MIMO

  - DL single-BS precoding with multi-BS coordination

  - DL multi-BS joint proceesing

  - Channel feedback for closed-loop transmit precoding

    - Sounding-based feedback

    - MIMO-coefficient-based feedback

    - Codebook-based feedback

    - Antenna-selection/grouping-index-based feedback

  - Channel quality measurement and report

  - Trigger conditions

- UL multi-BS MIMO

  - Multi-BS joint processing

  - Trigger conditions

# References

- IEEE 802.16-12-0394-06-Gdoc, PAR and Five Criteria for P802.16q (Amendment to IEEE Std 802.16 for Multi-tier Networks), July 2012.

- IEEE 802.16-12-0615-02-000q, Consolidated proposed text towards an initial draft System Requirements on IEEE 802.16q Multi-tier Networks, November 2012.

# Proposed Texts on Amendment Working Document (AWD) of IEEE 802.16q

[-------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy: Add the following texts in IEEE 802.16q AWD:]

## Interference Management

### Resource Management

### Multi-BS MIMO

Multi-BS MIMO techniques improve sector throughput and cell-edge throughput through multi-BS cooperative signaling. These include DL single-BS precoding with multi-BS coordination and DL/UL multi-BS joint processing.

#### DL Multi-BS MIMO

##### DL single-BS precoding with multi-BS coordination

When DL single-BS precoding with multi-BS coordination is enabled, interference from adjacent BSs is mitigated by coordinating the precoders applied in the adjacent BSs.

##### DL multi-BS joint processing

When DL multi-BS joint processing is enabled, radio resource allocation, data mapping, and pilot pattern allocation shall be aligned among coordinating BSs. The same data packet is transmitted by the coordinating BSs on the same time and frequency resources.

##### Channel feedback for closed-loop transmit precoding

###### Sounding-based feedback

[Note: In this subclause, operations of sounding-based feedback will be provided to support DL multi-BS MIMO.]

###### MIMO-coefficeint-based feedback

[Note: In this subclause, operations of MIMO-coefficient-based feedback will be provided to support DL multi-BS MIMO when MIMO midamble is supported.]

###### Codebook-based feedback

[Note: In this subclause, operations of codebook-based feedback will be provided to support DL multi-BS MIMO when MIMO midamble is supported.]

###### Antenna-selection/grouping-index-based feedback

[Note: In this subclause, operations of antenna-selection/grouping-index-based feedback will be provided to support DL multi-BS MIMO when MIMO midamble is supported.]

##### Channel quality measurement and report

An S-BS may request a subordinate MS to scan the neighbor BS(s) which are participating in cooperative transmission. The MS scans and reports the channel measurement result to the S-BS, if certain conditions are met.

##### Trigger conditions

[Note: In this subclause, trigger conditions for DL multi-BS MIMO will be provided.]

#### UL Multi-BS MIMO

##### UL multi-BS joint processing

When UL multi-BS joint processing is enabled, radio resource allocation, data mapping, and pilot pattern allocation shall be aligned among coordinating BSs. The same data packet is received by the coordinating BSs on the same time and frequency resources.

##### Trigger conditions

[Note: In this subclause, trigger conditions for UL multi-BS MIMO will be provided.]

[-------------------------------------End of Text Proposal---------------------------------------------------]