| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Narrowband Ad-Hoc Meeting Report |  |
| Date Submitted | 2015-03-12 |  |
| Source(s) | Tim Godfrey EPRI | Voice: E-mail: *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Session #96 Narrowband Discussion |  |
| Abstract | IEEE 802.16 Session #96 Narrowband Discussion |  |
| Purpose | To capture results of dicussion on Narrowband 1 MHz channel |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html> |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Status | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |

# IEEE 802.16 Working Group

Narrowband Ad Hoc Meeting Report

Estrel Hotel, Room 30210

Berlin, DE

March 11, 2015

This discussion is a follow up on the prior contribution 16-14-0015-01-Gcon (“Call For Interest: 802.16 enhancements for flexible spectrum support”).

To support the smaller spectrum allocations that are available for application of 802.16 in vertical markets, there is a need for standardized operational modes and profiles. In general, a standardized technique for operating in narrower channels is desired. Specifically, the availability of two 1 MHz –wide blocks of spectrum in the 700 MHz Upper A-Block that is suitable for utility field area networks and other vertical applications has precipitated the need for a suitable standard.

Based on previous discussions with vendors involved with the WiMAX Forum WiGRID profile, different options for technical approaches have been proposed to support a 1 MHz channel.

Discussion of options led to the development of a set of OFDMA parameters as represented in Table 1. It is believed that these parameters can be supported by chipsets currently used for commerical equipment, but that is yet to be confirmed with the vendors.

The group discussed the possible need to specify the use of a reduced preamble to operate within the narrow channel.

The group discussed the possibility of developing an amendment PAR to specify interoperable operation in narrow channel sizes including 1 MHz. It may be possible to consider ever narrower channels. Consideration should be given to adaptations to reduce overhead in such narrow channels.

| Nominal Channel Bandwidth |  |  | 1 MHz |
| --- | --- | --- | --- |
| Sampling factor |  |  | 28/25 |
| Sampling frequency (MHz) |  |  | 2.8 |
| FFT size |  |  | 512 |
| Subcarrier spacing (kHz) |  |  | 5.47 |
| Subchannels |  |  | 6 |
| Actual Bandwidth (centered on nominal channel) |  |  | 918.75 kHz |
| Frame Size |  |  | 10 ms |
| Useful symbol time (µs) |  |  | 182.86 |
| For CP ratio = 1/8 | OFDMA symbol time (µs) |  | 205.72 |
|  | FDD | OFDMA Symbols per 10 ms frame | n/a |
|  |  | Idle time (µs) | n/a |
|  | TDD | TR Gap Symbols | 1 |
|  |  | OFDMA Symbols per 10 ms frame | 47 |
|  |  | TTG + RTG (µs) | 331.42 |

Table 1