wORKING DOCUMENT TOWARDS A DRAFT REVISION OF RECOMMENDATION  ITU-R  M.1450-2*,**

Characteristics of broadband radio local area networks

(Questions ITU-R 212/8 and ITU-R 142/9)

(2000-2002-2003)

The ITU Radiocommunication Assembly,

considering

a)	that broadband radio local area networks (RLANs) are widely used for fixed, semi-fixed (transportable) and nomadic computer equipment for a variety of broadband applications;

b)	that broadband RLAN standards are compatible with current wired LAN standards;

c)	that it is desirable to establish guidelines for broadband RLANs in various frequency bands;

d)	that broadband RLANs should be implemented with careful consideration to compatibility with other radio applications;

e)	that the above guidelines should not limit the effectiveness of broadband RLANs but be used to enhance their application,

recommends

1	that for guidance on existing broadband RLAN standards, Table 2 can be referred to (Note 2);

2	that  for deails on methods of multiple access  and modulation techniques  for brodband RLANs in mobile applications, Table 32 can bereferred to;

guidance on the characteristics of broadband RLANs, Annex 1 can be referred to;IEEE 802.11 and its amendments can be referred to [are available from GetIEEE site http://standards.ieee.org/getieee802/]

6. that for guidance on fixed applications of RLANs refer to Recommendation ITU-R F.1244 [Editorial note:  the reference to Recommendation ITU-R F.1244 should ultimately be replaced with reference to the draft new Report [BWA.REQ], which has been developed by WP 9B and is intended to incorporate the relevant information from Rec. F.1244, after it is approved by Study Group 9.].

NOTE 1 – Acronyms and terminology used in this Recommendation are given in Table 1.

NOTE 2 – The Annex provides detailed information on how to obtain the complete standards described in Table 2.

TABLE  1

Acronyms and terms used in this Recommendation

| AP | Access point |
| --- | --- |
| BPSK | Binary phase shift keying |
| BRAN | Broadband Radio Access Networks |
| CSMA/CA | Carrier sensing multiple access with collision avoidance |
| DFS | Dynamic frequency selection |
| EIRP | Effective Isotropic Radiated Power |
| ETSI | European Telecommunications Standards Institute |
| HIPERLAN2 | High Performance Radio LAN 2 |
| HiSWAN | Hi Speed Wireless Access Network |
| HSWA | High Speed Wireless Access |
| IEEE | Institute of Electrical and Electronics Engineers |
| LAN | Local Area Network |
| MMAC | Multmedia Mobile Access Commmunication |
| OFDM | Orthogonal frequency division multiplexing |
| PSD | Power Spectral Density |
| RF | Radio frequency |
| RLAN | Radio Local Area Network |
| TPC | Transmit power control |
| Access method | Scheme used to provide multiple access to a channel |
| Bit rate | The rate of transfer of bit information from one network device to another |
| Channelization | Bandwidth of each channel and number of channels that can be contained in the RF bandwidth allocation |
| Frequency band | Nominal operating spectrum of application |
| Modulation | The method used to put digital information on an RF carrier |
| Tx power | (Transmitter power) – RF power in Watts produced by the transmitter |

RLAN operations: [Editorial note:  this text is just hanging, and to be part of the Recommendation, it needs to be attached to a Table, note or perhaps most logically, the introduction text of the Annex]

RLANs are  the extension to wired LANs utilizing radio as the connective media.

Some RLANs may utilize CSMA/CA, a listen before talk channel access method,  in the ransfer of data via radio links.  All stations listen for a clear channel prior to initiation of any transmission. There is a minimum interval when all stations must listen to the channel prior to attempting to transmit data.  And, clients associated to an AP will randomly transfer their data packets as the channel is available.

DFS is an interference mitigation technique that senses energy and moves the network to a different channel to avoid incumbents in the spectrum.

TABLE  2

RLAN characteristics for frequency sharing

| Characteristics | IEEE 802.11b | IEEE 802.11a (1) | IEEE 802.11g (1) | ETSI BRAN HIPERLAN 2 (1)(2) | MMAC HSWA HiSWAN a (1) |
| --- | --- | --- | --- | --- | --- |
| General: |  |  |  | [Should be reviewed] | [Should be reviewed] |
| Frequency Band | 2 400-2 483.5 MHz | 4 900-5 000 MHz 5 150-5 250 MHz 5 250-5 350 MHz 5470-5725 MHz 5 725-5 825 MHz Is there any RLAN in 5725-5850MHz | 2 400-2 483.5 MHz | 5 150-5 350 and 5 470-5 725 MHz(3) | 5 150 to 5 250 MHz(3), (6) |
| Emission type |  |  |  |  |  |
| Channelization | 5 MHz | 20 MHz | 5 MHz | 20 MHz | 20 MHz |
| Antenna: | Various | Various | Various |  |  |
| Gain (approximate) | 0-6 dBi (Omni) | 0-6 dBi (Omni) | 0-6 dBi (Omni) |  |  |
| Radiation Pattern | Omni, Directional | Omni, Directional | Omni, Directional |  |  |
| Occupied Bandwidth | 802.11b mask (Figure 2) | OFDM mask (Figure 1) | OFDM mask (Figure 1) | OFDM mask (Figure 1) | OFDM mask (Figure 1) |
| Transmitter: |  |  |  |  |  |
| Tx Power* | 1 000 mW, PSD 8 dBm/3 KHz | 5.150-5250MHz Tx 50 mW PSD 4 dBm/MHz 5 250-5 350 MHz 5 470-5 725 MHz Tx 200 mW [footnote that some administration have higher power] PSD 11 dBm/MHz 5 725-5 825 MHz Tx 1000 mW PSD 17 dBm/MHz 5 725-5 850 MHz 1 000 mW PSD 8 dBm/3 KHz | 1 000 mW, PSD 8 dBm/3 KHz | 5 150-5 350 MHz: 200 mW maximum mean e.i.r.p. and use of transmitter power control. 5 470-5 725 MHz: 1 W maximum mean e.i.r.p. and use of transmitter power control | 5 150 to 5 250 MHz 10 mW/MHz e.i.r.p(6) |

| Transmitter: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| EIRP* | 1 000 mW e.i.r.p (4) (US) 100 mW e.i.r.p.(5) ( CEPT) 10 mW/MHz e.i.r.p. density(6) | 5 150 to 5 250 MHz 10 mW/MHz 200 mW e.i.r.p. in 20 MHz channel Indoor Only 5 250-5 350 MHz 200 mW e.i.r.p. [some Admin -1000 mW] 5470-5725 MHz 1 000 mW w DFS [250 mW 5 725-5 825 MHz 4 000 mW e.i.r.p.(7) | 1 000 mW e.i.r.p.(4) 100 mW e.i.r.p.(5) 10 mW/MHz e.i.r.p. density(6) | 5 150-5 350 MHz: 200 mW maximum mean e.i.r.p. and use of transmitter power control. 5 470-5 725 MHz: 1000 mW maximum mean e.i.r.p. and use of transmitter power control | 5 150 to 5 250 MHz 10 mW/MHz e.i.r.p(6) |
| Interference Mitigation | CSMA/CA | DFS/TPC CSMA/CA | CSMA/CA | DFS/TPC |  |
| Receiver: |  |  |  |  |  |
| Sensitivity | Listed in standard | Listed in standard | Listed in standard |  |  |

*  Note that some administrations regulate transmitter power vs. EIRP.  Further, it should be recognized that power or EIRP limits may vary by region or by administration.

E  need to re-i(1)   Parameters for the physical layer are common between IEEE 802.11a and ETSI BRAN HIPERLAN 2 and HiSWANa.

(2)   WATM (Wireless ATM) and advanced IP with QoS are intended for use over ETSI BRAN HIPERLAN 2 physical transport.

(3)   For the band 5 150 to 5 250 MHz, No. 5.447 of the Radio Regulations (RR) applies.

(4)   This requirement refers to FCC 15.247 in the United States of America.

(5)  This requirement refers to EUROPE ETS 300-328.

(6)  This requirement refers to JAPAN MPHPT ordinance for Regulating Radio Equipment, Articles 49-20 and 49-21.

(7)   All values from FCC amendment of the Commission’s Rules to Docket No. 96-102 provide for operation of unlicensed NII (RM-8648) devices in the 5 GHz frequency range (RM-865). Also reflected in Canadian Radio Standard Specification RSS-210.

[Comment: Need to coordinate with ETSI/BRAN on updating HIPERLAN, and MMAC for HiSWAN

Comment:  ** There are three figures included in this document:

Figure 1 is the original spectrum mask for OFDM

Figure 2 is the .11b mask

Figure 3 is a combined mask set showing .11a/b/g spectrums

Request feedback on which mask/masks that should be included in this document.]

TABLE  3

Methods of multiple access and modulation techniques

| Frequency band | Multiple access | Modulation technique |
| --- | --- | --- |
| UHF | CSMA/CA | CCK |
|  | FDMA | PBCC |
|  | TDMA |  |
|  | SSMA-DS |  |
|  | SSMA-FH |  |
| SHF | CSMA/CA | GMSK/FSK |
|  | FDMA | BPSK-OFDM |
|  | TDMA-FDD | QPSK-OFDM |
|  | TDMA-TDD | BFSK 8-PSK-OFDM 16-QAM-OFDM |
|  | TDMA/EY-NPMA | 64-QAM-OFDM |

[Add Note that this spectrum mask [in Figure 1?] is for systems 802.11a, .11g,  HIPERLAN2, and HiSWAN]

Figure 2

802.11b Transmit spectrum mask

FIGURE 3

Spectrum masks for IEEE 802.11a, b, g

2

Methods of multiple access and modulation techniques

| Frequency band | Multiple access | Modulation technique |
| --- | --- | --- |

TABLE  3

ANNEX

Additional information on RLANs

The HYPERLAN 2 standards are TS 101 475 for the Physical Layer & TS 101 761 for the DLC Layer, and these can be downloaded from the ETSI Publications Download Area at http://pda.etsi.org/pda/queryform.asp

The IEEE 802 standards can be downloaded from http://standards.ieee.org/getieee802/

[Mike, Denis, we should consider moving the general description of RLANs from under Table 1 to here, as the introductory Section 1. (PeterMu)]

--

-

-

--

is about 20 dB under the frequency selective fading channel with 300 ns delay spread. The proposed physical layer approach allows us to use this high bit rate RLAN system not only in indoor areas but also outdoor areas forming parts of locations such as universities, factories, and shopping malls, etc.

TABLE  4

Major simulation parameters

| Raw data rate | 26.6 Mbit/s |
| --- | --- |
| Modulation/detection |  |

-

References

_______________

*	This Recommendation was jointly developed by Radiocommunication Study Groups 8 and 9, and future revisions should be undertaken jointly.

*	*	This Recommendation should be brought to the attention of Telecommunication Standardization Study Group 17, and Radiocommunication Study Groups 3 and 4.

