| Radiocommunication Study Groups Third Meeting of Working Party 5D Seoul, Korea, 8-15 October 2008 |  |
| --- | --- |
| Source: Revision 1 to Document 5D/TEMP/127 |  |
|  | 24 October 2008 |
|  | English only |
| Working Party 5D |  |
| liaison statement to external organizations |  |
| request for information for recommendations itu-r m.1580 and m.1581 (unwanted emission characteristics) |  |

# 1	Introduction

At the Seoul meeting of ITU-R Working Party (WP) 5D the work on revision 3 of Recommendations ITU-R M.1580 and M.1581 continued with a view to finalize the revisions at the 4th meeting of ITU-R WP 5D (10 - 17 February 2009).

Amongst other issues, some parameters have been discussed extensively within ITU-R WP 5D. WP 5D came to a common understanding that it will try to harmonize all 6 annexes to the maximum extent possible with regard to the parameters contained therein, while keeping the current scope of Recommendations ITU-R M.1580 and M.1581.

# 2	Parameters

## 2.1	Test Tolerance

As part of the above mentioned discussions, “test tolerances” have been addressed as an issue in relation to the harmonized content of these Recommendations and how “test tolerances” may have been accounted for in the provided parameters.

Hence ITU-R WP 5D kindly requests external organisations to provide test tolerance information as implemented in their specifications for the parameters provided for inclusion in Recommendations ITU-R M.1580 and M.1581.

ITU-R WP 5D will note which principle described in Recommendation ITU-R M.1545 has been used for each annex, considering the responses from the external organizations.

## 2.2	ACLR

In the view of ITU-R WP 5D, “ACLR” is considered to be one of the relevant parameters to be included in Recommendations ITU-R M.1580 and M.1581. Analysing the 6 annexes of Recommendations ITU-R M.1580 and M.1581, ITU-R WP 5D came to the conclusion that currently not all annexes include this parameter.

Hence ITU-R WP 5D kindly requests external organizations to provide intra-system and, where available, inter-system ACLR values for their RTT. To have a clear understanding, it is requested to also provide the definition of ACLR used, including all assumptions used in deriving these values. It should be noted that it is not intended to make the “ACLR” parameter a mandatory regulatory obligation, as it is up to the administrations which part of a Recommendation they want to use in national regulations.

# 3	Request for submission of information

To ensure the finalization of the draft revision of Recommendations ITU-R M.1580-2 and M.1581-2 at the 4th meeting of ITU-R WP 5D (10-17.Feb.2009), ITU-R WP 5D kindly requests exrternal organizations to provide the above mentioned parameters along with relevant information where:
(a) they are not already part of Recommendations ITU-R M.1580 and M.1581; and/or 
(b) new /additional values should be implemented in Recommendations ITU-R M.1580 and M.1581.

For convenience the latest working draft of the revised Recommendations are attached electronically (Source Documents TEMP/128(Rev.1) and TEMP/129(Rev.1)) so that the specific questions above may be answered.

ITU-R WP 5D thanks external organizations for their collaboration.

| Status: | For response |  |
| --- | --- | --- |
| Deadline: | 1600 hours UTC, 3 Febuary 2009 |  |
| Contact: | Colin Langtry Counsellor, ITU-R SG 5 | Tel: +41 22 730 6178 Email: colin.langtry@itu.int |

| Source: Revision 1 to Document 5D/TEMP/128 |  |
| --- | --- |
| Attachment 1 |  |
| Working document towards the draft revision of RECOMMENDATION ITU-R M.1580-2* |  |
| Generic unwanted emission characteristics of base stations using the terrestrial radio interfaces of IMT-2000 |  |
| (Question ITU-R 229/8) |  |

(2002-2005-2007)

Scope

This Recommendation provides the generic unwanted emission characteristics of base stations using the terrestrial radio interfaces of IMT-2000.

The ITU Radiocommunication Assembly,

considering

a)	that unwanted emissions consist of both spurious and out-of-band (OoB) emissions according to No. 1.146 of the Radio Regulation (RR) and that spurious and OoB emissions are defined in RR Nos. 1.145 and 1.144, respectively;

b)	that limitation of the maximum permitted levels of unwanted emissions of IMT-2000 base stations (BS) is necessary to protect other radio systems and services from interference and to enable coexistence between different technologies;

c)	that too stringent limits may lead to an increase in complexity of IMT-2000 BS;

d)	that every effort should be made to keep limits for unwanted emissions at the lowest possible values taking account of economic factors and technological limitations;

e)	that Recommendation ITU-R SM.329 relates to the effects, measurements and limits to be applied to spurious domain emissions;

f)	that the same spurious emission limits apply equally to BS of all radio interfaces;

g)	that Recommendation ITU-R SM.1541 relating to OoB emission specifies generic limits in the OoB domain which generally constitute the least restrictive OoB emission limits and encourages the development of more specific limits for each system;

h)	that the levels of spurious emissions of IMT-2000 BS shall comply with the limits specified in RR Appendix 3;

j)	that the harmonization of unwanted emission limits will facilitate global use and access to a global market; however national/regional variations in unwanted emission limits may exist;

k)	that additional work is needed in order to define unwanted emission limits for equipment operating in the other bands identified for IMT-2000 at the World Radiocommunication Conference (Istanbul, 2000) (WRC-2000);

l)	that unwanted emission limits are dependent on the transmitter emission characteristics, ITU spurious emission limits and national standards and regulations in addition to depending on services operating in other bands,

noting

a)	the work carried out by standardization bodies to define limits to protect other radio systems and services from interference and to enable coexistence between different technologies;

b)	that IMT-2000 base stations must comply with local, regional, and international regulations for out-of-band and spurious emissions relevant to their operations, wherever such regulations apply;

c)	with regard to Annex 6, IMT-2000 OFDMA TDD WMAN, additional urgent work, in particular on emission mask and ACLR, is needed to ensure geographical coexistence with other IMT-2000 radio interfaces,

recommends

1	that the unwanted emission characteristics of IMT-2000 base stations should be based on the limits contained in the technology specific Annexes 1 to 6 which correspond to the radio interface specifications described in § 5.1 to 5.6 of Recommendation ITU-R M.1457.

NOTE 1 – Except the cases stated in Note 2, Note 3, Note 4 and Note 5, the unwanted emission limits are defined for BS operating according to the following arrangement: frequency division duplex (FDD) uplink in the band 1 920-1 980 MHz, FDD downlink in the band 2 110-2 170 MHz and time division duplex (TDD) in the band 1 885-1 980 MHz and 2 010-2 025 MHz. Future versions of this Recommendation will include limits applicable to other frequency bands. Subject to further study, it is anticipated that such limits would be similar to those already contained in this Recommendation.

NOTE 2 – The unwanted emission limits defined in Annex 1 are for BS operating one of ,or a combination of, the following arrangements:

–	Frequency division duplex (FDD) uplink in the band 1 920-1 980 MHz, FDD downlink in the band 2 110-2 170 MHz, in Annex 1 referred to as FDD Band I in UTRA or Band 1 in E-UTRA.

–	FDD uplink in the band 1 850-1 910 MHz, FDD downlink in the band 1 930-1 990 MHz, in Annex 1 referred to as FDD Band II in UTRA or Band 2 in E-UTRA.

–	FDD uplink in the band 1 710-1 785 MHz, FDD downlink in the band 1 805-1 880 MHz, in Annex 1 referred to as FDD Band III in UTRA or Band 3 in E-UTRA.

–	FDD uplink in the band 1 710-1 755 MHz, FDD downlink in the band 2 110-2 155 MHz, in Annex 1 referred to as FDD Band IV in UTRA or Band 4 in E-UTRA.

–	FDD uplink in the band 824- 849 MHz, FDD downlink in the band 869-894 MHz, in Annex 1 referred to as FDD Band V in UTRA or Band 5 in E-UTRA.

–	FDD uplink in the band  830- 840 MHz, FDD downlink in the band  875- 885 MHz, in Annex 1 referred to as FDD Band VI in UTRA or Band 6 in E-UTRA.

–	FDD uplink in the band 2 500-2 570 MHz, FDD downlink in the band 2 620-2 690 MHz, in Annex 1 referred to as FDD Band VII in UTRA or Band 7 in E-UTRA.

–	FDD uplink in the band  880-915 MHz, FDD downlink in the band 925-960 MHz, in Annex 1 referred to as FDD Band VIII in UTRA or Band 8 in E-UTRA.

–	FDD uplink in the band 1 749.9-1 784.9 MHz, FDD downlink in the band 1 844.9-1 879.9 MHz, in Annex 1 to referred as FDD Band IX in UTRA or Band 9 in E-UTRA.

–	FDD uplink in the band 1 710-1 770 MHz, FDD downlink in the band 2 110-2 170 MHz, in Annex 1 referred to as FDD Band X in UTRA or Band 10 in E-UTRA.

–	FDD uplink in the band 1 427.9-1 452.9 MHz, FDD downlink in the band 1 475.9-1 500.9 MHz, in Annex 1 referred to as FDD Band XI in UTRA or Band 11 in E-UTRA.

–	FDD uplink in the band 698-716 MHz, FDD downlink in the band 728-746 MHz, in Annex 1 referred to as FDD Band XII in UTRA or Band 12 in E-UTRA.

–	FDD uplink in the band 777-787 MHz, FDD downlink in the band 746-756 MHz, in Annex 1 referred to as FDD Band XIII in UTRA or Band 13 in E-UTRA.

–	FDD uplink in the band 788-798 MHz, FDD downlink in the band 758-768 MHz, in Annex 1 referred to as FDD Band XIV in UTRA or Band 14 in E-UTRA.

Future versions of this Recommendation will include limits applicable to other frequency bands. Subject to further study, it is anticipated that such limits would be similar to those already contained in this Recommendation.

NOTE 3 – The unwanted emission limits defined in Annex 2 are for BS operating in the following arrangements and apply to both cdma2000 and HRPD operating modes except as noted:

| Band Class | Name | MS Transmit Frequency (MHz) | BS Transmit Frequency (MHz) |
| --- | --- | --- | --- |
| 0 | 800 MHz band | 824-849 | 869-894 |
| 1 | 1 900 MHz band | 1 850-1 910 | 1 930-1 990 |
| 2 | TACS band | 872-915 | 917-960 |
| 3 | JTACS band | 887-925 | 832-870 |
| 4 | Korean PCS band | 1 750-1 780 | 1 840-1 870 |
| 5 | 450 MHz band | 411-484 | 421-494 |
| 6 | 2 GHz band | 1 920-1 980 | 2 110-2 170 |
| 7 | 700 MHz band | 776-794 | 746-764 |
| 8 | 1 800 MHz band | 1 710-1 785 | 1 805-1 880 |
| 9 | 900 MHz band | 880-915 | 925-960 |
| 10 | Secondary 800 MHz band | 806-901 | 851-940 |
| 11 | 400 MHz European PAMR band | 411-484 | 421-494 |
| 12 | 800 MHz PAMR band | 870-876 | 915-921 |
| 13 | 2.5 GHz IMT-2000 Extension band | 2 500-2 570 | 2 620-2 690 |
| 14 | US PCS 1.9 GHz band | 1 850-1 915 | 1 930-1 995 |
| 15 | AWS band | 1 710-1 755 | 2 110-2 155 |
| 16* | US 2.5 GHz band | 2 502-2 568 | 2 624-2 690 |

*	No emissions specifications at this time.

NOTE 4 – The unwanted emission limits defined in Annex 3 are for BS operating one of or a combination of the following arrangements:

–	time division duplex (TDD) in the band 1 900-1 920 MHz and 2 010-2 025 MHz refered to as Band a) in UTRA or Band 33 and 34, respectively, in E-UTRA.

–	TDD in the band 1 850-1 910 MHz and 1 930-1 990 MHz refered to as band b) in UTRA or Band 35 and 36, respectively, in E-UTRA.

–	TDD in the band 1 910-1 930 MHz refered to as Band c) in UTRA or Band 37 in E-UTRA.

- TDD in the band 2 570-2 620 MHz refered to as Band d) in UTRA or Band 38 in E-UTRA.

- TDD in the band 1880-1920 MHz refered to as Band 39 in E-UTRA.

- TDD in the band 2300-2400 MHz refered to as Band 40 in E-UTRA.

Future versions of this Recommendation will include limits applicable to other frequency bands. Subject to further study, it is anticipated that such limits would be similar to those already contained in this Recommendation.

NOTE 5 – The unwanted emission limits defined in Annex 6 are for BS operating in the following arrangement:

–	TDD in the band 2 300-2 400 MHz;

–	TDD in the band 2 500-2 690 MHz;

–	TDD in the band 3 400-3 600 MHz.

Annex 1 –	IMT-2000 code division multiple access (CDMA) direct spread 
(universal terrestrial radio access (UTRA) FDD) base stations

Annex 2 –	IMT-2000 CDMA multi-carrier (cdma-2000) base stations

Annex 3 – 	IMT-2000 CDMA TDD (UTRA TDD) base stations

Annex 4 – 	IMT-2000 time division multiple access (TDMA) single-carrier (UWC-136) 
base stations

Annex 5 – 	IMT-2000 frequency division multiple access (FDMA)/TDMA 
(digital enhanced cordless telecommunications (DECT)) base stations

Annex 6 – 	IMT-2000 OFDMA TDD WMAN base stations.

Appendix 1 – Definitions and Abbreviations

Annex 1

IMT-2000 (code division multiple access (CDMA) direct spread (universal terrestrial radio access (UTRA) FDD) base stations

# 1	Measurement uncertainty

Values specified in this Annex differ from those specified in Recommendation ITU-R M.1457 since values in this Annex incorporate test tolerances defined in Recommendation ITU-R M.1545.

# 2	Spectrum mask

## 2.1	UTRA spectrum mask

The mask defined in Tables 1 to 4 below may be mandatory in certain regions. In other regions this mask may not be applied.

For regions where this clause applies, the requirement should be met by a BS transmitting on a single radio frequency (RF) carrier configured in accordance with the manufacturer’s specification. Emissions should not exceed the maximum level specified in Tables 1 to 4 for the appropriate BS maximum output power, in the frequency range from f  2.5 MHz to fmax from the carrier frequency, where:

–	f is the separation between the carrier frequency and the nominal –3 dB point of the measuring filter closest to the carrier frequency.

–	f_offset is the separation between the carrier frequency and the centre of the measurement filter:

–	f_offsetmax is either 12.5 MHz or the offset to the BS transmit band edge, whichever is the greater.

–	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

TABLE 1a

Spectrum emission mask values, BS maximum output power P  43 dBm

| Frequency offset of measurement filter –3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Requirement |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| 2.5 MHz f  2.7 MHz | 2.515 MHz  f_offset  2.715 MHz | 12.5 dBm |  | 30 kHz |
| 2.7 MHz f  3.5 MHz | 2.715 MHz  f_offset  3.515 MHz | 12.5  15 (f_offset  2.715) dBm |  | 30 kHz |
|  | 3.515 MHz  f_offset  4.0 MHz | 24.5 dBm |  | 30 kHz |
| 3.5 MHz f  fmax | 4.0 MHz  f_offset  f_offsetmax | 11.5 dBm |  | 1 MHz |

TABLE 2a

Spectrum emission mask values, BS maximum output power 39  P < 43 dBm

| Frequency offset of measurement filter –3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Requirement |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| 2.5 MHz  f < 2.7 MHz | 2.515 MHz  f_offset < 2.715 MHz | 12.5 dBm |  | 30 kHz |
| 2.7 MHz  f < 3.5 MHz | 2.715 MHz  f_offset < 3.515 MHz | 12.5  15 (f_offset  2.715) dBm |  | 30 kHz |
|  | 3.515 MHz  f_offset < 4.0 MHz | 24.5 dBm |  | 30 kHz |
| 3.5 MHz  f < 7.5 MHz | 4.0 MHz  f_offset < 8.0 MHz | 11.5 dBm |  | 1 MHz |
| 7.5 MHz  f  fmax MHz | 8.0 MHz  f_offset < f_offsetmax | P  54.5 dBm |  | 1 MHz |

TABLE 3a

Spectrum emission mask values, BS maximum output power 31  P < 39 dBm

| Frequency offset of measurement filter –3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Requirement |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| 2.5 MHz  f < 2.7 MHz | 2.515 MHz  f_offset < 2.715 MHz | P – 51.5 dBm |  | 30 kHz |
| 2.7 MHz  f < 3.5 MHz | 2.715 MHz  f_offset < 3.515 MHz | P – 51.5  15 (f_offset  2.715) dBm |  | 30 kHz |
|  | 3.515 MHz  f_offset < 4.0 MHz | P – 63.5 dBm |  | 30 kHz |
| 3.5 MHz  f < 7.5 MHz | 4.0 MHz  f_offset < 8.0 MHz | P – 50.5 dBm |  | 1 MHz |
| 7.5 MHz  f  fmax MHz | 8.0 MHz  f_offset < f_offsetmax | P – 54.5 dBm |  | 1 MHz |

TABLE 4a

Spectrum emission mask values, BS maximum output power P < 31 dBm

| Frequency offset of measurement filter –3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Requirement | Measurement bandwidth |
| --- | --- | --- | --- |
| 2.5 MHz  f < 2.7 MHz | 2.515 MHz  f_offset < 2.715 MHz | 20.5 dBm | 30 kHz |
| 2.7 MHz  f < 3.5 MHz | 2.715 MHz  f_offset < 3.515 MHz | 20.5  15 (f_offset  2.715) dBm | 30 kHz |
|  | 3.515 MHz  f_offset < 4.0 MHz | 32.5 dBm | 30 kHz |
| 3.5 MHz  f < 7.5 MHz | 4.0 MHz  f_offset < 8.0 MHz | 19.5 dBm | 1 MHz |
| 7.5 MHz  f  fmax MHz | 8.0 MHz  f_offset < f_offsetmax | 23.5 dBm | 1 MHz |

For operation in Band II, IV, V, X, XII, XIII and XIV, the applicable additional requirement in Tables 1AA, 2AA or 3AA apply in addition to the minimum requirements in Tables 1A to 4A.Table 1AA

TABLE 1aa

Additional spectrum emission limits for Bands II, IV, X

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Additional requirement | Measurement bandwidth |
| --- | --- | --- | --- |
| 2.5 MHz  f < 3.5 MHz | 2.515MHz  f_offset < 3.515MHz | -15 dBm | 30 kHz |
| 3.5 MHz  f  fmax | 4.0MHz  f_offset < f_offsetmax | -13 dBm | 1 MHz |

TABLE 2aa

Additional spectrum emission limits for Band V

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Additional requirement | Measurement bandwidth |
| --- | --- | --- | --- |
| 2.5 MHz  f < 3.5 MHz | 2.515MHz  f_offset < 3.515MHz | -15 dBm | 30 kHz |
| 3.5 MHz  f  fmax | 3.55MHz  f_offset < f_offsetmax | -13 dBm | 100 kHz |

TABLE 3aa

Additional spectrum emission limits for Bands XII, XIII, XIV

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Additional requirement | Measurement bandwidth |
| --- | --- | --- | --- |
| 2.5 MHz  f < 3.5 MHz | 2.515MHz  f_offset < 3.515MHz | -13 dBm | 30 kHz |
| 3.5 MHz  f  fmax | 3.55MHz  f_offset < f_offsetmax | -13 dBm | 100 kHz |

## 2.2	E-UTRA (LTE) spectrum mask

The Operating band unwanted emission limits are defined from 10 MHz below the lowest frequency of the BS transmitter operating band up to 10 MHz above the highest frequency of the BS transmitter operating band.

The requirements shall apply whatever the type of transmitter considered (single carrier or multi-carrier) and for all transmission modes foreseen by the manufacturer's specification.

The unwanted emission limits in the part of the operating band that falls in the spurious domain are consistent with Recommendation ITU-R SM.329.

Emissions should not exceed the maximum level specified in Tables below, where:

–	f is the separation between the channel edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

–	f_offset is the separation between the channel edge frequency and the centre of the measuring filter.

–	f_offsetmax is the offset to the frequency 10 MHz outside the BS transmitter operating band.

–	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multicarrier E-UTRA BS the definitions above apply to the lower edge of the carrier transmitted at the lowest carrier frequency and the higher edge of the carrier transmitted at the highest carrier frequency.

The requirements of either § 2.2.1 or § 2.2.2 shall apply.

The Additional operating band unwanted emission limits defined in § 2.2.2.1 may be mandatory in certain regions. In other regions it may not apply.

### 2.2.1	E-UTRA spectrum mask (Category A)

For E-UTRA BS operating in Bands 5, 6, 8, 12, 13 and 14, emissions shall not exceed the maximum levels specified in Table 1b-a) to 1b-c).

TABLE 1b

1b-a) General operating band unwanted emission limits for 1.4 MHz channel 
bandwidth (E-UTRA bands <1 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 1) |
| --- | --- | --- | --- |
| 0 MHz  f < 1.4 MHz | 0.05 MHz  f_offset < 1.45 MHz | +0.5dBm−101.4⋅foffsetMHz−0.05dB | 100 kHz |
| 1.4 MHz  f < 2.8 MHz | 1.45 MHz  f_offset < 2.85 MHz | -9.5 dBm | 100 kHz |
| 2.8 MHz  f  fmax | 2.85 MHz  f_offset < f_offsetmax | -13 dBm | 100 kHz |

1b-b) General operating band unwanted emission limits for 3 MHz channel 
bandwidth (E-UTRA bands <1 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 1) |
| --- | --- | --- | --- |
| 0 MHz  f < 3 MHz | 0.05 MHz  f_offset < 3.05 MHz | −3dBm−103foffsetMHz−0.05dB | 100 kHz |
| 3 MHz  f  fmax | 3.05 MHz  f_offset < f_offsetmax | -13 dBm | 100 kHz |

1b-c) General operating band unwanted emission limits for 5, 10, 15 and 20 MHz 
channel bandwidth (E-UTRA bands <1 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 1) |
| --- | --- | --- | --- |
| 0 MHz  f < 5 MHz | 0.05 MHz  f_offset < 5.05 MHz | −5.5dBm−75⋅foffsetMHz−0.05dB | 100 kHz |
| 5 MHz  f < 10 MHz | 5.05 MHz  f_offset < 10.05 MHz | -12.5 dBm | 100 kHz |
| 10 MHz  f  fmax | 10.05 MHz  f_offset < f_offsetmax | -13 dBm | 100 kHz |

For E-UTRA BS operating in Bands 1, 2, 3, 4, 7, 9, 10, and 11, emissions shall not exceed the maximum levels specified in Table 1b-d) to 1b-f):

1b-d) General operating band unwanted emission limits for 1.4 MHz channel 
bandwidth (E-UTRA bands >1 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 1) |
| --- | --- | --- | --- |
| 0 MHz  f < 1.4 MHz | 0.05 MHz  f_offset < 1.45 MHz | +0.5dBm−101.4⋅foffsetMHz−0.05dB | 100 kHz |
| 1.4 MHz  f < 2.8 MHz | 1.45 MHz  f_offset < 2.85 MHz | -9.5 dBm | 100 kHz |
| 2.8 MHz  f  fmax | 3.3 MHz  f_offset < f_offsetmax | -13 dBm | 1MHz |

1b-e) General operating band unwanted emission limits for 3 MHz channel 
bandwidth (E-UTRA bands >1 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 1) |
| --- | --- | --- | --- |
| 0 MHz  f < 3 MHz | 0.05 MHz  f_offset < 3.05 MHz | −3.5dBm−103⋅foffsetMHz−0.05dB | 100 kHz |
| 3 MHz  f < 6 MHz | 3.05 MHz  f_offset < 6.05 MHz | -13.5 dBm | 100 kHz |
| 6 MHz  f  fmax | 6.5 MHz  f_offset < f_offsetmax | -13 dBm | 1MHz |

1b-f) General operating band unwanted emission limits for 5, 10, 15 and 20 MHz 
channel bandwidth (E-UTRA bands >1 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 1) |
| --- | --- | --- | --- |
| 0 MHz  f < 5 MHz | 0.05 MHz  f_offset < 5.05 MHz | −5.5dBm−75⋅foffsetMHz−0.05dB | 100 kHz |
| 5 MHz  f < 10 MHz | 5.05 MHz  f_offset < 10.05 MHz | -12.5 dBm | 100 kHz |
| 10 MHz  f  fmax | 10.5 MHz  f_offset < f_offsetmax | -13 dBm | 1MHz |

### 2.2.2	E-UTRA spectrum mask (Category B)

For E-UTRA BS operating in Bands 5, 6, 8, 12, 13, and 14, emissions shall not exceed the maximum levels specified in Table 2b-a) to 2b-c):

TABLE 2b

2b-a) General operating band unwanted emission limits for 1.4 MHz channel 
bandwidth (E-UTRA bands <1 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 1) |
| --- | --- | --- | --- |
| 0 MHz  f < 1.4 MHz | 0.05 MHz  f_offset < 1.45 MHz | +0.5dBm−101.4⋅foffsetMHz−0.05dB | 100 kHz |
| 1.4 MHz  f < 2.8 MHz | 1.45 MHz  f_offset < 2.85 MHz | -9.5 dBm | 100 kHz |
| 2.8 MHz  f  fmax | 2.85 MHz  f_offset < f_offsetmax | -16 dBm | 100 kHz |

2b-b) General operating band unwanted emission limits for 3 MHz channel 
bandwidth (E-UTRA bands <1 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 1) |
| --- | --- | --- | --- |
| 0 MHz  f < 3 MHz | 0.05 MHz  f_offset < 3.05 MHz | −3.5dBm−103⋅foffsetMHz−0.05dB | 100 kHz |
| 3 MHz  f < 6 MHz | 3.05 MHz  f_offset < 6.05 MHz | -13.5 dBm | 100 kHz |
| 6 MHz  f  fmax | 6.05 MHz  f_offset < f_offsetmax | -16 dBm | 100 kHz |

2b-c) General operating band unwanted emission limits for 5, 10, 15 and 20 MHz 
channel bandwidth (E-UTRA bands <1 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 1) |
| --- | --- | --- | --- |
| 0 MHz  f < 5 MHz | 0.05 MHz  f_offset < 5.05 MHz | −5.5dBm−75⋅foffsetMHz−0.05dB | 100 kHz |
| 5 MHz  f < 10 MHz | 5.05 MHz  f_offset < 10.05 MHz | -12.5 dBm | 100 kHz |
| 10 MHz  f  fmax | 10.05 MHz  f_offset < f_offsetmax | -16 dBm | 100 kHz |

For E-UTRA BS operating in Bands 1, 2, 3, 4, 7, 9, 10, and 11 emissions shall not exceed the maximum levels specified in Table 2b-d) to 2b-f):

2b-d) General operating band unwanted emission limits for 1.4 MHz channel 
bandwidth (E-UTRA bands >1 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 1) |
| --- | --- | --- | --- |
| 0 MHz  f < 1.4 MHz | 0.05 MHz  f_offset < 1.45 MHz | +0.5dBm−101.4⋅foffsetMHz−0.05dB | 100 kHz |
| 1.4 MHz  f < 2.8 MHz | 1.45 MHz  f_offset < 2.85 MHz | -9.5 dBm | 100 kHz |
| 2.8 MHz  f  fmax | 3.3 MHz  f_offset < f_offsetmax | -15 dBm | 1MHz |

2b-e) General operating band unwanted emission limits for 3 MHz channel 
bandwidth (E-UTRA bands >1 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 1) |
| --- | --- | --- | --- |
| 0 MHz  f < 3 MHz | 0.05 MHz  f_offset < 3.05 MHz | −3.5dBm−103⋅foffsetMHz−0.05dB | 100 kHz |
| 3 MHz  f < 6 MHz | 3.05 MHz  f_offset < 6.05 MHz | -13.5 dBm | 100 kHz |
| 6 MHz  f  fmax | 6.5 MHz  f_offset < f_offsetmax | -15 dBm | 1MHz |

2b-f) General operating band unwanted emission limits for 5, 10, 15 and 20 MHz 
channel bandwidth (E-UTRA bands >1 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 1) |
| --- | --- | --- | --- |
| 0 MHz  f < 5 MHz | 0.05 MHz  f_offset < 5.05 MHz | −5.5dBm−75⋅foffsetMHz−0.05dB | 100 kHz |
| 5 MHz  f < 10 MHz | 5.05 MHz  f_offset < 10.05 MHz | -12.5 dBm | 100 kHz |
| 10 MHz  f  fmax | 10.5 MHz  f_offset < f_offsetmax | -15 dBm | 1MHz |

### 2.2.2.1	E-UTRA spectrum mask (additional limits)

The following requirements may apply in certain regions. For E-UTRA BS operating in Bands 5, emissions shall not exceed the maximum levels specified in Table 3b.

TABLE 3b

Additional operating band unwanted emission limits for E-UTRA bands <1 GHz

| Channel bandwidth | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 1) |
| --- | --- | --- | --- | --- |
| 1.4 MHz | 0 MHz  f < 1 MHz | 0.005 MHz  f_offset < 0.995 MHz | -12.5 dBm | 10 kHz |
|  | 1 MHz  f < 2.8 MHz | 1.05 MHz  f_offset < 2.85 MHz | -11.5 dBm | 100 kHz |
|  | 2.8 MHz  f < fmax | 2.85 MHz  f_offset < f_offsetmax | -13 dBm |  |
| 3 MHz | 0 MHz  f < 1 MHz | 0.015 MHz  f_offset < 0.985 MHz | -11.5 dBm | 30 kHz |
|  | 1 MHz  f < 6 MHz | 1.05 MHz  f_offset < 6.05 MHz | -11.5 dBm | 100 kHz |
|  | 6 MHz  f < fmax | 6.05 MHz  f_offset < f_offsetmax | -13 dBm |  |
| 5 MHz | 0 MHz  f < 1 MHz | 0.015 MHz  f_offset < 0.985 MHz | -13.5 dBm | 30 kHz |
|  | 1 MHz  f < 10 MHz | 1.05 MHz  f_offset < 10.05 MHz | -11.5 dBm | 100 kHz |
|  | 10 MHz  f < fmax | 10.05 MHz  f_offset < f_offsetmax | -13 dBm |  |
| 10 MHz | 0 MHz  f < 1 MHz | 0.05 MHz  f_offset < 0.95 MHz | -11.5 dBm | 100 kHz |
|  | 1 MHz  f < 20 MHz | 1.05 MHz  f_offset < 20.05 MHz | -11.5 dBm |  |
|  | 20 MHz  f < fmax | 20.05 MHz  f_offset < f_offsetmax | -13 dBm |  |
| 15 MHz | 0 MHz  f < 1 MHz | 0.05 MHz  f_offset < 0.95 MHz | -11.5 dBm | 100 kHz |
|  | 1 MHz  f < 30 MHz | 1.05 MHz  f_offset < 30.05 MHz | -11.5 dBm |  |
|  | 30 MHz  f < fmax | 30.05 MHz  f_offset < f_offsetmax | -13 dBm |  |
| 20 MHz | 0 MHz  f < 1 MHz | 0.05 MHz  f_offset < 0.95 MHz | -11.5 dBm | 100 kHz |
|  | 1 MHz  f < 40 MHz | 1.05 MHz  f_offset < 40.05 MHz | -11.5 dBm |  |
|  | 40 MHz  f < fmax | 40.05 MHz  f_offset < f_offsetmax | -13 dBm |  |

The following requirements may apply in certain regions. For E-UTRA BS operating in Bands 2, 4 and 10, emissions shall not exceed the maximum levels specified in Table 4b.

TABLE 4b

Additional operating band unwanted emission limits for E-UTRA bands>1 GHz

| Channel bandwidth | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 1) |
| --- | --- | --- | --- | --- |
| 1.4 MHz | 0 MHz  f < 1 MHz | 0.005 MHz  f_offset < 0.995 MHz | -12.5 dBm | 10 kHz |
|  | 1 MHz  f < 2.8 MHz | 1.5 MHz  f_offset < 3.3 MHz | -11.5 dBm | 1 MHz |
|  | 2.8 MHz  f < fmax | 3.3 MHz  f_offset < f_offsetmax | -13 dBm |  |
| 3 MHz | 0 MHz  f < 1 MHz | 0.015 MHz  f_offset < 0.985 MHz | -11.5 dBm | 30 kHz |
|  | 1 MHz  f < 6 MHz | 1.5 MHz  f_offset < 6.5 MHz | -11.5 dBm | 1 MHz |
|  | 6 MHz  f < fmax | 6.5 MHz  f_offset < f_offsetmax | -13 dBm |  |
| 5 MHz | 0 MHz  f < 1 MHz | 0.015 MHz  f_offset < 0.985 MHz | -13.5 dBm | 30 kHz |
|  | 1 MHz  f < 10 MHz | 1.5 MHz  f_offset < 10.5 MHz | -11.5 dBm | 1 MHz |
|  | 10 MHz  f < fmax | 10.5 MHz  f_offset < f_offsetmax | -13 dBm |  |
| 10 MHz | 0 MHz  f < 1 MHz | 0.05 MHz  f_offset < 0.95 MHz | -11.5 dBm | 100 kHz |
|  | 1 MHz  f < 20 MHz | 1.5 MHz  f_offset < 20.5 MHz | -11.5 dBm | 1 MHz |
|  | 20 MHz  f < fmax | 20.5 MHz  f_offset < f_offsetmax | -13 dBm |  |
| 15 MHz | 0 MHz  f < 1 MHz | 0.05 MHz  f_offset < 0.95 MHz | -13.5 dBm | 100 kHz |
|  | 1 MHz  f < 30 MHz | 1.5 MHz  f_offset < 30.5 MHz | -11.5 dBm | 1 MHz |
|  | 30 MHz  f < fmax | 30.5 MHz  f_offset < f_offsetmax | -13 dBm |  |
| 20 MHz | 0 MHz  f < 1 MHz | 0.05 MHz  f_offset < 0.95 MHz | -14.5 dBm | 100 kHz |
|  | 1 MHz  f < 40 MHz | 1.5 MHz  f_offset < 40.5 MHz | -11.5 dBm | 1 MHz |
|  | 40 MHz  f < fmax | 40.5 MHz  f_offset < f_offsetmax | -13 dBm |  |

The following requirements may apply in certain regions. For E-UTRA BS operating in Bands 12, 13, and 14, emissions shall not exceed the maximum levels specified in Table 4c.

TABLE 4c

Additional operating band unwanted emission limits for E-UTRA (bands 12, 13 and 14)

| Channel bandwidth | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 1) |
| --- | --- | --- | --- | --- |
| 1.4 MHz 3 MHz 5 MHz 10 MHz 15 MHz 20 MHz | 0 MHz  f < 100 kHz | 0.015 MHz  f_offset < 0.085 MHz | -11.5 dBm | 30 kHz |
| 1.4 MHz | 100 kHz  f < 2.8 MHz | 150 kHz  f_offset < 2.85 MHz | -11.5 dBm | 100 kHz |
|  | 2.8 MHz  f < fmax | 2.85 MHz  f_offset < f_offsetmax | -13 dBm |  |
| 3 MHz | 100 kHz  f < 6 MHz | 150 kHz  f_offset < 6.05 MHz | -11.5 dBm | 100 kHz |
|  | 6 MHz  f < fmax | 6.05 MHz  f_offset < f_offsetmax | -13 dBm |  |
| 5 MHz | 100 kHz  f < 10 MHz | 150 kHz  f_offset < 10.05 MHz | -11.5 dBm | 100 kHz |
|  | 10 MHz  f < fmax | 10.05 MHz  f_offset < f_offsetmax | -13 dBm |  |
| 10 MHz | 100 kHz  f < 20 MHz | 150 kHz  f_offset < 20.05 MHz | -11.5 dBm | 100 kHz |
|  | 20 MHz  f < fmax | 20.05 MHz  f_offset < f_offsetmax | -13 dBm |  |
| 15 MHz | 100 kHz  f < 30 MHz | 150 kHz  f_offset < 30.05 MHz | -11.5 dBm | 100 kHz |
|  | 30 MHz  f < fmax | 30.05 MHz  f_offset < f_offsetmax | -13 dBm |  |
| 20 MHz | 100 kHz  f < 40 MHz | 150 kHz  f_offset < 40.05 MHz | -11.5 dBm | 100 kHz |
|  | 40 MHz  f < fmax | 40.05 MHz  f_offset < f_offsetmax | -13 dBm |  |

NOTE 1 – As a general rule for the requirements in section 2.2, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

# 3	Adjacent channel leakage power ratio

Adjacent channel leakage power ratio (ACLR) is the ratio of the transmitted power to the power measured after a receiver filter in the adjacent channel(s).

## 3.1	ACLR for UTRA

For UTRA, both the transmitted power and the received power are measured through a matched filter (root raised cosine and roll-off 0.22) with a noise power bandwidth equal to the chip rate. The requirements should apply whatever the type of transmitter considered (single carrier or multi-carrier). It applies for all transmission modes foreseen by the manufacturer’s specification.

The limit for ACLR should be as specified in Table 5a.

TABLE 5a

BS ACLR limits for UTRA

| BS channel offset below the first or above the last carrier frequency used (MHz) | ACLR limit (dB) |
| --- | --- |
| 5 | 44.2 |
| 10 | 49.2 |
| NOTE 1 – In certain regions, the adjacent channel power (the root raised cosine (RRC) filtered mean power centred on an adjacent channel frequency) should be less than or equal to −7.2 dBm/3.84 MHz (for Band I, Band IX and Band XI) or +2.8 dBm/3.84 MHz (for Band VI) or as specified by the ACLR limit, whichever is the higher. |  |

## 3.2	ACLR for E-UTRA (LTE)

The ACLR is defined with a square filter of bandwidth equal to the transmission bandwidth configuration of the transmitted signal (BWConfig) centered on the assigned channel frequency and a filter centered on the adjacent channel frequency according to the tables below. The transmission bandwidth configuration is as specified in Table 5b.

TABLE 5b

Downlink transmission bandwidth configuration BWConfig

| Channel bandwidth BWChannel [MHz] | 1.4 | 3 | 5 | 10 | 15 | 20 |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission bandwidth configuration (BWConfig) [MHz] | 1.095 | 2.715 | 4.515 | 9.015 | 13.515 | 18.015 |

The ACLR is defined with a square filter of bandwidth equal to the transmission bandwidth configuration of the transmitted signal (BWConfig) centered on the assigned channel frequency and a filter centered on the adjacent channel frequency according to the tables below.

For Category A, either the ACLR limits in the tables below or the absolute limit of 
-13 dBm/MHz apply, whichever is less stringent.

For Category B, either the ACLR limits in the tables below or the absolute limit of 
-15 dBm/MHz apply, whichever is less stringent.

For operation in paired spectrum, the ACLR shall be higher than the value specified in Table 5c.

TABLE 5c

BS ACLR limits for E-UTRA (LTE) in paired spectrum

| E-UTRA transmitted signal channel bandwidth BWChannel [MHz] | BS adjacent channel centre frequency offset below the first or above the last carrier centre frequency used | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 1.4, 3.0, 5, 10, 15, 20 | BWChannel | E-UTRA of same BW | Square (BWConfig) | 44.2 dB |
|  | 2 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44.2 dB |
|  | BWChannel /2 + 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
|  | BWChannel /2 + 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
| NOTE 1: BWChannel and BWConfig are the channel bandwidth and transmission bandwidth configuration of the E-UTRA transmitted signal on the assigned channel frequency. . |  |  |  |  |

# 4	Transmitter spurious emission (conducted)

The spurious emission is measured at the BS RF output port.

For UTRA, the requirement applies at frequencies within the specified frequency ranges, which are more than 12.5 MHz under the first carrier frequency used or more than 12.5 MHz above the last carrier frequency used.

For E-UTRA (LTE), the requirement applies at frequencies within the specified frequency ranges, excluding the frequency range from 10 MHz below the lowest frequency of the BS transmitter operating band up to 10 MHz above the highest frequency of the BS transmitter operating band.

The requirement below should apply whatever the type of transmitter considered (single carrier or multi-carrier). It applies for all transmission modes foreseen by the manufacturer’s specification.

Unless otherwise stated, all requirements are measured as mean power (r.m.s.).

## 4.1	Mandatory requirements

The requirements of either§ 4.1.1 or§ 4.1.2 applies.

### 4.1.1	Category A

The following requirements should be met in areas where Category A limits for spurious emissions, as defined in Recommendation ITU-R SM.329, are applied.

The power of any spurious emission should not exceed the limit specified in Table 6.

TABLE 6

BS spurious emission limit, Category A

| Band | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 9 kHz-150 kHz | 13 dBm | 1 kHz | Bandwidth as in Recommendation ITU-R SM.329, § 4.1 |
| 150 kHz-30 MHz |  | 10 kHz | Bandwidth as in Recommendation ITU-R SM.329, § 4.1 |
| 30 MHz-1 GHz |  | 100 kHz | Bandwidth as in Recommendation ITU-R SM.329, § 4.1 |
| 1 GHz-12.75 GHz |  | 1 MHz | Upper frequency as in Recommendation ITU-R SM.329, § 2.5 Table 1 |

### 4.1.2	Category B

### 4.1.2.1	Category B for UTRA

The following requirements should be met in areas where Category B limits for spurious emissions, as defined in Recommendation ITU-R SM.329, are applied.

The power of any spurious emission should not exceed the limit specified in Tables 7a) and 7b).

TABLE 7

a)  BS mandatory spurious emission limits, operating Band I, II, III, IV, VII, X (Category B)

| Band | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 9 « 150 kHz | 36 dBm | 1 kHz | (1) |
| 150 kHz « 30 MHz | 36 dBm | 10 kHz | (1) |
| 30 MHz « 1 GHz | 36 dBm | 100 kHz | (1) |
| 1 GHz « Flow – 10 MHz | 30 dBm | 1 MHz | (1) |
| Flow – 10 MHz  Fhigh + 10 MHz | 15 dBm | 1 MHz | (2) |
| Fhigh + 10 MHz « 12.75 GHz | 30 dBm | 1 MHz | (3) |

b)  BS mandatory spurious emission limits, operating Band V, VIII, XII, XIII, XIV (Category B)

| Band | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 9 « 150 kHz | 36 dBm | 1 kHz | (1) |
| 150 kHz « 30 MHz | 36 dBm | 10 kHz | (1) |
| 30 MHz « Flow – 10 MHz | 36 dBm | 100 kHz | (1) |
| Flow – 10 MHz  Fhigh + 10 MHz | 16 dBm | 100 kHz | (2) |
| Fhigh + 10 MHz « 1 GHz | 36 dBm | 100 kHz | (1) |
| 1 GHz « 12.75 GHz | 30 dBm | 1 MHz | (3) |
| (1) Bandwidth as in Recommendation ITU-R SM.329, § 4.1. (2) Limit based on Recommendation ITU-R SM.329, § 4.3 and Annex 7. (3) Bandwidth as in Recommendation ITU-R SM.329, § 4.1. Upper frequency as in Recommendation ITU-R SM.329, § 2.5 Table 1. Flow: the lowest downlink frequency of the operating band. Fhigh: the highest downlink frequency of the operating band. |  |  |  |

### 4.1.2.2	Category B for E-UTRA

The following requirements should be met in areas where Category B limits for spurious emissions, as defined in Recommendation ITU-R SM.329, are applied.

The power of any spurious emission should not exceed the limit specified in Tables 7c.

TABLE 7c

| Band | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 9 kHz  150 kHz | -36 dBm | 1 kHz | Note 1 |
| 150 kHz  30 MHz | -36 dBm | 10 kHz | Note 1 |
| 30 MHz  1 GHz | -36 dBm | 100 kHz | Note 1 |
| 1 GHz  12.75 GHz | -30 dBm | 1 MHz | Note 2 |
| NOTE 1 - Bandwidth as in ITU-R SM.329, §4.1 NOTE 2 - Bandwidth as in ITU-R SM.329, §4.1. Upper frequency as in ITU-R SM.329, §2.5 Table 1 |  |  |  |

## 4.2	Coexistence with other systems in the same geographical area

### 4.2.1	Coexistence with other systems in the same geographical area for UTRA

These requirements may be applied for the protection of UE, MS and/or BS operating in other frequency bands in the same geographical area. The requirements may apply in geographic areas in which both UTRA FDD operating in frequency Bands I to XIV and a system operating in another frequency band than the FDD operating band are deployed. The system operating in the other frequency band may be GSM900, DCS1800, PCS1900, GSM850 and/or FDD operating in Bands I to XIV.

The power of any spurious emission should not exceed the limits of Table 8(a) for a BS where requirements for coexistence with the system listed in the first column apply.

TABLE 8(a)

BS spurious emission limits for UTRA BS in geographic coverage area 
of systems operating in other frequency bands

| System type operating in the same geographical area | Band for coexistence requirement | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| GSM900 | 921-960 MHz | −57 dBm | 100 kHz | This requirement does not apply to UTRA FDD operating in Band VIII |
|  | 876-915 MHz | −61 dBm | 100 kHz | For the frequency range 880-915 MHz, this requirement does not apply to UTRA FDD operating in Band VIII |
| DCS1800 | 1 805-1 880 MHz | −47 dBm | 100 kHz | This requirement does not apply to UTRA FDD operating in Band III |
|  | 1 710-1 785 MHz | −61 dBm | 100 kHz | This requirement does not apply to UTRA FDD operating in Band III |
| PCS1900 | 1 930-1 990 MHz | −47 dBm | 100 kHz | This requirement does not apply to UTRA FDD BS operating in frequency band II |
|  | 1 850-1 910 MHz | −61 dBm | 100 kHz | This requirement does not apply to UTRA FDD BS operating in frequency band II |
| GSM850 or CDMA850 | 869-894 MHz | −57 dBm | 100 kHz | This requirement does not apply to UTRA FDD BS operating in frequency band V |
|  | 824-849 MHz | −61 dBm | 100 kHz | This requirement does not apply to UTRA FDD BS operating in frequency band V |
| FDD Band I | 2 110-2 170 MHz | −52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in Band I |
|  | 1 920-1 980 MHz | −49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band I |
| FDD Band II | 1 930-1 990 MHz | −52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in Band II |
|  | 1 850-1 910 MHz | −49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band II |

TABLE 8(a) (End)

| System type operating in the same geographical area | Band for coexistence requirement | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| FDD Band III | 1 805-1 880 MHz | −52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in Band III |
|  | 1 710-1 785 MHz | −49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in Band III |
| FDD Band IV | 2 110-2 155 MHz | −52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in Band IV |
|  | 1 710-1 755 MHz | −49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in Band IV |
| FDD Band V | 869-894 MHz | −52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in Band V |
|  | 824-849 MHz | −49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in Band V |
| FDD Band VI | 860-895 MHz | −52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in Band VI |
|  | 815-850 MHz | −49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in Band VI |
| FDD Band VII | 2 620-2 690 MHz | −52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in Band VII |
|  | 2 500-2 570 MHz | −49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in Band VII |
| FDD Band VIII | 925-960 MHz | −52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in Band VIII |
|  | 880-915 MHz | −49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in Band VIII |
| FDD Band IX | 1 844.9-1 879.9 MHz | −52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in Band IX |
|  | 1 749.9-1 784.9 MHz | −49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in Band IX |
| FDD Band X | 2 110-2 170 MHz | −52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in Band X |
|  | 1 710-1 770 MHz | −49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in Band X |
| FDD Band XI | 1 475.9-1 500.9 MHz | −52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in Band XI |
|  | 1 427.9-1 452.9 MHz | −49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in Band XI |
| FDD Band XII | 728-746 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XII |
|  | 698-716 MHz | -49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XII |
| FDD Band XIII | 746-756 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XIII |
|  | 777-787 MHz | -49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XIII |
| FDD Band XIV | 758-768 MHz | -52 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XIV |
|  | 788-798 MHz | -49 dBm | 1 MHz | This requirement does not apply to UTRA FDD BS operating in band XIV |

### 4.2.2	Coexistence with other systems in the same geographical area for E-UTRA

These requirements may be applied for the protection of UE, MS and/or BS operating in other frequency bands in the same geographical area. The requirements may apply in geographic areas in which both E-UTRA BS and a system operating in another frequency band than the E-UTRA operating band are deployed. The system operating in the other frequency band may be GSM900, DCS1800, PCS1900, GSM850, UTRA FDD/TDD and/or E-UTRA.

The power of any spurious emission shall not exceed the limits of Table 8(b) for a BS where requirements for co-existence with the system listed in the first column apply.

TABLE 8(b)

BS spurious emission limits for E-UTRA BS in geographic coverage area 
of systems operating in other frequency bands

| System type operating in the same geographical area | Band for co-existence requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| GSM900 | 921 - 960 MHz | -57 dBm | 100 kHz | This requirement does not apply to E-UTRA BS operating in Band 8 |
|  | 876 - 915 MHz | -61 dBm | 100 kHz | For the frequency range 880-915 MHz, this requirement does not apply to E-UTRA BS operating in Band 8 |
| DCS1800 | 1805 - 1880 MHz | -47 dBm | 100 kHz | This requirement does not apply to E-UTRA BS operating in Band 3 |
|  | 1710 - 1785 MHz | -61 dBm | 100 kHz | This requirement does not apply to E-UTRA BS operating in Band 3 |
| PCS1900 | 1930 - 1990 MHz | -47 dBm | 100 kHz | This requirement does not apply to E-UTRA BS operating in frequency Band 2 or Band 36 |
|  | 1850 - 1910 MHz | -61 dBm | 100 kHz | This requirement does not apply to E-UTRA BS operating in frequency Band 2. This requirement does not apply to E-UTRA BS operating in frequency Band 35 |
| GSM850 | 869 - 894 MHz | -57 dBm | 100 kHz | This requirement does not apply to E-UTRA BS operating in frequency Band 5 |
|  | 824 - 849 MHz | -61 dBm | 100 kHz | This requirement does not apply to E-UTRA BS operating in frequency Band 5 |
| UTRA FDD Band I or E-UTRA Band 1 | 2110 - 2170 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 1 |
|  | 1920 - 1980 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 1 |
| UTRA FDD Band II or E-UTRA Band 2 | 1930 - 1990 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 2 |
|  | 1850 - 1910 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 2 |
| UTRA FDD Band III or E-UTRA Band 3 | 1805 - 1880 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 3 |
|  | 1710 - 1785 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 3 |
| UTRA FDD Band IV or E-UTRA Band 4 | 2110 - 2155 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 4 |
|  | 1710 - 1755 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 4 |
| UTRA FDD Band V or E-UTRA Band 5 | 869 - 894 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 5 |
|  | 824 - 849 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 5 |

TABLE 8(b) (End)

| System type operating in the same geographical area | Band for co-existence requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| UTRA FDD Band VI or E-UTRA Band 6 | 860-895 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 6 |
|  | 815-850 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 6 |
| UTRA FDD Band VII or E-UTRA Band 7 | 2 620-2 690 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 7 |
|  | 2 500-2 570 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 7 |
| UTRA FDD Band VIII or E-UTRA Band 8 | 925-960 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 8 |
|  | 880-915 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 8 |
| UTRA FDD Band IX or E-UTRA Band 9 | 1 844.9-1 879.9 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 9 |
|  | 1 749.9-1 784.9 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 9 |
| UTRA FDD Band X or E-UTRA Band 10 | 2 110-2 170 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 10 |
|  | 1 710-1 770 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 10 |
| UTRA FDD Band XI or E-UTRA Band 11 | 1 475.9-1 500.9 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 11 |
|  | 1 427.9-1 452.9 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 11 |
| UTRA TDD in Band a) or E-UTRA Band 33 | 1 900-1 920 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 33 |
| UTRA TDD in Band a) or E-UTRA Band 34 | 2 010-2 025 MHz | -52 dBm | 1 MHz | This requirement does not apply eto E-UTRA BS operating in Band 34 |
| UTRA TDD in Band b) or E-UTRA Band 35 | 1 850-1 910 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 35 |
| UTRA TDD in Band b) or E-UTRA Band 36 | 1 930-1 990 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 2 and 36 |
| UTRA TDD in Band c) or E-UTRA Band 37 | 1 910-1 930 MHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA BS operating in Band 37. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment |
| UTRA TDD in Band d) or E-UTRA Band 38 | 2 570-2 620 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in Band 38 |
| E-UTRA Band 39 | 1 880-1 920 MHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA BS operating in Band 39 |
| E-UTRA Band 40 | 2 300-2 400 MHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA BS operating in Band 40 |

NOTE 1 – As defined in the scope for spurious emissions in this clause (clause 4), the co-existence requirements in Table 8(b) do not apply for the 10 MHz frequency range immediately outside the BS transmit frequency range of an operating band (see Note 2 and Note 3 for the scope). This is also the case when the transmit frequency range is adjacent to the Band for the co-existence requirement in the table. Emission limits for this excluded frequency range may also be covered by local or regional requirements.

NOTE 2 – The table above assumes that two operating bands, where the frequency ranges defined either in Note 2 or Note 3 in the scope would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by this recommendation.

## 4.3	Coexistence with PHS

This requirement may be applied for the protection of PHS in geographic areas in which both PHS and UTRA FDD or E-UTRA FDD are deployed. For UTRA FDD, this requirement is also applicable at specified frequencies falling between 12.5 MHz below the first carrier frequency used and 12.5 MHz above the last carrier frequency used. For E-UTRA FDD, this requirement is also applicable at specified frequencies falling between 10 MHz below the lowest BS transmitter frequency of the operating band and 10 MHz above the highest BS transmitter frequency of the operating band.

The power of any spurious emission should not exceed:

TABLE 9

BS spurious emission limits for BS in geographic coverage area of PHS

| Band | Measurement bandwidth | Maximum level | Note |
| --- | --- | --- | --- |
| 1 884.5 to 1 919.6 MHz | 300 kHz | 41 dBm |  |

## 4.4	Coexistence with UTRA-TDD

This requirement may be applied to geographic areas in which both UTRA-TDD and UTRA-FDD are deployed.

The power of any spurious emission should not exceed:

TABLE 10

BS spurious emission limits for UTRA BS in geographic coverage area of UTRA-TDD

| Band | Measurement bandwidth | Maximum level | Note |
| --- | --- | --- | --- |
| 1 900 to 1 920 MHz | 1 MHz | 52 dBm |  |
| 2 010 to 2 025 MHz | 1 MHz | 52 dBm |  |
| 2 570 to 2 610 MHz | 1 MHz | 52 dBm |  |

# 5	Receiver spurious emission

The requirements apply to all BS with separate receiver and transmitter antenna port. The test should be performed when both transmitter and receiver are on with the transmitter port terminated.

For all BS with common receiver and transmitter antenna ports the transmitter spurious emission as specified above is valid.

The power of any spurious emission should not exceed the limit specified in Tables 11a) and 11b).

For E-UTRA, in addition to the requirements in Table 11, the power of any spurious emission should not exceed the levels specified for Co-existence with other systems in the same geographical area in Clause 4.2.2 and Clause 4.3.

TABLE 11

a)  Receiver spurious emission limits

| Band | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz-1 GHz | 57 dBm | 100 kHz |  |
| 1-12.75 GHz | 47 dBm | 1 MHz |  |
| NOTE 1 – For UTRA, frequencies between 12.5 MHz below the first carrier frequency and 12.5 MHz above the last carrier frequency used by the BS transmitter are excluded. NOTE 2 – For E-UTRA, the frequency range between 2.5 * BWChannel below the first carrier frequency and 2.5 * BWChannel above the last carrier frequency transmitted by the BS, where BWChannel is the channel bandwidth, may be excluded from the requirement. However, frequencies that are more than 10 MHz below the lowest frequency of the BS transmitter operating band or more than 10 MHz above the highest frequency of the BS transmitter operating band shall not be excluded from the requirement. |  |  |  |

TABLE 11 (End)

b)  Additional spurious emission requirements for UTRA

| Operating Band | Band | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| I | 1 920-1 980 MHz | −78 dBm | 3.84 MHz |  |
| II | 1 850-1 910 MHz | −78 dBm | 3.84 MHz |  |
| III | 1 710-1 785 MHz | −78 dBm | 3.84 MHz |  |
| IV | 1 710-1 755 MHz | −78 dBm | 3.84 MHz |  |
| V | 824-849 MHz | −78 dBm | 3.84 MHz |  |
| VI | 815-850 MHz | −78 dBm | 3.84 MHz |  |
| VII | 2 500-2 570 MHz | −78 dBm | 3.84 MHz |  |
| VIII | 880-915 MHz | −78 dBm | 3.84 MHz |  |
| IX | 1 749.9-1 784.9 MHz | −78 dBm | 3.84 MHz |  |
| X | 1 710-1 770 MHz | −78 dBm | 3.84 MHz |  |
| XI | 1 427.9-1 452.9 MHz | −78 dBm | 3.84 MHz |  |
| XII | 698 - 716 MHz | -78 dBm | 3.84 MHz |  |
| XIII | 777 - 787 MHz | -78 dBm | 3.84 MHz |  |
| XIV | 788 - 798 MHz | -78 dBm | 3.84 MHz |  |

In addition, the requirements in Table 11c) may be applied to geographic areas in which both IMT-2000 CDMA TDD and IMT-2000 CDMA DS are deployed.

c)  Additional spurious emission requirements for the TDD bands for UTRA

| Operating Band |  | Band |  | Maximum level | Measurement bandwidth |  | Note |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I |  | 1 900-1 920 MHz 2 010-2 025 MHz |  | −78 dBm | 3.84 MHz |  | Not applicable in Japan |  |
|  |  | 2 010-2 025 MHz |  | −52 dBm | 1 MHz |  | Applicable in Japan |  |
| VI, IX, XI |  | 2 010-2 025 MHz |  | −52 dBm | 1 MHz |  |  |  |

Annex 2

IMT-2000 CDMA Multi-carrier (cdma-2000) Base Stations

# 1	Spectrum mask

The emissions when transmitting on a single or all RF carriers supported by the BS and configured in accordance with the manufacturer’s specification shall be less than the limits specified below. The spectrum emission mask values in Table 1 apply to Band Classes 0, 2, 5, 7, 9, and 10 and shall be met when transmitting on a single or all RF carriers supported by the BS as indicated by the entries in the column Active Carriers.

TABLE 1

Band Classes 0, 2, 5, 7, 9, and 10 Spectrum Emission Mask Values

| For ¦Δf ¦ within the Range | Active Carriers | Emission Limit |
| --- | --- | --- |
| 750 kHz to 1.98 MHz | Single | 45 dBc/30 kHz |
| 1.98 to 4.00 MHz | Single | -60 dBc / 30 kHz, HRPD -60 dBc / 30 kHz; Pout  33 dBm, cdma2000 -27 dBm / 30 kHz; 28 dBm  Pout < 33 dBm, cdma2000 -55 dBc / 30 kHz; Pout < 28 dBm, cdma2000 |
| 3.25 to 4.00 MHz (Band Class 7 only) | All | 46 dBm / 6.25 kHz |
| NOTE 1 – All frequencies in the measurement bandwidth shall satisfy the restrictions on ¦Δf ¦ where Δf  centre frequency – closer edge frequency ( f ) of the measurement filter. For multiple-carrier testing, f is defined for positive f as the centre frequency of the highest carrier – closer measurement edge frequency ( f ) and for negative f as the centre frequency of the lowest carrier – closer measurement edge frequency ( f ). |  |  |

The spectrum emission mask values  in Table 2 apply to Band Classes 1, 4, 6, 8, 13, 14, and 15 and shall be met when transmitting on a single or all RF carriers supported by the BS as indicated by the entries in the column Active Carriers.

Table 2

Band Classes 1, 4, 6, 8, 13, 14, and 15 Spectrum Emission Mask Values

| For ¦ Δf¦ within the Range | Active Carriers | Emission Limit |
| --- | --- | --- |
| 885 kHz to 1.25 MHz | Single | -45 dBc / 30 kHz |
| 1.25 to 1.98 MHz | Single | More stringent of -45 dBc / 30 kHz or -9 dBm / 30 kHz |
| 1.25 to 2.25 MHz (MC tests only) | All | -9 dBm / 30 kHz |
| 1.25 to 1.45 MHz (Band Classes 6 , 8, and 13) | All | -13 dBm / 30 kHz |
| 1.45 to 2.25 MHz (Band Classes 6 , 8, and 13) | All | -[13 + 17  (f – 1.45 MHz)] dBm / 30 kHz |
| 1.98 MHz to 2.25 MHz | Single | -55 dBc / 30 kHz, HPRD -55 dBc / 30 kHz; Pout  33 dBm, cdma2000 -22 dBm / 30 kHz; 28 dBm  Pout < 33 dBm, cdma2000 -50 dBc / 30 kHz; Pout < 28 dBm, cdma2000 |
| 2.25 MHz to 4.00 MHz | All | -13 dBm / 1 MHz |

NOTE - All frequencies in the measurement bandwidth shall satisfy the restrictions on |f|. The emissions requirements shall apply for all values of f regardless of whether the measurement frequency falls inside or outside of the band or block edge. For single-carrier testing, f = center frequency - closer measurement edge frequency (f). For multiple-carrier testing, f is defined for positive f as the closer measurement edge frequency (f) - center frequency of the highest carrier and for negative f as the closer measurement edge frequency (f) - center frequency of the lowest carrier.

The spectrum emission mask values  in Table 3 apply to Band Classes 11 and 12 and shall be met when transmitting on a single or all RF carriers supported by the BS as indicated by the entries in the column Active Carriers.

Table 3

Band Classes 11 and 12 Spectrum Emission Mask Values

| For ¦f¦ within the Range | Active Carriers | Emission Limit |
| --- | --- | --- |
| 750 to 885 KHz | Single | -45-15(¦f¦-750)/135 dBc in 30 kHz |
| 885 to 1125 KHz | Single | -60-5(¦f¦-885)/240 dBc in 30 kHz |
| 1.125 to 1.98 MHz | Single | -65 dBc / 30kHz |
| 1.98 to 4.00 MHz | Single | -75 dBc / 30kHz |
| NOTE - All frequencies in the measurement bandwidth shall satisfy the restrictions on ¦f¦ where f = center frequency - closer measurement edge frequency (f). f is positive offset from the highest valid CDMA channel in the band subclass or negative offset from the lowest valid CDMA channel in the band subclass. The emission limits for Band Classes 11 and 12 (European PAMR bands) are designed to allow co-existence with incumbent services in Europe and are tighter than ITU Category B requirements. |  |  |

The spectrum emission mask values in Table 4 apply to Band Class 3 and shall be met when transmitting on a single or all RF carriers supported by the BS as indicated by the entries in the column Active carriers.

Table 4

Band Class 3 Spectrum Emission Mask Values

| Measurement Frequency | Active Carriers | For ¦ Δf ¦ within the Range | Emission Limit |
| --- | --- | --- | --- |
| > 832 MHz and Error! Objects cannot be created from editing field codes.834 MHz, > 838 MHz and Error! Objects cannot be created from editing field codes.846 MHz, > 860 MHz and Error! Objects cannot be created from editing field codes. 895 MHz | Single | Error! Objects cannot be created from editing field codes. 750 kHz and < 1.98 MHz | -45 dBc / 30 kHz |
|  | Single | Error! Objects cannot be created from editing field codes. 1.98 MHz | 25 mW (-16 dBm) / 100 kHz; Pout Error! Objects cannot be created from editing field codes. 30 dBm -60 dBc / 100 kHz; 30dBm < Pout Error! Objects cannot be created from editing field codes. 47 dBm Less stringent of 50 mW (-13 dBm) / 100 kHz or -70 dBc / 100 kHz; Pout > 47 dBm |
| > 810 MHz and Error! Objects cannot be created from editing field codes. 860 MHz, except > 832 MHz and Error! Objects cannot be created from editing field codes. 834 MHz, > 838 MHz and Error! Objects cannot be created from editing field codes. 846 MHz | Single | < 1.98 MHz | 25 mW (-16 dBm) / 30 kHz; Pout Error! Objects cannot be created from editing field codes. 30 dBm More stringent of -60 dBc / 30 kHz and 25 mW (-16 dBm) / 30 kHz; Pout > 30 dBm |
|  | Single | Error! Objects cannot be created from editing field codes. 1.98 MHz | 25 mW (-16 dBm) / 100 kHz; Pout Error! Objects cannot be created from editing field codes. 30 dBm More stringent of -60 dBc / 100 kHz and 25 mW (-16 dBm) / 100 kHz; Pout > 30 dBm |
| Error! Objects cannot be created from editing field codes. 810 MHz and > 895 MHz | All | N/A | 25 mW (-16 dBm) / 1 MHz; Pout Error! Objects cannot be created from editing field codes. 44 dBm -60 dBc / 1 MHz; 44 dBm < Pout Error! Objects cannot be created from editing field codes. 47 dBm Less stringent of 50 mW (-13 dBm) / 1 MHz or -70 dBc / 1 MHz; Pout > 47 dBm |
| NOTE - All frequencies in the measurement bandwidth shall satisfy the restrictions on ¦Δf¦. The emissions requirements shall apply for all values of Δf regardless of whether the measurement frequency falls inside or outside of the band or block edge. For single-carrier testing, Δf = center frequency - closer measurement edge frequency (f). For multiple-carrier testing, Δf is defined for positive Δf as the closer measurement edge frequency (f) - center frequency of the highest carrier and for negative Δf as the closer measurement edge frequency (f) - center frequency of the lowest carrier. The upper and lower limits of the frequency measurement are currently 10 MHz and 3 GHz in Japan radio measurement documents. |  |  |  |

# 2	Transmitter Spurious Emission

In areas where Category A limits for spurious emissions, as defined in Recommendation ITU-R SM.329, are applied, the spurious emissions when transmitting on all RF carriers supported by the BS and configured in accordance with the manufacturer’s specification shall be less than the limits specified in Tables 5A and 5B.

TABLE 5A

BS Spurious Emission Limits, Category A

| For ¦Δf ¦ within the Range | Emission Limit |  |
| --- | --- | --- |
|  4.00 MHz | 9 kHz  f  150 kHz 150 kHz  f  30 MHz 30 MHz  f  1 GHz 1 GHz  f  12.75 GHz | 13 dBm/1 kHz 13 dBm/10 kHz 13 dBm/100 kHz 13 dBm/1 MHz |
| NOTE 1 – All frequencies in the measurement bandwidth shall satisfy the restrictions on ¦Δf ¦ where f  centre frequency – closer edge frequency ( f ) of the measurement filter. For multiple-carrier testing, f is defined for positive f as the centre frequency of the highest carrier – closer measurement edge frequency ( f ) and for negative f as the centre frequency of the lowest carrier – closer measurement edge frequency ( f ). |  |  |

TABLE 5B

Additional Spurious Emission Limits in addition to Category A Limits in Areas where PHS is Deployed

| Measurement Frequency | Measurement Bandwidth | Emission Limit | For Protection of |
| --- | --- | --- | --- |
| 1 884.5 to 1 919.6 MHz | 300 kHz | 41 dBm | PHS |

In areas where Category B limits for spurious emissions, as defined in Recommendation ITU-R SM.329, are applied, the spurious emissions when transmitting on a single or all RF carriers supported by the BS and configured in accordance with the manufacturer’s specification shall be less than the limits specified in Tables 6A and 6B. The emission limits in Table 6A shall be met when transmitting on all RF carriers supported by the BS. The emission limits in Table 6B shall be met when transmitting on a single or all RF carriers supported by the BS as indicated by the entries in the column Active Carriers.

TABLE 6A

Spurious Emission Limits, Category B

| For ¦Δf ¦ within the Range | Emission Limit |  |
| --- | --- | --- |
|  4.00 MHz | 9 kHz  f  150 kHz 150 kHz  f  30 MHz 30 MHz  f  1 GHz 1 GHz  f  12.75 GHz | 36 dBm/1 kHz 36 dBm/10 kHz 36 dBm/100 kHz 30 dBm/1 MHz |
| NOTE 1 – All frequencies in the measurement bandwidth shall satisfy the restrictions on ¦Δf ¦ where f  centre frequency – closer edge frequency ( f ) of the measurement filter. For multiple-carrier testing, f is defined for positive f as the centre frequency of the highest carrier – closer measurement edge frequency ( f ) and for negative f as the centre frequency of the lowest carrier – closer measurement edge frequency ( f ). |  |  |

TABLE 6B

Additional Spurious Emission Limits in 
addition to Category B Limits

| Measurement Frequency | Active Carriers | Emission Limit | For Protection of |
| --- | --- | --- | --- |
| 921 to 960 MHz | All | 57 dBm/100 kHz | GSM 900 MS receive band |
| 1 805 to 1 880 MHz | All | 47 dBm/100 kHz | DCS 1800 MS receive band |
| 1 900 to 1 920 MHz 2 010 to 2 025 MHz | All | 52 dBm/1 MHz | IMT-2000 CDMA TDD |
| 1 920 to 1 980 MHz | Single | 86 dBm/1 MHz | FDD BS receive band |

When transmitting in Band Classes 0, 7, 9, and 10, the spurious emissions shall be less than the limits specified in Table 7A and 7B when transmitting on a single or all RF carriers supported by the BS as indicated by the entries in the column Active carriers.

Table 7A

Additional Band Classes 0, 7, 9, and 10 Spurious Emission Limits for ITU Category B only

| For ¦ Δf ¦ within the Range | Active Carriers | Emission Limit |  |
| --- | --- | --- | --- |
| > 4.00 MHz [Band Classes 0, 7, 9, and 10] (ITU Category B only) | All | -36 dBm / 1 kHz; -36 dBm / 10 kHz; -36 dBm / 100 kHz; -30 dBm / 1 MHz; | 9 kHz < f < 150 kHz 150 kHz < f < 30 MHz 30 MHz < f < 1 GHz 1 GHz < f < 12.5 GHz |

Table 7b

Additional Band Classes 0, 7, 9, and 10 Spurious Emission Limits for ITU Category B only

| Frequency Range | Active Carriers | Emission Limit |
| --- | --- | --- |
| 30 MHz < f < flow – 4.0 MHz | All | -36 dBm/100 kHz |
| flow – 4.0 MHz Error! Objects cannot be created from editing field codes. f Error! Objects cannot be created from editing field codes. fc – 4.0 MHz | All | -16 dBm/100 kHz |
| Fc + 4.0 MHz Error! Objects cannot be created from editing field codes. f Error! Objects cannot be created from editing field codes. fhigh + 4.0 MHz | All | -16 dBm/100 kHz |
| fhigh + 4.0 MHz < f < 1.0 GHz | All | -36 dBm/100 kHz |
| flow = The center frequency of the lowest valid carrier in the band fhigh = The center frequency of the highest valid carrier in the band |  |  |

When transmitting in Band Classes 2 and 5, the spurious emissions shall be less than the limits specified in Table 8 when transmitting on a single or all RF carriers supported by the BS as indicated by the entries in the column Active carriers.

Table 8

Additional Band Classes 2 and 5 Spurious Emission Limits for ITU Category B only

| For ¦ Δf ¦ within the Range | Active Carriers | Emission Limit |  |
| --- | --- | --- | --- |
| > 4.00 MHz [Band Classes 2 and 5] (ITU Category B only) | All | -36 dBm / 1 kHz; -36 dBm / 10 kHz; -30 dBm / 1 MHz; | 9 kHz < f < 150 kHz 150 kHz < f < 30 MHz 1 GHz < f < 12.5 GHz |
| 4.00 to 6.40 MHz (Band Classes 2 and 5) (ITU Category B only) | All | -36 dBm / 1 kHz | 30 MHz < f < 1 GHz |
| 6.40 to 16 MHz (Band Classes 2 and 5) (ITU Category B only) | All | -36 dBm / 10 kHz | 30 MHz < f < 1 GHz |
| > 16 MHz (Band Classes 2 and 5) (ITU Category B only) | All | -36 dBm / 100 kHz | 30 MHz < f < 1 GHz |

When transmitting in Band Classes 11 and 12, the spurious emissions shall be less than the limits specified in Tables 9A and 9B.

Table 9A

Additional Band Classes 11 and 12 Spurious Emission Limits for ITU Category B only

| For ¦ Δf¦ within the Range |  | Active Carriers |  | Emission Limit |  |
| --- | --- | --- | --- | --- | --- |
| > 6.00 MHz | All |  | -36 dBm / 1 kHz; -36 dBm / 10 kHz; -45 dBm/100 kHz; -30 dBm / 1 MHz; |  | 9 kHz < f < 150 kHz 150 kHz < f < 30 MHz 30 MHz < f < 1 GHz 1 GHz < f < 12.75 GHz |

Table 9B

Additional Band Classes 11 and 12 Spurious Emission Limits

| For ¦ Δf ¦ within the Range | Active Carriers | Emission Limit |
| --- | --- | --- |
| 4.00 to 6.00 MHz | All | -36 dBm / 100kHz |
| > 6.00 MHz | All | -45 dBm / 100kHz |
| The emission limits for Band Classes 11 and 12 (European PAMR bands) are designed to allow co-existence with incumbent services in Europe and are tighter than ITU Category B requirements. |  |  |

When transmitting in Band Classes 1, 4, 6, 8, 13, 14, and 15, the spurious emissions shall be less than the limits specified in Table 10A. When transmitting in Band Class 6, the spurious emissions shall be less than the limits specified in Table 10B.

Table 10a

Additional Band Classes 1, 4, 6, 8, 13, 14, and 15 Spurious 
Emission Limits for ITU Category B only

| Frequency Range | Active Carriers | Emission Limit |
| --- | --- | --- |
| flow – 4.0 MHz < f < fc – 4.0 MHz | All | -30 dBm/30 kHz |
| fc + 4.0 MHz < f < fhigh + 4.0 MHz | All | -30 dBm/30 kHz |
| 1 GHz < f < flow – 4.0 MHz | All | -30 dBm/1 MHz |
| fhigh + 4.0 MHz < f < 12.5 GHz | All | -30 dBm/1 MHz |
| flow = The center frequency of the lowest valid carrier in the band fhigh = The center frequency of the highest valid carrier in the band. |  |  |

Table 10b

Additional Band Class 6 Spurious Emission Limits

| Measurement Frequency | Active Carriers | Emission Limit | When Coverage Overlaps with |
| --- | --- | --- | --- |
| 1 884.5 to 1 919.6 MHz | Single | -41 dBm / 300 kHz | PHS |
| 824 to 849 MHz | No | -98 dBm / 100 kHz (co-located only) -61 dBm / 100 kHz (non-colocated) | GSM 850 CDMA 850 |
| 869 to 894 MHz | Yes | –57 dBm / 100 kHz | GSM 850 CDMA 850 |
| 876 to 915 MHz | Single | –98 dBm / 100 kHz (co-located only) –61 dBm / 100 kHz (non-co-located) | GSM 900 |
| 921 to 960 MHz | All | -57 dBm / 100 kHz | GSM 900 |
| 1 710 to 1 785 MHz | Single | –98 dBm / 100 kHz (co-located only) –61 dBm / 100 kHz (non-co-located) | DCS 1800 |
| 1 805 to 1 880 MHz | All | – 47 dBm / 100 kHz | DCS 1800 |
| 1 900 to 1 920 MHz and 2 010 to 2 025 MHz | Single | – 86 dBm / 1 MHz (co-located only) | UTRA-TDD |
| 1 900 to 1 920 MHz and 2 010 to 2 025 MHz | All | –52 dBm / 1 MHz | UTRA-TDD |
| 1 920 to 1 980 MHz | Single | – 86 dBm / 1 MHz | Always |

When transmitting in Band Class 10 in North America the spurious emissions shall be less than the limits specified in Table 11.

Table 11

Additional Band Class 10 Spurious Emission Limits for North American Operation

| Measurement Frequency | Emission Limit |
| --- | --- |
| 854.75 to 861 MHz | -40 dBm / 30 kHz |
| 866 to 869 MHz | -40 dBm / 30 kHz |
| NOTE - The Band Class 10 spurious emissions limit is designed to allow marginal co-existence with North American PMRS 800 MHz Public Safety services and is far tighter than the CFR 47 Part 90.691(a)(2) requirement. |  |

When transmitting in Band Class 7, the spurious emissions shall also be less than the limits specified in Table 12.

Table 12

Additional Band Class 7 Spurious Emission Limits

| Transmission Frequency (MHz) | Measurement Frequency (MHz) | Emission Limit | Victim Band |
| --- | --- | --- | --- |
| 746-758 | 763-775 and 793-805 | -46 dBm/6.25 kHz | Public Safety |
| 758-768 | 769-775 and 799-805 | -46 dBm/6.25 kHz | Public Safety |

# Base Station ACLR Limits

# 3	Receiver Spurious Emission

This requirement only applies if the BS is equipped with a separate RF input port. The conducted spurious emissions at the BS RF input ports shall be not greater than the limits in Tables 13 and 14.

TABLE 13

General Receiver Spurious Emission Requirements

| Frequency Band | Measurement Bandwidth | Maximum Level | Note |
| --- | --- | --- | --- |
| 30 MHz  f  1 GHz | 100 kHz | 57 dBm |  |
| 1 GHz  f  12.75 GHz | 1 MHz | 47 dBm | With the exception of the frequencies covered by Table 15, for which additional receiver spurious emission requirements apply |

For all frequencies within the mobile station receive and transmit bands, the conducted emissions shall be below the limits in Table 14.

TABLE 14

Additional Receiver Spurious Emission Requirements

|  | Measurement Bandwidth (kHz) | Maximum Level (dBm) | Note |
| --- | --- | --- | --- |
|  | 30 | 80 | Base receive band |
|  | 30 | 60 | Base transmit band |

Annex 3

IMT-2000 CDMA TDD (UTRA TDD) base stations

# 1	Measurement uncertainty

Values specified in this Annex differ from those specified in Recommendation ITU-R M.1457 since values in this Annex incorporate test tolerances defined in Recommendation ITU-R M.1545.

# 2	Spectrum mask

## 2.1	UTRA 3.84 Mchip/s TDD option

The spectrum emission mask specifies the limit of the transmitter OoB emissions at frequency offsets from the assigned channel frequency of the wanted signal between 2.5 MHz and 12.5 MHz.

The requirement should be met by a BS transmitting on a single RF carrier configured in accordance with the manufacturer’s specification. Emissions should not exceed the maximum level specified in Tables 17a to 20a  in the frequency range of f_offset from 2.515 MHz to fmax from the carrier frequency, where:

–	f_offset is the separation between the carrier frequency and the centre of the measurement filter:

	f_offsetmax is either 12.5 MHz or the offset to the universal mobile telecommunications system (UMTS) transmit band edge, whichever is the greater.

	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

The spectrum emissions measured should not exceed the maximum level specified in Tables 17a to 20a for the appropriate BS rated output power.

TABLE 17a

Spectrum emission mask values, BS maximum output power P   43 dBm

| Frequency offset of measurement filter −3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Maximum level | Measure-ment bandwidth |
| --- | --- | --- | --- |
| 2.5 MHz  f < 2.7 MHz | 2.515 MHz  f_offset < 2.715 MHz | −12.5 dBm | 30 kHz |
| 2.7 MHz  f < 3.5 MHz | 2.715 MHz  f_offset < 3.515 MHz | −12.5 dBm – 15 f_offsetMHz – 2.715 dB | 30 kHz |
| (see Note) | 3.515 MHz  f_offset < 4.0 MHz | −24.5 dBm | 30 kHz |
| 3.5 MHz  f  fmax | 4.0 MHz  f_offset < f_offsetmax | −11.5 dBm | 1 MHz |

TABLE 18a

Spectrum emission mask values, BS maximum output power 39  P < 43 dBm

| Frequency offset of measurement filter −3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Maximum level | Measure-ment bandwidth |
| --- | --- | --- | --- |
| 2.5 MHz  f < 2.7 MHz | 2.515 MHz  f_offset < 2.715 MHz | −12.5 dBm | 30 kHz |
| 2.7 MHz  f < 3.5 MHz | 2.715 MHz  f_offset < 3.515 MHz | −12.5 dBm – 15 f_offsetMHz – 2.715 dB | 30 kHz |
| (see Note) | 3.515 MHz  f_offset < 4.0 MHz | −24.5 dBm | 30 kHz |
| 3.5 MHz  f < 7.5 MHz | 4.0 MHz  f_offset < 8.0 MHz | −11.5 dBm | 1 MHz |
| 7.5 MHz  f  fmax | 8.0 MHz  f_offset < f_offsetmax | P – 54.5 dB | 1 MHz |

TABLE 19a

Spectrum emission mask values, BS maximum output power 31  P < 39 dBm

| Frequency offset of measurement filter −3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Maximum level | Measure-ment bandwidth |
| --- | --- | --- | --- |
| 2.5 MHz  f < 2.7 MHz | 2.515 MHz  f_offset < 2.715 MHz | P – 51.5 dB | 30 kHz |
| 2.7 MHz  f < 3.5 MHz | 2.715 MHz  f_offset < 3.515 MHz | P−51.5 dB – 15 f_offsetMHz – 2.715 dB | 30 kHz |
| (see Note) | 3.515 MHz  f_offset < 4.0 MHz | P – 63.5 dB | 30 kHz |
| 3.5 MHz  f < 7.5 MHz | 4.0 MHz  f_offset < 8.0 MHz | P – 50.5 dB | 1 MHz |
| 7.5 MHz  f  fmax | 8.0 MHz  f_offset < f_offsetmax | P – 54.5 dB | 1 MHz |

TABLE 20a

Spectrum emission mask values, BS maximum output power P < 31 dBm dB

| Frequency offset of measurement filter −3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Maximum level | Measure-ment bandwidth |
| --- | --- | --- | --- |
| 2.5 MHz  f < 2.7 MHz | 2.515 MHz  f_offset < 2.715 MHz | −20.5 dBm | 30 kHz |
| 2.7 MHz  f < 3.5 MHz | 2.715 MHz  f_offset < 3.515 MHz | −20.5 dBm – 15 f_offsetMHz – 2.715 dB | 30 kHz |
| (see Note) | 3.515 MHz  f_offset < 4.0 MHz | −32.5 dBm | 30 kHz |
| 3.5 MHz  f < 7.5 MHz | 4.0 MHz  f_offset < 8.0 MHz | −19.5 dBm | 1 MHz |
| 7.5 MHz  f  fmax | 8.0 MHz  f_offset < f_offsetmax | −23.5 dBm | 1 MHz |

NOTE – This frequency range ensures that the range of values of f_offset is continuous.

## 2.2	UTRA 1.28 Mchip/s TDD option

The spectrum emission mask specifies the limit of the transmitter OoB emissions at frequency offsets from the assigned channel frequency of the wanted signal between 0.8 MHz and 4.0 MHz.

The requirement should be met by a BS transmitting on a single RF carrier configured in accordance with the manufacturer’s specification. Emissions should not exceed the maximum level specified in Tables 17b to 19b in the frequency range of f_offset from 0.815 MHz to fmax from the carrier frequency, where:

–	f_offset is the separation between the carrier frequency and the centre of the measurement filter:

	f_offsetmax is either 4.0 MHz or the offset to the universal mobile telecommunication system (UMTS) transmit band edge, whichever is the greater.

	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

The spectrum emissions measured should not exceed the maximum level specified in Tables 17b to 19b for the appropriate BS rated output power.

TABLE 17b

Spectrum emission mask values, BS maximum output power P  34 dBm

| Frequency offset of measurement filter −3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Maximum level | Measure-ment bandwidth |
| --- | --- | --- | --- |
| 0.8 MHz  f < 1.0 MHz | 0.815 MHz  f_offset < 1.015 MHz | −18.5 dBm | 30 kHz |
| 1.0 MHz  f < 1.8 MHz | 1.015 MHz  f_offset < 1.815 MHz | −18.5 dBm – 10 f_offsetMHz – 1.015 dB | 30 kHz |
| (see Note) | 1.815 MHz  f_offset < 2.3 MHz | −26.5 dBm | 30 kHz |
| 1.8 MHz  f fmax | 2.3 MHz  f_offset < f_offsetmax | −11.5 dBm | 1 MHz |

TABLE 18b

Spectrum emission mask values, BS maximum output power 26  P < 34 dBm

| Frequency offset of measurement filter −3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Maximum level | Measure-ment bandwidth |
| --- | --- | --- | --- |
| 0.8 MHz  f < 1.0 MHz | 0.815 MHz  f_offset < 1.015 MHz | P − 52.5 dB | 30 kHz |
| 1.0 MHz  f < 1.8 MHz | 1.015 MHz  f_offset < 1.815 MHz | P−52.5 dB – 10 f_offsetMHz – 1.015 dB | 30 kHz |
| (see Note) | 1.815 MHz  f_offset < 2.3 MHz | P − 60.5 dB | 30 kHz |
| 1.8 MHz  f fmax | 2.3 MHz  f_offset < f_offsetmax | P – 45.5 dB | 1 MHz |

TABLE 19b

Spectrum emission mask values, BS maximum output power P < 26 dBm

| Frequency offset of measurement filter −3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Maximum level | Measure-ment bandwidth |
| --- | --- | --- | --- |
| 0.8 MHz  f < 1.0 MHz | 0.815 MHz  f_offset < 1.015 MHz | −26.5 dBm | 30 kHz |
| 1.0 MHz  f < 1.8 MHz | 1.015 MHz  f_offset < 1.815 MHz | −26.5 dB – 10 f_offsetMHz – 1.015 dB | 30 kHz |
| (see Note) | 1.815 MHz  f_offset < 2.3 MHz | −34.5 dBm | 30 kHz |
| 1.8 MHz  f fmax | 2.3 MHz  f_offset < f_offsetmax | −19.5 dBm | 1 MHz |

NOTE – This frequency range ensures that the range of values of f_offset is continuous.

## 2.3	UTRA 7.68 Mchip/s TDD option

The spectrum emission mask specifies the limit of the transmitter OoB emissions at frequency offsets from the assigned channel frequency of the wanted signal between 5 MHz and 25 MHz.

The requirement should be met by a BS transmitting on a single RF carrier configured in accordance with the manufacturer’s specification. Emissions should not exceed the maximum level specified in Tables 17c to 20c in the frequency range of f_offset from 5.015 MHz to fmax from the carrier frequency, where:

–	f_offset is the separation between the carrier frequency and the centre of the measurement filter:

	f_offsetmax is either 25 MHz or the offset to the universal mobile telecommunications system (UMTS) transmit band edge, whichever is the greater.

	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

The spectrum emissions measured should not exceed the maximum level specified in Tables 17c to 19c for the appropriate BS rated output power.

TABLE 17c

Spectrum emission mask values, BS maximum output power P  43 dBm

| Frequency offset of measurement filter −3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Maximum level | Measurement bandwidth |
| --- | --- | --- | --- |
| 5 MHz  f < 5.2 MHz | 5.015 MHz  f_offset < 5.215 MHz | −15.5 dBm | 30 kHz |
| 5.2 MHz  f < 6 MHz | 5.215 MHz  f_offset < 6.015 MHz | −15.5 dBm – 15 f_offsetMHz – 5.215 dB | 30 kHz |
| (see Note) | 6.015 MHz  f_offset < 6.5 MHz | −27.5 dBm | 30 kHz |
| 6 MHz  f  fmax | 6.5 MHz  f_offset < f_offsetmax | −14.5 dBm | 1 MHz |

TABLE 18c

Spectrum emission mask values, BS maximum output power 39  P < 43 dBm

| Frequency offset of measurement filter −3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Maximum level | Measurement bandwidth |
| --- | --- | --- | --- |
| 5 MHz  f < 5.2 MHz | 5.015 MHz  f_offset < 5.215 MHz | −15.5 dBm | 30 kHz |
| 5.2 MHz  f < 6 MHz | 5.215 MHz  f_offset < 6.015 MHz | −15.5 dBm – 15 f_offsetMHz – 5.215 dB | 30 kHz |
| (see Note) | 6.015 MHz  f_offset < 6.5 MHz | −27.5 dBm | 30 kHz |
| 6 MHz  f < 15 MHz | 6.5 MHz  f_offset < 15.5 MHz | −14.5 dBm | 1 MHz |
| 15 MHz  f  fmax | 15.5 MHz  f_offset < f_offsetmax | P – 57.5 dB | 1 MHz |

TABLE 19c

Spectrum emission mask values, BS maximum output power 31  P < 39 dBm

| Frequency offset of measurement filter −3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Maximum level | Measure-ment bandwidth |
| --- | --- | --- | --- |
| 5 MHz  f < 5.2 MHz | 5.015 MHz  f_offset < 5.215 MHz | P – 54.5 dB | 30 kHz |
| 5.2 MHz  f < 6 MHz | 5.215 MHz  f_offset < 6.015 MHz | P−54.5 dBm – 15 f_offsetMHz – 5.215 dB | 30 kHz |
| (see Note) | 6.015 MHz  f_offset < 6.5 MHz | P – 66.5 dB | 30 kHz |
| 6 MHz  f < 15 MHz | 6.5 MHz  f_offset < 15.5 MHz | P – 53.5 dB | 1 MHz |
| 15 MHz  f  fmax | 15.5 MHz  f_offset < f_offsetmax | P – 57.5 dB | 1 MHz |

TABLE 20c

Spectrum emission mask values, BS maximum output power P < 31 dBm

| Frequency offset of measurement filter −3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Maximum level | Measure-ment bandwidth |
| --- | --- | --- | --- |
| 5 MHz  f < 5.2 MHz | 5.015 MHz  f_offset < 5.215 MHz | −23.5 dBm | 30 kHz |
| 5.2 MHz  f < 6 MHz | 5.215 MHz  f_offset < 6.015 MHz | −23.5 dBm – 15 f_offsetMHz – 5.215 dB | 30 kHz |
| (see Note) | 6.015 MHz  f_offset < 6.5 MHz | −35.5 dBm | 30 kHz |
| 6 MHz  f < 15 MHz | 6.5 MHz  f_offset < 15.5 MHz | −22.5 dBm | 1 MHz |
| 15 MHz  f  fmax | 15.5 MHz  f_offset < f_offsetmax | −26.5 dBm | 1 MHz |

NOTE – This frequency range ensures that the range of values of f_offset is continuous.

## 2.4	E-UTRA (LTE) spectrum mask

The Operating band unwanted emission limits are defined from 10 MHz below the lowest frequency of the BS transmitter operating band up to 10 MHz above the highest frequency of the BS transmitter operating band.

The requirements shall apply whatever the type of transmitter considered (single carrier or multi-carrier) and for all transmission modes foreseen by the manufacturer's specification.

The unwanted emission limits in the part of the operating band that falls in the spurious domain are consistent with Recommendation ITU-R SM.329.

Emissions should not exceed the maximum level specified in Tables below, where:

–	f is the separation between the channel edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency.

–	f_offset is the separation between the channel edge frequency and the centre of the measuring filter.

–	f_offsetmax is the offset to the frequency 10 MHz outside the BS transmitter operating band.

–	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multicarrier E-UTRA BS the definitions above apply to the lower edge of the carrier transmitted at the lowest carrier frequency and the higher edge of the carrier transmitted at the highest carrier frequency.

The requirements of either § 2.4.1 or § 2.4.2 shall apply.

The Additional operating band unwanted emission limits defined in § 2.4.2.1 may be mandatory in certain regions. In other regions it may not apply.

### 2.4.1	E-UTRA spectrum mask (Category A)

For E-UTRA BS operating in TDD bands, emissions shall not exceed the maximum levels specified in Table 17d:

TABLE 17d

d-a) General operating band unwanted emission limits for 1.4 MHz channel 
bandwidth (E-UTRA bands >1 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 1) |
| --- | --- | --- | --- |
| 0 MHz  f < 1.4 MHz | 0.05 MHz  f_offset < 1.45 MHz | +0.5dBm−101.4⋅foffsetMHz−0.05dB | 100 kHz |
| 1.4 MHz  f < 2.8 MHz | 1.45 MHz  f_offset < 2.85 MHz | -9.5 dBm | 100 kHz |
| 2.8 MHz  f  fmax | 2.85 MHz  f_offset < f_offsetmax | -13 dBm | 100 kHz |

d-b) General operating band unwanted emission limits for 3 MHz channel 
bandwidth (E-UTRA bands >1 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 1) |
| --- | --- | --- | --- |
| 0 MHz  f < 3 MHz | 0.05 MHz  f_offset < 3.05 MHz | −3.5dBm−103⋅foffsetMHz−0.05dB | 100 kHz |
| 3 MHz  f < 6 MHz | 3.05 MHz  f_offset < 6.05 MHz | -13.5 dBm | 100 kHz |
| 6 MHz  f  fmax | 6.5 MHz  f_offset < f_offsetmax | -13 dBm | 1MHz |

d-c) General operating band unwanted emission limits for 5, 10, 15 and 20 MHz 
channel bandwidth (E-UTRA bands >1 GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 1) |
| --- | --- | --- | --- |
| 0 MHz  f < 5 MHz | 0.05 MHz  f_offset < 5.05 MHz | −5.5dBm−75⋅foffsetMHz−0.05dB | 100 kHz |
| 5 MHz  f < 10 MHz | 5.05 MHz  f_offset < 10.05 MHz | -12.5 dBm | 100 kHz |
| 10 MHz  f  fmax | 10.5 MHz  f_offset < f_offsetmax | -13 dBm | 1MHz |

### 2.4.2	E-UTRA spectrum mask (Category B)

For E-UTRA BS operating in bands 33, 34, 35, 36, 37, 38,39 and 40, emissions shall not exceed the maximum levels specified in Table 18d:

TABLE 18d

d-a) General operating band unwanted emission limits for 1.4 MHz channel 
bandwidth (E-UTRA bands >1 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 1) |
| --- | --- | --- | --- |
| 0 MHz  f < 1.4 MHz | 0.05 MHz  f_offset < 1.45 MHz | +0.5dBm−101.4⋅foffsetMHz−0.05dB | 100 kHz |
| 1.4 MHz  f < 2.8 MHz | 1.45 MHz  f_offset < 2.85 MHz | -9.5 dBm | 100 kHz |
| 2.8 MHz  f  fmax | 3.3 MHz  f_offset < f_offsetmax | -15 dBm | 1MHz |

d-b) General operating band unwanted emission limits for 3 MHz channel 
bandwidth (E-UTRA bands >1 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 1) |
| --- | --- | --- | --- |
| 0 MHz  f < 3 MHz | 0.05 MHz  f_offset < 3.05 MHz | −3.5dBm−103⋅foffsetMHz−0.05dB | 100 kHz |
| 3 MHz  f < 6 MHz | 3.05 MHz  f_offset < 6.05 MHz | -13.5 dBm | 100 kHz |
| 6 MHz  f  fmax | 6.5 MHz  f_offset < f_offsetmax | -15 dBm | 1MHz |

d-c) General operating band unwanted emission limits for 5, 10, 15 and 20 MHz channel bandwidth (E-UTRA bands >1 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 1) |
| --- | --- | --- | --- |
| 0 MHz  f < 5 MHz | 0.05 MHz  f_offset < 5.05 MHz | −5.5dBm−75⋅foffsetMHz−0.05dB | 100 kHz |
| 5 MHz  f < 10 MHz | 5.05 MHz  f_offset < 10.05 MHz | -12.5 dBm | 100 kHz |
| 10 MHz  f  fmax | 10.5 MHz  f_offset < f_offsetmax | -15 dBm | 1MHz |

### 2.4.2.1	E-UTRA spectrum mask (additional limits)

The following requirements may apply in certain regions. For E-UTRA BS operating bands 35, and 36, emissions shall not exceed the maximum levels specified in Table 19d.

TABLE 19d

Additional operating band unwanted emission limits for E-UTRA bands>1 GHz

| Channel bandwidth | Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 1) |
| --- | --- | --- | --- | --- |
| 1.4 MHz | 0 MHz  f < 1 MHz | 0.005 MHz  f_offset < 0.995 MHz | -12.5 dBm | 10 kHz |
|  | 1 MHz  f < 2.8 MHz | 1.5 MHz  f_offset < 3.3 MHz | -11.5 dBm | 1 MHz |
|  | 2.8 MHz  f < fmax | 3.3 MHz  f_offset < f_offsetmax | -13 dBm |  |
| 3 MHz | 0 MHz  f < 1 MHz | 0.015 MHz  f_offset < 0.985 MHz | -11.5 dBm | 30 kHz |
|  | 1 MHz  f < 6 MHz | 1.5 MHz  f_offset < 6.5 MHz | -11.5 dBm | 1 MHz |
|  | 6 MHz  f < fmax | 6.5 MHz  f_offset < f_offsetmax | -13 dBm |  |
| 5 MHz | 0 MHz  f < 1 MHz | 0.015 MHz  f_offset < 0.985 MHz | -13.5 dBm | 30 kHz |
|  | 1 MHz  f < 10 MHz | 1.5 MHz  f_offset < 10.5 MHz | -11.5 dBm | 1 MHz |
|  | 10 MHz  f < fmax | 10.5 MHz  f_offset < f_offsetmax | -13 dBm |  |
| 10 MHz | 0 MHz  f < 1 MHz | 0.05 MHz  f_offset < 0.95 MHz | -11.5 dBm | 100 kHz |
|  | 1 MHz  f < 20 MHz | 1.5 MHz  f_offset < 20.5 MHz | -11.5 dBm | 1 MHz |
|  | 20 MHz  f < fmax | 20.5 MHz  f_offset < f_offsetmax | -13 dBm |  |
| 15 MHz | 0 MHz  f < 1 MHz | 0.05 MHz  f_offset < 0.95 MHz | -13.5 dBm | 100 kHz |
|  | 1 MHz  f < 30 MHz | 1.5 MHz  f_offset < 30.5 MHz | -11.5 dBm | 1 MHz |
|  | 30 MHz  f < fmax | 30.5 MHz  f_offset < f_offsetmax | -13 dBm |  |
| 20 MHz | 0 MHz  f < 1 MHz | 0.05 MHz  f_offset < 0.95 MHz | -14.5 dBm | 100 kHz |
|  | 1 MHz  f < 40 MHz | 1.5 MHz  f_offset < 40.5 MHz | -11.5 dBm | 1 MHz |
|  | 40 MHz  f < fmax | 40.5 MHz  f_offset < f_offsetmax | -13 dBm |  |

NOTE 1 – As a general rule for the requirements in section 2.4, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

# 3	ACLR

ACLR is the ratio of the transmitted power to the power measured after a receiver filter in the adjacent channel(s).

## 3.1	ACLR for UTRA

For UTRA, both the transmitted power and the received power are measured through a matched filter (root raised cosine and roll-off 0.22) with a noise power bandwidth equal to the chip rate. The requirements should apply whatever the type of transmitter considered (single carrier or multi-carrier). It applies for all transmission modes foreseen by the manufacturer’s specification.

The ACLR of a single carrier BS or a multi-carrier BS with contiguous carrier frequencies should be higher than the value specified in Tables 21a), 21b) and 21c).

TABLE 21

a)  BS ACLR limits for 3.84 Mchip/s TDD option

| BS adjacent channel offset below the first or above the last carrier frequency used (MHz) | ACLR limit (dB) |
| --- | --- |
| 5 | 44.2 |
| 10 | 54.2 |

b)  BS ACLR limits for 1.28 Mchip/s TDD option

| BS adjacent channel offset below the first or above the last carrier frequency used (MHz) | ACLR limit (dB) |
| --- | --- |
| 1.6 | 39.2 |
| 3.2 | 44.2 |

c)  BS ACLR limits for 7.68 Mchip/s TDD option

| BS adjacent channel offset below the first or above the last carrier frequency used (MHz) |  | ACLR limit (dB) |
| --- | --- | --- |
| 10.0 |  | 44.2 |
| 20.0 |  | 54.2 |

If a BS provides multiple non-contiguous single carriers or multiple non-contiguous groups of contiguous single carriers, the above requirements should be applied individually to the single carriers or group of single carriers.

## 3.2	ACLR for E-UTRA (LTE)

The ACLR is defined with a square filter of bandwidth equal to the transmission bandwidth configuration of the transmitted signal (BWConfig) centred on the assigned channel frequency and a filter centred on the adjacent channel frequency according to the tables below. The transmission bandwidth configuration is as specified in Table 21b.

TABLE 21b

Downlink transmission bandwidth configuration BWConfig

| Channel bandwidth BWChannel [MHz] | 1.4 | 3 | 5 | 10 | 15 | 20 |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission bandwidth configuration (BWConfig) [MHz] | 1.095 | 2.715 | 4.515 | 9.015 | 13.515 | 18.015 |

For Category A limits in the tables below or the absolute limit of -13 dBm/MHz apply, whichever is less stringent.

For Category B, either the ACLR limits in the tables below or the absolute limit of 
-15 dBm/MHz apply, whichever is less stringent.

For operation in unpaired spectrum, the ACLR shall be higher than the value specified in Table 21c.

TABLE 21c

Base Station ACLR in unpaired spectrum with synchronized operation

| E-UTRA transmitted signal channel bandwidth BWChannel [MHz] | BS adjacent channel centre frequency offset below the first or above the last carrier centre frequency used | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 1.4, 3.0 | BWChannel | E-UTRA of same BW | Square (BWConfig) | 44.2 dB |
|  | 2 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44.2 dB |
|  | BWChannel /2 + 0.8 MHz | 1.28 Mcps UTRA | RRC (1.28 Mcps) | 44.2 dB |
|  | BWChannel /2 + 2.4 MHz | 1.28 Mcps UTRA | RRC (1.28 Mcps) | 44.2 dB |
| 5, 10, 15, 20 | BWChannel | E-UTRA of same BW | Square (BWConfig) | 44.2 dB |
|  | 2 x BWChannel | E-UTRA of same BW | Square (BWConfig) | 44.2 dB |
|  | BWChannel /2 + 0.8 MHz | 1.28 Mcps UTRA | RRC (1.28 Mcps) | 44.2 dB |
|  | BWChannel /2 + 2.4 MHz | 1.28 Mcps UTRA | RRC (1.28 Mcps) | 44.2 dB |
|  | BWChannel /2 + 2.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
|  | BWChannel /2 + 7.5 MHz | 3.84 Mcps UTRA | RRC (3.84 Mcps) | 44.2 dB |
|  | BWChannel /2 + 5 MHz | 7.68 Mcps UTRA | RRC (7.68 Mcps) | 44.2 dB |
|  | BWChannel /2 + 15 MHz | 7.68 Mcps UTRA | RRC (7.68 Mcps) | 44.2 dB |
| NOTE 1 – BWChannel and BWConfig are the channel bandwidth and transmission bandwidth configuration of the E-UTRA transmitted signal on the assigned channel frequency. NOTE 2 – The RRC filter shall be equivalent to a root raised cosine fitler with roll-off 0.22, with a chip rate as defined in this table. |  |  |  |  |

# 4	Transmitter spurious emission (conducted)

The conducted spurious emissions are measured at the BS RF output port.

Unless otherwise stated, all requirements are measured as mean power.

The requirements should apply to BS intended for general-purpose applications.

The requirements should apply whatever the type of transmitter considered (single carrier or multi-carrier). It applies for all transmission modes foreseen by the manufacturer.

For the 3.84 Mchip/s UTRA TDD option, either requirement (except the case of coexistence with PHS) applies at frequencies within the specified frequency ranges which are more than 12.5 MHz under the first carrier frequency used or more than 12.5 MHz above the last carrier frequency used.

For the 1.28 Mchip/s UTRA TDD option, either requirement applies at frequencies within the specified frequency ranges which are more than 4 MHz under the first carrier frequency used or more than 4 MHz above the last carrier frequency used.

For the 7.68 Mchip/s UTRA TDD option, either requirement (except the case of coexistence with PHS) applies at frequencies within the specified frequency ranges which are more than 25 MHz under the first carrier frequency used or more than 25 MHz above the last carrier frequency used.

For E-UTRA (LTE), the requirement applies at frequencies within the specified frequency ranges, excluding the frequency range from 10 MHz below the lowest frequency of the BS transmitter operating band up to 10 MHz above the highest frequency of the BS transmitter operating band.

In areas where Category A limits for spurious emissions, as defined in Recommendation ITU-R SM.329, are applied, the power of any spurious emission should not exceed the maximum levels given in Table 22a.

TABLE 22a

BS Mandatory spurious emissions limits, Category A

| Band | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 9-150 kHz | 13 dBm | 1 kHz | Bandwidth as in Recommendation ITU-R SM.329, § 4.1 |
| 150 kHz-30 MHz |  | 10 kHz | Bandwidth as in Recommendation ITU-R SM.329, § 4.1 |
| 30 MHz-1 GHz |  | 100 kHz | Bandwidth as in Recommendation ITU-R SM.329, § 4.1 |
| 1-12.75 GHz |  | 1 MHz | Upper frequency as in Recommendation ITU-R SM.329, § 2.5, Table 1 |
| NOTE 1 – The requirements reported in the present table are applicable for the 3.84 Mchip/s, 1.28 Mchip/s, 7.68 Mchip/s and E-UTRA(LTE) TDD options. |  |  |  |

In areas where Category B limits for spurious emissions, as defined in Recommendation ITU-R SM.329, are applied, the power of any spurious emission should not exceed the maximum levels given in Tables 22b, 22c, 22d and 22e.

TABLE 22b

BS spurious emission limits for UTRA 3.84 Mchip/s option, Category B

| Band | Maximum level | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 9 ↔ 150 kHz | 36 dBm | 1 kHz | (1) |
| 150 kHz ↔ 30 MHz | 36 dBm | 10 kHz | (1) |
| 30 MHz ↔ 1 GHz | 36 dBm | 100 kHz | (1) |
| 1 GHz ↔ Fl – 10 MHz | 30 dBm | 1 MHz | (1) |
| Fl – 10 MHz ↔ Fu + 10 MHz | 15 dBm | 1 MHz | (2) |
| Fu + 10 MHz ↔ 12.5 GHz | 30 dBm | 1 MHz | (3) |

TABLE 22c

BS spurious emission limits for UTRA 1.28 Mchip/s option, Category B

| Band | Maximum level | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 9 ↔ 150 kHz | 36 dBm | 1 kHz | (1) |
| 150 kHz ↔ 30 MHz | 36 dBm | 10 kHz | (1) |
| 30 MHz ↔ 1 GHz | 36 dBm | 100 kHz | (1) |
| 1 GHz ↔ Flow – 10 MHz | 30 dBm | 1 MHz | (1) |
| Flow – 10 MHz  Fhigh+ 10 MHz | 15 dBm | 1 MHz | (2) |
| Fhigh+ 10 MHz « 12.75 GHz | 30 dBm | 1 MHz | (3) |

TABLE 22d

BS spurious emission limits for UTRA 7.68 Mchip/s option, Category B

| Band | Maximum level | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 9 ↔ 150 kHz | 36 dBm | 1 kHz | (1) |
| 150 kHz ↔ 30 MHz | 36 dBm | 10 kHz | (1) |
| 30 MHz ↔ 1 GHz | 36 dBm | 100 kHz | (1) |
| 1 GHz ↔ Flow – 10 MHz | 30 dBm | 1 MHz | (1) |
| Flow – 10 MHz  Fhigh+ 10 MHz | 15 dBm | 1 MHz | (2) |
| Fhigh+ 10 MHz « 12.75 GHz | 30 dBm | 1 MHz | (3) |
| (1) Bandwidth as in Recommendation ITU-R SM.329, § 4.1. (2) Specification in accordance with Recommendation ITU-R SM.329, § 4.3 and Annex 7. (3) Bandwidth as in ITU-R SM.329, §4.3 and Annex 7. Upper frequency as in Recommendation ITU-R SM.329, § 2.5, Table 1. Flow: The lowest downlink frequency of the operating band. Fhigh: The highest downlink frequency of the operating band. |  |  |  |

TABLE 22e

BS spurious emission limits for E-UTRA, Category B

| Band | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 9 « 150 kHz | 36 dBm | 1 kHz | (1) |
| 150 kHz « 30 MHz | 36 dBm | 10 kHz | (1) |
| 30 MHz « 1 GHz | 36 dBm | 100 kHz | (1) |
| 1 GHz « 12.75 GHz | 30 dBm | 1 MHz | (2) |
| (1) Bandwidth as in Recommendation ITU-R SM.329, § 4.1. (2) Bandwidth as in Recommendation ITU-R SM.329, § 4.1. Upper frequency as in Recommendation ITU-R SM.329, § 2.5 Table 1. |  |  |  |

4.1	Coexistence with GSM 900

This requirement may be applied for the protection of GSM 900 MS and GSM 900 BTS receivers in geographic areas in which both GSM 900 and UTRA are deployed.

TABLE 23a

BS spurious emissions limits for UTRA BS in geographic coverage area
of GSM 900 MS and GSM 900 BTS receivers

| Band | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 876-915 MHz | −61 dBm | 100 kHz |  |
| 921-960 MHz | −57 dBm | 100 kHz |  |

## 4.2	Coexistence with DCS 1800

This requirement may be applied for the protection of DCS 1800 MS and DCS 1800 BTS receivers in geographic areas in which both DCS 1800 and UTRA are deployed.

TABLE 23b

BS spurious emissions limits for UTRA BS in geographic coverage area 
of DCS 1800 MS and DCS 1800 BTS receivers

| Band | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 1 710-1 785 MHz | −61 dBm | 100 kHz |  |
| 1 805-1 880 MHz | −47 dBm | 100 kHz |  |

## 4.3	Coexistence with UTRA-FDD

This requirement may be applied to geographic areas in which both UTRA-TDD and UTRA-FDD operating in bands specified in Table 23c are deployed.

For TDD base stations which use carrier frequencies within the band 2 010-2 025 MHz the requirements apply at all frequencies within the specified frequency bands in Table 23c. For the 3.84 Mchip/s TDD option base stations which use a carrier frequency within the band 1 900-1 920 MHz, the requirement apply at frequencies within the specified frequency range which are more than 12.5 MHz above the last carrier used in the frequency band 1 900-1 920 MHz. For the 1.28 Mchip/s TDD option base stations which use carrier frequencies within the band 1 900-1 920 MHz, the requirement applies at frequencies within the specified frequency range which are more than 4 MHz above the last carrier used in the frequency band 1 900-1 920 MHz. For the 7.68 Mchip/s TDD option base stations which use a carrier frequency within the band 1 900-1 920 MHz, the requirement applies at frequencies within the specified frequency range which are more than 25 MHz above the last carrier used in the frequency band 1 900-1 920 MHz.

The power of any spurious emission should not exceed values reported in Table 23c.

TABLE 23c

BS spurious emissions limits for UTRA BS in geographic coverage area of UTRA-FDD

| BS class | Band | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| Wide area BS | 1 920-1 980 MHz | −43 dBm(1) | 3.84 MHz |  |
| Wide area BS | 2 110-2 170 MHz | −52 dBm | 1 MHz |  |
| Wide area BS | 2 500-2 570 MHz | −43 dBm(2) | 3.84 MHz |  |
| Wide area BS | 2 620-2 690 MHz | −52 dBm | 1 MHz |  |
| Wide area BS | 815-850 MHz | −43 dBm(3) | 3.84 MHz | Applicable in Japan |
| Wide area BS | 860-895 MHz | −52 dBm(3) | 1 MHz | Applicable in Japan |
| Wide area BS | 1 427.9 MHz-1 452.9 MHz | −43 dBm(4) | 3.84 MHz | Applicable in Japan |
| Wide area BS | 1 475.9 MHz-1 500.9 MHz | −52 dBm(4) | 1 MHz | Applicable in Japan |
| Wide area BS | 1 749.9-1 784.9 MHz | −43 dBm(3) | 3.84 MHz | Applicable in Japan |
| Wide area BS | 1 844.9-1 879.9 MHz | −52 dBm(3) | 1 MHz | Applicable in Japan |
| Local area BS | 1 920-1 980 MHz | −40 dBm(1) | 3.84 MHz |  |
| Local area BS | 2 110-2 170 MHz | −52 dBm | 1 MHz |  |
| Local area BS | 2 500-2 570 MHz | −40 dBm(2) | 3.84 MHz |  |
| Local area BS | 2 620-2 690 MHz | −52 dBm | 1 MHz |  |
| (1) For the 3.84 Mchip/s TDD option base stations which use carrier frequencies within the band 1 900-1 920 MHz, the requirement should be measured RRC filtered mean power with the lowest centre frequency of measurement at 1 922.6 MHz or 15 MHz above the highest TDD carrier used, whichever is higher. For the 1.28 Mchip/s TDD option base stations which use carrier frequencies within the band 1 900-1 920 MHz, the requirement should be measured RRC filtered mean power with the lowest centre frequency of measurement at 1 922.6 MHz or 6.6 MHz above the highest TDD carrier used, whichever is higher. For the 7.68 Mchip/s TDD option base stations which use carrier frequencies within the band 1 900-1 920 MHz, the requirement should be measured RRC filtered mean power with the lowest centre frequency of measurement at 1 922.6 MHz or 30 MHz above the highest TDD carrier used, whichever is higher. (2) For the 3.84 Mchip/s TDD option base stations which use carrier frequencies within the band 2 570-2 620 MHz, the requirement should be measured RRC filtered mean power with the highest centre frequency of measurement at 2 567.5 MHz or 15 MHz below the lowest TDD carrier used, whichever is lower. For the 1.28 Mchip/s TDD option base stations which use carrier frequencies within the band 2 570-2 620 MHz, the requirement should be measured RRC filtered mean power with the highest centre frequency of measurement at 2 567.5 MHz or 6.6 MHz below the lowest TDD carrier used, whichever is lower. For the 7.68 Mchip/s TDD option base stations which use carrier frequencies within the band 2 570-2 620 MHz, the requirement should be measured RRC filtered mean power with the highest centre frequency of measurement at 2 567.5 MHz or 30 MHz below the lowest TDD carrier used, whichever is lower. (3) This is applicable only in Japan for the 3.84 Mchip/s and 7.68 Mchip/s TDD options operating in 2 010-2 025 MHz. (4) This is applicable only to the 7.68 Mchip/s TDD option operating in 2 010-2 025 MHz. |  |  |  |  |

The requirements for wide area BS in Table 23c are based on a coupling loss of 67 dB between the TDD and FDD base stations. The requirements for local area BS in Table 23c are based on a coupling loss of 70 dB between TDD and FDD wide area base stations.

## 4.4	Coexistence with PHS

This requirement may be applied for the protection of PHS in geographic areas in which both PHS and UTRA TDD or E-UTRA TDD are deployed. For the 3.84 Mchip/s TDD option, this requirement is also applicable at specified frequencies falling between 12.5 MHz below the first carrier frequency used and 12.5 MHz above the last carrier frequency used. For the 7.68 Mchip/s TDD option, this requirement is also applicable at specified frequencies falling between 25 MHz below the first carrier frequency used and 25 MHz above the last carrier frequency used. For E-UTRA TDD, this requirement is also applicable at specified frequencies falling between 10 MHz below the lowest BS transmitter frequency of the operating band and 10 MHz above the highest BS transmitter frequency of the operating band.

The power of any spurious emission should not exceed values reported in Table 23d.

TABLE 23d

BS spurious emissions limits for UTRA BS 
(3.84 Mchip/s and 7.68 Mchip/s TDD options) and E-UTRA BS in geographic
coverage area of PHS

| Band | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 1 884.5-1 919.6 MHz | −41 dBm | 300 kHz | Applicable for transmission in 2 010-2 025 MHz |

# 5	Receiver spurious emission

The requirements apply to all BS with separate receive and transmit antenna ports. The test should be performed when both transmitter and receiver are on with the transmitter port terminated.

For BS equipped with only a single antenna connector for both transmitter and receiver, the requirements of transmitter spurious emissions should apply to this port, and this test need not be performed.

The requirements in this subclause should apply to BS intended for general-purpose applications.

The power of any spurious emission should not exceed the values given in the Tables below.

For E-UTRA, in addition to the requirements in Table 24f, the power of any spurious emission should not exceed the levels specified for Co-existence with other systems in the same geographical area in Clause 4.4.

## 5.1	3.84 Mchip/s UTRA TDD option

TABLE 24a

Receiver spurious emission requirements

| Band | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz-1 GHz | −57 dBm | 100 kHz |  |
| 1 GHz-1.9 GHz, 1.98 GHz-2.01 GHz and 2.025 GHz-2.5 GHz | −47 dBm | 1 MHz | With the exception of frequencies between 12.5 MHz below the first carrier frequency and 12.5 MHz above the last carrier frequency used by the BS |
| 1.9 GHz-1.98 GHz, 2.01 GHz-2.025 GHz and 2.5 GHz-2.62 GHz | −78 dBm | 3.84 MHz | With the exception of frequencies between 12.5 MHz below the first carrier frequency and 12.5 MHz above the last carrier frequency used by the BS |
| 2.62 GHz-12.75 GHz | −47 dBm | 1 MHz | With the exception of frequencies between 12.5 MHz below the first carrier frequency and 12.5 MHz above the last carrier frequency used by the BS |

TABLE 24b

Additional receiver spurious emission requirements

| Band | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 815 MHz-850 MHz 1 749.9 MHz-1 784.9 MHz | −78 dBm | 3.84 MHz | Applicable in Japan. With the exception of frequencies between 12.5 MHz below the first carrier frequency and 12.5 MHz above the last carrier frequency used by the BS |

## 5.2	1.28 Mchip/s UTRA TDD option

TABLE 24c

Receiver spurious emission requirements

| Band | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz-1 GHz | −57 dBm | 100 kHz |  |
| 1 GHz-1.9 GHz, 1.98 GHz-2.01 GHz and 2.025 GHz-2.3 GHz 2.4-2.50 GHz | −47 dBm | 1 MHz | With the exception of frequencies between 4 MHz below the first carrier frequency and 4 MHz above the last carrier frequency used by the BS |
| 1.9 GHz-1.98 GHz, 2.01 GHz-2.025 GHz 2.3 GHz to 2.4 GHz and 2.5 GHz-2.62 GHz | −83 dBm | 1.28 MHz | With the exception of frequencies between 4 MHz below the first carrier frequency and 4 MHz above the last carrier frequency used by the BS |
| 2.62 GHz-12.75 GHz | −47 dBm | 1 MHz | With the exception of frequencies between 4 MHz below the first carrier frequency and 4 MHz above the last carrier frequency used by the BS |

## 5.3	7.68 Mchip/s UTRA TDD option

TABLE 24d

Receiver spurious emission requirements

| Band | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz-1 GHz | −57 dBm | 100 kHz |  |
| 1 GHz-1.9 GHz, 1.98 GHz-2.01 GHz 2.025 GHz-2.5 GHz | −47 dBm | 1 MHz | With the exception of frequencies between 25 MHz below the first carrier frequency and 25 MHz above the last carrier frequency used by the BS |
| 1.9 GHz-1.98 GHz, 2.01 GHz-2.025 GHz 2.5 GHz-2.62 GHz | −75 dBm | 7.68 MHz | With the exception of frequencies between 25 MHz below the first carrier frequency and 25 MHz above the last carrier frequency used by the BS |
| 2.62 GHz-12.75 GHz | −47 dBm | 1 MHz | With the exception of frequencies between 25 MHz below the first carrier frequency and 25 MHz above the last carrier frequency used by the BS |

TABLE 24e

Additional receiver spurious emission requirements

| Band | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 815 MHz-850 MHz 1 427.9 MHz-1 452.9 MHz 1 749.9 MHz-1 784.9 MHz | −78 dBm | 3.84 MHz | Applicable in Japan. With the exception of frequencies between 25 MHz below the first carrier frequency and 25 MHz above the last carrier frequency used by the BS |

## 5.4	E-UTRA TDD option

TABLE 24f

Receiver spurious emission limits

| Band | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz-1 GHz | 57 dBm | 100 kHz |  |
| 1-12.75 GHz | 47 dBm | 1 MHz |  |
| NOTE – For E-UTRA, the frequency range between 2.5 * BWChannel below the first carrier frequency and 2.5 * BWChannel above the last carrier frequency transmitted by the BS, where BWChannel is the channel bandwidth, may be excluded from the requirement. However, frequencies that are more than 10 MHz below the lowest frequency of the BS transmitter operating band or more than 10 MHz above the highest frequency of the BS transmitter operating band shall not be excluded from the requirement. |  |  |  |

Annex 4 

TDMA Single-Carrier (UWC-136) mobile stations

Part A

Conformance requirements (30 kHz)

# 1	Spectrum mask

Spectrum noise suppression is the restraint of sideband energy outside the active transmit channel. This RF spectrum is the result of power ramping, modulation and all sources of noise. The spectrum is primarily the result of events that do not occur at the same time: digital modulation and power ramping (switching transients). The RF spectrum from these two events are specified separately.

Adjacent and first or second alternate channel power is that part of the mean power output of the transmitter resulting from the modulation and noise which falls within a specified passband centred on either of the adjacent or first or second alternate channels.

The emission power should not exceed the limits specified in Table 19.

TABLE  19

Adjacent and alternate channel power requirements

| OoB power arising from switching transients is the peak power of the spectrum, arising from the ramping-on and ramping-off of the transmitter, that fall within defined frequency bands outside the active transmit channel. Channel | Maximum level |
| --- | --- |
| In either adjacent channel, centred 30 kHz from the centre frequency | 26 dB below the mean output power |
| In either alternate channel, centred 60 kHz from the centre frequency | 45 dB below the mean output power |
| In either second alternate channel centred 90 kHz from the centre frequency | 45 dB below the mean output power or –13 dBm measured in 30 kHz bandwidth, whichever is the lower power |

The peak emission power should not exceed the limits specified in Table 20.

TABLE  20

Switching transients requirements

| 2 Transmitter spurious emissions (conducted) Channel | Maximum level |
| --- | --- |
| In either adjacent channel, centred 30 kHz from the centre frequency | 26 dB below the peak output power reference |
| In either alternate channel, centred 60 kHz from the centre frequency | 45 dB below the peak output power reference |
| In either second alternate channel centred 90 kHz from the centre frequency | 45 dB below the peak output power reference or –13 dBm measured in 30 kHz bandwidth, whichever is the lower power |

The power of any spurious emission should not exceed the limits specified in Table 21.

TABLE  21

MS spurious emission limits

| 2.1 Coexistence with services in adjacent frequency bands Band ( f ) (1) | Maximum level (dBm) | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 9 kHz  f  150 kHz | –36 | 1 kHz | (2) |
| 150 kHz  f  30 MHz | –36 | 10 kHz | (2) |
| 30 MHz  f  1 000 MHz | –36 | 100 kHz | (2) |
| 1 000 MHz  f  1 920 MHz | –30 | 1 MHz | (2) |
| 1 920 MHz  f  1 980 MHz | –30 | 30 kHz | (3) |
| 1 980 MHz  f  2 110 MHz | –30 | 1 MHz | (2) |
| 2 110 MHz  f  2 170 MHz | –70 | 30 kHz | (4) |
| 2 170 MHz  f  12.75 GHz | –30 | 1 MHz | (2) |
| (1) f is the frequency of the spurious emission. (2) In accordance with the applicable clauses of Recommendation ITU-R SM.329. (3) MS transmit band. (4) MS receive band. |  |  |  |

This requirement provides for the protection of receivers operating in bands adjacent to the MS transmit frequency band of 1 920 to 1 980 MHz: GSM 900, R-GSM and UTRA TDD.

NOTE 1 – UTRA FDD operates in the same frequency band as UWC-136.

The power of any spurious emission should not exceed the limits specified in Table 22.

TABLE  22

Additional spurious emissions requirements

| 3 Receiver spurious emissions (idle mode) Service | Frequency band | Measurement bandwidth (kHz) | Limit (dBm) |
| --- | --- | --- | --- |
| R-GSM | 921  f  925 MHz | 100 | –60 |
| R-GSM | 925  f  935 MHz | 100 | –67 |
| GSM 900/R-GSM | 935  f  960 MHz | 100 | –79 |
| DCS 1800 | 1 805  f  1 880 MHz | 100 | –71 |
| UTRA TDD | 1 900  f  1 920 MHz | 100 | –62 |
| UTRA TDD | 2 010  f  2 025 MHz | 100 | –62 |
| NOTE 1 – The measurements are made on frequencies which are integer multiples of 200 kHz. Up to five exceptions of up to –36 dBm are permitted in the GSM 900, DCS 1800 and UTRA bands, and up to three exceptions of up to –36 dBm are permitted in the GSM 400 bands. |  |  |  |

The power of any spurious emissions should not exceed the limits given in Table 23.

TABLE  23

General receiver spurious emission requirements

| Frequency band | Measurement bandwidth | Maximum level (dBm) | Note |
| --- | --- | --- | --- |
| 30 MHz  f  1 GHz | 100 kHz | –57 |  |
| 1 GHz  f  12.75 GHz | 1 MHz | – 47 | With the exception of the frequencies covered by the table below, for which additional receiver spurious emission requirements apply(1) |
| (1) (Editorial Note) – In TFES Harmonized Standard v1.0.2, no additional receiver spurious emission is specified; yet, it is expected that there will be a table added, in the same form as for the other technologies (see Tables 5, 12 and 18). |  |  |  |

Part B

Conformance requirements (200 kHz)

The 200 kHz channel provides packet data service and employs both 8-level phase shift keying (8-PSK) and Gaussian minimum shift keying (GMSK) modulations.

# 1	Spectrum mask

Output RF spectrum is the relationship between the frequency offset from the carrier and the power, measured in a specified bandwidth and time, produced by the MS due to the effects of modulation and power ramping.

The specifications contained in this subclause apply in frequency hopping as well as in non-frequency hopping modes.

Due to the bursty nature of the signal, the output RF spectrum results from two effects: the modulation process, and the power ramping up and down (switching transients).

–	The level of the output RF spectrum due to GMSK and 8-PSK modulations should be no more than that given in Tables 24 and 25.

–	The level of the output RF spectrum due to switching transients should be no more than that given in Table 26.

–	The power emitted should not exceed –71 dBm in frequency band 2 110-2 170 MHz.

# 2	Spectrum due to the modulation and wideband noise

The output RF modulation spectrum is specified in Tables 24 and 25. This specification applies for all RF channels supported by the equipment.

The specification applies to the entire relevant transmit band and up to 2 MHz either side.

The limits should be met under the following measurement conditions:

–	Zero frequency scan, filter bandwidth and video bandwidth of 30 kHz up to 1 800 kHz from the carrier and 100 kHz at 1 800 kHz and above from the carrier, with averaging done over 50% to 90% of the useful part of the transmitted bursts, excluding the midamble, and then averaged over at least 200 such burst measurements. Above 1 800 kHz from the carrier, only measurements centred on 200 kHz multiples are taken with averaging over 50 bursts.

–	When tests are done in frequency hopping mode, the averaging should include only bursts transmitted when the hopping carrier corresponds to the nominal carrier of the measurement. The limits then apply to the measurement results for any of the hopping frequencies.

The figures in Table 24, at the vertically listed power level (dBm) and at the horizontally listed frequency offset from the carrier (kHz), are then the maximum allowed level (dB) relative to a measurement in 30 kHz on the carrier.

NOTE 1 – This approach of specification has been chosen for convenience and speed of testing. It does however require careful interpretation if there is a need to convert figures in the following tables into spectral density values, in that only part of the power of the carrier is used as the relative reference, and in addition different measurement bandwidths are applied at different offsets from the carrier.

TABLE  24

Relative maximum level due to modulation

| The following exceptions should apply, using the same measurement conditions as specified above: Carrier power (dBm) | Frequency offset (kHz) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 100 | 200 | 250 | 400 | 600 1 200 | 1 200 1 800 | 1 800 6 000 | 6 000 |
| 33 | 0.5 | –30 | –33 | –60 | –60 | –60 | –68 | –76 |
| 32 | 0.5 | –30 | –33 | –60 | –60 | –60 | –67 | –75 |
| 30 | 0.5 | –30 | –33 | –60 | –60(1) | –60 | –65 | –73 |
| 28 | 0.5 | –30 | –33 | –60 | –60(1) | –60 | –63 | –71 |
| 26 | 0.5 | –30 | –33 | –60 | –60(1) | –60 | –61 | –69 |
| 24 | 0.5 | –30 | –33 | –60 | –60(1) | –60 | –59 | –67 |
| (1) For equipment supporting 8-PSK, the requirement for 8-PSK modulation is –54 dB. |  |  |  |  |  |  |  |  |

–	In the combined range of 600 kHz to 6 MHz above and below the carrier, in up to three bands of 200 kHz width centred on a frequency which is an integer multiple of 200 kHz, exception levels of up to –36 dBm are allowed.

–	Above 6 MHz offset from the carrier in up to 12 bands of 200 kHz width centred on a frequency which is an integer multiple of 200 kHz, exception levels of up to –36 dBm are allowed.

Using the same measurement conditions as specified above, if a requirement in Table 24 results in lower than the power limit given in Table 25, then the latter should be applied instead.

TABLE  25

Absolute maximum level due to modulation

| 3 Spectrum due to switching transients Frequency offset from the carrier (kHz) | Level (dBm) |
| --- | --- |
|  600 | –36 |
|  600,  1 800 | –56 |
| 1 800 | –51 |

These effects are also measured in the time domain and the specifications assume the following measurement conditions: zero frequency scan, filter bandwidth 30 kHz, peak hold, and video bandwidth 100 kHz. Table 26 specifies the limits.

TABLE  26

Maximum levels due to switching transients

| Carrier power level (dBm) | Maximum level measured at various frequency offsets |  |  |  |
| --- | --- | --- | --- | --- |
|  | 400 kHz | 600 kHz | 1 200 kHz | 1 800 kHz |
| 39 | –21 dBm | –26 dBm | –32 dBm | –36 dBm |
|  37 | –23 dBm | –26 dBm | –32 dBm | –36 dBm |

NOTE 1 – The relaxation for carrier power level 39 dBm is in line with the modulated spectra and thus causes negligible additional interference to an analogue system by an UWC-136 200 kHz signal.

NOTE 2 – The near-far dynamics with this specification has been estimated to be approximately 58 dB for MS operating at a power level of 8 W or 49 dB for MSs operating at a power level of 1 W. The near-far dynamics then gradually decreases by 2 dB per power level down to 32 dB for MSs operating in cells with a maximum allowed output power of 20 mW or 29 dB for MS operating at 10 mW.

NOTE 3 – The possible performance degradation due to switching transient leaking into the beginning or the end of a burst, was estimated and found to be acceptable with respect to the BER due to co-channel interference (C/I).

# 4	Transmitter spurious emissions (conducted)

The power of any spurious emission should not exceed the limits specified in Table 27.

TABLE  27

MS spurious emission limits

| Band ( f )(1) | Measurement bandwidth | Maximum level (dBm) | Note |
| --- | --- | --- | --- |
| 9 kHz  f  150 kHz | 1 kHz | –36 | (2) |
| 150 kHz  f  30 MHz | 10 kHz | –36 | (2) |
| 30 MHz  f  1 000 MHz | 100 kHz | –36 | (2) |
| 1 000 MHz  f  1 920 MHz | 1 MHz | –30 | (2) |
| 1 920 MHz  f  1 980 MHz | 100 kHz | –36 | (3) |
| 1 980 MHz  f  2 110 MHz | 1 MHz | –30 | (2) |
| 2 110 MHz  f  2 170 MHz | 100 kHz | –66 | (4) |
| 2 170 MHz  f  12.75 GHz | 1 MHz | –30 | (2) |
| (1) f is the frequency of the spurious emission. (2) In accordance with the applicable clauses of Recommendation ITU-R SM.329. (3) MS transmit band. (4) MS receive band. |  |  |  |

# 5	Coexistence with services in adjacent frequency bands

This requirement provides for the protection of receivers operating in bands adjacent to the MS transmit frequency band of 1 920 MHz to 1 980 MHz: GSM 900, R-GSM, UTRA TDD.

The power of any spurious emission should not exceed the limits specified in Table 28.

TABLE  28

Additional spurious emissions requirements

| 6 Receiver spurious emissions (idle mode) Service | Frequency band | Measurement bandwidth (kHz) | Minimum requirement (dBm) |
| --- | --- | --- | --- |
| R-GSM | 921  f  925 MHz | 100 | –60 |
| R-GSM | 925  f  935 MHz | 100 | –67 |
| GSM 900/R-GSM | 935  f  960 MHz | 100 | –79 |
| DCS 1800 | 1 805  f  1 880 MHz | 100 | –71 |
| UTRA TDD | 1 900  f  1 920 MHz | 100 | –62 |
| UTRA TDD | 2 010  f  2 025 MHz | 100 | –62 |
| NOTE 1 – The measurements are made on frequencies which are integer multiples of 200 kHz. Up to five exceptions of up to –36 dBm are permitted in the GSM 900, DCS 1800 and UTRA bands, and up to three exceptions of up to –36 dBm are permitted in the GSM 400 bands. |  |  |  |

The power of any spurious emissions should not exceed the limits given in Table 29.

TABLE  29

General receiver spurious emission requirements

| Frequency band | Measurement bandwidth | Maximum level (dBm) | Note |
| --- | --- | --- | --- |
| 30 MHz  f  1 GHz | 100 kHz | –57 |  |
| 1 GHz  f  12.75 GHz | 1 MHz | –47 | With the exception of the frequencies covered by the table below, for which additional receiver spurious emission requirements apply(1) |
| (1) (Editorial Note) – In TFES Harmonized Standard v1.0.2, no additional receiver spurious emission is specified; yet, it is expected that there will be a table added, in the same form as for the other technologies (see Tables 5, 12 and 18). |  |  |  |

Annex 5

FDMA/TDMA (digital enhanced cordless telecommunications
(DECT)) mobile stations

# 1	Spectrum mask

If the equipment under test (EUT) is equipped with antenna diversity, the EUT should have the diversity operation defeated for the following tests.

# 2	Emissions due to modulation

The unwanted emission(s) due to modulation is the power measured in any DECT RF channel other than the one in which the EUT is transmitting, integrated over a bandwidth of 1 MHz.

With transmissions on physical channel Ra (K, L, M, N) in successive frames, the power in physical channel Ra (K, L, Y, N) should be less than the values given in Table 30.

TABLE  30

Emissions modulation

| 3 Emissions due to transmitter transients Emissions on RF channel “Y ” | Measurement bandwidth | Maximum power level |
| --- | --- | --- |
| Y  M  1 | (1) | 160 mW (–8 dBm) |
| Y  M  2 | (1) | 1 mW (–30 dBm) |
| Y  M  3 | (1) | 80 nW (–41 dBm) |
| Y  any other DECT channel | (1) | 40 nW (–44 dBm)(2) |
| (1) The power in RF channel Y is defined by integration over a bandwidth of 1 MHz centred on the nominal centre frequency, FY, averaged over at least 60% but less than 80% of the physical packet, and starting before 25% of the physical packet has been transmitted but after the synchronization word. (2) For Y  “any other DECT channel”, the maximum power level should be less than 40 nW (–44 dBm) except for one instance of a 500 nW (–33 dBm) signal. |  |  |

The power level of all modulation products (including AM components due to the switching on or off of the modulated RF carrier) in a DECT RF channel as a result of a transmission on another DECT RF channel.

The power level of all modulation products (including AM products due to the switching on or off of a modulated RF carrier) arising from a transmission on RF channel M should, when measured using a peak hold technique, be less than the values given in Table 31.

TABLE  31

Emissions due to transmitter transients

| 4 Transmitter spurious emissions (conducted) Emissions on RF channel “Y ” | Measurement bandwidth | Maximum power level |
| --- | --- | --- |
| Y  M  1 | (1) | 250 mW (–6 dBm) |
| Y  M  2 | (1) | 40 mW (–14 dBm) |
| Y M  3 | (1) | 4 mW (–24 dBm) |
| Y  any other DECT channel | (1) | 1 mW (–30 dBm) |
| (1) The measurement bandwidth should be 100 kHz and the power should be integrated over a 1 MHz bandwidth centred on the DECT frequency, FY. |  |  |

## 4.1	Spurious emissions when allocated a transmit channel

The spurious emissions, when a radio end point has an allocated physical channel, should meet the requirements of Table 32. The requirements of Table 32 are only applicable for frequencies which are greater than 12.5 MHz away from the centre frequency, fc, of a carrier.

TABLE  32

Spurious emissions requirements

| Measurements should not be made for transmissions on the RF channel closest to the nearest band edge for frequency offsets of up to 2 MHz. Frequency | Minimum requirement/ Reference bandwidth |
| --- | --- |
| 30 MHz  f  1 000 MHz | –36 dBm/100 kHz |
| 1 GHz  f  12.75 GHz | –30 dBm/1 MHz |
| fc – 12.5 MHz  f  fc  12.5 MHz | Not defined |

# 5	Receiver spurious emissions (idle mode)

## 5.1	Spurious emissions when the EUT has no allocated transmit channel

The power level of any spurious emissions when the radio end point has no allocated transmit channel should not exceed the limits specified in Table 33.

TABLE  33

Receiver spurious emissions

| 5.2 In the DECT band Frequency band | Measurement bandwidth | Maximum level (dBm) | Note |
| --- | --- | --- | --- |
| 30 MHz  f  1 GHz | 100 kHz (1) | –57 |  |
| 1 GHz  f  12.75 GHz | 1 MHz (1) | –47 | With the exception of the frequencies within the DECT band, covered by Table 34 |
| (1) The power should be measured using a peak hold technique. |  |  |  |

The power level of any receiver spurious emissions within the DECT band should not exceed the limit in Table 34.

TABLE  34

Receiver spurious emissions within DECT band

| Frequency band (MHz) | Measurement bandwidth (MHz) | Maximum level (dBm) |
| --- | --- | --- |
| 1 900-1 920 2 010-2 025 | 1 | –57(1) |
| (1) The following exceptions are allowed: – in one 1 MHz band, the maximum allowable e.r.p. should be less than 20 nW; – in up to two bands of 30 kHz, the maximum e.r.p. should be less than 250 nW. |  |  |

Annex 6

IMT-2000 OFDMA TDD WMAN base stations

# 1	Introduction

This Annex identifies unwanted emission limits for IMT-2000 OFDMA TDD WMAN base stations.

# 2	Spectrum emission mask

## 2.1	Spectrum emission mask for equipment operating in the band 2 300-2 400 MHz

TBD

## 2.2	Spectrum emission mask for equipment operating in the band 2 500-2 690 MHz

The spectrum emission mask of base stations applies to frequency offsets between 2.5 MHz and 12.5 MHz away from the base station centre frequency for the 5 MHz carrier and between 5 MHz and 25 MHz away from the base station centre frequency for the 10 MHz carrier. f is defined as the frequency offset in MHz from the channel centre frequency.

TABLE 47

Spectrum emission mask for 5 MHz carrier

| Frequency offset from centre | Allowed emission level | Measurement bandwidth |
| --- | --- | --- |
| 2.5  f  3.5 MHz | 13 dBm | 50 kHz |
| 3.5  f < 12.5 MHz | 13 dBm | 1 MHz |

TABLE 48

Spectrum emission mask for 10 MHz carrier

| Frequency offset from centre | Allowed emission level | Measurement bandwidth |
| --- | --- | --- |
| 5  f  6 MHz | 13 dBm | 100 kHz |
| 6  f < 25 MHz | 13 dBm | 1 MHz |

TABLE 49

Adjacent channel leakage power – Japan

| Channel size | Measurement frequency range (MHz) | Allowed adjacent channel leakage power (dBm) |
| --- | --- | --- |
| 5 MHz | 2.6 < f < 7.4 | 7 |
| 10 MHz | 5.25 < f < 14.75 | 3 |

TABLE 50

Spectrum emission mask for 5 MHz carrier – Japan

| Frequency offset from centre | Allowed emission level | Measurement bandwidth |
| --- | --- | --- |
| 7.5 MHz  f < 12.25 | 151.4 × (f 7.5) dBm | 1 MHz |
| 12.25  f < 22.5 MHz | 22 dBm | 1 MHz |
| NOTE – The adjacent channel leakage power for the 5 MHz channel from 2.6 MHz to 7.4 MHz is shown in Table 49. |  |  |

TABLE 51

Spectrum emission mask for 10 MHz carrier – Japan

| Frequency offset from centre | Allowed emission level | Measurement bandwidth |
| --- | --- | --- |
| 15  f < 25 MHz | 22 dBm | 1 MHz |
| NOTE – The adjacent channel leakage power for the 10 MHz channel from 5.25 MHz to 14.75 MHz is shown in Table 49. |  |  |

## 2.3	Spectrum emission mask for equipment operating in the band 3 400-3 600 MHz

TBD

# 3	Transmitter spurious emissions (conducted)

## 3.1	Transmitter spurious emissions

IMT-2000 OFDMA TDD WMAN base stations comply with the limits recommended in Recommendation ITU-R SM.329-10. The limits shown in Tables 52 and 53 are only applicable for frequency offsets which are greater than 12.5 MHz away from the base station centre frequency for the 5 MHz carrier and greater than 25 MHz for the 10 MHz carrier. f is the frequency of the spurious domain emissions. fc is the base station centre frequency.

The emission levels in Table 52 should be met in areas where Category A limits for spurious emissions, as defined in Recommendation ITU-R SM.329-10, are applicable. The emission levels in Table 53 should be met in areas where Category B limits for spurious emissions, as defined in Recommendation ITU-R SM.329-10, are applicable.

TABLE 52

Base station spurious emission limit, Category A

| Band | Allowed emission level | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz-1 GHz | 13 dBm | 100 kHz | Bandwidth as in Recommendation ITU-R SM.329-10, § 4.1 |
| 1 GHz-13.45 GHz |  | 1 MHz | Upper frequency as in Recommendation ITU-R SM.329-10, § 2.5, Table 1 |

TABLE 53

Base station spurious emissions limit, Category B

| Band | Measurement bandwidth | Allowed emission level |
| --- | --- | --- |
| 30 MHz  f < 1 000 MHz | 100 kHz | –36 dBm |
| 1 GHz  f < 13.45 GHz | 30 kHz If 2.5 × BW <= ¦ fc − f ¦ < 10 × BW 300 kHz If 10 × BW <= ¦ fc − f ¦ < 12 × BW 1 MHz If 12 × BW <= ¦ fc − f ¦ | –30 dBm |

TABLE 54

Base station spurious emission limit, Japan

| Frequency bandwidth | Measurement bandwidth | Allowed emission level (dBm) |
| --- | --- | --- |
| 9 kHz  f < 150 kHz | 1 kHz | −13 |
| 150 kHz  f < 30 MHz | 10 kHz | −13 |
| 30 MHz  f < 1 000 MHz | 100 kHz | −13 |
| 1 000 MHz  f < 2 505 MHz | 1 MHz | −13 |
| 2 505 MHz  f < 2 535 MHz | 1 MHz | −42 |
| 2 535 MHz  f < 2 630 MHz | 1 MHz | −13(1) |
| 2 630 MHz  f < 2 634.75 MHz | 1 MHz | −15 − 7/5 × (f −2 629.75) |
| 2 634.75 MHz  f < 2 655 MHz | 1 MHz | −22 |
| 2 655 MHz  f | 1 MHz | −13 |
| (1) The allowed emission level for the frequency band between 2 535 MHz and 2 630 MHz shall be applied for the frequency range greater than 2.5 times the channel size from the centre frequency. |  |  |

## 3.2	Coexistence with other systems in the same geographical/service area

These requirements may be applied for the protection of UE, MS and/or BS operating in other frequency bands in the same geographical area. The requirements may apply in geographical/service areas as applicable in which both OFDMA-TDD-WMAN and a system operating in another frequency band than the OFDMA-TDD-WMAN operating band are deployed. The systems operating in the other frequency band may be GSM900, DCS1800, PCS1900, GSM850, PHS, UTRA-TDD (3.84 Mchip/s, 7.68 Mchip/s, 1.28 Mchip/s options) and UTRA-FDD.

The power of any spurious emission should not exceed the limits of Table 57 for a BS where requirements for coexistence with the system listed in the first column apply.

TABLE 55

BS spurious emission limits for OFDMA-TDD-WMAN BS in geographic coverage 
area of systems operating in other frequency bands

| System type operating in the same geographical area | Band for coexistence requirement | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| GSM900 | 921-960 MHz | −57 dBm | 100 kHz |  |
|  | 876-915 MHz | −61 dBm | 100 kHz |  |
| DCS1800 | 1 805-1 880 MHz | −47 dBm | 100 kHz |  |
|  | 1 710-1 785 MHz | −61 dBm | 100 kHz |  |
| PCS1900 | 1 930-1 990 MHz | −47 dBm | 100 kHz |  |
|  | 1 850-1 910 MHz | −61 dBm | 100 kHz |  |
| GSM850 | 869-894 MHz | −57 dBm | 100 kHz |  |
|  | 824-849 MHz | −61 dBm | 100 kHz |  |
| PHS | 1 884.5-1 919.6 MHz | 41 dBm | 300 kHz |  |
| FDD Band I | 2 110-2 170 MHz | −52 dBm | 1 MHz |  |
|  | 1 920-1 980 MHz | −49 dBm | 1 MHz |  |
| FDD Band II | 1 930-1 990 MHz | −52 dBm | 1 MHz |  |
|  | 1 850-1 910 MHz | −49 dBm | 1 MHz |  |
| FDD Band III | 1 805-1 880 MHz | −52 dBm | 1 MHz |  |
|  | 1 710-1 785 MHz | −49 dBm | 1 MHz |  |
| FDD Band IV | 2 110-2 155 MHz | −52 dBm | 1 MHz |  |
|  | 1 710-1 755 MHz | −49 dBm | 1 MHz |  |
| FDD Band V | 869-894 MHz | −52 dBm | 1 MHz |  |
|  | 824-849 MHz | −49 dBm | 1 MHz |  |

| System type operating in the same geographical area | Band for coexistence requirement | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| FDD Band VI | 860-895 MHz | −52 dBm | 1 MHz |  |
|  | 815-850 MHz | −49 dBm | 1 MHz |  |
| FDD Band VII | 2 620-2 690 MHz | −52 dBm | 1 MHz | This requirement does not apply to OFDMA TDD WMAN operating in Band VII |
|  | 2 500-2 570 MHz | −49 dBm | 1 MHz | This requirement does not apply to OFDMA TDD WMAN operating in Band VII |
| FDD Band VIII | 925-960 MHz | −52 dBm | 1 MHz |  |
|  | 880-915 MHz | −49 dBm | 1 MHz |  |
| FDD Band IX | 1 844.9-1 879.9 MHz | −52 dBm | 1 MHz |  |
|  | 1 749.9-1 784.9 MHz | −49 dBm | 1 MHz |  |
| FDD Band X | 2 110-2 170 MHz | −52 dBm | 1 MHz |  |
|  | 1 710-1 770 MHz | −49 dBm | 1 MHz |  |
| UTRA-TDD | 1 900-1 920 MHz | 52 dBm | 1 MHz |  |
|  | 2 010-2 025 MHz | 52 dBm | 1 MHz |  |
|  | 2 300-2 400 MHz | 52 dBm | 1 MHz | This requirement does not apply to OFDMA TDD WMAN operating in the band 2 300-2 400 MHz |
|  | 2 570-2 610 MHz | 52 dBm | 1 MHz | This requirement does not apply to OFDMA TDD WMAN operating in the band 2 500-2 690 MHz |
| NOTE 1 – The values in this table are considered as preliminary values only, and are subject to further study that could lead to a revision of this Recommendation. |  |  |  |  |

# 4	Receiver spurious emissions (conducted)

The receiver spurious emissions in Table 56 are applied in Japan.

TABLE 56

Receiver spurious emission requirements

| Frequency band | Total allowed emission level (dBm) |
| --- | --- |
| f < 1 GHz | –54 |
| 1 GHz  f | –47 |

# 5	Adjacent channel leakage ratio (ACLR)

## 5.1	ACLR values for equipment operating in the band 2 300-2 400 MHz

TBD

## 5.2	ACLR values for equipment operating in the band 2 500-2 690 MHz

Within this Annex, and in a similar manner to other annexes, the ACLR is defined as the ratio of the on-channel transmitted power to the power transmitted in adjacent channels as measured at the output of the receiver filter. In order to measure ACLR it is necessary to consider a measurement filter for the transmitted signal as well as a receiver measurement bandwidth for the adjacent channel (victim) system.

## Inter-system and intra-system scenarios

There are two specific coexistence requirements that must be considered; the intra-system and inter-system. In this section only the following scenarios are considered:

–	OFDMA TDD WMAN adjacent to OFDMA TDD WMAN within the same network;

–	OFDMA TDD WMAN adjacent to UTRA technologies, which might operate using FDD or unsynchronized TDD techniques. The ACLR in this case also takes into account the boundary coexistence conditions between an OFDMA TDD WMAN system and a UTRA system, which could happen in the case of deployments in adjacently assigned spectrum blocks.

In this text, only one inter-system scenario is discussed, that pertaining to UTRA. Two classes of ACLR figures are defined in this Annex to describe the two relevant scenarios as follows:

Intra-system scenario: A classification that identifies a level of minimum required ACLR performance generally appropriate for intra-system operation in contiguous channel assignments within the same network, i.e. OFDMA TDD WMAN adjacent to OFDMA TDD WMAN. In this Annex, intra-system ACLR is based on the following receiver bandwidths with the OFDMA TDD WMAN system operated on-channel and adjacent channel:

–	4.75 MHz for a 5 MHz channelized system, and

–	9.5 MHz for a 10 MHz channelized system.

UTRA scenario: A classification that identifies a level of minimum required ACLR performance appropriate for more demanding interoperator/coexistence scenarios at adjacent frequency block boundaries.

The following receiver bandwidths are assumed for the UTRA system:

–	3.84 MHz for a 5 MHz channelized system, and

–	7.68 MHz for a 10 MHz channelized system.

In each scenario, the passband of the receiver filter is centred on the first or second adjacent channel centre frequency. In the case where the adjacent system is OFDMA TDD WAN, both the transmitted power and the received power are measured with a rectangular filter. For adjacent UTRA systems the transmitted power is measured using a rectangular filter and the received power using a RRC filter with a roll-off factor of 0.22.

The ACLR values for the two relevant scenarios are provided in the following tables.

TABLE 57

a)  BS ACLR for 5 MHz channel bandwidth – intra-system scenario

| Adjacent channel centre frequency | Minimum required ACLR (dB) |
| --- | --- |
| BS channel centre frequency ± 5 MHz | 45 |
| BS channel centre frequency ± 10 MHz | 55 |

b)  BS ACLR for 5 MHz channel bandwidth – UTRA scenario

| Adjacent channel centre frequency | Minimum required ACLR (dB) |
| --- | --- |
| BS channel centre frequency ± 5 MHz | 53.5 |
| BS channel centre frequency ± 10 MHz | 66 |

c)  BS ACLR for 10 MHz channel bandwidth – intra-system scenario

| Adjacent channel centre frequency | Minimum required ACLR (dB) |
| --- | --- |
| BS channel centre frequency ± 10.0 MHz | 45 |
| BS channel centre frequency ± 20.0 MHz | 55 |

d)  BS ACLR for 10 MHz channel bandwidth –UTRA scenario

| Adjacent channel centre frequency | Minimum required ACLR (dB) |
| --- | --- |
| BS channel centre frequency ± 10.0 MHz | 53.5 |
| BS channel centre frequency ± 20.0 MHz | 66 |

Additional information may be provided in future revisions of this Recommendation.

NOTE 1 – Further study is necessary for other systems wherever applicable, as well as the relationship between ACLR and the emission mask.

## 5.3	ACLR of equipment operating in the frequency range 3 400-3 600 MHz

TBD

Appendix 1

Definitions and Abbreviations

Test tolerance

With reference to Recommendation ITU-R M.1545, “test tolerance” is the relaxation value referred to in recommends 2 of ITU-R M.1545, i.e. the difference between the core specification value and the test limit, evaluated applying the shared risk principle as per figures 2 and 3 of annex 1 of ITU-R M.1545. In case the core specification value is equal to the test limit (figure 3 of annex 1 of ITU-R M.1545) the “test tolerances” are equal to 0.

| Source: Revision 1 to Document 5D/TEMP/129 |  |
| --- | --- |
| Attachment 2 |  |
| WORKING DOCUMENT TOWARDS the DRAFT REVISion of RECOMMENDATION ITU-R M.1581-2* |  |
| Generic unwanted emission characteristics of mobile stations using the terrestrial radio interfaces of IMT-2000 |  |
| (Question ITU-R 229/8) |  |

(2002-2003-2007)

Scope

This Recommendation provides the generic unwanted emission characteristics of mobile stations using the terrestrial radio interfaces of IMT-2000, suitable for establishing the technical basis for global circulation of IMT-2000 terminals.”

The ITU Radiocommunication Assembly,

considering

a)	that unwanted emissions consist of both spurious and out-of-band (OoB) emissions according to No. 1.146 of the Radio Regulations (RR) and that spurious and OoB emissions are defined in RR Nos. 1.145 and 1.144, respectively;

b)	that limitation of the maximum permitted levels of unwanted emissions of IMT-2000 mobile stations (MSs) is necessary to protect other radio systems and services from interference and to enable coexistence between different technologies;

c)	that too stringent limits may lead to an increase in size or in complexity of IMT-2000 radio equipment;

d)	that every effort should be made to keep limits for unwanted emissions at the lowest possible values taking account of economic factors and technological limitations;

e)	that Recommendation ITU-R SM.329 relates to the effects, measurements and limits to be applied to spurious domain emissions;

f)	that the same spurious emission limits apply equally to MSs of all radio interfaces;

g)	that Recommendation ITU-R SM.1541 relating to OoB emission specifies generic limits in the OoB domain which generally constitute the least restrictive OoB emission limits and encourages the development of more specific limits for each system;

h)	that the levels of spurious emissions of IMT-2000 terminals shall comply with the limits specified in RR Appendix 3;

j)	that Recommendation ITU-R M.1579 establishes the technical basis for global circulation of IMT-2000 MSs;

k)	that one of the basic requirements of global circulation is that the MS does not cause harmful interference in any country where it is taken;

l)	that the harmonization of unwanted emission limits will facilitate global use and access to a global market;

m)	that additional work is needed in order to define unwanted emission limits for equipment operating in the bands identified for IMT-2000 at the World Radiocommunication Conference (Istanbul, 2000) (WRC-2000);

n)	that unwanted emission limits are dependent on the transmitter emission characteristics in addition to depending on services operating in other bands,

noting

a)	the work carried out by standardization bodies to define limits to protect other radio systems and services from interference and to enable coexistence between different technologies;

b)	that IMT-2000 mobile stations must comply with local, regional, and international regulations for out-of-band and spurious emissions relevant to their operations, wherever such regulations apply;

c)	with regard to Annex 6, IMT-2000 OFDMA TDD WMAN, additional urgent work, in particular on emission mask and ACLR, is needed to ensure geographical coexistence with other IMT-2000 radio interfaces,

recommends

1	that the unwanted emission characteristics of IMT-2000 MSs should be based on the limits contained in the technology specific Annexes 1 to 6 which correspond to the radio interface specifications described in § 5.1 to 5.6 of Recommendation ITU-R M.1457.

NOTE 1 – Except the cases stated in Note 2, Note 3, Note 4 and Note 5, the unwanted emission limits are only defined for mobile stations operating according to the following arrangement: frequency division duplex (FDD) uplink in the band 1 920-1 980 MHz, FDD downlink in the band 2 110-2 170 MHz and time division duplex (TDD) in the bands 1 885-1 980 and 2 010-2 025 MHz. Future versions of this Recommendation will include limits applicable to other frequency bands. Subject to further study, it is anticipated that such limits would be similar to those already contained in this Recommendation.

NOTE 2 – The unwanted emission limits defined in Annex 1 are for MS operating one of or a combination of the following arrangements:

–	Frequency division duplex (FDD) uplink in the band 1 920-1 980 MHz, FDD downlink in the band 2 110-2 170 MHz, in Annex 1 referred to as FDD Band I in UTRA or Band 1 in 
E-UTRA.

–	FDD uplink in the band 1 850-1 910 MHz, FDD downlink in the band 1 930-1 990 MHz, in Annex 1 referred to as FDD Band II in UTRA or Band 2 in E-UTRA.

–	FDD uplink in the band 1 710-1 785 MHz, FDD downlink in the band 1 805-1 880 MHz, in Annex 1 referred to as FDD Band III in UTRA or Band 3 in E-UTRA.

–	FDD uplink in the band 1 710-1 755 MHz, FDD downlink in the band 2 110-2 155 MHz, in Annex 1 referred to as FDD Band IV in UTRA or Band 4 in E-UTRA.

–	FDD uplink in the band 824-849 MHz, FDD downlink in the band 869-894 MHz, in Annex 1 referred to as FDD Band V in UTRA or Band 5 in E-UTRA.

–	FDD uplink in the band 830-840 MHz, FDD downlink in the band 875-885 MHz, in Annex 1 referred to as FDD Band VI in UTRA or Band 6 in E-UTRA.

–	FDD uplink in the band 2 500-2 570 MHz, FDD downlink in the band 2 620-2 690 MHz, in Annex 1 referred to as FDD Band VII in UTRA or Band 7 in E-UTRA.

–	FDD uplink in the band 880-915 MHz, FDD downlink in the band 925-960 MHz, in Annex 1 referred to as FDD Band VIII in UTRA or Band 8 in E-UTRA.

–	FDD uplink in the band 1 749.9-1 784.9 MHz, FDD downlink in the band 1 844.9-1 879.9 MHz, in Annex 1 referred to as FDD Band IX in UTRA or Band 9 in 
E-UTRA.

–	FDD uplink in the band 1 710-1 770 MHz, FDD downlink in the band 2 110-2 170 MHz, in Annex 1 referred to as FDD Band X in UTRA or Band 10 in E-UTRA.

–	FDD uplink in the band 1 427.9-1 452.9 MHz, FDD downlink in the band 1 475.9-1 500.9 MHz, in Annex 1 referred to as FDD Band XI in UTRA or Band 11 in 
E-UTRA.

–	FDD uplink in the band 698-716 MHz, FDD downlink in the band 728-746 MHz, 
in Annex 1 referred to as FDD Band XII in UTRA.

–	FDD uplink in the band 777-787 MHz, FDD downlink in the band 746-756 MHz, 
in Annex 1 referred to as FDD Band XIII in UTRA or Band 13 in E-UTRA.

–	FDD uplink in the band 788-798 MHz, FDD downlink in the band 758-768 MHz, 
in Annex 1 referred to as FDD Band XIV in UTRA or Band 14 in E-UTRA.

Future versions of this Recommendation will include limits applicable to other frequency bands. Subject to further study, it is anticipated that such limits would be similar to those already contained in this Recommendation.

NOTE 3 – The unwanted emission limits defined in Annex 2 are for MS operating in the following arrangements and apply to both cdma2000 and HRPD operating modes except as noted:

| Band Class | Name | MS Transmit Frequency (MHz) | BS Transmit Frequency (MHz) | Band Group |
| --- | --- | --- | --- | --- |
| 0 | 800 MHz band | 824-849 | 869-894 | 800 |
| 1 | 1 900 MHz band | 1 850-1 910 | 1 930-1 990 | 1 900 |
| 2 | TACS band | 872-915 | 917-960 | 800 |
| 3 | JTACS band | 887-925 | 832-870 | 800 |
| 4 | Korean PCS band | 1 750-1 780 | 1 840-1 870 | 1 900 |
| 5 | 450 MHz band | 411-484 | 421-494 | 450 |
| 6 | 2 GHz band | 1 920-1 980 | 2 110-2 170 | 1 900 |
| 7 | 700 MHz band | 776-794 | 746-764 | 800 |
| 8 | 1 800 MHz band | 1 710-1 785 | 1 805-1 880 | 1 900 |
| 9 | 900 MHz band | 880-915 | 925-960 | 800 |
| 10 | Secondary 800 MHz band | 806-901 | 851-940 | 800 |
| 11 | 400 MHz European PAMR band | 411-484 | 421-494 | 450 |
| 12 | 800 MHz PAMR band | 870-876 | 915-921 | 800 |
| 13 | 2.5 GHz IMT-2000 extension band | 2 500-2 570 | 2 620-2 690 | N/A |
| 14 | US PCS 1.9 GHz band | 1 850-1 915 | 1 930-1 995 | 1 900 |
| 15 | AWS band | 1 710-1 755 | 2 110-2 155 | 1 900 |
| 16* | US 2.5 GHz band | 2 502-2 568 | 2 624-2 690 | N/A |

*	No emissions specifications at this time.

NOTE 4 – The unwanted emission limits defined in Annex 3 are for MS operating one of, or a combination of, the following arrangements:

–	Time division duplex (TDD) in the band 1 900-1 920 MHz and 2 010-2 025 MHz referred to as Band b) in UTRA or Band 33 and 34, respectively, in E-UTRA.

–	TDD in the band 1 850-1 910 MHz and 1930-1990 MHz refered to as Band .b) in UTRA or Band 35 and 36, respectively, in E-UTRA.

–	TDD in the band 1 910-1 930 MHz refered to as Band c) in UTRA or Band 37 in E-UTRA TDD.

–	TDD in the band 2 570-2 620 MHz refered to as Band d) in UTRA or Band 38 in E-UTRA TDD.

–	TDD in the band 1880-1920MHz refered to as Band 39 in E-UTRA.

–	TDD in the band 2300-2400MHz refered to as Band e) in UTRA or Band 40 in E-UTRA.

Future versions of this Recommendation will include limits applicable to other frequency bands. Subject to further study, it is anticipated that such limits would be similar to those already contained in this Recommendation.

NOTE 5 – The out-of-band emission limits defined in Annex 6 are for MS operating in the following arrangement:

–	TDD in the band 2 300-2 400 MHz;

–	TDD in the band 2 500-2 690 MHz;

–	TDD in the band 3 400-3 600 MHz.

Annex 1:	IMT-2000 Code division multiple access (CDMA) Direct Spread mobile stations

Annex 2:	IMT-2000 CDMA Multi-Carrier mobile stations

Annex 3:	IMT-2000 CDMA TDD mobile stations

Annex 4:	IMT-2000 TDMA Single-Carrier mobile stations

Annex 5:	IMT-2000 FDMA/TDMA mobile stations

Annex 6: 	IMT-2000 OFDMA TDD WMAN mobile stations

Appendix 1 – Definitions and Abbreviations

Annex 1 

CDMA Direct Spread (universal terrestrial radio access
(UTRA) FDD) mobile stations

# 1	Measurement uncertainty

Values specified in this Annex differ from those specified in Recommendation ITU-R M.1457 since values in this Annex incorporate test tolerances defined in Recommendation ITU-R M.1545.

# 2	Spectrum mask

## 2.1	UTRA spectrum mask

The spectrum emission mask of the MS applies to frequencies, which are between 2.5 MHz and 12.5 MHz away from the MS centre carrier frequency. The out-of-channel emission is specified relative to the root raised cosine (RRC) filtered mean power of the UE carrier, where the RRC filtered mean power is the mean power measured through a root raised cosine filter with a roll-off factor of 0.22 and a bandwidth equal to the chip rate of 3.84 MHz. The power of any UE emission should not exceed the levels specified in Table 1.

The absolute requirement is based on a −48.5 dBm/3.84 MHz minimum power threshold for the UE. This limit is expressed for the narrower measurement bandwidths as −54.3 dBm/1 MHz and −69.6 dBm/30 kHz.

TABLE 1

Spectrum emission mask requirement (UTRA FDD MS)

| Δf in MHz (Note 1) | Minimum requirement (Note 2) |  | Additional requirements Band II, Band IV, Band V, and Band X (Note 3) | Measurement bandwidth (Note 6) |
| --- | --- | --- | --- | --- |
|  | Relative requirement | Absolute requirement (in measurement bandwidth) |  |  |
| 2.5-3.5 | −33.5−15⋅ΔfMHz−2.5 dBc | −69.6 dBm | −15 dBm | 30 kHz (Note 4) |
| 3.5-7.5 | −33.5−1⋅ΔfMHz−3.5 dBc | −54.3 dBm | −13 dBm (Note 7) | 1 MHz (Note 5) |
| 7.5-8.5 | −37.5−10⋅ΔfMHz−7.5 dBc | −54.3 dBm | −13 dBm (Note 7) | 1 MHz (Note 5) |
| 8.5-12.5 | −47.5 dBc | −54.3 dBm | −13 dBm (Note 7) | 1 MHz (Note 5) |
| NOTE 1 – f is the separation between the carrier frequency and the centre of the measurement bandwidth. NOTE 2 – The minimum requirement is calculated from the relative requirement or the absolute requirement, whichever is the higher power. NOTE 3 – For operation in Band II, IV, V, and Band X only, the minimum requirement is calculated in Note 2 or the additional requirement for band II, IV, V, X, whichever is the lower power. NOTE 4 – The first and last measurement positions with a 30 kHz filter at f equals 2.515 MHz and 3.485 MHz. NOTE 5 – The first and last measurement positions with a 1 MHz filter at f equals 4 MHz and 12 MHz. NOTE 6 – As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. NOTE 7 – For operation in Band V, the measurement bandwidth of this requirement shall be 100 kHz. |  |  |  |  |

## 2.2	E-UTRA spectrum mask

The output UE transmitter spectrum consists of the three components; the emission within the occupied bandwidth (channel bandwidth), the Out Of Band (OOB) emissions and the far out spurious emission domain (Figure 1).

Figure 1

The spectrum emission mask of the MS applies to frequencies (ΔfOOB) starting from the ± edges of the assigned E-UTRA channel bandwidth.  For frequencies greater than (ΔfOOB) as specified in Table 1a the spurious requirements in §4 are applicable.

### 2.2.1	E-UTRA spectrum mask

The power of any MS emission shall not exceed the levels specified in Table 1a for the specified channel bandwidths.

TABLE 1a

E-UTRA spectrum emission mask

| ΔfOOB (MHz) | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
|  0-1 |  |  | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
|  1-2.5 |  |  | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
|  2.5-2.8 |  |  | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
|  2.8-5 |  |  | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
|  5-6 |  |  | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
|  6-10 |  |  | -23.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
|  10-15 |  |  |  | -23.5 | -11.5 | -11.5 | 1 MHz |
|  15-20 |  |  |  |  | -23.5 | -11.5 | 1 MHz |
|  20-25 |  |  |  |  |  | -23.5 | 1 MHz |

NOTE – As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

Note- the values for 1.4 MHz and 3.0 MHz channel bandwidths will be addressed at a later stage

# 3	Adjacent channel leakage power ratio (ACLR)

## 3.1	UTRA ACLR

ACLR is the ratio of the RRC filtered mean power centred on the assigned channel frequency to the RRC filtered mean power centred on an adjacent channel frequency.

The limit for ACLR should be as specified in Table 2.

TABLE 2

MS ACLR limits

| 3.2 E-UTRA ACLR Power class | MS channel offset below the first or above the last carrier frequency used (MHz) | ACLR limit (dB) |
| --- | --- | --- |
| 3, 4 | 5 | 32.2 |
| 3, 4 | 10 | 42.2 |

ACLR is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. The ACLR requirements are specified for two scenarios: for (an) adjacent E -UTRA and/or UTRA channels (Figure 2).

Figure 2

### 3.2.1	Limits for E-UTRA

E-UTRA Adjacent Channel Leakage power Ratio (E-UTRAACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.  The E-UTRA on channel and adjacent channel power is measured with a rectangular measurement bandwidth filter. The limits should be as specified in Table 2a.

TABLE 2a

General requirements for E-UTRAACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB |
| E-UTRA channel Measurement bandwidth |  |  | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UE channel |  |  | +5MHz or -5MHz | +10MHz or -10MHz | +15MHz or -15MHz | +20MHz or -20MHz |

### 3.2.2	Limits for E-UTRA for UTRA co-existence in the same geographical area

For adjacent UTRA carriers the limits should be as specified in Table 2b.

UTRA Adjacent Channel Leakage power Ratio (UTRAACLR) is the ratio of the filtered mean power centred on the assigned E-UTRA channel frequency to the filtered mean power centred on an adjacent(s) UTRA channel frequency.

UTRA Adjacent Channel Leakage power Ratio is specified for both the first UTRA 5 MHz adjacent channel (UTRAACLR1) and the 2nd UTRA 5MHz adjacent channel (UTRAACLR2) .The UTRA channel is measured with a 3.84 MHz RRC bandwidth filter with roll-off factor =0.22. The E-UTRA channel is measured with a rectangular measurement bandwidth filter.

TABLE 2b

Additional requirements

|  | Channel bandwidth / UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB |
| UTRAACLR2 | - | - | 35.2 dB | 35.2 dB | 35.2 dB | 35.2 dB |
| E-UTRA channel Measurement bandwidth |  | - | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA channel Measurement bandwidth | - | - | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UE channel for UTRA ACLR1 | +2.5MHz from upper band edge or -2.5MHz from lower band edge |  |  |  |  |  |
| UE channel for UTRA ACLR2 | +7.5MHz from upper band edge or -7.5MHz from lower band edge |  |  |  |  |  |

### 3.2.3	Additional ACLR limits

Additional ACLR requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

The additional ACLR limits are specified for the 2nd UTRA 5MHz adjacent channel (UTRAACLR2). The UTRA channel is measured with a 3.84 MHz RRC bandwidth filter with roll-off factor =0.22. The E-UTRA channel is measured with a rectangular measurement bandwidth filter.

TABLE 2c

Additional requirements (UTRAACLR2)

|  | Channel bandwidth / UTRAACLR2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR2bis | - | - | 42.2 dB | 42.2 dB | - | - |
| E-UTRA channel Measurement bandwidth | - | - | 4.5 MHz | 9.0 MHz | - | - |
| UTRA channel Measurement bandwidth | - | - | 3.84 MHz | 3.84 MHz | - | - |
| UE channel for UTRAACLR2bis | +7.5MHz from upper band edge or -7.5MHz from lower band edge |  |  |  |  |  |

# 4	Transmitter spurious emissions (conducted)

## 4.1	Transmitter spurious emissions for UTRA

For UTRA, the limits shown in Tables 3 and 4 are only applicable for frequencies which are greater than 12.5 MHz away from the MS centre carrier frequency.

TABLE  3

General spurious emissions requirements

| Frequency bandwidth | Measurement bandwidth | Minimum requirement (dBm) |
| --- | --- | --- |
| 9 kHz  f  150 kHz | 1 kHz | –36 |
| 150 kHz  f  30 MHz | 10 kHz | –36 |
| 30 MHz  f  1 000 MHz | 100 kHz | –36 |
| 1 GHz  f  12.75 GHz | 1 MHz | –30 |

TABLE 4

Additional spurious emissions requirements for UTRA

| Operating Band | Frequency bandwidth | Measurement bandwidth | Minimum requirement |
| --- | --- | --- | --- |
| I | 860 MHz  f  895 MHz | 3.84 MHz | −60 dBm |
|  | 921 MHz  f < 925 MHz | 100 kHz | −60 dBm (see Note 1) |
|  | 925 MHz  f  935 MHz | 100 kHz 3.84 MHz | −67 dBm (see Note 1) −60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz | −79 dBm (see Note 1) |
|  | 1 475.9 MHz  f  1 500.9 MHz | 3.84 MHz | -60 dBm |
|  | 1 805 MHz  f  1 880 MHz | 100 kHz | −71 dBm (see Note 1) |
|  | 1 844.9 MHz  f  1 879.9 MHz | 3.84 MHz | −60 dBm |
|  | 1 884.5 MHz < f < 1 919.6 MHz | 300 kHz | −41 dBm |
|  | 2 110 MHz  f  2 170 MHz | 3.84 MHz | −60 dBm |
|  | 2 620 MHz  f  2 690 MHz | 3.84 MHz | −60 dBm |
| II | 869 MHz  f  894 MHz | 3.84 MHz | −60 dBm |
|  | 1 930 MHz  f  1 990 MHz | 3.84 MHz | −60 dBm |
|  | 2 110 MHz  f  2 170 MHz | 3.84 MHz | −60 dBm |

TABLE 4 (cont’)

| Operating Band | Frequency bandwidth | Measurement bandwidth | Minimum requirement |
| --- | --- | --- | --- |
| III | 921 MHz  f < 925 MHz | 100 kHz | −60 dBm (see Note 1) |
|  | 925 MHz  f  935 MHz | 100 kHz 3.84 MHz | −67 dBm (see Note 1) −60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz | −79 dBm (see Note 1) |
|  | 1 805 MHz  f  1 880 MHz | 3.84 MHz | −60 dBm |
|  | 2 110 MHz  f  2 170 MHz | 3.84 MHz | −60 dBm |
|  | 2 620 MHz  f  2 690 MHz | 3.84 MHz | −60 dBm |
| IV | 869 MHz  f  894 MHz | 3.84 MHz | −60 dBm |
|  | 1 930 MHz  f  1 990 MHz | 3.84 MHz | −60 dBm |
|  | 2 110 MHz  f  2 170 MHz | 3.84 MHz | −60 dBm |
| V | 869 MHz  f  894 MHz | 3.84 MHz | −60 dBm |
|  | 1 930 MHz  f  1 990 MHz | 3.84 MHz | −60 dBm |
|  | 2 110 MHz  f  2 170 MHz | 3.84 MHz | −60 dBm |
| VI | 860 MHz  f < 875 MHz | 1 MHz | −37 dBm |
|  | 875 MHz  f  895 MHz | 3.84 MHz | −60 dBm |
|  | 1 475.9 MHz  f  1 500.9 MHz | 3.84 MHz | -60 dBm |
|  | 1 844.9 MHz  f  1 879.9 MHz | 3.84 MHz | −60 dBm |
|  | 1 884.5 MHz  f  1 919.6 MHz | 300 kHz | −41 dBm |
|  | 2 110 MHz  f  2 170 MHz | 3.84 MHz | −60 dBm |
| VII | 921 MHz  f < 925 MHz | 100 kHz | −60 dBm (see Note 1) |
|  | 925 MHz  f  935 MHz | 100 kHz 3.84 MHz | −67 dBm (see Note 1) −60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz | −79 dBm (see Note 1) |
|  | 1 805 MHz  f  1 880 MHz | 100 kHz | −71 dBm (see Note 1) |
|  | 2 110 MHz  f  2 170 MHz | 3.84 MHz | −60 dBm |
|  | 2 620 MHz  f  2 690 MHz | 3.84 MHz | −60 dBm |
|  | 2 590 MHz  f  2 620 MHz | 3.84 MHz | −50 dBm |
| VIII | 925 MHz  f  935 MHz | 100 kHz 3.84 MHz | −67 dBm (see Note 1) −60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz 3.84 MHz | −79 dBm (see Note 1) −60 dBm |
|  | 1 805 MHz < f  1 830 MHz | 100 kHz 3.84 MHz | −71 dBm (see Notes 1 and 2) −60 dBm (see Note 2) |
|  | 1 830 MHz < f  1 880 MHz | 100 kHz 3.84 MHz | −71 dBm (see Note 1) −60 dBm |
|  | 2 110 MHz  f  2 170 MHz | 3.84 MHz | −60 dBm |
|  | 2 620 MHz  f  2 640 MHz | 3.84 MHz | −60 dBm |
|  | 2 640 MHz < f  2 690 MHz | 3.84 MHz | −60 dBm (see Note 2) |

TABLE 4 (end)

| Operating Band | Frequency bandwidth | Measurement bandwidth | Minimum requirement |
| --- | --- | --- | --- |
| IX | 860 MHz  f  895 MHz | 3.84 MHz | −60 dBm |
|  | 1 475.9 MHz  f  1 500.9 MHz | 3.84 MHz | -60 dBm |
|  | 1 844.9 MHz  f  1 879.9 MHz | 3.84 MHz | −60 dBm |
|  | 1 884.5 MHz  f  1 919.6 MHz | 300 kHz | −41 dBm |
|  | 2 110 MHz  f  2 170 MHz | 3.84 MHz | −60 dBm |
| X | 869 MHz  f  894 MHz | 3.84 MHz | −60 dBm |
|  | 1 930 MHz  f  1 990 MHz | 3.84 MHz | −60 dBm |
|  | 2 110 MHz  f  2 170 MHz | 3.84 MHz | −60 dBm |
| XI | 860 MHz  f  895 MHz | 3.84 MHz | -60 dBm |
|  | 1 475.9 MHz  f  1 500.9 MHz | 3.84 MHz | -60 dBm |
|  | 1 844.9 MHz  f  1 879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1 884.5 MHz  f  1 919.6 MHz | 300 kHz | -41 dBm |
|  | 2 110 MHz  f  2 170 MHz | 3.84 MHz | -60 dBm |
| NOTE 1 – The measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, up to five measurements with a level up to the applicable requirements defined in Table 3 are permitted for each channel used in the measurement. NOTE 2 – The measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, measurements with a level up to the applicable requirements defined in Table 3 are permitted for each channel used in the measurement due to second or third harmonic spurious emissions. |  |  |  |

## 4.2	Transmitter spurious emissions for E-UTRA

For E-UTRA, the spurious emission limits apply for frequency ranges that are more than ΔfOOB (MHz) from the edge of the channel bandwidth (Table 3a).

TABLE 3a

Boundary between E-UTRA ΔfOOB and spurious emission domain

| Channel bandwidth | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) |  |  | 10 | 15 | 20 | 25 |

Note- the values for 1.4 MHz and 3.0 MHz channel bandwidths will be addressed at a later stage

The spurious emission limits in Table 3b apply for all E-UTRA transmitter band configurations and channel bandwidths.

TABLE 3b

Spurious emissions limits

| Table 4b specifies the requirements for the specified E-UTRA band Frequency range | Measurement bandwidth | Maximum level |
| --- | --- | --- |
| 9 kHz  f  150 kHz | 1 kHz | –36 dBm |
| 150 kHz  f  30 MHz | 10 kHz | –36 dBm |
| 30 MHz  f  1 000 MHz | 100 kHz | –36 dBm |
| 1 GHz  f  12.75 GHz | 1 MHz | –30 dBm |

TABLE 4b

Spurious emissions requirements for UE co-existence in E-UTRA bands

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Level (dBm) | Bandwidth (MHz) | Comment |
| 1 | E-UTRA Band 1, 3, 7, 8, 9, 11, 34, 38, 40 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 |  |
|  | E-UTRA band 33 | 1900 | - | 1920 | -50 | 1 | Note 3 |
|  | E-UTRA band 39 | 1880 | - | 1920 | -50 | 1 | Note 3 |
| 2 | E-UTRA Band 2, 4, 5, 10, 13, 14 | FDL_low | - | FDL_high | -50 | 1 |  |
| 3 | E-UTRA Band 1, 3, 7, 8, 9, 11, 33, 34, 38 | FDL_low | - | FDL_high | -50 | 1 |  |
| 4 | E-UTRA Band 2, 4, 5, 10, 13, 14 | FDL_low | - | FDL_high | -50 | 1 |  |
| 5 | E-UTRA Band 2, 4, 5, 10, 13, 14 | FDL_low | - | FDL_high | -50 | 1 |  |
| 6 | E-UTRA Band 1, 9, 11, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 875 | -37 | 1 |  |
|  | Frequency range | 875 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 |  |
| 7 | E-UTRA Band 1, 3, 7, 8, 33, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 38 | 2570 | - | 2620 | -50 | 1 | Note 3 |
| 8 | E-UTRA Band 1, 8, 7, 33, 34, 38, 39, 40 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | 1805 | - | 1830 | -50 | 1 | Note 4 |
|  | E-UTRA band 3 | 1805 | - | 1880 | -36 | 0.1 | Note 2,4 |
|  | E-UTRA band 3 | 1830 | - | 1880 | -50 | 1 | Note 4 |
|  | E-UTRA band 7 | 2640 | - | 2690 | -50 | 1 | Note 4 |
|  | E-UTRA band 7 | 2640 | - | 2690 | -36 | 0.1 | Note 2,4 |
| 9 | E-UTRA Band 1, 9, 11, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 |  |
| 10 | E-UTRA Band 2, 4, 5, 10, 13, 14 | FDL_low | - | FDL_high | -50 | 1 |  |
| 11 | E-UTRA Band 1, 9, 11, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 |  |
| 13 | E-UTRA Band 2, 4, 5, 10, 13, 14 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 763 | - | 775 | -35 | 0.00625 |  |
| 14 | E-UTRA Band 2, 4, 5, 10, 13, 14 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 763 | - | 775 | -35 | 0.00625 |  |
| Note |  |  |  |  |  |  |  |
| 1 | FDL_low and FDL_high refer to each E-UTRA frequency band specified in Note 2 of the Scope |  |  |  |  |  |  |
| 2 | A number of exceptions are permitted and is FFS. These exceptions include both spurious due to LO mixing and I/Q imbalance for specific values of NRB. For these exceptions the requirements of Table 3b are applicable. |  |  |  |  |  |  |
| 3 | To meet these requirements some restriction will be needed for either the operating band or protected band |  |  |  |  |  |  |
| 4 | Requirements are specified in terms of E-UTRA sub-bands |  |  |  |  |  |  |
| 5 | For non synchronised TDD operation to meet these requirements some restriction will be needed for either the operating band or protected band |  |  |  |  |  |  |

# 5	Receiver spurious emissions (conducted)

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector.

## 5.1	Receiver spurious emissions for UTRA

For UTRA, the power of any narrow-band continuous wave (CW) spurious emission should not exceed the maximum level specified in Tables 5 and 6.

TABLE 5

General receiver spurious emission requirements

| Frequency band | Measurement bandwidth | Maximum level | Note |
| --- | --- | --- | --- |
| 30 MHz  f  1 GHz | 100 kHz | −57 (dBm) |  |
| 1 GHz  f  12.75 GHz | 1 MHz | −47 (dBm) |  |

For UTRA the following additional spurious emission limits are applicable

TABLE 6

Additional receiver spurious emission requirements

| Band | Frequency band | Measurement bandwidth | Maximum level | Note |
| --- | --- | --- | --- | --- |
| I |  |  |  |  |
|  | 1 475.9 MHz  f  1 500.9 MHz | 3.84 MHz | −60 dBm |  |
|  | 1 844.9 MHz £ f  1 879.9 MHz | 3.84 MHz | −60 dBm |  |
|  | 1 920 MHz  f  1 980 MHz | 3.84 MHz | −60 dBm | UE transmit band |
|  | 2 110 MHz  f  2 170 MHz | 3.84 MHz | −60 dBm | UE receive band |
| II |  |  |  |  |
|  | 1 850 MHz  f  1 910 MHz | 3.84 MHz | −60 dBm | UE transmit band |
|  | 1 930 MHz  f  1 990 MHz | 3.84 MHz | −60 dBm | UE receive band |

TABLE 6 (cont’)

Additional receiver spurious emission requirements

| Band | Frequency band | Measurement bandwidth | Maximum level | Note |
| --- | --- | --- | --- | --- |
| III |  |  |  |  |
|  | 1 710 MHz  f  1 785 MHz | 3.84 MHz | −60 dBm | UE transmit band |
|  | 1 805 MHz  f  1 880 MHz | 3.84 MHz | −60 dBm | UE receive band |
| IV | 869 MHz  f < 894 MHz | 3.84 MHz | −60 dBm |  |
|  | 1 710 MHz  f < 1 755 MHz | 3.84 MHz | −60 dBm | UE transmit band in URA_PCH, Cell_PCH and idle state |
|  | 1 930 MHz  f  1 990 MHz | 3.84 MHz | −60 dBm |  |
|  | 2 110 MHz f  2 170 MHz | 3.84 MHz | −60 dBm | UE receive band (see note 1) |
|  | 2 110 MHz f  2 170 MHz | 3.84 MHz | −60 dBm | UE receive band (see note 2) |
| V | 824 MHz  f  849 MHz | 3.84 MHz | −60 dBm | UE transmit band |
|  | 869 MHz  f < 894 MHz | 3.84 MHz | −60 dBm | UE receive band |
| VI | 815 MHz  f  850 MHz | 3.84 MHz | −60 dBm |  |
|  | 860 MHz  f  895 MHz | 3.84 MHz | −60 dBm |  |
|  | 1 475.9 MHz  f  1 500.9 MHz | 3.84 MHz | −60 dBm |  |
|  | 1 844.9 MHz £ f  1 879.9 MHz | 3.84 MHz | −60 dBm |  |
|  | 2 110 MHz  f  2 170 MHz | 3.84 MHz | −60 dBm |  |
| VII | 921 MHz  f < 925 MHz | 100 kHz | −60 dBm * |  |
|  | 925 MHz  f  935 MHz | 100 kHz −3.84 MHz | −67 dBm* −60 dBm |  |
|  | 935 MHz < f  960 MHz | 100 kHz | −79 dBm* |  |
|  | 1 805 MHz  f  1 880 MHz | 100 kHz | −71 dBm* |  |
|  | 2 110 MHz  f  2 170 MHz | 3.84 MHz | −60 dBm |  |
|  | 2 500 MHz  f  2 570 MHz | 3.84 MHz | −60 dBm | UE transmit band |
|  | 2 620 MHz  f  2 690 MHz | 3.84 MHz | −60 dBm | UE receive band |

TABLE 6 (end)

| Band | Frequency band | Measurement bandwidth | Maximum level | Note |
| --- | --- | --- | --- | --- |
| VIII | 880 MHz  f 915 MHz | 3.84 MHz | −60 dBm |  |
|  | 921 MHz  f < 925 MHz | 100 kHz | −60 dBm* |  |
|  | 925 MHz  f  MHz | 100 kHz 3.84 MHz | −67 dBm* −60 dBm |  |
|  | 935 MHz < f  960 MHz | 100 kHz | −79 dBm* |  |
|  | 1 805 MHz < f  1 880 MHz | 3.84 MHz | −60 dBm |  |
|  | 2 110 MHz  f  2 170 MHz | 3.84 MHz | −60 dBm |  |
|  | 2 620 MHz  f  2 690 MHz | 3.84 MHz | −60 dBm |  |
| IX | 860 MHz £ f  895 MHz | 3.84 MHz | −60 dBm |  |
|  | 1 475.9 MHz  f  1 500.9 MHz | 3.84 MHz | −60 dBm |  |
|  | 1 749.9 MHz £ f £ 1 784.9 MHz | 3.84 MHz | −60 dBm | UE transmit band |
|  | 1 844.9 MHz £ f  1 879.9 MHz | 3.84 MHz | −60 dBm | UE receive band |
|  | 2 110 MHz £ f £ 2 170 MHz | 3.84 MHz | −60 dBm |  |
| X | 869 MHz  f < 894 MHz | 3.84 MHz | −60 dBm |  |
|  | 1 710 MHz  f < 1 770 MHz | 3.84 MHz | −60 dBm | UE transmit band |
|  | 1 930 MHz  f  1 990 MHz | 3.84 MHz | −60 dBm |  |
|  | 2 110 MHz  f  2 170 MHz | 3.84 MHz | −60 dBm | UE receive band |
| XI | 860 MHz £ f  895 MHz | 3.84 MHz | −60 dBm |  |
|  | 1 427.9 MHz  f  1 452.9 MHz | 3.84 MHz | −60 dBm | UE transmit band |
|  | 1 475.9 MHz  f  1 500.9 MHz | 3.84 MHz | −60 dBm | UE receive band |
|  | 1 844.9 MHz £ f  1 879.9 MHz | 3.84 MHz | −60 dBm |  |
|  | 2 110 MHz £ f £ 2 170 MHz | 3.84 MHz | −60 dBm |  |
| * The measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, up to five measurements with a level up to the applicable requirements defined in Table 5 are permitted for each channel used in the measurement. NOTE 1 – For UEs that conform to Release 6 and support Band IV shall support the defined frequency bandwidth. NOTE 2 – For UEs that conform to Release 7 and later releases and support Band IV shall support the defined frequency bandwidth. |  |  |  |  |

## 5.2	Receiver spurious emissions for E-UTRA

The power of any narrow band CW spurious emission shall not exceed the maximum level specified in Table 7.

TABLE 7

General receiver spurious emission requirements for E-UTRA

| Frequency Band | Measurement Bandwidth | Maximum level | Note |
| --- | --- | --- | --- |
| 30MHz  f < 1GHz | 100 kHz | -57 dBm |  |
| 1GHz  f  12.75 GHz | 1 MHz | -47 dBm |  |

Annex 2

IMT 2000 CDMA Multi-Carrier (CDMA-2000) mobile stations

NOTE – Single carrier HRPD spectrum emission mask or spurious emission limits are only applicable for spreading Rate 1.

# 1	Spectrum Mask

## 1.1	Spreading Rate 1

When transmitting with Spreading Rate 1, the spectrum emission mask for Band Classes 0, 2, 5, 7, 9, 10, 11 and 12 shall be less than the limits specified in Table 1.

TABLE  7

Transmitter spectrum emission limits for spreading rate 1

| TABLE 1 For ¦ |  |
| --- | --- |
|  | Less stringent of –42 dBc/30 kHz or –54 dBm/1.23 MHz |

Band Classes 0, 2, 5, 7, 9, 10, 11 and 12 Spectrum Emission Mask for spreading Rate 1

| For ¦f ¦ within the Range (MHz) | Emission Limit |
| --- | --- |
| 885-1.98 | Less stringent of –42 dBc/30 kHz or –54 dBm/1.23 MHz |
| 1.25 to 4.00 (Band Class 10 only) | –13 dBm/30 kHz |
| 1.98-4.00 | Less stringent of –54 dBc/30 kHz or –54 dBm/1.23 MHz |
| 2.25-4.00 (Band Class 7 only) | –35 dBm/6.25 kHz |
| NOTE 1 – All frequencies in the measurement bandwidth shall satisfy the restrictions on ¦f ¦ where f  centre frequency – closer edge frequency, f, of the measurement filter. |  |

When transmitting with Spreading Rate 1, the spectrum emission mask for Band Classes 1, 4, 6, 8, 13, 14 and 15 shall be less than the limits specified in Table 2.

TABLE 2

Band Classes 1, 4, 6, 8, 13, 14 and 15 Spectrum Emission Mask for spreading Rate 1

| For ¦f ¦ within the Range (MHz) | Emission Limit |
| --- | --- |
| 1.25 to 1.98 | less stringent of -42 dBc/30 kHz or -54 dBm/1.23 MHz |
| 1.98 to 4.00 | less stringent of -50 dBc/30 kHz or -54 dBm/1.23 MHz |
| 2.25 to 4.00 (Band Classes 6, 8 and 13 only) | -[13 + 1  (f – 2.25 MHz)] dBm/1 MHz |
| NOTE 1 – All frequencies in the measurement bandwidth shall satisfy the restrictions on ¦f ¦ where f = centre frequency – closer edge frequency, f, of the measurement filter. |  |

When transmitting in Band Class 3, the spectrum emission mask shall be less than the limits specified in Table 3.

Table 3

Band Class 3 Spectrum Emission Mask for Spreading Rate 1

| Measurement Frequency (MHz) | For ¦f¦ within the Range | Emission Limit |
| --- | --- | --- |
| > 815 and  850, > 887 and  889, > 893 and  901, > 915 and  925 |  900 kHz and < 1.98 MHz | -42 dBc / 30 kHz |
|  |  1.98 MHz | 25 mW (-16 dBm) / 100 kHz; Pout  30 dBm -54 dBc / 100 kHz; Pout > 30 dBm |
| > 885 and  958, except > 887 and  889, > 893 and  901, > 915 and  925 | < 1.98 MHz | 25 mW (-16 dBm) / 30 kHz; Pout  30 dBm Less stringent of -60 dBc / 30 kHz or 2.5 mW (-26 dBm) / 30 kHz; Pout > 30 dBm |
|  |  1.98 MHz | 25 mW (-16 dBm) / 100 kHz; Pout  30 dBm Less stringent of -60 dBc / 100 kHz or 2.5 mW (-26 dBm) / 100 kHz; Pout > 30 dBm |
|  885 and > 958, except 815-850 | < 1.98 MHz | 25 mW (-16 dBm) / 30 kHz; Pout  30 dBm Less stringent of -60 dBc / 30 kHz or 2.5 mW (-26 dBm) / 30 kHz; Pout > 30 dBm |
|  |  1.98 MHz | 25 mW (-16 dBm) / 1 MHz; Pout  44 dBm More stringent of -60 dBc / 1 MHz and 20 mW (13 dBm) / 1 MHz; Pout > 44 dBm |

NOTE – All frequencies in the measurement bandwidth shall satisfy the restrictions on |f| where f = center frequency - closer measurement edge frequency (f). The lower and upper limits of the frequency measurement are currently 10 MHz and 3 GHz in Japan radio measurement documents.

When transmitting in Band Class 11 or 12 with spreading Rate 1, the spectrum emission mask shall also be less than the requirements in Table 4A for cdma2000 and Table 4B for HRPD.

Table 4a

Additional cdma2000 Band Classes 11 and 12 Spectrum Emission Mask for spreading Rate 1

| For ¦f¦ within the Range | Emission Limit |
| --- | --- |
| 885 to 1.125 | -47 – 7  (¦f¦ – 885) / 240 dBc in 30 kHz |
| 1.125 to 1.98 | -54 – 13  (¦f¦ – 1125) / 855 dBc in 30 kHz |
| 1.98 to 4.00 | -67 – 15  (¦f¦ – 1980) / 2020 dBc in 30 kHz |
| 4.00 to 10.00 | -51 dBm in 100 kHz |

NOTE – All frequencies in the measurement bandwidth shall satisfy the restrictions on |f| where f = center frequency - closer measurement edge frequency (f). f is positive offset from the highest valid CDMA channel in the band subclass or negative offset from the lowest valid CDMA channel in the band subclass. The emission limits for Band Classes 11 and 12 (European PAMR bands) are designed to allow co-existence with incumbent services in Europe and are tighter than ITU Category B requirements.

Table 4B

Additional HRPD Band Classes 11 and 12 Spectrum Emission Mask

| For ¦f¦ within the Range (MHz) | Emission Limit Band Class 11 Subclasses 4, 5; Band Class 12 Subclass 1 | Emission Limit Band Class 11 Subclasses 0, 1, 2, 3; Band Class 12 Subclass 0 |
| --- | --- | --- |
| .885 to 1.12 | -47 – 7 ´ (¦f¦ – 885) / 235 dBc in 30 kHz | Not specified |
| 1.12 to 1.98 | -54 – 13 ´ (¦f¦ – 1120) / 860 dBc in 30 kHz | Not specified |
| 1.98 to 4.00 | -67 – 15 ´ (¦f¦ – 1980) / 2020 dBc in 30 kHz | Not specified |

NOTE – All frequencies in the measurement bandwidth shall satisfy the restrictions on |f| where f = center frequency - closer measurement edge frequency (f).  f is positive offset from the highest valid CDMA channel in the band subclass or negative offset from the lowest valid CDMA channel in the band subclass. The emission limits for Band Classes 11 and 12 (European PAMR bands) are designed to allow co-existence with incumbent services in Europe and are tighter than ITU Category B requirements.

## 1.2	Multicarrier HRPD

When transmitting in Band Classes 0, 2, 3, 5, 7, 9, 10, 11 or 12 for a HRPD Rev B capable terminal configured with two reverse link channels with maximum frequency separation, the spectrum emission masks with ten or more averages shall be less than the limits specified in Table 5A.

Table 5A

Band Classes 0, 2, 3, 5, 7, 9, 10, 11, and 12 Spectrum Emission Mask for Multicarrier HRPD

| For ¦¦f¦¦ within the Range (MHz) | Emission Limit |
| --- | --- |
| 885 kHz to 1.885 | 6 dBm / 1 MHz |
| > 1.885 | -13 dBm / 1 MHz |
| NOTE – All frequencies in the measurement bandwidth shall satisfy the restrictions on ¦f¦ where f is measured as the frequency offset from the center frequency of each reverse CDMA channel. Emission Limits shall apply between the reverse CDMA channels when MaxReverseLinkBandwidth 41.23 MHz. |  |

When transmitting in Band Classes 1, 4, 6, or 8 for a HRPD Rev B capable terminal configured with two reverse link channels with maximum frequency separation, the spectrum emissions mask with ten or more averages shall be less than the limits specified in Table 5B.

Table 5B

Band Classes 1, 4, 6, and 8 Spectrum Emission Mask for Multicarrier HRPD

| For ¦f ¦ within the Range (MHz) | Emission Limit |
| --- | --- |
| 1.25 to 2.25 | 6 dBm / 1 MHz |
| > 2.25 | -13 dBm / 1 MHz |
| NOTE – All frequencies in the measurement bandwidth shall satisfy the restrictions on ¦f¦ where f where f is measured as the frequency offset from the center frequency of each channel. Emission Limits shall apply between the carriers when MaxReverseLinkBandwidth 41.25 MHz. |  |

A single exception will be allowed for spurious emission frequencies between the two reverse CDMA channels (for both Tables 5A and 5B).

For adjacent reverse CDMA channels, the spectrum emission mask with ten or more averages shall be less than the limits specified in Tables 6A and 6B.

Table 6A

Adjacent Multi-carrier Spectrum Emission Limits for number 
of adjacent reverse CDMA Channels, N = 3

| For ¦f ¦ within the Range (MHz) | Emission Limit |
| --- | --- |
| 2.5 to 2.7 | -14 dBm / 30 kHz |
| 2.7 to 3.5 | -[14 + 15  (f – 2.7 MHz)] dBm / 30 kHz |
| 3.08 (Band Class 6 only) | -33 dBc / 3.84 MHz |
| 3.5 to 7.5 | -[13 + 1  (f – 3.5 MHz)] dBm / 1 MHz |
| 7.5 to 8.5 | -[17 + 10  (f – 7.5 MHz)] dBm / 1 MHz |
| 8.08 (Band Class 6 only) | -43 dBc / 3.84 MHz |
| 8.5 to 12.5 | -27 dBm / 1 MHz |
| NOTE – All frequencies in the measurement bandwidth shall satisfy the restrictions on ¦f ¦ where f = center frequency of the middle reverse CDMA channel- closer measurement edge frequency (f). The requirements at offsets of 3.08 and 8.08 MHz are equivalent to ACLR requirements of 33 and 43 dB from a Spreading Rate 3 mobile station transmitter into a Spreading Rate 3 or IMT-DS mobile station receiver offset by 5 and 10 MHz respectively. ITU Category B is intended to apply to only Band Classes 6, 8, 9, 11 and 12. |  |

Table 6B

Adjacent multi-carrier Spectrum Emission Limits for number of adjacent reverse CDMA Channels, N≠ 3

| For ¦f ¦ within the Range (MHz) | Emission Limit |
| --- | --- |
| 2.5 + to 3.5 + | -13 dBm / (12.5 kHz × N) kHz |
| 3.5 + to 3.125× (N+1) | -13 dBm / 1 MHz |
| NOTE – All frequencies in the measurement bandwidth shall satisfy the restrictions on ¦f ¦ where f = center frequency - closer measurement edge frequency (f). ITU Category B is intended to apply to only Band Classes 6, 8, 9, 11 and 12. = (N-3) × 625 kHz, where N is the number of carriers (N ≥ 2). Operation outside North America is for future study. |  |

## Spreading Rate 3

When transmitting with Spreading Rate 3, the spectrum emission mask shall be less than the limits specified in Table 7.

TABLE 7

Spectrum Emission Limits for Spreading Rate 3

| 2 Transmitter Spurious Emissions (Conducted) For ¦f ¦ within the Range (MHz) | Emission Limit |
| --- | --- |
| 2.5-2.7 | –14 dBm/30 kHz |
| 2.7-3.5 | –(14 + 15  (f – 2.7 MHz)) dBm/30 kHz |
| 3.08 (Band Class 6 only) | –33 dBc/3.84 MHz |
| 3.5-7.5 | –(13 + 1  (f – 3.5 MHz)) dBm/1 MHz |
| 7.5-8.5 | –(17 + 10  (f – 7.5 MHz)) dBm/1 MHz |
| 8.08 (Band Class 6 only) | –43 dBc/3.84 MHz |
| 8.5-12.5 | –27 dBm/1 MHz |
| NOTE : All frequencies in the measurement bandwidth shall satisfy the restrictions on ¦f ¦ where f  centre frequency – closer edge frequency, f, of the measurement filter. The requirements at offsets of 3.08 and 8.08 MHz are equivalent to ACLR requirements of 33 and 43 dB from a Spreading Rate 3 mobile station transmitter into a Spreading Rate 3 or IMT-2000 CDMA Direct Spread mobile station receiver offset by 5 and 10 MHz respectively. ITU Category B is intended to apply to only Band Classes 5, 6, 8, 9, 11 and 12. |  |

When transmitting with Spreading Rate 1 or Spreading Rate 3, the spurious emissions shall  be less than the limits specified in Tables 8A and 8B.

TABLE 8A

Transmitter Spurious Emission Limits for Spreading Rates 1 and 3, Respectively (Category A)

| Table 8B For ¦f ¦ within the Range | Frequency Bandwidth | Measurement Bandwidth | Emission Limit (dBm) |
| --- | --- | --- | --- |
| > 4 MHz for Spreading Rate 1 > 12.5 MHz for Spreading Rate 3 | 9 kHz < f < 150 kHz 150 kHz < f < 30 MHz 30 MHz < f < 1 GHz 1 GHz < f < 12.75 GHz | 1 kHz 10 kHz 100 kHz 1 MHz | –13 –13 –13 –13 |
| NOTE 1 – All frequencies in the measurement bandwidth shall satisfy the restrictions on ¦f ¦ where f  centre frequency – closer edge frequency, f, of the measurement filter. |  |  |  |

Transmitter Spurious Emission Limits for Spreading Rates 1 and 3, 
Respectively (Category B)

| When transmitting with spreading Rate 1 or spreading Rate 3 in Band Class 6, the spurious emissions shall be less than the limits specified in Table 9. For ¦f ¦ within the Range | Frequency Bandwidth | Measurement Bandwidth | Emission Limit (dBm) |
| --- | --- | --- | --- |
| > 4 MHz for spreading rate 1 > 12.5 MHz for spreading rate 3 | 9 kHz < f < 150 kHz 150 kHz < f < 30 MHz 30 MHz < f < 1 GHz 1 GHz < f < 12.75 GHz | 1 kHz 10 kHz 100 kHz 1 MHz | –36 –36 –36 –30 |
| NOTE 1 – All frequencies in the measurement bandwidth shall satisfy the restrictions on ¦f ¦ where f  centre frequency – closer edge frequency, f, of the measurement filter. |  |  |  |

TABLE 9

Additional Band Class 6 Transmitter Spurious Emission Limits 
for Spreading Rates 1 and 3, Respectively

| When transmitting in Band Class 7, the transmitter spurious emissions with ten or more averages shall also be less than the requirements in Table 10. Measurement Frequency (MHz) | Measurement Bandwidth (kHz) | Emission Limit (dBm) | Victim Band |
| --- | --- | --- | --- |
| 1 884.5-1 919.6 | 300 | 41 | PHS |
| 925-935 | 100 | 67 | GSM 900 |
| 935-960 | 100 | 79 | GSM 900 |
| 1 805-1 880 | 100 | 71 | DCS 1800 |
| NOTE 1 – Measurements apply only when the measurement frequency is at least 11.25 MHz (Spreading Rate 1) or 12.5 MHz (Spreading Rate 3) from the CDMA centre frequency. The non-PHS system band measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, up to five measurements with a level up to the spurious emission limits in Table 8B are allowed. |  |  |  |

Table 10

Additional Band Class 7 Transmitter Spurious Emission Limits

| Transmission Frequency (MHz) | Measurement Frequency (MHz) | Emission Limit | Victim Band |
| --- | --- | --- | --- |
| 776-788 | 763-775 | -35 dBm/6.25 kHz | Public Safety |
| 788-793 | 769-775 | -35 dBm/6.25 kHz | Public Safety |
| 776-788 | 793-805 | -35 dBm/6.25 kHz | Public Safety |
| 788-793 | 799-805 | -35 dBm/6.25 kHz | Public Safety |

When transmitting in Band Classes 11 and 12, the transmitter spurious emissions with ten or more averages shall also be less than the requirements in Table 11.

Table 11

Additional Band Classes 11 and 12 Transmitter Spurious Emission Limits

| For ¦f¦ within the Range | Emission Limit Band Class 11 Subclasses 4, 5; Band Class 12 Subclass 1 | Emission Limit Band Class 11 Subclasses 0, 1, 2, 3; Band Class 12 Subclass 0 |
| --- | --- | --- |
| 4.00 MHz to 10.0 MHz | -51 dBm in 100 kHz | Not specified |
| NOTE – All frequencies in the measurement bandwidth shall satisfy the restrictions on ¦f¦ where f = center frequency – closer measurement edge frequency (f). f is positive offset from the highest valid CDMA channel in the band subclass or negative offset from the lowest valid CDMA channel in the band subclass. The emission limits for Band Classes 11 and 12 (European PAMR bands) are designed to allow co-existence with incumbent services in Europe and are tighter than ITU Category B requirements. |  |  |

3	Receiver Spurious Emissions (Conducted)

The conducted spurious emissions when not transmitting for a MS shall be less than the limits in Table 12.

TABLE 12

General Receiver Spurious Emission Requirements

| For all frequencies within the mobile station receive and transmit bands, the conducted emissions shall be below the limits in Table 13. Frequency Band | Measurement Bandwidth | Maximum Level (dBm) | Note |
| --- | --- | --- | --- |
| 30 MHz  f < 1 GHz | 100 kHz | 57 | Band Class 6 only |
| 1 GHz  f  12.75 GHz | 30 kHz | –54 | With the exception of the frequencies covered by Table 14, for which additional receiver spurious emission requirements apply Band Class 3 only |
| 1 GHz  f  12.75 GHz | 1 MHz | 47 | With the exception of the frequencies covered by Table 12, for which additional receiver spurious emission requirements apply |
| 1 GHz  f  12.75 GHz | 30 kHz | –47 | With the exception of the frequencies covered by Table 14, for which additional receiver spurious emission requirements apply All Band Classes except 3 and 6 |

TABLE 13

Additional Receiver Spurious Emission Requirements

|  | Measurement Bandwidth (MHz) | Maximum Level (dBm) | Note |
| --- | --- | --- | --- |
|  | 1 | –61 | Mobile transmit band |
|  | 1 | –76 | Mobile receive band All Band Classes except 3 |
|  | 1 | –81 | Mobile receive band Band Class 3 |

Annex 3

CDMA TDD (UTRA TDD) mobile stations

# 1	Measurement uncertainty

Values specified in this Annex differ from those specified in Recommendation ITU-R M.1457 since values in this Annex incorporate test tolerances defined in Recommendation ITU-R M.1545.

# 2	Spectrum mask

## 2.1	Spectrum mask (3.84 Mchip/s UTRA TDD option)

The spectrum emission mask of the MS applies to frequency offsets (f) between 2.5 and 12.5 MHz on both sides of the carrier frequency.

The out-of-channel emission is specified as a power level relative to the MS output power in a frequency band of 3.84 MHz bandwidth.

The power of any MS emission should not exceed −48.5 dBm/3.84 MHz or the levels specified in Table 13a, whichever is higher.

TABLE 13a

Spectrum emission mask requirement (3.84 Mchip/s TDD option)

| Δf(1) (MHz) | Minimum requirement | Measurement bandwidth |
| --- | --- | --- |
| 2.5-3.5 | −33.5 − 15(1) (f/MHz − 2.5) dBc | 30 kHz(2) |
| 3.5-7.5 | –33.5 − 1(1) (f/MHz − 3 dBc Hz f                  5 . 3 1 33.55 .5) dBc | 1 MHz(3) |
| 7.5-8.5 | −37.5 − 10(1) (f/MHz − 7.5) dBc | 1 MHz(3) |
| 8.5-12.5 | −47.5 dBc | 1 MHz(3) |
| (1) f is the separation between the carrier frequency and the centre of the measuring filter. (2) The first and last measurement positions with a 30 kHz filter at f equals 2.515 MHz and 3.485 MHz. (3) The first and last measurement positions with a 1 MHz filter at f equals 4 MHz and 12 MHz. As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be different from the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. NOTE 1 – The lower limit should be − 48.5 dBm/3.84 MHz or the minimum requirement presented in this table, whichever is the higher. |  |  |

## 2.2	Spectrum mask (1.28 Mchip/s UTRA TDD option)

The spectrum emission mask of the MS applies to frequency offsets between 0.8 and 4.0 MHz on both sides of the carrier frequency.

The out-of-channel emission is specified as a power level relative to the MS output power in a frequency band of 1.6 MHz bandwidth.

TABLE 13b

Spectrum emission mask requirement (1.28 Mchip/s TDD option)

| Δf(1) (MHz) | Minimum requirement | Measurement bandwidth |
| --- | --- | --- |
| 0.8 | −33.5 dBc(3) | 30 kHz(2) |
| 0.8-1.8 | −33.5 − 14(1)(f/MHz − 0.8) dBc(3) | 30 kHz(2) |
| 1.8-2.4 | −47.5 − 17(1)(f/MHz − 1.8) dBc(3) | 30 kHz(2) |
| 2.4-4.0 | −42.5 dBc(3) | 1 MHz(3) |
| (1) f is the separation between the carrier frequency and the centre of the measuring filter. (2) The first and last measurement positions with a 30 kHz filter at f equals to 0.815 MHz and 2.385 MHz. (3) The first and last measurement positions with a 1 MHz filter at f equals 2.9 MHz and 3.5 MHz. As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be different from the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. NOTE 1 – The lower limit should be −53.5 dBm/1.28 MHz or the minimum requirement presented in this table, whichever is the higher. |  |  |

## 2.3	Spectrum mask (7.68 Mchip/s UTRA TDD option)

The spectrum emission mask of the UE applies to frequencies which are between 5 MHz and 25 MHz from the UE centre carrier frequency. The out-of-channel emission is specified relative to the RRC filtered mean power of the UE carrier.

The power of any UE emission should not exceed the levels specified in Table 13c.

TABLE 13c

Spectrum emission mask requirement (7.68 Mchip/s TDD option)

| Δf(1) (MHz) | Minimum requirement | Measurement bandwidth |
| --- | --- | --- |
| 5.0-5.75 | −36.5−10.67⋅ΔfMHz−5.0 dBc | 30 kHz(2) |
| 5.75-7.0 | −44.5−5.6⋅ΔfMHz−5.75 dBc | 30 kHz(2) |
| 7.0-15 | −36.5−0.5⋅ΔfMHz−7.0 dBc | 1 MHz(3) |
| 15.0-17.0 | −40.5−5.0⋅ΔfMHz−15.0 dBc | 1 MHz(3) |
| 17.0-25.0 | −51.5 dBc | 1 MHz(3) |
| (1) f is the separation between the carrier frequency and the centre of the measuring filter. (2) The first and last measurement positions with a 30 kHz filter at f equals 5.015 MHz and 6.985 MHz. (3) The first and last measurement positions with a 1 MHz filter at f equals 7.5 MHz and 24.5 MHz. As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be different from the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth. NOTE 1 – The lower limit should be −47 dBm/7.68 MHz or the minimum requirement presented in this table, whichever is the higher. |  |  |

# 2.4	E-UTRA spectrum mask

The output UE transmitter spectrum consists of the three components; the emission within the occupied bandwidth (channel bandwidth), the Out Of Band (OOB) emissions and the far out spurious emission domain (Figure 3).

Figure 3

The spectrum emission mask of the MS applies to frequencies (ΔfOOB) starting from the ± edges of the assigned E-UTRA channel bandwidth.  For frequencies greater than (ΔfOOB) as specified in Table 13d the spurious requirements in §4 are applicable.

### 2.4.1	General E-UTRA spectrum mask

The power of any MS emission shall not exceed the levels specified in Table 13d for the specified channel bandwidths.

TABLE 13d

General E-UTRA spectrum emission mask

| ΔfOOB (MHz) | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
|  0-1 |  |  | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
|  1-2.5 |  |  | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
|  2.5-2.8 |  |  | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
|  2.8-5 |  |  | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
|  5-6 |  |  | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
|  6-10 |  |  | -23.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
|  10-15 |  |  |  | -23.5 | -11.5 | -11.5 | 1 MHz |
|  15-20 |  |  |  |  | -23.5 | -11.5 | 1 MHz |
|  20-25 |  |  |  |  |  | -23.5 | 1 MHz |

Note- the values for 1.4 MHz and 3.0 MHz channel bandwidths will be addressed at a later stage

NOTE – As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

# 3	ACLR

## 3.1	ACLR for UTRA

ACLR is the ratio of the transmitted power to the power measured after a receiver filter in the adjacent channel(s). Both the transmitted power and the received power are measured through a matched filter (root raised cosine and roll-off 0.22) with a noise power bandwidth equal to the chip rate. The requirements should apply whatever the type of transmitter considered (single carrier or multi-carrier). It applies for all transmission modes foreseen by the manufacturer’s specification. The limit for ACLR should be as specified in Table 14a-14c).

Table 14a)  MS ACLR limits for 3.84 Mchip/s TDD option

| Power class | Adjacent channel | ACLR limit (dB) |
| --- | --- | --- |
| 2, 3 | MS channel ± 5 MHz | 32.2 |
| 2, 3 | MS channel ± 10 MHz | 42.2 |

Table 14b)  MS ACLR limits for 1.28 Mchip/s TDD option

| Power class | Adjacent channel | ACLR limit (dB) |
| --- | --- | --- |
| 2, 3 | MS channel ± 1.6 MHz | 32.2 |
| 2, 3 | MS channel ± 3.2 MHz | 42.2 |

Table 14c)  MS ACLR limits for 7.68 Mchip/s TDD option

| Power class | Adjacent channel | Chip rate for RRC measurement filter (MHz) | ACLR limit (dB) |
| --- | --- | --- | --- |
| 2, 3 | MS channel ± 7.5 MHz | 3.84 | 32.2 |
| 2, 3 | MS channel ± 12.5 MHz | 3.84 | 42.2 |
| 2, 3 | MS channel ± 10.0 MHz | 7.68 | 32.2 |
| 2, 3 | MS channel ± 20.0 MHz | 7.68 | 42.2 |

## 3.2	ACLR for E-UTRA

ACLR is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. The ACLR requirements are specified for two scenarios: for (an) adjacent E -UTRA and/or UTRA channels (Figure 4).

FIGURE 4

### 3.2.1	Limits for E-UTRA

E-UTRA Adjacent Channel Leakage power Ratio (E-UTRAACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.  The E-UTRA on channel and adjacent channel power is measured with a rectangular measurement bandwidth filter. The limits should be as specified in Table 14D.

TABLE 14d

General requirements for E-UTRAACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | -29.2 dB | -29.2 dB | -29.2 dB | -29.2 dB | -29.2 dB | -29.2 dB |
| E-UTRA channel Measurement bandwidth |  |  | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UE channel |  |  | +5MHz or -5MHz | +10MHz or -10MHz | +15MHz or -15MHz | +20MHz or -20MHz |

### 3.2.2	Limits for E-UTRA for UTRA co-existence in the same geographical area

For adjacent UTRA carriers the limits should be as specified in Table 14e.

UTRA Adjacent Channel Leakage power Ratio (UTRAACLR) is the ratio of the filtered mean power centred on the assigned E-UTRA channel frequency to the filtered mean power centred on an adjacent(s) UTRA channel frequency.

UTRA Adjacent Channel Leakage power Ratio is specified for both the first UTRA 5 MHz adjacent channel (UTRAACLR1) and the 2nd UTRA 5MHz adjacent channel (UTRAACLR2) .The UTRA channel is measured with a 3.84 MHz RRC bandwidth filter with roll-off factor =0.22. The E-UTRA channel is measured with a rectangular measurement bandwidth filter.

TABLE 14e

Additional requirements

|  | Channel bandwidth / UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB |
| UTRAACLR2 | - | - | 35.2 dB | 35.2 dB | 35.2 dB | 35.2 dB |
| E-UTRA channel Measurement bandwidth |  | - | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA channel Measurement bandwidth | - | - | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UE channel for UTRAACLR1 | +2.5MHz from upper band edge or -2.5MHz from lower band edge |  |  |  |  |  |
| UE channel for UTRAACLR2 | +7.5MHz from upper band edge or -7.5MHz from lower band edge |  |  |  |  |  |

### 3.2.3	Additional ACLR limits

Additional ACLR requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

The additional ACLR limits are specified for the 2nd UTRA adjacent channel (UTRAACLR2). .The UTRA channel is measured with a 3.84 MHz RRC bandwidth filter with roll-off factor =0.22. The E-UTRA channel is measured with a rectangular measurement bandwidth filter.

TABLE 14f

Additional requirements (UTRAACLR2)

|  | Channel bandwidth / UTRAACLR2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR2bis | - | - | 42.2 dB | 42.2 dB | - | - |
| E-UTRA channel Measurement bandwidth | - | - | 4.5 MHz | 9.0 MHz | - | - |
| UTRA channel Measurement bandwidth | - | - | 3.84 MHz | 3.84 MHz | - | - |
| UE channel for UTRAACLR2bis | +7.5MHz from upper band edge or -7.5MHz from lower band edge |  |  |  |  |  |

# 4	Transmitter spurious emissions (conducted)

## 4.1	Transmitter spurious emissions for UTRA

For UTRA, the spurious emissions should be less than the limits specified in Tables 15, 16a), 16b) and 16c). The following requirements are only applicable for MS centre carrier frequency offsets greater than 12.5 MHz (3.84 Mchip/s TDD option), 4 MHz (1.28 Mchip/s TDD option) or 25 MHz (7.68 Mchip/s TDD option).

TABLE 15

General spurious emissions requirements for UTRA

| Frequency band | Measurement bandwidth | Minimum requirement (dBm) |
| --- | --- | --- |
| 9 kHz  f  150 kHz | 1 kHz | –36 |
| 150 kHz  f  30 MHz | 10 kHz | –36 |
| 30 MHz  f  1 000 MHz | 100 kHz | –36 |
| 1 GHz  f  12.75 GHz | 1 MHz | –30 |

TABLE 16

a)  Additional spurious emissions requirements (3.84 Mchip/s TDD option)

| Frequency bandwidth | Measurement bandwidth | Minimum requirement (dBm) |
| --- | --- | --- |
| 925 MHz  f  935 MHz | 100 kHz | −67 (Note 1) |
| 935 MHz < f  960 MHz | 100 kHz | −79 (Note 1) |
| 1 805 MHz  f  1 880 MHz | 100 kHz | −71 (Note 1) |
| 1 884.5 MHz  f  1 919.6 MHz | 300 kHz | −41 (Note 2) |
| NOTE 1 – The measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, up to five measurements with a level up to the applicable requirements defined in Table 15 are permitted for each absolute RF channel used in the measurement. NOTE 2 – Applicable for transmission in 2 010-2 025 MHz. |  |  |

b)  Additional spurious emissions requirements (1.28 Mchips TDD option)

| Operating Band | Frequency Bandwidth | Measurement Bandwidth | Minimum requirement |
| --- | --- | --- | --- |
| a | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm (note1) |
|  | 925 MHz  f  935 MHz | 100 kHz | -67 dBm (note1) |
|  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm (note1) |
|  | 1805 MHz  f  1880 MHz | 100 kHz | -71 dBm (note1) |
|  | 2010 MHz  f  2025 MHz | 1MHz | -65 dBm (Note2) |
|  | 1900 MHz  f  1920 MHz | 1MHz | -65 dBm (Note 3) |
| b | 1850 MHz  f  1910 MHz | 1 MHz | -65 dBm (Note 4) |
|  | 1930 MHz  f  1990 MHz | 1 MHz | -65 dBm (Note 5) |
|  | 2010 MHz  f  2025 MHz | 1MHz | -65 dBm |
| c | 2010 MHz  f  2025 MHz | 1 MHz | -65 dBm |
| d | 1900 MHz  f  1920 MHz | 1 MHz | -65 dBm |
|  | 2010 MHz  f  2025 MHz | 1 MHz | -65 dBm |
|  | 2620 MHz  f  2690 MHz | 3.84 MHz | -37 dBm |
| e | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm (note1) |
|  | 925 MHz  f  935 MHz | 100 kHz | -67 dBm (note1) |
|  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm (note1) |
|  | 1805 MHz  f  1880 MHz | 100 kHz | -71 dBm (note1) |
|  | 1900 MHz  f  1920 MHz | 1 MHz | -65 dBm |
|  | 2010 MHz  f  2025 MHz | 1 MHz | -65 dBm |
| NOTE 1 –The measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, up to five measurements with a level up to the applicable requirements defined in Table 6.7c are permitted for each UARFCN used in the measurement. NOTE 2 – This requirement is only applicable when UE operating in 1900-1920MHz of band a. NOTE 3 – This requirement is only applicable when UE operating in 2010-2025MHz of band a. NOTE 4 – This requirement is only applicable when UE operating in 1930-1990MHz of band b. NOTE 5 – This requirement is only applicable when UE operating in 1850-1910MHz of band b. |  |  |  |

c)  Additional spurious emissions requirements (7.68 Mchips TDD option)

| Frequency bandwidth | Measurement bandwidth | Minimum requirement (dBm) |
| --- | --- | --- |
| 921 MHz  f < 925 MHz | 100 kHz | −60 (Note 1) |
| 925 MHz  f  935 MHz | 100 kHz | −67 (Note 1) |
| 935 MHz < f  960 MHz | 100 kHz | −79 (Note 1) |
| 1 805 MHz  f  1 880 MHz | 100 kHz | −71 (Note 1) |
| 2 620 MHz  f  2 690 MHz | 3.84 MHz | −37 (Note 1) |
| 1 884.5 MHz  f  1 919.6 MHz | 300 kHz | −41 (Note 2) |
| NOTE 1 – The measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, up to five measurements with a level up to the applicable requirements defined in Table 15 are permitted for each absolute RF channel used in the measurement. NOTE 2 – Applicable for transmission in 2 010-2 025 MHz. |  |  |

# 4.2	Transmitter spurious emissions for E-UTRA

For E-UTRA, the spurious emission limits apply for frequency ranges that are more than ΔfOOB (MHz) from the edge of the channel bandwidth (Table 16d).

TABLE 16d

Boundary between E-UTRA ΔfOOB and spurious emission domain

| Channel bandwidth | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) |  |  | 10 | 15 | 20 | 25 |

Note- the values for 1.4 MHz and 3.0 MHz channel bandwidths will be addressed at a later stage

The spurious emission limits in Table 16e apply for all E-UTRA transmitter band configurations and channel bandwidths.

TABLE 16e

General spurious emissions requirements for E-UTRA

| Frequency band | Measurement bandwidth | Minimum requirement (dBm) |
| --- | --- | --- |
| 9 kHz  f  150 kHz | 1 kHz | –36 |
| 150 kHz  f  30 MHz | 10 kHz | –36 |
| 30 MHz  f  1 000 MHz | 100 kHz | –36 |
| 1 GHz  f  12.75 GHz | 1 MHz | –30 |

TABLE 16f

Spurious emissions requirements for UE co-existance in E-UTRA bands

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Level (dBm) | Bandwidth (MHz) | Comment |
| 33 | E-UTRA Band 1, 3, 8, 34, 38, 39, 40 | FDL_low | - | FDL_high | -50 | 1 | Note 5 |
| 34 | E-UTRA Band 1, 3, 7, 8, 9, 11, 33, 38,39, 40 | FDL_low | - | FDL_high | -50 | 1 | Note 5 |
|  | Frequency range | 860 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 |  |
| 35 |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |
| 37 |  |  | - |  |  |  |  |
| 38 | E-UTRA Band 1,3, 33, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
| 39 | E-UTRA Band 34, 40 | FDL_low | - | FDL_high | -50 | 1 |  |
| 40 | E-UTRA Band 1, 3, 33, 34, 39 | FDL_low | - | FDL_high | -50 | 1 |  |
| Note |  |  |  |  |  |  |  |
| 1 | FDL_low and FDL_high refer to each E-UTRA frequency band specified in Note 3 of the Scope |  |  |  |  |  |  |
| 2 | A number of exceptions are permitted and is FFS. These exceptions include both spurious due to LO mixing and I/Q imbalance for specific values of NRB. For these exceptions the requirements of Table 16e are applicable. |  |  |  |  |  |  |
| 3 | To meet these requirements some restriction will be needed for either the operating band or protected band |  |  |  |  |  |  |
| 4 | Requirements are specified in terms of E-UTRA sub-bands |  |  |  |  |  |  |
| 5 | For non synchronised TDD operation to meet these requirements some restriction will be needed for either the operating band or protected band |  |  |  |  |  |  |

# 5	Receiver spurious emissions (conducted)

## 5.1	Receiver spurious emissions for UTRA

For UTRA, the power of any spurious emissions from the receiver should not exceed the limits given in Tables  18a), 18b) and 18c).

TABLE 18

a)  Receiver spurious emission requirements (3.84 Mchip/s UTRA TDD option)

| Band | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz-1 GHz | −57 dBm | 100 kHz |  |
| 1 GHz-1.9 GHz and 1.92 GHz-2.01 GHz and 2.025 GHz-2.11 GHz and 2.17 GHz- 2.57 GHz | −47 dBm | 1 MHz |  |
| 1.9 GHz-1.92 GHz and 2.01 GHz-2.025 GHz and 2.11 GHz-2.170 GHz and 2.57 GHz-2.69 GHz | −60 dBm | 3.84 MHz |  |
| 2.69 GHz-12.75 GHz | −47 dBm | 1 MHz |  |

b)  Receiver spurious emission requirements (1.28 Mchip/s UTRA TDD option)

| Band | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz-1 GHz | −57 dBm | 100 kHz |  |
| 1 GHz-1.9 GHz and 1.92 GHz-2.01 GHz and 2.025 GHz-2.11 GHz and 2.17 GHz-2.30 GHz and 2.40 GHz - 2.57 GHz | −47 dBm | 1 MHz |  |
| 1.9 GHz-1.92 GHz and 2.01 GHz-2.025 GHz and 2.11 GHz-2.170 GHz and 2.30 GHz-2.40 GHz and 2.57 GHz-2.69 GHz | −64 dBm | 1.28 MHz |  |
| 2.69 GHz 12.75 GHz | −47 dBm | 1 MHz |  |

TABLE 18 (end)

c)  Receiver spurious emission requirements (7.68 Mchip/s UTRA TDD option)

| Band | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz-1 GHz | −57 dBm | 100 kHz |  |
| 1 GHz-1.9 GHz and 1.92 GHz-2.01 GHz and 2.025 GHz-2.11 GHz and 2.17 GHz-2.57 GHz | −47 dBm | 1 MHz | With the exception of frequencies between 25 MHz below the first carrier frequency and 25 MHz above the last carrier frequency used by the MS. |
| 1.9 GHz-1.92 GHz and 2.01 GHz-2.025 GHz and 2.11 GHz-2.170 GHz and 2.57 GHz-2.69 GHz | −57 dBm | 7.68 MHz | With the exception of frequencies between 25 MHz below the first carrier frequency and 25 MHz above the last carrier frequency used by the MS. |
| 2.69 GHz-12.75 GHz | −47 dBm | 1 MHz |  |

# 5.2	Receiver spurious emissions for E-UTRA

The power of any narrow band CW spurious emission shall not exceed the maximum level specified in Table 19.

TABLE 19

General receiver spurious emission requirements for E-UTRA

| Frequency Band | Measurement Bandwidth | Maximum level | Note |
| --- | --- | --- | --- |
| 30MHz  f < 1GHz | 100 kHz | -57 dBm |  |
| 1GHz  f  12.75 GHz | 1 MHz | -47 dBm |  |

Annex 4 

TDMA Single-Carrier (UWC-136) mobile stations

Part A

Conformance requirements (30 kHz)

# 1	Spectrum mask

Spectrum noise suppression is the restraint of sideband energy outside the active transmit channel. This RF spectrum is the result of power ramping, modulation and all sources of noise. The spectrum is primarily the result of events that do not occur at the same time: digital modulation and power ramping (switching transients). The RF spectrum from these two events are specified separately.

Adjacent and first or second alternate channel power is that part of the mean power output of the transmitter resulting from the modulation and noise which falls within a specified passband centred on either of the adjacent or first or second alternate channels.

The emission power should not exceed the limits specified in Table 19.

TABLE  19

Adjacent and alternate channel power requirements

| Channel | Maximum level |
| --- | --- |
| In either adjacent channel, centred 30 kHz from the centre frequency | 26 dB below the mean output power |
| In either alternate channel, centred 60 kHz from the centre frequency | 45 dB below the mean output power |
| In either second alternate channel centred 90 kHz from the centre frequency | 45 dB below the mean output power or –13 dBm measured in 30 kHz bandwidth, whichever is the lower power |

OoB power arising from switching transients is the peak power of the spectrum, arising from the ramping-on and ramping-off of the transmitter, that fall within defined frequency bands outside the active transmit channel.

The peak emission power should not exceed the limits specified in Table 20.

TABLE  20

Switching transients requirements

| Channel | Maximum level |
| --- | --- |
| In either adjacent channel, centred 30 kHz from the centre frequency | 26 dB below the peak output power reference |
| In either alternate channel, centred 60 kHz from the centre frequency | 45 dB below the peak output power reference |
| In either second alternate channel centred 90 kHz from the centre frequency | 45 dB below the peak output power reference or –13 dBm measured in 30 kHz bandwidth, whichever is the lower power |

# 2	Transmitter spurious emissions (conducted)

The power of any spurious emission should not exceed the limits specified in Table 21.

TABLE  21

MS spurious emission limits

| Band ( f ) (1) | Maximum level (dBm) | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 9 kHz  f  150 kHz | –36 | 1 kHz | (2) |
| 150 kHz  f  30 MHz | –36 | 10 kHz | (2) |
| 30 MHz  f  1 000 MHz | –36 | 100 kHz | (2) |
| 1 000 MHz  f  1 920 MHz | –30 | 1 MHz | (2) |
| 1 920 MHz  f  1 980 MHz | –30 | 30 kHz | (3) |
| 1 980 MHz  f  2 110 MHz | –30 | 1 MHz | (2) |
| 2 110 MHz  f  2 170 MHz | –70 | 30 kHz | (4) |
| 2 170 MHz  f  12.75 GHz | –30 | 1 MHz | (2) |
| (1) f is the frequency of the spurious emission. (2) In accordance with the applicable clauses of Recommendation ITU-R SM.329. (3) MS transmit band. (4) MS receive band. |  |  |  |

## 2.1	Coexistence with services in adjacent frequency bands

This requirement provides for the protection of receivers operating in bands adjacent to the MS transmit frequency band of 1 920 to 1 980 MHz: GSM 900, R-GSM and UTRA TDD.

NOTE 1 – UTRA FDD operates in the same frequency band as UWC-136.

The power of any spurious emission should not exceed the limits specified in Table 22.

TABLE  22

Additional spurious emissions requirements

| Service | Frequency band | Measurement bandwidth (kHz) | Limit (dBm) |
| --- | --- | --- | --- |
| R-GSM | 921  f  925 MHz | 100 | –60 |
| R-GSM | 925  f  935 MHz | 100 | –67 |
| GSM 900/R-GSM | 935  f  960 MHz | 100 | –79 |
| DCS 1800 | 1 805  f  1 880 MHz | 100 | –71 |
| UTRA TDD | 1 900  f  1 920 MHz | 100 | –62 |
| UTRA TDD | 2 010  f  2 025 MHz | 100 | –62 |
| NOTE 1 – The measurements are made on frequencies which are integer multiples of 200 kHz. Up to five exceptions of up to –36 dBm are permitted in the GSM 900, DCS 1800 and UTRA bands, and up to three exceptions of up to –36 dBm are permitted in the GSM 400 bands. |  |  |  |

# 3	Receiver spurious emissions (idle mode)

The power of any spurious emissions should not exceed the limits given in Table 23.

TABLE  23

General receiver spurious emission requirements

| Frequency band | Measurement bandwidth | Maximum level (dBm) | Note |
| --- | --- | --- | --- |
| 30 MHz  f  1 GHz | 100 kHz | –57 |  |
| 1 GHz  f  12.75 GHz | 1 MHz | – 47 | With the exception of the frequencies covered by the table below, for which additional receiver spurious emission requirements apply(1) |
| (1) (Editorial Note) – In TFES Harmonized Standard v1.0.2, no additional receiver spurious emission is specified; yet, it is expected that there will be a table added, in the same form as for the other technologies (see Tables 5, 12 and 18). |  |  |  |

Part B

Conformance requirements (200 kHz)

The 200 kHz channel provides packet data service and employs both 8-level phase shift keying (8-PSK) and Gaussian minimum shift keying (GMSK) modulations.

# 1	Spectrum mask

Output RF spectrum is the relationship between the frequency offset from the carrier and the power, measured in a specified bandwidth and time, produced by the MS due to the effects of modulation and power ramping.

The specifications contained in this subclause apply in frequency hopping as well as in non-frequency hopping modes.

Due to the bursty nature of the signal, the output RF spectrum results from two effects: the modulation process, and the power ramping up and down (switching transients).

–	The level of the output RF spectrum due to GMSK and 8-PSK modulations should be no more than that given in Tables 24 and 25.

–	The level of the output RF spectrum due to switching transients should be no more than that given in Table 26.

–	The power emitted should not exceed –71 dBm in frequency band 2 110-2 170 MHz.

# 2	Spectrum due to the modulation and wideband noise

The output RF modulation spectrum is specified in Tables 24 and 25. This specification applies for all RF channels supported by the equipment.

The specification applies to the entire relevant transmit band and up to 2 MHz either side.

The limits should be met under the following measurement conditions:

–	Zero frequency scan, filter bandwidth and video bandwidth of 30 kHz up to 1 800 kHz from the carrier and 100 kHz at 1 800 kHz and above from the carrier, with averaging done over 50% to 90% of the useful part of the transmitted bursts, excluding the midamble, and then averaged over at least 200 such burst measurements. Above 1 800 kHz from the carrier, only measurements centred on 200 kHz multiples are taken with averaging over 50 bursts.

–	When tests are done in frequency hopping mode, the averaging should include only bursts transmitted when the hopping carrier corresponds to the nominal carrier of the measurement. The limits then apply to the measurement results for any of the hopping frequencies.

The figures in Table 24, at the vertically listed power level (dBm) and at the horizontally listed frequency offset from the carrier (kHz), are then the maximum allowed level (dB) relative to a measurement in 30 kHz on the carrier.

NOTE 1 – This approach of specification has been chosen for convenience and speed of testing. It does however require careful interpretation if there is a need to convert figures in the following tables into spectral density values, in that only part of the power of the carrier is used as the relative reference, and in addition different measurement bandwidths are applied at different offsets from the carrier.

TABLE  24

Relative maximum level due to modulation

| Carrier power (dBm) | Frequency offset (kHz) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 100 | 200 | 250 | 400 | 600 1 200 | 1 200 1 800 | 1 800 6 000 | 6 000 |
| 33 | 0.5 | –30 | –33 | –60 | –60 | –60 | –68 | –76 |
| 32 | 0.5 | –30 | –33 | –60 | –60 | –60 | –67 | –75 |
| 30 | 0.5 | –30 | –33 | –60 | –60(1) | –60 | –65 | –73 |
| 28 | 0.5 | –30 | –33 | –60 | –60(1) | –60 | –63 | –71 |
| 26 | 0.5 | –30 | –33 | –60 | –60(1) | –60 | –61 | –69 |
| 24 | 0.5 | –30 | –33 | –60 | –60(1) | –60 | –59 | –67 |
| (1) For equipment supporting 8-PSK, the requirement for 8-PSK modulation is –54 dB. |  |  |  |  |  |  |  |  |

The following exceptions should apply, using the same measurement conditions as specified above:

–	In the combined range of 600 kHz to 6 MHz above and below the carrier, in up to three bands of 200 kHz width centred on a frequency which is an integer multiple of 200 kHz, exception levels of up to –36 dBm are allowed.

–	Above 6 MHz offset from the carrier in up to 12 bands of 200 kHz width centred on a frequency which is an integer multiple of 200 kHz, exception levels of up to –36 dBm are allowed.

Using the same measurement conditions as specified above, if a requirement in Table 24 results in lower than the power limit given in Table 25, then the latter should be applied instead.

TABLE  25

Absolute maximum level due to modulation

| Frequency offset from the carrier (kHz) | Level (dBm) |
| --- | --- |
|  600 | –36 |
|  600,  1 800 | –56 |
| 1 800 | –51 |

# 3	Spectrum due to switching transients

These effects are also measured in the time domain and the specifications assume the following measurement conditions: zero frequency scan, filter bandwidth 30 kHz, peak hold, and video bandwidth 100 kHz. Table 26 specifies the limits.

TABLE  26

Maximum levels due to switching transients

| Carrier power level (dBm) | Maximum level measured at various frequency offsets |  |  |  |
| --- | --- | --- | --- | --- |
|  | 400 kHz | 600 kHz | 1 200 kHz | 1 800 kHz |
| 39 | –21 dBm | –26 dBm | –32 dBm | –36 dBm |
|  37 | –23 dBm | –26 dBm | –32 dBm | –36 dBm |

NOTE 1 – The relaxation for carrier power level 39 dBm is in line with the modulated spectra and thus causes negligible additional interference to an analogue system by an UWC-136 200 kHz signal.

NOTE 2 – The near-far dynamics with this specification has been estimated to be approximately 58 dB for MS operating at a power level of 8 W or 49 dB for MSs operating at a power level of 1 W. The near-far dynamics then gradually decreases by 2 dB per power level down to 32 dB for MSs operating in cells with a maximum allowed output power of 20 mW or 29 dB for MS operating at 10 mW.

NOTE 3 – The possible performance degradation due to switching transient leaking into the beginning or the end of a burst, was estimated and found to be acceptable with respect to the BER due to co-channel interference (C/I).

# 4	Transmitter spurious emissions (conducted)

The power of any spurious emission should not exceed the limits specified in Table 27.

TABLE  27

MS spurious emission limits

| Band ( f )(1) | Measurement bandwidth | Maximum level (dBm) | Note |
| --- | --- | --- | --- |
| 9 kHz  f  150 kHz | 1 kHz | –36 | (2) |
| 150 kHz  f  30 MHz | 10 kHz | –36 | (2) |
| 30 MHz  f  1 000 MHz | 100 kHz | –36 | (2) |
| 1 000 MHz  f  1 920 MHz | 1 MHz | –30 | (2) |
| 1 920 MHz  f  1 980 MHz | 100 kHz | –36 | (3) |
| 1 980 MHz  f  2 110 MHz | 1 MHz | –30 | (2) |
| 2 110 MHz  f  2 170 MHz | 100 kHz | –66 | (4) |
| 2 170 MHz  f  12.75 GHz | 1 MHz | –30 | (2) |
| (1) f is the frequency of the spurious emission. (2) In accordance with the applicable clauses of Recommendation ITU-R SM.329. (3) MS transmit band. (4) MS receive band. |  |  |  |

# 5	Coexistence with services in adjacent frequency bands

This requirement provides for the protection of receivers operating in bands adjacent to the MS transmit frequency band of 1 920 MHz to 1 980 MHz: GSM 900, R-GSM, UTRA TDD.

The power of any spurious emission should not exceed the limits specified in Table 28.

TABLE  28

Additional spurious emissions requirements

| Service | Frequency band | Measurement bandwidth (kHz) | Minimum requirement (dBm) |
| --- | --- | --- | --- |
| R-GSM | 921  f  925 MHz | 100 | –60 |
| R-GSM | 925  f  935 MHz | 100 | –67 |
| GSM 900/R-GSM | 935  f  960 MHz | 100 | –79 |
| DCS 1800 | 1 805  f  1 880 MHz | 100 | –71 |
| UTRA TDD | 1 900  f  1 920 MHz | 100 | –62 |
| UTRA TDD | 2 010  f  2 025 MHz | 100 | –62 |
| NOTE 1 – The measurements are made on frequencies which are integer multiples of 200 kHz. Up to five exceptions of up to –36 dBm are permitted in the GSM 900, DCS 1800 and UTRA bands, and up to three exceptions of up to –36 dBm are permitted in the GSM 400 bands. |  |  |  |

# 6	Receiver spurious emissions (idle mode)

The power of any spurious emissions should not exceed the limits given in Table 29.

TABLE  29

General receiver spurious emission requirements

| Frequency band | Measurement bandwidth | Maximum level (dBm) | Note |
| --- | --- | --- | --- |
| 30 MHz  f  1 GHz | 100 kHz | –57 |  |
| 1 GHz  f  12.75 GHz | 1 MHz | –47 | With the exception of the frequencies covered by the table below, for which additional receiver spurious emission requirements apply(1) |
| (1) (Editorial Note) – In TFES Harmonized Standard v1.0.2, no additional receiver spurious emission is specified; yet, it is expected that there will be a table added, in the same form as for the other technologies (see Tables 5, 12 and 18). |  |  |  |

Annex 5

FDMA/TDMA (digital enhanced cordless telecommunications
(DECT)) mobile stations

# 1	Spectrum mask

If the equipment under test (EUT) is equipped with antenna diversity, the EUT should have the diversity operation defeated for the following tests.

# 2	Emissions due to modulation

The unwanted emission(s) due to modulation is the power measured in any DECT RF channel other than the one in which the EUT is transmitting, integrated over a bandwidth of 1 MHz.

With transmissions on physical channel Ra (K, L, M, N) in successive frames, the power in physical channel Ra (K, L, Y, N) should be less than the values given in Table 30.

TABLE  30

Emissions modulation

| Emissions on RF channel “Y ” | Measurement bandwidth | Maximum power level |
| --- | --- | --- |
| Y  M  1 | (1) | 160 mW (–8 dBm) |
| Y  M  2 | (1) | 1 mW (–30 dBm) |
| Y  M  3 | (1) | 80 nW (–41 dBm) |
| Y  any other DECT channel | (1) | 40 nW (–44 dBm)(2) |
| (1) The power in RF channel Y is defined by integration over a bandwidth of 1 MHz centred on the nominal centre frequency, FY, averaged over at least 60% but less than 80% of the physical packet, and starting before 25% of the physical packet has been transmitted but after the synchronization word. (2) For Y  “any other DECT channel”, the maximum power level should be less than 40 nW (–44 dBm) except for one instance of a 500 nW (–33 dBm) signal. |  |  |

# 3	Emissions due to transmitter transients

The power level of all modulation products (including AM components due to the switching on or off of the modulated RF carrier) in a DECT RF channel as a result of a transmission on another DECT RF channel.

The power level of all modulation products (including AM products due to the switching on or off of a modulated RF carrier) arising from a transmission on RF channel M should, when measured using a peak hold technique, be less than the values given in Table 31.

TABLE  31

Emissions due to transmitter transients

| Emissions on RF channel “Y ” | Measurement bandwidth | Maximum power level |
| --- | --- | --- |
| Y  M  1 | (1) | 250 mW (–6 dBm) |
| Y  M  2 | (1) | 40 mW (–14 dBm) |
| Y M  3 | (1) | 4 mW (–24 dBm) |
| Y  any other DECT channel | (1) | 1 mW (–30 dBm) |
| (1) The measurement bandwidth should be 100 kHz and the power should be integrated over a 1 MHz bandwidth centred on the DECT frequency, FY. |  |  |

# 4	Transmitter spurious emissions (conducted)

## 4.1	Spurious emissions when allocated a transmit channel

The spurious emissions, when a radio end point has an allocated physical channel, should meet the requirements of Table 32. The requirements of Table 32 are only applicable for frequencies which are greater than 12.5 MHz away from the centre frequency, fc, of a carrier.

TABLE  32

Spurious emissions requirements

| Frequency | Minimum requirement/ Reference bandwidth |
| --- | --- |
| 30 MHz  f  1 000 MHz | –36 dBm/100 kHz |
| 1 GHz  f  12.75 GHz | –30 dBm/1 MHz |
| fc – 12.5 MHz  f  fc  12.5 MHz | Not defined |

Measurements should not be made for transmissions on the RF channel closest to the nearest band edge for frequency offsets of up to 2 MHz.

# 5	Receiver spurious emissions (idle mode)

## 5.1	Spurious emissions when the EUT has no allocated transmit channel

The power level of any spurious emissions when the radio end point has no allocated transmit channel should not exceed the limits specified in Table 33.

TABLE  33

Receiver spurious emissions

| Frequency band | Measurement bandwidth | Maximum level (dBm) | Note |
| --- | --- | --- | --- |
| 30 MHz  f  1 GHz | 100 kHz (1) | –57 |  |
| 1 GHz  f  12.75 GHz | 1 MHz (1) | –47 | With the exception of the frequencies within the DECT band, covered by Table 34 |
| (1) The power should be measured using a peak hold technique. |  |  |  |

## 5.2	In the DECT band

The power level of any receiver spurious emissions within the DECT band should not exceed the limit in Table 34.

TABLE  34

Receiver spurious emissions within DECT band

| Frequency band (MHz) | Measurement bandwidth (MHz) | Maximum level (dBm) |
| --- | --- | --- |
| 1 900-1 920 2 010-2 025 | 1 | –57(1) |
| (1) The following exceptions are allowed: – in one 1 MHz band, the maximum allowable e.r.p. should be less than 20 nW; – in up to two bands of 30 kHz, the maximum e.r.p. should be less than 250 nW. |  |  |

Annex 6

IMT 2000 OFDMA TDD WMAN mobile stations

This Annex defines the unwanted emission limits for IMT-2000 OFDMA TDD WMAN mobile stations.

# 1	Spectrum emission mask

## 1.1	Spectrum emission mask for equipment operating in the band 2 300-2 400 MHz

TBD

## 1.2	Spectrum emission mask for equipment operating in the band 2 500-2 690 MHz

The spectrum emission mask of user equipment applies to frequencies between 2.5 MHz and 12.5 MHz away from the user equipment centre frequency for the 5 MHz carrier and between 5 MHz and 25 MHz away from the user equipment centre frequency for the 10 MHz carrier.

TABLE 35

Spectrum emission mask for 10 MHz carrier

| Segment number | Offset from channel centre frequency (MHz) | Integration bandwidth (kHz) | Allowed emission level (dBm/integration bandwidth) |
| --- | --- | --- | --- |
| 1 | 5 to < 6 | 100 | −13.00 |
| 2 | 6 to < 10 | 1 000 | −13.00 |
| 3 | 10 to < 11 | 1 000 | −13 − 12(f − 10) |
| 4 | 11 to < 15 | 1 000 | −25.00 |
| 5 | 15 to < 20 | 1 000 | If PTx,max ≤ +23 dBm and 2 550 ≤ fc≤ 2 620 MHz then −21 − 32/19 × (f −10.5) else −25 |
| 6 | 20 to < 25 | 1 000 | If PTx,max ≤ +23 dBm and 2 550 ≤ fc≤ 2 620 MHz then −37.00 else −25 |
| NOTE 1 – Maximum transmitter output power of user equipment is 23 dBm or smaller in Japan, and the frequency band of operation is limited to 2 545-2 625 MHz. |  |  |  |

In Table 35:

–	Channel bandwidth is 10 MHz.

–	Integration bandwidth refers to the frequency range over which the emission power is integrated.

–	f is defined as the frequency offset in MHz from the channel centre frequency.

- PTx,max is the maximum declared output power for the user equipment.

TABLE 36

Spectrum emission mask for 5 MHz carrier

| Segment number | Offset from channel centre frequency (MHz) | Integration bandwidth (kHz) | Allowed emission level (dBm/integration bandwidth) |
| --- | --- | --- | --- |
| 1 | 2.5 to < 3.5 | 50 | −13.00 |
| 2 | 3.5 to < 7.5 | 1 000 | −13.00 |
| 3 | 7.5 to < 8 | 1 000 | If PTx,max ≤ +23 dBm and 2 547.5 ≤ fc≤ 2 622.5 MHz then −20 − 2.28 × (f − 7.5) else −13.00 |
| 4 | 8 to < 10.4 | 1 000 | −25.00 |
| 5 | 10.4 to < 12.5 | 1 000 | If PTx,max ≤ +23 dBm and 2 547.5 ≤ fc≤ 2 622.5 MHz then −21 − 1.68 × (f − 8) else −25 |
| NOTE 1 – Maximum transmitter output power of user equipment is 23 dBm or smaller in Japan, and the frequency band of operation is limited to 2 545-2 625 MHz. |  |  |  |

In Table 36:

–	Channel bandwidth is 5 MHz.

–	Integration bandwidth refers to the frequency range over which the emission power is integrated.

## 1.3	Spectrum emission mask for equipment operating in the band 3 400-3 600 MHz

TBD

# 2	Transmitter spurious emissions (conducted)

IMT-2000 OFDMA TDD WMAN user equipment complies with the limits recommended in Recommendation ITU-R SM.329-10. The limits shown in Tables 37a and 37b and 38 are only applicable for frequency offsets which are greater than 12.5 MHz away from the user equipment centre frequency for the 5 MHz carrier and greater than 25 MHz for the 10 MHz carrier. f is the frequency of the spurious domain emissions. fc is the user equipment center frequency.

The emission levels in Table 37a should be met in areas where Category A levels for spurious emissions, as defined in Recommendation ITU-R SM.329-10, are applicable. The emission levels in Table 37b should be met in areas where Category B levels for spurious emissions, as defined in Recommendation ITU-R SM.329-10, are applicable.

TABLE 37a

User equipment spurious emissions limits – Category A

| Band | Measurement bandwidth | Allowed emission level |
| --- | --- | --- |
| 9 kHz  f  150 kHz | 1 kHz | −13 dBm |
| 150 kHz  f  30 MHz | 10 kHz | −13 dBm |
| 30 MHz  f  1 000 MHz | 100 kHz | –13 dBm |
| 1 GHz  f  13.45 GHz | 1 MHz | –13 dBm |

TABLE 37b

User equipment spurious emissions limits – Category B

| Band | Measurement bandwidth | Allowed emission level |
| --- | --- | --- |
| 9 kHz  f  150 kHz | 1 kHz | −13 |
| 150 kHz  f  30 MHz | 10 kHz | −13 |
| 30 MHz  f  1 000 MHz | 100 kHz | –36 dBm |
| 1 GHz  f  13.45 GHz | 30 kHz If 2.5 × BW ≤ ¦ fc − f ¦ < 10 × BW 300 kHz If 10 × BW ≤ ¦ fc − f ¦ < 12 × BW 1 MHz If 12 × BW ≤ ¦ fc − f ¦ | –30 dBm |

TABLE 38

Additional user equipment spurious emission limit

for equipment operating in the range 2 500-2 690 MHz

| Frequency bandwidth | Measure-ment bandwidth | Minimum requirement (dBm) | Note |
| --- | --- | --- | --- |
| 1 000 MHz  f  2 505 MHz | 1 MHz | −13 |  |
| 2 505 MHz  f  2 530 MHz | 1 MHz | −37 |  |
| 2 530 MHz  f  2 535 MHz | 1 MHz | 1.7f − 4 338 |  |
| 2 535 MHz  f  2 630 MHz | 1 MHz | −21 − 1.68*(f − 8) 12.5 MHz < f < 17.5 MHz −37 17.5 MHz < f < 22.5 MHz −18 22.5 MHz < f | For 5 MHz channel size |
|  |  | −18 25 MHz < f | For 10 MHz channel size |
| 2 630 MHz  f  2 630.5 MHz | 1 MHz | −13 − 8/3.5 × (f − 2 627) |  |
| 2 630.5 MHz  f  2 640 MHz | 1 MHz | −21 − 16/9.5× (f − 2 630.5) |  |
| 2 640 MHz  f  2 655 MHz | 1 MHz | −37 |  |
| 2 655 MHz  f | 1 MHz | −13 |  |
| NOTE 1 – The allowed emission level shall be applied for the frequency range greater than 2.5 times the channel size from the centre frequency. f is the offset from channel centre frequency. NOTE 2 – This additional requirement provides for the protection of satellite systems in the bands 2 500-2 535 MHz and 2 630-2 690 MHz in Japan, and applies only to terminals operating in the frequency band 2 545-2 625 MHz with powers of 23 dBm or smaller. NOTE 3 – The update of the values in this Table is for further study. |  |  |  |

# 3	Receiver spurious emissions (conducted)

The power of any narrow-band spurious emission should not exceed the maximum level specified in Table 39.

TABLE 39

General receiver spurious emission requirements

| Band | Measurement bandwidth | Allowed emission level (dBm) |
| --- | --- | --- |
| 30 MHz  f  1 GHz | 100 kHz | −57 |
| 1 GHz  f  13.45 GHz | 30 kHz If 2.5 × BW ≤ ¦ fc − f ¦ < 10 × BW 300 kHz If 10 × BW ≤ ¦ fc − f ¦ < 12 × BW 1 MHz If 12 × BW ≤ ¦ fc − f ¦ | −47 |

# 4	Adjacent channel leakage ratio (ACLR)

## 4.1 	ACLR of equipment operating in the frequency range 2 300-2 400 MHz

TBD

## 4.2	ACLR of equipment operating in the frequency range 2 500-2 690 MHz

Within this Annex, and in a similar manner to other annexes, the ACLR is defined as the ratio of the on-channel transmitted power to the power transmitted in adjacent channels as measured at the output of the receiver filter. In order to measure ACLR, it is necessary to consider a measurement filter for the transmitted signal as well as a receiver measurement bandwidth for the adjacent channel (victim) system.

In this Annex, data is provided that is relevant to the case where the adjacent system is OFDMA TDD WMAN (intra-system) or the case where the adjacent system is UTRA (inter-system).

ACLR is therefore specified considering the following receiver bandwidths:

When the adjacent system is OFDMA TDD WMAN:

–	4.75 MHz for a 5 MHz channelized system, and

–	9.5 MHz for a 10 MHz channelized system.

When the adjacent system is UTRA:

–	3.84 MHz for a 5 MHz channelized system, and

–	7.68 MHz for a 10 MHz channelized system.

The measurement bandwidth for the measurement of on-channel power of the OFDMA TDD WMAN carrier is:

–	4.75 MHz for a 5 MHz channelized system, and

–	9.5 MHz for a 10 MHz channelized system.

The passband of the receiver filter is centred on the first or second adjacent channel centre frequency. In the case where the adjacent system is OFDMA TDD WAN, both the transmitted power and the received power are measured with a rectangular filter. For adjacent UTRA systems the transmitted power is measured using a rectangular filter and the received power using a RRC filter with a roll-off factor of 0.22.

The ACLR values for the two relevant cases are provided in the Tables 40 and 41.

TABLE 40

MS ACLR for 5 MHz channel bandwidth

|  | Minimum required ACLR relative to assigned channel frequency (dB) |  |
| --- | --- | --- |
| Adjacent channel centre frequency | OFDMA TDD WMAN case | UTRA(1) case |
| MS channel centre frequency ± 5 MHz | 30 | 33 |
| MS channel centre frequency ± 10 MHz | 44 | 43 |
| (1) These are similar to the minimum requirements for UTRA systems (see Annexes 1 and 3 to this Recommendation) and in practice may be expected to be larger. |  |  |

TABLE 41

MS ACLR for 10 MHz channel bandwidth

|  | Minimum required ACLR relative to assigned channel frequency (dB) |  |
| --- | --- | --- |
| Adjacent channel centre frequency | OFDMA TDD WMAN case | UTRA(1) case |
| MS channel centre frequency ± 10 MHz | 30 | 33 |
| MS channel centre frequency ± 20 MHz | 44 | 43 |
| (1) These are similar to the minimum requirements for UTRA systems (see Annexes 1 and 3 to this Recommendation) and in practice may be expected to be larger. |  |  |

Additional information may be provided in future revisions of this Recommendation.

NOTE 1 – Further study is necessary for other systems wherever applicable, as well as the relationship between ACLR and the emission mask.

## 4.3	ACLR of equipment operating in the frequency range 3 400-3 600 MHz

TBD

Appendix 1 – Definitions and Abbreviations

Test tolerance

With reference to Recommendation ITU-R M.1545, “test tolerance” is the relaxation value referred to in recommends 2 of ITU-R M.1545, i.e. the difference between the core specification value and the test limit, evaluated applying the shared risk principle as per figures 2 and 3 of annex 1 of 
ITU-R M.1545. In case the core specification value is equal to the test limit (Figure 3 of Annex 1 of ITU-R M.1545) the “test tolerances” are equal to 0.

______________

Remark: With reference to Recommendation ITU-R M.1545, “test tolerance” is the relaxation value referred to in recommends 2 of Recommendation ITU-R M.1545, i.e. the difference between the core specification value and the test limit, evaluated applying the shared risk principle as per Figures 2 and 3 of Annex 1 of Recommendation ITU-R M.1545. In case the core specification value is equal to the test limit (Figure 3 of Annex 1 of Recommendation ITU-R M.1545) the “test tolerances” are equal to 0.

*	This Recommendation should be brought to the attention of Radiocommunication Study Group 1.

*	This Recommendation should be brought to the attention of Radiocommunication Study Group 1.